A Causal Graph Attention Learning for Interpretable Cascading Failure Predictions in Power Systems with Renewable Generation via HVDC

Shiqu Xiao, Yang Fu*, Xiangjing Su, Shaohua Zhang, Jiajia Yang, Cuo Zhang, Zhaoyang Dong

Abstract—The emergence of high-voltage direct current integrated (HVDC) power systems introduces significant out-of-distribution (OOD) challenges to machine learning-based analysis of cascading failures (CFs). Existing algorithms often suffer from poor interpretability and generalization in handling diverse CF propagation modes. To address these issues, this paper proposes a novel Causal Graph Attention (CGAT) model that captures critical information from CF events to accurately predict propagation paths and impacts. Specifically, by employing a graph attention mechanism, the model extracts both causal and trivial features as distinctive attention dual subgraphs, which aids in differentiating shortcuts in causal variables and improving prediction accuracy. These subgraphs also provide visual representations that highlight the key elements influencing CF propagation. By incorporating interventions, the CGAT model enhance prediction capabilities across a variety of scenarios that include HVDC control strategies and renewable energy sources (RES). Extensive experiments on augmented IEEE 30-bus and 2737-sop systems demonstrate the effectiveness and superiority of the proposed CGAT model.

Index Terms—cascading failure predictions, causal learning, graph neural network

I. INTRODUCTION

T

HE integration of renewable energy sources (RESs) and the extensive deployment of high-voltage direct current (HVDC) technology have increased the vulnerability of power grid operations. Localized fluctuations introduced by extreme weather events and potential malicious attacks can escalate into cascading failures (CFs), leading to widespread power outages [1], [2]. Therefore, it is crucial for grid operators to identify CF propagation paths and evaluate their impacts to devise effective preventive strategies and mitigate losses [3].

Recent incidents have highlighted unique characteristics of CFs across different regions [4]. For example, the UK experienced major blackouts in July and August 2019 due to HVDC system shutdowns and severe weather, resulting in power losses of 1,000 and 1,878 MW, respectively. In 2021, Texas faced severe cold conditions that significantly reduced RES generation, leading to a series of CFs with over 20,000 MW load shedding, affecting approximately 4.8 million customers. The integration of RESs in remote locations like deserts and deep seas through HVDC connections presents new operational challenges for the power grid, reducing its ability to withstand extreme weather events. Existing simulation models including OPA [5], [6], CFS [7], [8], and Manchester [9], have been employed to study individual CF events, but these methods often lack the computational speed required for real-time response in modern power systems.

Graph neural networks (GNNs) particularly effective in handling CF problems in power grid topologies have been explored in various studies [10], [11], [12], [13], [14]. Reference [10] presents a GCN-LSTM model specifically designed for predicting CF propagation paths, where the GCN module is used to capture both the topological features and electrical states. In [11], a node-edge conversion concept is used to enhance the GCN inputs, thereby incorporating branch indices, such as the line outage distribution factor (LODF), into the model. A multilayer graph-transformer is proposed in [12], and it processes both branch and node features directly, obviating any intermediary node-edge conversion. These GNN methods attempt to establish a mapping relation between the input data and target labels to capture the dynamics of information during CF propagation [13]. However, minor shifts in the topology or the domain distribution of operating states can significantly reduce the effectiveness of these models. Moreover, due to the black-box nature, GNNs cannot determine the specific features that play critical roles in CF propagation, leading to a lack of interpretability and transparency.

The interpretability of ML models is critical as it provides credibility and reliability to decisions in power system applications. Layer-wise Relevance Propagation (LRP) has been used to identify features that influence labels positively or negatively, thus measuring the contributions to predictions [10]. However, LRP requires approximation methods to assign correlations in nonlinear networks that can compromise prediction accuracy. The Attention (AT) mechanism has been proven effective, where a graph attention network (GAT) model predicts CF propagation paths by incorporating power system constraints into the loss function during training to enhance interpretability [13]. Our previous study also explored certain GAT applications [15], [16]. However, GAT tends to exploit shortcuts as it assigns weights to all features equally. These shortcuts typically emerge from biases, noise features, or other trivial patterns within the graph data, which are non-causal but distinctive in the training data [17], [18], [19]. Consequently, the GAT assigns unreliable weights to shortcuts, while the weights to components such as converters or DC transmission lines, may not align with their actual significance during CF events. Therefore, new methods need to be explored to clarify the GNN applications in the CF analysis for HVDC-RR power systems.

Several state-of-art studies utilized causal inference to address the widespread out-of-distribution (OOD) problems in the data-driven applications of power system problems and enhance model generalizability [20], [21], [22]. Reference [23] first applied a causal inference-based neural network model to predict CF events, using traditional causal graphs to represent causal relationships and weights between fault lines, and employing non-Gaussian noise as interventions to enhance model robustness. However, the model still fails to effectively address the degradation effects on model performance caused by shortcuts, which limits prediction accuracy.

To address these challenges, this paper introduces a novel Causal Graph Attention Network (CGAT) model for predicting CFs in power grids with extensive RES integration. This dual-task model predicts the propagation paths and quantifies the losses of CF events. The contributions of this paper are summarized as follows:

- The model captures causal and trivial features as attention subgraphs to disentangle causal variables from shortcut features, and their visualization offers interpretable insights into the structural vulnerabilities of power systems.

- The framework of the CGAT is effectively designed based on Structural Casual Model (SCM), where loss function is designed to incorporate disentanglement and intervention, aiming to maintain a stable prediction and encourage the invariant relationships between the causal patterns and the predicting targets.

- Via comprehensive case study, it is a new finding that accuracy and stability can be achieved by emphasizing invariant relationships between causal features and prediction targets, thus weakening the influence of shortcuts. CGAT exhibits strong generalization as an essential tool for enhancing the resilience of power systems with a growing level of RES integration and complex conditions.

The remainder of the article is as follows. Section II introduce the dual-task prediction model. In Section III, the proposed CGAT model is formulated. In Section IV, the implement procedures are demonstrated. In Section V, the proposed model is validated on test systems in different scales. Finally, conclusions are presented in Section VI.

II. Problem Formulation

This section begins by detailing CF scenarios in the HVDC-RR power systems with HVDC connected large-scale RES stations. It then introduces the fundamental concepts and tasks of CF prediction.

- A. Modeling Failure Scenarios in Power Systems with HVDC

In recent years, HVDC technology has been widely applied in renewable energy integration due to its advantages of long-distance transmission, high-capacity, and efficiency [24], [25]. This paper investigates CF events within a system consisting of main grid, an HVDC system, and several renewable energy stations as shown in Fig. 1.

Fig. 1. HVDC Power system with large-scale renewable energy integration

Taking the Rudong HVDC project as an example, which is built on the modular multilayer converter-based HVDC (MMC-HVDC) technology as shown in Fig. 2. The feature of MMC converter v is formulated as dv=Psv,Qsv,Usv,δsv,Pcv,Qcv,Ucv,δcv,Pdcv,Udcv,  where Usv∠δsv is the fundamental vector of the AC bus voltage connected to the MMC, Ucv∠δcv is the fundamental vector of the MMC AC side voltage, Psv and Qsv are the power output of the AC system, Pcv and Qcv are the power absorbed by the MMC, Pdcv  and Udcv are the DC power output and voltage of the MMC DC side, Rv and Xv are the equivalent impedance.

Fig. 2. Steady-state model of the MMC station

The probabilities of three typical failures in the HVDC integrated grid are modeled. From the prospective of AC transmission system, the fragility function is widely used to roughly describe the line failure probability as

probs=Φ1λlnsS#(1)

where S is the median value of engineering demand parameter when the stress s reaches the threshold of the damage state; λ is the standard deviation of the natural logarithm of engineering demand parameter; and Φ· denotes the standard normal cumulative distribution function. Notably, the stress s=vijw  is represented by wind speed on transmission line ij in this paper. The detailed parameters of fragility functions can be obtained from domain knowledge [26].

From the perspective of MMC-HVDC system, the probability of submodule-failure-induced valve blocking is modeled as shown in (2). The failure probability of each submodule is assumed as an intendent event and follows Poisson process. Then, the protection maloperation in transformers and generators are formulated as shown in (3), to consider the interactions between AC and DC systems.

probk>kThre=1-k=0kThre-1e-λ λkk!#2

probΔIvdc=ΦΔIvdcI0-kvuIσvuI×e-dvu#3

where k is the number of failure submodule; kThre is the tolerable number of failure submodule within an MMC converter; λ is the parameter of Poisson distribution; ∆Ivdc is the transient amplitude of DC current of converter v; I0 is the rated value of DC current;  kvuI and σvuI are the coefficients between the affected device u and converter v; dvu is the electrical distances between affected device u and converter v.

- B. The Formulation of Cascading Failure Chain

In this paper, initial failure events are defined as N-k failures. Cascading failure chains (CFCs) [27] are employed to represent the temporal and spatial relationships of CF events. Assuming an initial failure event Ii triggers the whole CF event Fi, which can be formulated as

Fi=Ii,Ri,EENSi#4

where Fi=Fijj=1,2,…,Ni is the set of failure events, Ni denotes the number of propagation phases, Fij represents the jth phase of failure event i, the initial failure event Ii=Fi1. Ri=RijRij=Fij→Fij+1 is the set of tempo-spatial relationships between adjacent phases, which is a binary vector denoting the status transition power line at the j→j+1 process. In this paper, expected energy not supplied (EENSi) is used as the loss index, and is used to represent the severity of the damages.

The dual-task prediction model proposed in this paper is designed to achieve two objectives. Task 1 predicts the CFC (i.e., CF propagation path). Task 2 forecasts the EENS of CF event i. The first task answers the question of “Which stage does line survive to?”, while the second one addresses “How much loss is caused by the CF i?”

III. Scheme Overview For The Proposed CGAT-Based CF Prediction Model

This section presents the implementation scheme of the proposed interpretable and generalizable learning method for CF prediction. Then, the inputs and outputs of the proposed CGAT model are introduced, followed by a detailed explanation of steps in the proposed model.

- A. CGAT Prediction Scheme for Cascading Failure Analysis

Traditional ML methods assume that there is only one domain D=I,PYX, i.e., the independent and identically distributed (i.i.d.) assumption. This domain includes the data space I and the marginal distribution PYX for all input samples X and target labels Y. However, real-time CF predictions face challenges posed by multiple domains, each with distinct characteristics, such as varying topologies, uncertain RES and load power, as well as multiple HVDC control status. Methods based on empirical risk minimization (ERM) struggle to capture the propagation process and their impacts on real power grids. This paper adopts a causal perspective to learn from multiple source domains and make inferences in unseen target domains.

The proposed CGAT is constructed based on the Structural Causal Model (SCM) [18], [19], as shown in Fig. 3. It presents the causalities among five variables: input data X, causal factor C, trivial factor T, graph representation R, and prediction Y. The SCM categorizes factors that influence the prediction into causal and non-causal, which avoids the misleading of shortcut features and biases. In the relationship T←X→C, C denotes the causal feature that truly reflects the intrinsic property of input X, while T represents the trivial feature that introduces biases or shortcuts. With regard to the T→R←C relationship, the variable R is the representation of input X to extract discriminative information from the causal feature C and the trivial feature T. The R→Y relationship represents the ultimate goal of making prediction Y based on the representation R.

Fig. 3. Structural causal model

The proposed CGAT model is comprised of three components: the graph attention module, the causal and trivial representation module, and the regression module. Each module of the CGAT model is designed corresponding to the structure of SCM, as presented in Fig. 4. First, the input data includes electrical data from AC and DC nodes, topology information of power grid, and initial failure event. Then, the input data is processed through the graph attention models to compute attention scores for nodes and edges. The causal and trivial features are respectively captured by causal and trivial modules as attention subgraphs. Finally, the disentanglement and intervention are designed as loss functions to enhance the model’s capabilities in CF predicting tasks.

- B. Input and Output of the Model

The initial state of CF event i in power grid, including the

Fig. 4 The proposed CGAT network structure diagram

adjacency matrix A∈RNl×RNl, initial failure event Ii∈RNl×1, and initial operation data set zi=Pibus,Qibus,Uibus,θibus,di∈R(4Nv+Nd)×1, is collected by the SCADA system and considered as inputs. Here, Nl, Nv, and Nd are the numbers of AC power lines, AC nodes, and DC nodes, respectively. Pibus, Qibus, Uibus, and θibus are the AC nodes’ vectors of active power, reactive power, voltage, and angle, respectively. di=dvv=1,2,…,Nv denotes the vector of converter nodes. The data labels for the two predicting tasks are the CFC vector yichain=ji,lrijl>0,l=1,2,…,Nl and the EENS value yiEENS=Si of CF event i, respectively. With the above definitions, the output of the CF predictor can be expressed as

yichain=ftask1zi,A,IiyiEENS=ftask2zi,A,Ii#5

where ftask1∙ and ftask2∙ are the prediction function of task 1 and task 2, respectively.

- C. Causal Graph Attention Model

1) Node- and Edge-Level Attention Mechanism

The proposed CGAT employs attention modules, creating two branches for the causal and trivial subgraphs. Given a GCN encoder GConv∙ and a graph data G=A⨀Ii,zi, the node representation can be obtained as

H=ReLUGConvA⨀Ii,zi#6

where ⊙ denotes the Hadama product. Then, two multilayer perceptrons (MLPs) are applied to calculate the node-level and edge-level attention scores, respectively. The node attention defines a self-attention mask layer to select the most significant nodes for the node-level representation, as shown in (7). The edge attention is obtained by aggregating the weighted node representations, as shown in (8).

αvC,node,αvT,node=σMLPnodehv#7

αvuC,edge,αvuT,edge=σMLPedgehvhu#8

where v,u∈Nv indicates the index of nodes; αvC,node and αvT,node represent the node-level causal and trivial weights for node i, respectively; αvuC,edge and αvuT,edge denote the edge-level causal and trivial weights for edge vu, respectively; || denotes concatenation operation; σ ∙ denotes the sigmoid nonlinear activation function.

The node-level and edge-level attention weight vectors are then normalized using the softmax function, as shown in (9) and (10).

α'vnode=expαvm∈Mαv,m,αv∈αC,node,αT,node#9

α'vuedge=expαvum∈Mαvu,m,αvu∈αC,edge,αT,edge#10

where m∈M is the index of node features. Note that the decoupling of causal and trivial features can be demonstrated by α'vC,node+α'vT,node=1 and α'vuC,edge+α'vuT,edge=1. These attention weights describe how much the GAT models focus on each node or edge in the corresponding causal-trivial graphs.

2) Causal and trivial subgraphs

To investigate the causalities among CF propagations and system topologies, the normalized attention weight vectors α'C,node,α'T,node,α'C,edge, and α'T,edge are applied to represent causal attention subgraph GCand trivial attention subgraph GT, as shown in (11) and (12). Note that the two subgraphs extracted in this paper are different from the causal graph in traditional causal inference method. GC and GT are constructed as the subgraphs of the original topology of power grid and represent the causal and non-causal relationships between the input data and CF labels.

GC=A⨀α'C,edge,z⨀α'C,node#11

GT=A⨀α'T,edge,z⨀α'T,node#12

Notably, the causal- and trivial subgraphs not only guide the representation learning of the CGAT model but also clarify the underlying mechanisms that drive the model predictions. As a result, the CGAT exploits both subgraphs, highlighting their significant applications in explainability, privacy, and fairness.

3) Introducing Disentanglement and Intervention in Loss Function

After obtaining the causal and trivial subgraphs, every node and edge are distributed attention scores in causal and trivial subgraphs. Their representations are used to capture the causal and non-causal features. Two GCN layers, as shown in (13) and (14), are designed to obtain the causal representation HGC and non-causal representation HGT, respectively.

HGC=GConvCA⨀α'C,edge,z⨀α'C,node#13

HGT=GConvTA⨀α'T,edge,z⨀α'T,node#14

Three readout regression modules are designed for the objectives, as illustrated in Fig. 5. The causal and trivial readout modules are formulized for the disentanglement between causal and shortcut features, as shown in (15) and (16).

yCpred=fCreadoutHGC#15

yTpred=fTreadoutHGT#16

where yCpred and yTpred are the predicted values of the two readout modules, respectively. Hence, the loss functions for the two disentanglement regression modules can be defined as

fCloss=-1Di∈DyiCFClogyC,iCFC,pred+yiEENSlogyC,iEENS,pred#17

fTloss=-1Di∈DKLyiCFC,unif,yT,iCFC,pred+KLyiEENS,unif,yT,iEENS,pred#18

where D is the batch size, KL∙ denotes the KL-divergence function, yCFC,unif and yEENS,unif are two uniform distributions. fCloss indicates the cross-entropy of the two-task model’s prediction results based on causal features. fTloss ensures that the predictions by trivial features distribute evenly across all possible labels. By optimizing the two objectives, the causal and trivial features are effectively disentangled.

The random readout module is designed to introduce interventions, accounting for the various operation scenarios in CF propagations, as shown in (19). By combining the additional representation HGR with the target causal representation HGC, a counterfactual graph is generated by modifying the original trivial subgraph.

yRpred=fRreadoutHGC,HGR#19

where yRpred is the predicted value of the random readout module, HGR is defined as an additional representation obtained from a set of additional representation. Due to the CF propagation paths are irregular graph data, it is impossible to make the intervention on the data level. To this end, the intervention on representations can be guided by introducing backdoor adjustment as

fRloss=-1D⋅TRi∈Dr∈TRyiCFClogyR,rCFC,pred+yiEENSlogyR,rEENS,pred#20

where TR is the estimated stratification set of the trivial subgraph, which collects the trivial features from training data. The trivial subgraph aims to approach the shortcuts that unnecessary for prediction. fRloss makes the intervened predictions to be invariant and stable across different stratifications.

Fig. 5 Structure of the Readout Modules

Finally, the objective of CGAT can be defined as the sum of the losses:

floss=fCloss+λ1fTloss+λ2fRloss#21

where λ1 and λ2 are hyper-parameters that determine the strength of disentanglement and intervention, respectively.

IV. Experiment Procedure

In this section, the experimental procedures are outlined as shown in Fig. 6. Firstly, the CF prediction procedures after training the CGAT model are demonstrated. Furthermore, the CF simulation process is introduced.

- A. Cascading Failure Predicting Procedure

To predict CF path and impact, the detailed procedures of the CGAT model are as follows:

Step 1: Preprocess the system’s operational data and initial contingency information as inputs for the model. The operational data is formatted into a batch_size  node_num  feature_num matrix z, where

- batch_size is the batch size.

- node_num denotes the number of system nodes.

- feature_num represents the feature dimension of nodes.

The adjacent matrix A, representing the connection state of power system, is created as a node_num  node_num matrix.

Fig. 6. Implementation of the proposed model

The initial contingency information of CF event i is formed as a batch_size  node_num  2 matrix Ii. Finally, the preprocessed data z, A, and Ii are used as inputs to the CGAT model.

Step 2: Input the preprocessed data into the GCN module to obtain the general representation H. Then, the node and edge attention modules extract the causal and trivial attention weight vectors  [αVC,αLC] and [αVT,αLT], respectively. After normalizing the attention vectors, the adjacent matrix A, feature matrix z, and the normalized attention vectors are processed to generate causal and trivial subgraphs GC and GT, which represent the causality between  prediction labels and inputs.

Step 3: Two GCN modules GConvC(∙) and GConvT(∙) are applied to compute the  representations of the causal and trivial subgraphs HGC and HGT, respectively. The causal and trivial readout modules disentangle the causal and confounding features. Meanwhile, an additional representation HGR, selected from the additional representation set, is combined with the target causal representation HGC as input of the random readout module. Finally, the representations processed by the CGAT model are fed into the regression modules. The regressors predicts the CF propagation paths or losses uses the full connection layer, ReLU activation function and the loss function designed in (18). Among them, the causal and trivial regressors respectively make predictions according to the representations HGC and HGT. And predictions from the random regressor introduce external interventions HGR by paring causal features with confounders.

- B. The Simulation of CF Samples

The alterations of system topology and HVDC control strategy introduce the fundamental changes of system operational modes. Furthermore, load fluctuations and renewable energy output variations introduce additional uncertainties within each topology, increasing the complexity and diversity of the data. To measuring the predictive accuracy of the CF predictions from the leaned models, it is essential to valid the generation of ground truth data. This paper simulates CF events using the CFS oracle [5] and AC/DC power flow model [28]. The simulation accounts not only for failures within power networks, but also for RES station output fluctuations and the derating operation of the integration system. The CFS process is summarized as the following steps

CFS Step 1: Initialize system’s power flow state based on the loads and RES outputs.

CFS Step 2: Create an initial failure event Ii∈RNl×1, which denotes an N-k contingency containing transmission line outage, converter commutation failure, and the protection maloperation of transformers and generators.

CFS Step 3: Identify islands within the network, and recalculate power flow for each island.

CFS Step 4: Find and cut-off the overloaded lines. If there are no overloaded lines, the cascade ends and the simulation reinitializes from Step 1. If there exist overloaded lines, sequentially remove each overloaded line and recalculates the power flow from Step 3.

- C. Performance Metrics

To analyze the performance of the models in CF predictions, the five metrics proposed in [8] are evaluated. The average CFC prediction accuracy lchain and the average loss prediction accuracy  lEENS represent the models’ general predicting accuracy. The average false prediction rate lfal is defined as the ratio of instances where normal operating components are incorrectly predicted to be in a failure state within CFCs. The average missing prediction rate lmis indicates the proportion of actual component faults that are not predicted. The average time error rate ltime measures the error in predicting the stages of CFCs. The formulas for the indicators are provided below.

lCFC=1Nti∈Ntvu∈Nedgeεyi,vuCFC,pred=yi,vuCFCNedge#22

lEENS=1Nti∈NtεyiEENS,pred=yiEENSyiEENS#23

lerr=1Nti∈Ntvu∈Nedgeεyi,vuCFC,pred<yi,vuCFC#24

lmis=1Nti∈Ntvu∈Nedgeεyi,vuCFC,pred>yi,vuCFC#25

ltime=1NtNedgei∈Ntvu∈Nedgeyi,vuCFC,pred-yi,vuCFC#26

where ε[∙] is the indicator function that takes a value of 1 if the condition within is satisfied, and 0 otherwise; Nt is the test dataset; Nedge is the set of power lines.

V. Case Study

- A. Datasets Generation

In this paper, a modified IEEE 30-bus system is firstly utilized as a benchmark system for estimating CF events. Two offshore wind farms are incorporated to simulate the scenario of large-scale integration of RES energy. The system configuration is illustrated in Fig. 7. The peak load of the main grid is approximately 5000 MW, with thermal power generation capacity around 4800 MW. One of the offshore windfarms is integrated via a HVDC system at node 5 with 1100 MW installed capacity, and another one replaces the former generation unit at node 13 with 200 MW.

Fig. 7. Modified IEEE 30-bus system integrated with an off-shore power HVDC system

Table 1: Performance Comparison in Test set

| Model | Precision Rate (%) |  | Error Rate (%) |  |  |  | Time Consumption |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | lCFC | lEENS | False Alarm lerr | Miss Alarm lmis | Time Error ltime |  | Total Training Time (s) | Single Sample Prediction Time (ms) |
| CGAT | 87.57 | 92.89 | 7.05 | 6.44 |  | 7.21 | 569 | 1.72 |
| GAT | 82.91 | 90.47 | 9.75 | 8.34 |  | 9.98 | 481 | 1.61 |
| GCN | 80.51 | 90.96 | 10.13 | 9.36 |  | 13.62 | 441 | 1.54 |
| CNN | 77.89 | 89.92 | 11.04 | 11.07 |  | 12.85 | 402 | 1.49 |
| MLP | 78.19 | 88.74 | 10.79 | 11.02 |  | 14.07 | 355 | 1.62 |

Original dataset is firstly generated based on the CF simulation process described in Section IV.B. The simulations are set at scenarios where the power system exhibits a single initial topological structure with 3 different HVDC control strategies, as shown in Fig. 7. Besides, the loads and the offshore wind power outputs are set to fluctuate by non-Gaussian noise. The initial failure events for CFs are introduced as random N-1, N-2, and N-3 contingencies. The original dataset of 20,000 CF samples is generated, and is divided into 16,000 training samples and 4,000 testing samples at a ratio of 4:1. To validate the effectiveness and reliability of the proposed CGAT model, the performances are compared with traditional deep learning methods MLP and CNN, as well as GNNs like GCN and GAT. The batch size for all models was set to 64, with 250 training epochs. The Adam optimizer was used with an initial learning rate of 0.01. In both CGAT and GAT, the number of heads was set to 4. The CNN has 3 convolutional layers with kernel size of 1×1, and the number of maximum pooling layer is set to 1×1. For GCN, the Chebyshev polynomial filter order was set to 3, with 2 sub-modules. The model loss during the training process for each algorithm is shown in Fig. 8, and the prediction results are presented in Table 1.

Fig. 8. Validation loss curves for CGAT and other learning-based models

- B. Results for Modified IEEE 30-bus System

1) Analysis of Prediction Performance

As shown in Fig. 8, the convergence stability of the loss curves of MLP (Fig. 8(e)) and CNN (Fig. 8(d)) is inferior to that of the other GNN-based models, thereby indicating the inadequacy of linear models in addressing the graph-based predictions. In the GNNs, such as GCN (Fig. 8(c)), GAT (Fig. 8(b)), and CGAT (Fig. 8(a)), the electrical features of nodes and the topology information are considered during the learning process. Therefore, it is observed that the training curves are more stable than those using deep learning algorithms, such as CNN and MLP, which only focus on electrical features.

Table 1 shows that CGAT has the highest accuracy of all models for CFC and ENNS predictions, reaching 87.57% and 92.89% respectively. The main challenge of the CFC predicting task is to determine the component’s state in each cascading stage. Traditional statistical methods such as CNN and GCN only achieved prediction accuracies of 77.89% and 80.51% respectively, while GAT was slightly higher at 82.91%. Other metrics related to the CFC predicting task are also demonstrated in Table 1. In terms of model error predicting indices, the proposed CGAT has an average false prediction rate lfal of 7.05%, an average missing prediction rate lmis of 6.44%, and an average time error rate ltime of 7.21%. It is better than 9.75%, 8.34% and 9.98% for the GAT method, and better than 11.04%, 11.07% and 12.85% for the traditional CNN algorithm. In terms of computational time, the total training time and single sample prediction time of the proposed CGAT model are 569 seconds and 1.72 milliseconds, respectively, which are slightly longer but acceptable compared to other models.

2) Analysis of Prediction Performance

The causal-trivial dual-subgraphs for the two tasks are depicted as shown in Fig. 9. The two subgraphs are constructed using the nodes and edges’ attention weights. In Fig. 9, the assignment of darker colors to nodes and edges indicates that they are assigned higher attention scores and play a causal role in CF propagations. It can be observed that the majority of the darker colored nodes and edges are distributed in the critical and deterministic region, both at the node level e.g., nodes 6, 10 and 12, and at the edge level e.g., lines L5-38, L9-11 and L31-39. For instance, L31-39 and L5-38 serve as the primary collection points for wind farms and the HVDC system, respectively. There failures absolutely cause the outage of the HVDC system. Additionally, nodes 6, 10, and 12 represent hub nodes within the system, exhibiting a degree of 7, 6, and 5, respectively. Furthermore, node 2, L6-7, and L7-10 have been assigned a relatively high attention score in task 2. Node 2, with a capacity of 800 MW, represents the highest conventional generator unit. The failure of L6-7 and L7-10 has the potential to affect the active power transmitted on transmission lines such as L27-28 and L6-9, resulting in significant power flow shifts. The causal subgraphs of the two tasks demonstrate that the proposed CGAT model is capable of identifying the essential nodes and lines for a novel power system.

Fig. 9. Visualizations of the extracted causal-trivial dual-subgraphs via CGAT

The relative importance rankings of nodes and edges identified by each method are illustrated in Fig. 10(a) and (b), respectively. Figure 10(a) depicts the top three critical power lines identified by power flow betweenness, directed flow betweenness, and CGAT as L5-38, L9-11, and L31-39, whereas GAT identifies L9-10, L6-9, and L4-12 as the most crucial. In Fig. 10(b), the top two nodes identified by CGAT, NFB, and GAT are 6 and 10, while the identification of the third important node varies. It is observed that the proposed CGAT model is in accordance with the rankings of the physical mechanism-based indices in identifying critical power lines and nodes. The ranking produced by GAT is unable to focus effectively on HVDC-related nodes and lines, such as L5-38, L31-39, L38-39, and nodes 38, 39. In parallel checks with existing state-of-the-art vulnerability indices, the proposed CGAT method demonstrates superior performance in capturing critical power lines and nodes in CF propagations using causal-trivial graphs.

(a) Comparison of link prioritization in different methods

(b) Comparison of node prioritization in different methods

Fig. 10. Vulnerable power components identification results

3) Model Ablation Analysis

This subsection investigates the impact of the load and the RES output variations and the hyperparameters {λ1,λ2}.

a) Performance on Different Load and RES Levels: The variations in loads and RES outputs alter the ground truth distributions while the model performance needs to be evaluated. The CGAT model is tested on the modified IEEE-30 bus system via 16 cases under the increased loading conditions of 100%,115%, 130%,and 145%×base_load, along with the increased RES conditions of 50%,65%, 80%,and 95%×rated_power, resulting in 5000 ground truth CF events for each case. The results of these tests are illustrated in Fig. 11.

Fig. 11. Precisions for different load and RES levels

With increases of the loading and RES levels, we can observe a minor increase in predicting precisions of both tasks. Despite these changes, the CGAT model demonstrates consistent performance across all load and RES levels. In practice scenarios, the output power on RES nodes varies with huge volatility and uncertainty. And the redistribution of power flow tends to follow an uncertain pattern. Consequently, the trained causal-trivial dual subgraphs via CGAT model effectively captures the pattern and predict the most likely CF events across different loading and RES scenarios.

b) Performance on Loss Coefficients λ1 and λ2: According to (21), λ1 represents the strength of disentanglement between the causal and trivial features, where its increasing denotes more attention on shortcuts. While λ2 controls the strength of the causal intervention. To analyze their impacts, one coefficient is fixed as 0.5 and change the other on in [0, 1] with a step size of 0.1. The results for the both tasks are shown in Fig. 12.

The impacts on task 1 are depicted in Fig. 12(a), the results show that λ1 achieves better performance in a range of 0.3 ~ 0.6. Smaller values will cause performance degradation. While λ2 achieves the best point at 0.2. The lager value will cause a continuous performance degradation to task 1. In Fig. 12(b), λ1 achieves better performance in a range of 0.4 ~ 0.7 for task 2. Too small or too large values will cause performance degradation. The situation of λ2 is similar to the task 1. Its optimal value at 0.3 and the subsequent performance decrease with its increasing.

Fig. 12. Parameter sensitivity of hyperparameters λ1 and λ2

- C. Results for Modified2737-sop System

To further investigate the scalability of the proposed CGAT model in larger power systems, this case study is conducted on a modified 2737-sop grid [29], which is composed of 2744 buses, 403 generator, 3512 AC lines, 1 DC lines, and 2 converters. The training data and corresponding meteorological data are simulated to construct about 20000 CF samples. The sample set is divided into training set and test set with a ratio of 8:2.

For the larger system, the training becomes more expensive as the sizes of causal and trivial attention subgraphs increasing. However, once the model is trained, the appropriate hyperparameters {λ1,λ2} can be obtained, which leads to a trade-off between accuracy and complexity. Fig. 13 depicts the extracted causal-trivial attention subgraphs in the larger grid. It can be observed that the significant nodes and edges are captured. For instance, the two converter nodes of wind farm integration with the largest RES capacity are attributed with high attention scores, meaning the outage of MMC stations might cause the further deterioration of CF propagation. Furthermore, the important hub components, such as node 1115 and 1233, as well as edges L25-28 and L178-83, are identified by the attention subgraphs.

We expand the analysis of hyperparameters’ impacts on the larger grid in Fig.14, where one coefficient is fixed as 0.5 and change the other on in [0, 1] with a step size of 0.1. Although there is an approximate 5% decrease in precision for the 2737-sop system test case, the model still demonstrates the consistent performance of the CGAT model for the larger grid. In task 1, λ1 achieves better performance in a range of 0.5 ~ 0.8, and  λ2 achieves the best point at 0.5. While in task 2, the reasonable values of λ1 and λ2 are achieved in ranges of 0.4 ~ 0.6. The results demonstrate that the model require more attention on the shortcut features and interventions as the increasing of the system scale.

Fig. 13. Visualizations of the causal-part subgraphs for the modified 2737-sop test system

Fig. 14. Parameter sensitivity of hyperparameters λ1 and λ2 for the 2737-sop test system

V. Conclusion

This paper proposes a novel interpretable and generalizable dual-task CGAT model for predicting CF events. The model identifies the causal and shortcut features which are extracted as causal and trivial attention subgraphs, and introduces interventions into loss function. The model overcomes the inefficiencies in capturing the critical components during CF propagation, and disentangling the interactions between causal and shortcut features. Empirical assessments conducted on the modified IEEE 30-bus and 2737-sop test systems validate the efficacy of the CGAT framework in predicting CF chains and impacts within the system. By utilizing observational data and without comprehensive ground-truth training sample, the model outperforms compared to the existing state-of-the-art baseline.

References

[1]	Z. X. A. H. J. Ruan, "Towards interdisciplinary integration of electrical engineering and earth science," Nature Reviews Electrical Engineering, vol. 1, no. 5, pp. 278-279, 2024.

[2]	Y. Li et al., "A Spatio‐Temporal Attention–Enhanced LSTM Model for Critical Fault‐Set Identification Under Wildfire Conditions," IET Gener. Transm. Distrib., vol. 19, no. 1, p. e70138, 2025.

[3]	A. Stankovski, B. Gjorgiev, and G. Sansavini, "Multi‐zonal method for cascading failure analyses in large interconnected power systems," IET Gener. Transm. Distrib., vol. 16, no. 20, pp. 4040-4053, 2022.

[4]	F. Yu, Q. Guo, J. Wu, Z. Qiao, and H. Sun, "Early warning and proactive control strategies for power blackouts caused by gas network malfunctions," Nat. Commun., vol. 15, no. 1, p. 4714, 2024.

[5]	S. M. Shengwei Mei, Y. N. Yixin Ni, G. W. Gang Wang, and S. W. Shengyu Wu, "A Study of Self-Organized Criticality of Power System Under Cascading Failures Based on AC-OPF With Voltage Stability Margin," IEEE Trans. Power Syst., vol. 23, no. 4, pp. 1719-1726, 2008.

[6]	S. M. Shengwei Mei, F. H. Fei He, X. Z. Xuemin Zhang, S. W. Shengyu Wu, and G. W. Gang Wang, "An Improved OPA Model and Blackout Risk Assessment," IEEE Trans. Power Syst., vol. 24, no. 2, pp. 814-823, 2009.

[7]	M. J. Eppstein and P. D. H. Hines, "A “Random Chemistry” Algorithm for Identifying Collections of Multiple Contingencies That Initiate Cascading Failure," IEEE Trans. Power Syst., vol. 27, no. 3, pp. 1698-1705, 2012.

[8]	X. Wu, D. Wu, and E. Modiano, "Predicting Failure Cascades in Large Scale Power Systems via the Influence Model Framework," IEEE Trans. Power Syst., vol. 36, no. 5, pp. 4778-4790, 2021.

[9]	M. A. Rios, D. S. Kirschen, D. Jayaweera, D. P. Nedic, and R. N. Allan, "Value of security: modeling time-dependent phenomena and weather conditions," IEEE Trans. Power Syst., vol. 17, no. 3, pp. 543-548, 2002.

[10]	Y. Zhu, Y. Zhou, W. Wei, and L. Zhang, "Real-Time Cascading Failure Risk Evaluation With High Penetration of Renewable Energy Based on a Graph Convolutional Network," IEEE Trans. Power Syst., vol. 38, no. 5, pp. 4122-4133, 2023.

[11]	Y. Liu, N. Zhang, D. Wu, A. Botterud, R. Yao, and C. Kang, "Searching for Critical Power System Cascading Failures With Graph Convolutional Network," IEEE Trans. Control Netw. Syst., vol. 8, no. 3, pp. 1304-1313, 2021.

[12]	A. Varbella, B. Gjorgiev, and G. Sansavini, "Geometric deep learning for online prediction of cascading failures in power grids," Reliab. Eng. Syst. Saf., vol. 237, p. 109341, 2023.

[13]	C. C. C. W. Supapo LONAPALAWONG, "Interpreting the vulnerability of power systems in cascading failures usingmulti-graph convolutional networks," Front. Inform. Technol. Elect. Eng., vol. 23, no. 12, pp. 1848-1861, 2022.

[14]	X. Gu, T. Liu, S. Li, X. Yang, and X. Cao, "Identification of vulnerable nodes in power grids based on graph deep learning algorithm," IET Gener. Transm. Distrib., vol. 17, no. 9, pp. 2015-2027, 2023.

[15]	X. Su et al., "DAMGAT-Based Interpretable Detection of False Data Injection Attacks in Smart Grids," IEEE Trans. Smart Grid, vol. 15, no. 4, pp. 4182-4195, 2024.

[16]	Y. Zhu, Y. Zhou, W. Wei, and N. Wang, "Cascading Failure Analysis Based on a Physics-Informed Graph Neural Network," IEEE Trans. Power Syst., pp. 1-10, 2023.

[17]	M. L. P. L. S. Miao, "Interpretable and Generalizable Graph Learning via Stochastic Attention Mechanism," presented at the PMLR, 2022.

[18]	F. L. S. B. Bernhard Schölkopf, "Towards Causal Representation Learning," Proc. IEEE, vol. 109, no. 5, pp. 612-634, 2021.

[19]	Y. Sui, X. Wang, J. Wu, M. Lin, X. He, and T. Chua, "Causal Attention for Interpretable and Generalizable Graph Classification,", New York, NY, USA, 2022, ACM, pp. 1696-1705, doi: 10.1145/3534678.3539366. [Online]. Available: https://go.exlibris.link/LTTQfg2W

[20]	Q. Guo, G. Li, and J. Lin, "A Domain Generalization Network Exploiting Causal Representations and Non-Causal Representations for Three-Phase Converter Fault Diagnosis," IEEE Trans. Instrum. Meas., vol. 73, pp. 1-13, 2024.

[21]	L. Massidda and M. Marrocu, "Total and thermal load forecasting in residential communities through probabilistic methods and causal machine learning," Appl. Energy, vol. 351, p. 121783, 2023.

[22]	Z. Wang, Y. Zhou, Q. Guo, and H. Sun, "The Total Transfer Capability Assessment of Transmission Interfaces Combining Causal Inference and Multi-Task Learning," IEEE Trans. Power Syst., vol. 39, no. 1, pp. 453-464, 2024.

[23]	S. S. Ghosh, A. Dwivedi, A. Tajer, K. Yeo, and W. M. Gifford, "Cascading Failure Prediction via Causal Inference," IEEE Trans. Power Syst., vol. 40, no. 4, pp. 3361-3373, 2025.

[24]	"Analysis and suppression of subsynchronous oscillation of Rudong wind power through MMC-HVDC system," presented at the 2021 Annual Meeting of CSEE Study Committee of HVDC and Power Electronics (HVDC 2021), 2021.

[25]	M. X. Z. Z. XIANG Wang, "Topology Design of ±800 kV/10 000 MW HVDC Transmission for Inland Large-scale Renewable Energy Bases," High Voltage Engineering, vol. 50, no. 5, pp. 2052-2066, 2024.

[26]	M. Panteli, C. Pickering, S. Wilkinson, R. Dawson, and P. Mancarella, "Power System Resilience to Extreme Weather: Fragility Modeling, Probabilistic Impact Assessment, and Adaptation Measures," IEEE Trans. Power Syst., vol. 32, no. 5, pp. 3747-3757, 2017.

[27]	X. Wei, S. Gao, T. Huang, E. Bompard, R. Pi, and T. Wang, "Complex Network-Based Cascading Faults Graph for the Analysis of Transmission Network Vulnerability," IEEE Trans. Ind. Inform., vol. 15, no. 3, pp. 1265-1276, 2019.

[28]	X. Su, S. Wu, S. Xiao, X. Liu, Y. Tan, and L. Liu, "A Novel AC/DC Power Flow Algorithm Considering MMC-HVDC Connected Offshore Wind Farms and System Regulation Characteristics,", 2024, IEEE, pp. 1-5, doi: 10.1109/APPEEC61255.2024.10922477. [Online]. Available: https://go.exlibris.link/GNdvJRjp

[29]	Matpower, "CASE2737SOP Power flow data for Polish system - summer 2004 off-peak." https://matpower.org/docs/ref/matpower6.0/case2737sop.html(accessed.

This work is supported by National Key R&D Program of China “Remote monitoring and fault diagnosis technology for offshore wind power grid connected systems (2023YFB2406900)”.

S. Xiao, Y. Fu, and S. Zhang are with the School of Mechatronic Engineering and Automation, Shanghai University, Shanghai, China (xiaoshiquovo@foxmail.com, mfudong@126.com, eeshzhang@shu.edu.cn).

X. Su is with the Engineering Research Center of Offshore Wind Technology Ministry of Education, Shanghai University of Electric Power, Shanghai, China (xiangjing_su@126.com).

J. Yang is with the James Cook University, Townsville, Australia (jiajia.yang@jcu.edu.au).

C. Zhang is with the School of Electrical & Computer Engineering, the University of Sydney, Sydney, Australia (cuo.zhang@sydney.edu.au).

Z. Dong is with the Department of Electrical Engineering, City University of Hongkong, Hongkong, China (zydong@ieee.org).