Spatiotemporal variations of groundwater level and gully impact in two peatland watersheds in the Source Region of the Yellow River

Zhiwei Li a, *, Bingyu Zhou b, Peng Gao c, *, Xiwei Guo c, Bang Chen d, Shimin Tian e

a State Key Laboratory of Water Resources Engineering and Management, Wuhan University, Wuhan 430072, China

b Xiangtan Water Resources and Hydropower Survey and Design Institute Co., Ltd, Xiangtan 411100, China

c  Department of Geography and the Environment, Syracuse University, Syracuse, New York 13244, USA

e  School of Hydraulic and Environmental Engineering, Changsha University of Science & Technology, Changsha 410114, China

e  Key Laboratory of Yellow River Sediment, Yellow River Institute of Hydraulic Research, Zhengzhou 450003, China

* Correspondence author:

lizw2003@whu.edu.cn; pegao@syr.edu

Abstract: The spatiotemporal variability of groundwater level (GWL) is an important property of peatland hydrology that directly affects fluctuations of water storage. Nonetheless, current understanding of the variations of GWL in different time scales still remains unclear. In this study, two peatland watersheds (0.151 km2 for W1 and 0.844 km2 for W2) in the Zoige Basin in the Source Region of the Yellow River (SRYR) were selected for monitoring the temporal variability of GWL using self-recorded water loggers during 2017-2021. The main results demonstrate that: (1) GWL variations tended to be controlled by gully drainage in sites adjacent to the gully and be more synchronized with rainfall in sites distant from the gully. The GWL near the gully that cuts through the peat layer was lower than that near the gully without cutting through the peat layer, with a maximum difference between the former and the latter of 58.3 cm, indicating the effect of longitudinal attenuation of the GWL in W1. (2) Because rainfall had a lag effect on the GWL, the length of lag gradually decreased with increased rainfall intensity (i.e., the lag time of sites far away from the gully was about 18 min shorter than that of sites close to the gully in W1). (3) The peak values of the GWL occurred simultaneously with the maximum and minimum rainfall in W2, and the peak occurrence time was related to the ratio of precipitation to evaporation. In the downstream sites, GWL fluctuated more intensively than the upstream ones in W2. Moreover, the average GWL of the upstream sites was 14.3 cm higher than that of the middle ones, indicating a decreasing trend of water storage along the gully. (4) The GWL discrepancy between wet and dry seasons was explicit, but the difference was smaller in the upstream sites due to limited gully incision and higher water storage within the peat layer. Additionally, rainy days dominate the GWL change in wet and dry seasons, but the different rainfall intensity resulted in a stable GWL in the dry season and an oscillating GWL in the wet season in W2. This study uncovers the spatio-temporal variation of groundwater level in two peatland watersheds, which is of great significance for understanding runoff variation, ecohydrological processes, and wetland shrinkage in the SRYR.

Key words: peatland hydrology, groundwater level, rainfall, gully impact, lag effect

1. Introduction

Peatland is the dominant type of wetland in high latitude, high elevation, and cold environments, in which peat is gradually formed by plant litter under water-submerged environment with hypoxia and incomplete decay (Li et al., 2018a; Ratcliffe et al., 2021; Turetsky et al., 2014). Global peatlands account for only 3% of the Earth's land surface but store about 25% of the world’s soil organic carbon and hence play a crucial role in stabilizing global climate and compensating greenhouse effects (Evans et al., 2021; Fenner and Freeman, 2011; Huang et al., 2021). Peatland is also a natural reservoir storing abundant water resources that has important ecological-service functions, such as water storage, water temperature regulation, and biodiversity conservation (Ishikura et al., 2017; Li et al., 2020; Liu et al., 2018; Wu et al., 2018a). The amount of water storage of peatlands does not partially depend on the thickness of the peat layer but is affected by physical properties of peat and spatiotemporal changes of groundwater level (GWL) (Chen et al., 2014; Ishikura et al., 2017; Li et al., 2018a). Nonetheless, the artificial drainage in peatlands, headward erosion of natural river network system, and excessive use of water in peatland for civil and grazing purposes have decreased GWL and water storage (Holden et al., 2006; Holden et al., 2011; Li and Gao, 2019; Li et al., 2018b; Li et al., 2015; Wilson et al., 2010; Zhang et al., 2016). Its long-term effect on peatland degradation is still not clear and calls for long-term in-situ monitoring for a more systematic evaluation.

Hydrologically, rainfall can directly recharge peatlands or generate surface runoff and subsurface flow that converge into wetlands (Regan et al., 2019; Sheng et al., 2020). The total amount, intensity, and duration of rainfall may affect ecohydrological regulation and water storage of peatlands (Li and Gao, 2020; Li et al., 2019). Climate change may intensify the global hydrological cycle by alternating precipitation, evapotranspiration and runoff, which profoundly changes hydrological processes and water balance of peatlands (Evans et al., 2021; Fossey and Rousseau, 2016; Gunther et al., 2020; Levison et al., 2014). These external hydrological changes often increase GWLs. The increased GWLs may decrease by recharging rivers connected to peatlands during non-rainfall periods (Hare et al., 2017; Li et al., 2019; Rahman et al., 2017). Over a long time period, these variations of GWLs affect the formation, development, and water storage functions of peatlands. Thus, monitoring temporal variations of GWL is critical for identifying controlling factors and further evaluation of water storage in peatlands (Hikouei et al., 2023; Rahman et al., 2017).

As the largest alpine peatland in China, the Zoige Basin is the most important water source in the Source Region of the Yellow River (SRYR). It features peatland that area once had the area of about 4,600 km2 and stored organic carbon of about 0.477 Pg (Chen et al., 2014). The annual water supply of the Zoige Basin to the Yellow River was 6.71 ± 1.49 ×109 m3 in 1981-2011 (Lu et al., 2019). In recent decades, water supply from the Zoige Basin continued to decrease at a rate of 0.48×108 m3 per year, which is related to the shrinkage of the peatland area by 52% since 1960 (Li et al., 2015). The area reduction of peatland may be related to gully erosion, peat mining, artificial ditch drainage, and overgrazing activities (Li and Gao, 2019; Li et al., 2019; Li et al., 2018b). Artificial ditches accelerate GWL decline, causing the decrease of water storage in peatlands, which subsequently result in their area reduction and degraded ecological function. Gully and headward erosion are the key geomorphic processes of lowering GWL (Holden et al., 2011; Li and Gao, 2019; Li and Gao, 2020). Previous studies have mainly focused on quantifying peat hydraulic conductivity, weight, and porosity, and analyzing the spatial distribution of the GWLs under various peat depths (Dixon et al., 2017; Li et al., 2018a; Li et al., 2019; Liu et al., 2018; Mercer and Westbrook, 2016; Young et al., 2017). In addition, studies have shown that gully drainage increases peat hydraulic conductivity and reduces peat water storage, which further lowers the peat GWL (Ishii et al., 2016; Li et al., 2018a; Price et al., 2003; Ramchunder et al., 2009). These impacts on peat physical properties and hydrological functions vary between gullies whose beds cut through the peat layer or not (Holden et al., 2011; Li and Gao, 2019; Li et al., 2019). However, how the effect of gullies on GWLs of peatland is still unknown.

The aim of this study is to reveal the spatiotemporal variations of groundwater level in two peatland watersheds and identify the different impacts of the gully (cutting through peat and non-cutting through peat) on GWL change based on in-situ observations in Zoige Basin of the SRYR on the northern Qinghai-Tibet Plateau from 2017 to 2021. First, variation of GWL in hourly and daily scales is analyzed for measured sites near and far from the gully in W1. Second, spatiotemporal variations of GWL in annual and seasonal scales are quantified and further comparatively analyzed in wet and dry seasons in W2. Finally, we reveal the main causes controlling the GWL variation in the two peatland watersheds.

2. Materials and methods

## 2.1 Study area

The Source Region of the Yellow River (SRYR) is situated on the northeast side of the Qinghai-Tibet Plateau, where the elevation is > 3,000 m. It has a cold alpine climate influenced strongly by the East Asian monsoon. The SRYR watershed is one of the major components of the “Asian Water Tower” (Yao et al., 2022; Zhang et al., 2021) and accounts for about 17% and 35% of the Yellow River Basin by area and the average annual runoff, respectively. The SRYR develops peatlands widely across the region. The total peatland area is about 10,750 km2, 48.3% of which is distributed in the Zoige Basin (Figure 1a, b). The Zoige Basin has an average elevation of 3,500 m, where the annual mean temperature is 0.6-1.2 °C, the mean annual rainfall is about 765 mm, and the mean relative humidity is 64-73% (Li et al., 2020). The peat layer is about 3 m deep on average, storing groundwater up to 4.5×109 m3. The annual water supply of the Zoige Basin to the Upper Yellow River exhibits a decreasing trend in recent years (Li et al., 2014; Li et al., 2015; Wang et al., 2018; Zhang et al., 2011).

This study focuses on two small watersheds in the upper reaches of the Black River in the Zoige Basin, which is about 46 km away from Hongyuan County (Figure 1b). The first watershed (W1) is located in the upper reaches of the Black River (Figure 1c) with an area of about 0.151 km2. The upper ground layer contains peat soil of 0.5-2.0 m deep, and the lower layer mainly consists of weakly permeable silts with the median particle size of 0.651 and 0.039 mm for the former and latter, respectively. According to our field observations, natural gullies can be divided into two types based on whether the peat layer is cut through or not. To analyze the difference in GWL changes in different sites, eight automatic water-level loggers (AWLLs) were placed in W1 (Fig. 1c). Among them, P1-P5 were installed near the gully, and P6-P8 were installed far away from the gully for comparison. Also, P1 and P2 were located near the gully that cuts through the peat layer, whereas P3-P5 were located near the gully that does not cut through the peat layer (Table 1).

The area of the second watershed W2 is about 0.844 km2, in which the peatland area is about 0.289 km2, surrounded by hillslopes on three sides (i.e., 13.6°, 6.7°, and 10.3° for the left, right, and headward slope, respectively). The monitoring sites (S2-S7) by six AWLLs are located along the main gully with their distances to the gully edge of 5.3, 2.2, 4.0, 3.2, 4.8, and 6.3 m, respectively (Table 1). The bed of the main gully has been subject to headward incision. From upstream to downstream, the incision depths (h) of the gully close to S7 and S2 are 0.31 and 1.50 m, respectively, demonstrating headward erosion along the peat gully (Figure 2a, 2b, 2c).

Figure 1  Geographic setting of two studied peatland watersheds (W1 and W2). (a) Location of the Zoige Basin (outlined in red) within the Qinghai-Tibet Plateau; (b) Distributions of peatland (gray shaded areas) and major river networks within the Zoige Basin (WR: White River; BR: Black River) and two studied watersheds; (c) Locations of 8 installed AWLLs along the gully G1 in the W1; (d) Locations of 7 installed AWLLs along the gully in the W2.

Figure 2  (a) A photo of the gully with cutting through the peat in W2; (b) A photo of the gully with non-cutting through the peat in W2; (c) Incision depths and groundwater levels of S1-S7 sites in W2 based on the measured data on July 27, 2021; (d) A schematic diagram a photo show the installing method of water level logger.

Table 1 Basic parameters and installation information of 15 water level loggers

| No. | Longitude and latitude | Location | Types of the gully | Embedded depth (cm) | Slope of the site to the gully (o) |
| --- | --- | --- | --- | --- | --- |
| P1 | 32°57'48.70"N，103° 0'26.30"E | near gully | Cutting through the peat | 79.5 | 8.5 |
| P2 | 32°57'45.57"N，103° 0'29.50"E | near gully | Cutting through the peat | 64.5 | 4.5 |
| P3 | 32°57'36.23"N，103° 0'25.25"E | near gully | Non-cutting through the peat | 36.5 | 4.6 |
| P4 | 32°57'35.11"N，103° 0'25.44"E | near gully | Non-cutting through the peat | 55.5 | 3.2 |
| P5 | 32°57'34.22"N，103° 0'24.83"E | near gully | Non-cutting through the peat | 69.5 | 3.5 |
| P6 | 32°57'44.06"N，103° 0'30.19"E | far away the gully | - | 68 | 11.1 |
| P7 | 32°57'43.23"N，103° 0'27.12"E | far away the gully | - | 73.5 | 11.9 |
| P8 | 32°57'43.48"N，103° 0'30.71"E | far away the gully | - | 40 | 7.9 |
| S1 | 33°04'46.80"N，102° 53'46.19"E | in the gully | Cut through the peat | 33 | - |
| S2 | 33°04'48.70"N，102° 53'43.17"E | near gully | Cut through the peat | 76 | 1.8 |
| S3 | 33°04'50.48"N，102° 53'39.30"E | near gully | Cut through the peat | 51.2 | 2.6 |
| S4 | 33°04'54.03"N，102° 53'33.35"E | near gully | Non-cutting through the peat | 64 | 2.3 |
| S5 | 33°04'55.02"N，102° 53'31.84"E | near gully | Non-cutting through the peat | 66 | 8.0 |
| S6 | 33°04'56.94"N，102° 53'28.80"E | near gully | Non-cutting through the peat | 37 | 0.8 |
| S7 | 33°04'59.20"N，102° 53'25.28"E | near gully | Non-cutting through the peat | 24.3 | 4.1 |

## 2.2 Field measurements

In W1, GWL was observed in situ from May to September 2017 using 8 automatic water-level loggers (AWLLs) (P1-P8) (HOBO, U20L-04). In W2, it was recorded at six sites (S2-S7) along the peat gully. An additional AWLL was installed at the gully outlet for obtaining continuous water depths from May 2018 to May 2021 (Figure 1d and Table 1). Each AWLL was connected to the ground surface using a fine steel pipe and fastened to a nearby thick steel pipe (Figure 2d). The AWLL can detect the water depth up to 4 m. It has a wide sampling time interval from 1 s to 18 h, a high data transmission speed, a measurement accuracy of 0.1%, a short response time, a high temperature resolution (0.1 °C), a long power-supply time, and a water-level accuracy of ± 4 mm. Therefore, the AWLL satisfies the requirements of GWL monitoring in peatlands. The sampling interval of 15 minutes was set at the observation points in this study.

The AWLL adopts the pressure measurement principle to obtain the groundwater pressure. It is based on the distance between the probe and the water surface (the position of the probe remains unchanged) to determine the groundwater level (Figure 2d):

(1)

where WL is the groundwater level (the ground surface is 0, such that the GWLs are negative values); D1 is the underwater atmospheric pressure, which is converted to the distance of the probe from the water surface; D2 is the buried depth, which is the distance from the top of the logger to the surface; D3 = 0.154 m, which is the logger length.

P1-P8 recorded the GWL data in W1 from May 16, 2017 to September 18, 2017. S1-S7 recorded the GWL data in W2 from May 29, 2018 to May 28, 2021. For W1, the monitoring periods are divided into three parts: T1w1 (18: 00pm May 15, 2017 to 11:00am June 8, 2017), T2w1 (12:00pm June 8, 2017 to 10:00am July 27, 2017), and T3w1 (11:00am July 27, 2017 to 15:00pm September 18, 2017). For W2,

three analysis periods were defined (i.e., May 29, 2018 to May 28, 2019, May 29, 2019 to May 28, 2020, and May 29, 2020 to May 28, 2021) as T1w2, T2w2 and T3w2, respectively, to facilitate analysis of inter-annual changes and subsequent interpretations of hydrological processes.

In addition, we performed an unmanned aerial vehicle (UAV) survey (DJI Advanced 4 Pro) in 2018 for W2. The flight height was fixed at 160 m, and the overlap rate of the images was 85%. A group of RTK GPS points were created in the study area for error estimation. The spatial resolution of the UAV aerial survey images is 42 mm. The finally generated DEM (digital elevation model) is used to calculate the slope and the gully incision depth at different locations of the gully (Figure 2c).

## 2.3 Data of precipitation and evapotranspiration

Among three meteorological stations (i.e., Zoige, Hongyuan, and Maqu) (Figure 1b) in the Zoige Basin, Hongyuan station is the closest to W1 and W2. Thus, meteorological data recorded at this station were verified using China Daily Ground Climate Dataset (V3.0) of China Meteorological Data Network and utilized for comparison in this study. According to the rainfall intensity standard adopted by the Chinese meteorological department, light rain is defined as a cumulated rainfall of < 5 mm within 12 hours, moderate rain is defined as a cumulated rainfall between 5.0 and 14.9 mm within 12 hours, heavy rain is defined as a cumulated rainfall between 15.0 and 29.9 mm in 12 hours, and very heavy rain large than 30 mm.

The cumulative precipitation from 18: 00pm May 15, 2017 to 11:00am June 8, 2017 (T1w1) was 68.5 mm, and the average rainfall intensity was about 0.9 mm/h (Figure 3a). Eighteen of the 23 rainfall periods had moderate and light rain. From 12:00pm June 8, 2017 to 10:00am July 27, 2017 (T2w1), the cumulative rainfall was 103.3 mm, and the average rainfall intensity was about 0.9 mm/h. Eighteen of the 22 rainfall periods were moderate and light rain. From 11:00am July 27, 2017 to 15:00pm September 18, 2017 (T3w1), the cumulative precipitation was 200.6 mm and the average rainfall intensity was about 1.4 mm/h, and 12 of the 36 rainfall periods are heavy and rainstorm that mainly concentrated between late August and mid-September. These data matched the periods of GWL measurements in W1. For W2, the period of rainfall data collection was from May 2018 to May 2021. The annual rainfall in T1w2, T2w3 and T1w3 were 982.4, 827.7 and 1004.8 mm, respectively (Figure 3b).

The potential evapotranspiration (ET0) was calculated according to the P-M Formula recommended in FAO56 Drainage and Irrigation Manual (Allen et al., 1998). The land cover type and its corresponding area were extracted according to remote sensing images, and the actual evapotranspiration ETc was calculated by using the area of various land cover as the weight factor. ETc of grassland accounts for about 74.28%, wetland for about 23.85%, and the sum of desert and water body about 1.87%. For W1, mean and std value of daily ETc is 1.83 mm and 0.64 mm, respectively (Fig. 3a). For W2, mean and std value of daily ETc is 1.43 mm and 0.64 mm, respectively. ETc in T1w2, T2w2, and T3w2 are 491.2, 510.0, and 525.2 mm, respectively (Figure 3b). The period between May and October each year has the strongest evapotranspiration, accounting for 61.4%, 60.5%, and 65.9% for T1w2, T2w2, and T3w2, respectively.

Figure 3  Variations of precipitation and calculated evapotranspiration at Hongyuan Meteorological Station  (a) Daily values from May 8 to September 18 2017 in W1, (b) Daily values from May 1 2018 to May 31 2021 in W2.

## 2.4 Analyses of groundwater levels

2.4.1 Analysis of GWLs in W1

The analysis of GWL data in W1 follows methods adopted in the steps:

(1) We analyzed the temporal trend of GWLs variations at each site and recorded the occurrence time of the peatland saturation with the GWLs greater than zero.

(2) The sampling sites (P1-P8) were then categorized based on their adjacency to the gully (i.e., close to the gully and distant from the gully). Recall that these observation points are located in both the gully that cuts through the peat layer and the gully without cutting through peat layer.

(3) We then determined the periods of heavy rain and no rain, and further analyzed the relations between hourly rainfall and GWL change.

(4) Next, we analyzed whether the lag effect appears between the GWLs and the rainfall. If there is a lag effect, the lag time is estimated according to the classification of the rainfall intensity. The lag time is expressed as:

(2)

in which tWL is the time of groundwater level change, tPRE is the duration of the rainfall period,is the lag time of groundwater level affected by the rainfall.

(5) Lastly, we quantitatively compared differences in the GWLs of P1-P8 based on the adjaceny to the gully and infer causes of the spatial variability.

2.4.2 Analysis of GWLs in W2

The analysis of GWL data in W2 follows methods adopted in the steps:

(1) Changes of GWL over time mainly involve inter-annual and intra-annual periods. We calculated the average annual GWLs and its annual differences in the three periods and determined the inter-annual changes of GWLs. The monthly GWLs and its CV (coefficient of variation) were calculated, and the differences of groundwater levels in different months were compared and analyzed as well.

(2) We then conducted a comparative analysis of the change of GWL along S2-S7 in different months, wet and dry seasons, and rainy and no-rainy days.

(3) Lastly, the responses of GWLs to rainfall, air temperature and evapotranspiration were assessed. The changing trend between the average monthly GWL and monthly precipitation was analyzed. In order to characterize the relationship between GWL change and precipitation and evapotranspiration, the ratio of precipitation to evapotranspiration (RPE) is defined below.

(3)

where P is the precipitation in a certain period of time, ETc is the actual evapotranspiration. If RPE is greater than 1, precipitation is the predominant factor. In comparison of RPE with GWL, we further analyzed the response of GWL to precipitation and evapotranspiration in different months of each sampling year.

# 3. Results

## 3.1 Spatiotemporal variation of groundwater level in the hourly and daily scales in W1

3.1.1 The measured sites near the gully

Five AWLLs (P1-P5) near the gully were used to analyze the change of the GWLs at different time scales (hours and days). In the P1, the GWL first increased, then decreased, and finally increased again in T1w1, T2w1, and T3w1, respectively. During the heavy rainfall in August and September, the GWLs experienced dramatic and abrupt rises (Figure 4), and the difference between the initial change value and the maximum value of the GWLs reached 73 cm. In the P2, the GWLs in the T1w1 and T2w1 periods changed more mildly that fluctuated around 50 cm below the surface. During T3w1, due to the abundant rainfall, the GWL increased significantly, with an average increase of 6.1 cm/h and a maximum increase of 30 cm in 1 hour (Figure 4). Overall, P3, P4, and P5 fluctuated around 0 cm in T1w1, and appeared the falling and rising trend in T2w1 and T3w1, respectively.

Among the five observation sites near the gully, only P3 had the GWL value above zero, indicating a saturation state of the peat layer (Fig. 5). Specifically, from 9: 35am, July 12, 2017 to 20:15pm, August 29, 2017, GWL was 7.3 cm at 13:15pm July, 27 2017, and 8.1 cm and 2.9 cm at 12:15pm and 12: 30pm, respectively, in August 20, 2017, while remaining below zero at other times. The rainfall data of the weather station from 9:45 am, July 7, 2017 to 20:15 pm,August 29, 2017 showed that there was very limited precipitation, resulting in five sites that remained unsaturated. Due to the recharge of groundwater on nearby slopes, the unsaturation time of P3 was delayed to July 13, 2017 and reached saturated early on August 29, 2017. The unsaturation time of the remaining four observation points (P1, P2, P4, P5) were on June 28, 2017 and the four loggers finally reached saturation state on August 30, 2017.

In terms of topography, the changing trends of the GWLs at the five observation sites near the gully are related to their locations (Table 1). The observation site of P1 was installed in the northwest of the watershed W1, in a low-lying land around the high terrain (8.5o slope). Furthermore, P1 is located downstream of the gully, so the GWLs change at P1 was relatively stable. P2 is located in a flat terrain (4.5o slope) in the northeast of the watershed W1. P3, P4, and P5 are located directly to the south side of the watershed W1, on a gentle slope beside the shallow gully, with a slope of 4.6o, 3.2o, and 3.5o, respectively. Since the three sites were installed in the upstream side of the gully, the GWLs fluctuated largely. Compared to the rainfall at the same period, it is found that the GWLs of P3-P5 was more responsive to rainfall than that of P1 and P2.

Figure 4  Groundwater level and rainfall of the five monitoring points (P1-P5) near the gully in the W1.

During moderate and light rains (T1w1 and T2w1), the GWLs of P1 and P2 (the gully cutting through the peat layer) were lower than that of P3-P5 (the gully non-cutting through the peat layer), with a maximum difference of 58.3 cm, which was caused by the former having a stronger drainage capacity than the later. Furthermore, P1 had a lower GWL than P2 because P1 is closer the gully than P2. The GWLs near the gully that cuts through peat layer showed a strong lateral distance attenuation effect, with the GWLs near the gully notably lower than that distant from the gully. Only P3, P4 and P5 differed greatly because of the weak drainage capacity of the gully without cutting through peat layer. The surrounding peat replenishes the gully less, which made the water on the nearby ground seep into the peat layer, causing increases in the GWLs.

We selected a heavy rain period (10:00am-23:00pm, August 29), in which the total rainfall is 17.5 mm and the rainfall intensity is 1.3 mm /h, and a no rainfall period (0:00 am August 8-20:00pm, August 11) for comparison. The GWLs in P1 fluctuated most during the heavy rain time (Figure 4). The difference between the maximum GWL and the initial GWL values is 40.9 cm, 16.7 cm, 28.1 cm, 20.9 cm and 16.6 cm for P1-P5, respectively, with the average hourly increasing rate of 6.8 cm/h, 2.1 cm/h, 2.8 cm/h, 2.3 cm/h and 1.8 cm/h, respectively. This obvious discrepancy may be related to the fact that P1 is closer to the gully, although the impact of the gully to P2 is very limited because the distance of the P2 to the gully is 33 m. The GWL of P3, P4, and P5 varied less because the three observation points are close to the gully without cutting through the peat layer. In the no rainfall period, the GWLs of the five observation sits did not change much (Figure 5), and the extreme difference of GWLs in P1-P5 were 5.7 cm, 13.4 cm, 22.5 cm, 21.8 cm and 19.9 cm, respectively, which were more stable compared with the heavy rain period, indicating that the occurrence of rainfall events is an important driver for GWL variation.

3.1.2 The measured sites far away from the gully

The GWLs of P6, P7, and P8 that are distant from the gully showed roughly the similar trend over the study period. During the T1w1, T2w1, and T3w1 periods, the GWLs first rose briefly, then continuously decreased, and then rose rapidly, showing a similar trend with the rainfall in the corresponding period (Figure 5). The GWLs in P6, P7 and P8 are all greater than zero, indicating the saturation state of the peat layer, even though their corresponding occurring times are different (i.e., 19:45pm, August 29, 2017 in P6, 22:30pm, June 3, 2017 in P7, and 18:15pm, May 16, 2017).

We selected a heavy rain period (10:00am-23:00pm, August 29, 2017) and a non-rainfall period (0:00 am August 8-20:00pm, August 11, 2017) as well as the above analysis. Specifically, GWL variation in P6 during the heavy rain is not significant compared with that in P7 and P8 (Figure 5). The differences between the maximum GWL and real GWL in the starting time were 21.3 cm, 31.5 cm, and 52.6 cm for P6, P7, and P8, respectively, with an average hourly increasing rate of 2.4 cm/h, 4.5 cm/h and 5.8 cm/h, respectively. This difference is related to the differing location of P6, P7, and P8, i.e., headward incision in P8, leading to a more obvious occurrence of GWL fluctuation. Nevertheless, in the non-rainfall period, the GWLs of P6, P7 and P8 did not change considerably (Figure 5), and the extreme difference of the GWLs were 9.2 cm, 15.8 cm, and 11 cm respectively, which were more stable compared with those during the heavy rain period. It also indicates that the occurrence of rainfall events indeed caused the change of GWLs. The fluctuation of GWLs in P6-P8 was similar in time to the observed sites near the gully (P1-P5), and the fluctuation range of GWLs increases with increasing rainfall. Among them, the GWL fluctuation range of the P8 was significantly larger compared with that of P6 and P7. The difference between the upper and lower limits was 90 cm, which was related to the headward incision process of the gully in P8. Overall, the mean GWLs in the P6-P8 sites that are distant from the gully fluctuatd more mildly than the GWLs of the sites near the gully.

Figure 5  Groundwater level and precipitation of P6, P7, and P8 far away from the gully in the W1.

3.1.3  Groundwater level changes at the day and night scale

To further explore the short-term change of GWLs in peatland within a single day, we counted the average GWL changes in the day and night within one week and compared the five AWLLs close to the gully with the three ones distant from the gully (Figure 6). In the P1, the daytime GWLs were generally higher than that at night, but the high GWLs in some nights reached the peak. For example, the evening GWL on September 16 and September 8 reached 17.40 cm and 14.75 cm, respectively, 11.05 cm and 3.75 cm higher than that in the daytime. This was caused by the heavy rainfall (9.3 mm and 5.7 mm on September 16 and September 8, respectively) and the water recharge into the peat layer. In the P2, except on September 16, the daytime GWLs were higher than that at night, indicating that the evapotranspiration during the day consumed water, and that water flowing into the gully did not recharge the peatland at night. In the P3, the GWLs during daytime were 3 cm higher than that at night on average, with the highest value reaching 9.9 cm. Given the even amount of precipitation within this small watershed, it is suggested that the higher GWLs at P3 during the day were replenished by the surrounding high terrain, such that high GWLs could be maintained during daytime. In the P4, the difference of GWLs between day and night decreased, and GWLs at night were higher than that in the daytime in some time periods. GWL patterns in P5 were distinct from the previous ones, with the GWL fluctuation being small during the day that maintained between 2.2-6.6 cm, while the GWLs at night being at 0-2.9 cm. It indicates that the location in P5 is very flat and difficult to be replenished by the surrounding slopes or peatland, and it is also difficult to produce water accumulation by precipitation and GWLs maintain within the limited change range. In general, the rising water level in the gully generated a replenishment effect in the P1 during the rainfall. The gully drainage lowered GWLs in P2 and P3. P4 was synchronously affected by precipitation recharge and gully drainage. The GWLs in P5 were basically not affected by precipitation and channel drainage, in which water level change at night is close to 0, indicating a very low level of evening evapotranspiration.

For P6, P7 and P8, which are distant from the gully, the GWLs at the day and night displayed some new characteristics (Figure 6). In the P6, the change of daytime GWLs was much higher than that at night, which may be related to the shorter showers during the daytime. However, in the night of August 28, the GWL change was greater than that during the day due to the continuous rainfall at night, resulting a greater evening GWL change than that of daytime. In the P7, the daytime change of GWLs reached the maximum value of 13.7 cm on July 27, which indicates that heavy rainfall led to high GWL on the surface, while the GWL change in other periods during the day was higher than that at night. The evening GWL change on August 27, September 8 and September 16 was greater than that during the day, because obviously a large rainfall event occurred at night. In the P8, the GWL changes between day and night were quite different, which shows that, in addition to rainfall, terrain and hillslope seepage also played a role in the complex variational patterns of GWL between day and night.

Figure 6  Groundwater level in day and night scale (a-e) near the gully (P1-P5)  (f-h) far away the gully (P6-P8) (i) Daily precipitation from May 16 to September 16, 2017.

## 3.2 Spatiotemporal variation of groundwater level in the annual and seasonal scales in W2

The spatiotemporal changes of GWL in the peatland reflect the interactions among rainfall, evapotranspiration, and infiltration. This section characterizes the spatial and temporal variations of GWLs in S2-S7 and the gully impact in W2. During T1w2, T2 w2, and T3 w2 in the annual scale, the highest values of GWL in S2-S7 appeared to be in October, September, and September, respectively, and the lowest values appeared to be in February, January, and February, respectively (Figure 7). In addition, four peaks and two troughs existed in GWL changes in S2-S7 during T1w2 and T2 w2. Specifically, four water drops existed in July, August, September and January during T1w2, and two water drops existed in June, August, September and January during T2w2. This demonstrates that, in the T1w2 period, the precipitation in August decreased by 106.9 mm compared with that in July, the precipitation in September increased by 63.8 mm, and the precipitation in January was only 3.2 mm. In the T2 w2 period, the sudden increase of GWL also occurred in the intermittent rainstorm. Therefore, the annual change of GWL and precipitation in the peatland kept the same pace. From the perspective of inter-annual changes of GWL, the average annual GWL of S2-S7 were -0.37, -0.37, and -0.31 m in T1w2, T2 w2, and T3w2, respectively. The fluctuating amplitude of GWL was little, but the GWL during the monitoring period increased by about 6 cm from 2018 to 2021. Spatially, the mean difference between the highest and lowest GWL values of S2-S7 during T1w2, T2 w2, and T3 w2 were 0.97, 0.81, 0.85, 0.74, 0.42, and 0.39 m, respectively. Since S2 in the upstream watershed is the first supplied site by the hillside confluence, which is easier to reach the saturation state, the variational amplitude of GWLs at the upstream observed sites are found smaller than that at the downstream ones.

Combined with the difference between the highest and lowest values of S2-S7 and patterns of fluctuation in Figure 7, we find that the most drastic change occurred in S2, where the GWL was significantly lower than other observed sites, and the difference from the nearest site S3 is 73.0 cm, with an average of 35.9 cm. S3, S4 and S5 are in the middle parts within the watershed, of which the GWLs differed mildly in different periods. The difference in GWLs between the three observed sites (S3, S4, S5) fluctuated between 0.2 cm and 21.0 cm, with an average difference of 6.0 cm. In S6 and S7, the groundwater levels had little fluctuations but were higher than that of the other sites. The average GWL of the two observed sites (S6 and S7) was about 14.3 cm higher than that of S3, S4 and S5, and the average water level difference between S6 and S7 was 11.0 cm. Consequently, the change of GWL in the peat layer showed an explicit linkage with precipitation.

The distances of sites S2-S7 to the gully edge were used to analyze spatial changes of GWL, where the distance between two adjacent observation sites (i.e., S2-S3, S3-S4…S6-S7) were 114.39, 189.00, 49.87, 98.49, and 114.78 m, respectively, and elevation difference between the ground surface and gully bottom in S2-S7 (Figure 2). The elevation from S2 to S7 increased continuously along the gully, with a total elevation difference of 13 m. The GWL in T1w2-T3w2 generally increased in each period, and the changing rates of GWL with elevation were 4.2, 3.6, and 3.1 cm/m, respectively (Figure 7). The average water level differences between S2 and S3 and between S3 and S4 were 36.4 and 2.3 cm, respectively, with the distance of the former (S2-S3) less than the latter (S3-S4). The incision depths, therefore, were 1.50, 0.80, and 0.45 m, respectively, for S2, S3, and S4, suggesting gully incision draining water out of the peatland as a main driver in lowering GWL during non-rainfall periods. Furthermore, the largest variation of GWL in S2 occurred in T1w2-T3w2 indicates that the lower gully cutting through the peat layer leads to groundwater dropping significantly.

In order to compare the relationship between GWL change and precipitation along the gully, the May-September period was selected as an example. With the increase of elevation, the GWL generally increased in T1w2-T3w2, and the average changing rate of GWL with elevation in May-September were 2.1, 1.3, 1.3, 2.0, and 1.9 cm/m between any two adjacent sites (i.e., S2-S3, S3-S4, S4-S5, S5-S6, and S6-S7, respectively). During the T1w2-T3w2 period, the average rainfall from May to September were 112.7, 189.6, 153.4, 129.1, and 137.1 mm, respectively. With decreasing elevation, the GWL in S3-S6 showed diverse changes, but the highest precipitation occurred in June and the lowest GWL occurs in August. The precipitation difference between June and August was 60.5 mm, and the average difference of GWL was 13.7 cm. S7 at the upstream location showed a gradual rise from May to September. The GWL was 6.6 cm in S7, with an average rise of 1.7 cm per month with less fluctuation, which was related to its upstream position that had a shallow gully easily recharged by precipitation.

Figure 7  Monthly variations of the mean groundwater level in S2-S7 and precipitation during T1w2-T3w2.

S2-S7 can be divided into three groups according to their amplitude and trend of groundwater level change, namely S2 (Pattern 1), S3-S5 (Pattern 2) and S6-S7 (Pattern 3) (Figure 8a). Among them, Pattern 1 was located downstream of G1, with a large slope of 7.2°, where the GWL changed greatly, affected by the local topography. The slopes of Pattern 2 and Pattern 3, located more upstream, were relatively slow, with the averages of 2.8° and 3.6°, respectively. Given that the GWL points of Patterns 2 and 3 had smaller changes of GWL, such a classification enables further analysis of spatial variation of groundwater level. During the GWL monitoring time from 2018 to 2021, RPE was generally much greater than one during May-October, with the maximum of 48.8. From November to next April, RPE values remained small due to limited precipitation, with a mean value of 0.9. Although evapotranspiration was relatively low in winters, RPE still occasionally reached above one. Combining the daily GWL and RPE of Pattern 1, Pattern 2 and Pattern 3, the peak occurrence time of the GWL and the RPE was close to the trough value. The RPE was 25.0 on November 17, 2018, and then decreased by 24.6 in the next day. In the following 100 days, the average RPE was 0.2, and the GWL of Pattern 1 - Pattern 3 first decreased by 66.9, 66.7, and 17.9 cm, respectively, and then increased by 26.8, 30.4, and 11.9 cm, with roughly the same downward trend (Figure 8a). On July 8, 2018, RPE reached to 48.8, while GWL of the three patterns all showed a sudden increase by a mean of 6.7 cm and the maximum of 13.9 cm. In the period of low precipitation, for a few rainfall events, RPE also had a significant impact on the GWL. Specifically, RPE was 25.0 on November 17, 2018, while GWL of the three patterns all increased by a mean of 4.5 cm and a maximum of 6.6 cm.

Clearly, both precipitation and evapotranspiration have influenced GWL in this peatland watershed, with GWL and RPE showing a positive relationship. Owing to the influence of air temperature, GWL was affected more by RPE in summer than in winter. In addition, CV of GWL changed greatly in summer but less in winter, which further showed that GWL fluctuated more drastically in summer with higher rainfall and air temperature (Figure 9b). Pattern 2 had an abrupt change in the T3w2 period due to its location that maked it easier to reach saturation during and immediately following rainfall events and then the water level dropped rapidly after the completion of rainfall. However, this phenomenon did not appear in Pattern 2 in T1w2. The fluctuating ranges of Patterns 1 and 3 in T1w2-T3w2 period shared a similar trend, with Pattern 2 having a larger fluctuation in T3w2, indicating that the function of water storage declined in the peatland around the middle zone of the gully.

Figure 8 Temporal changes of groundwater level in three different modes and its response to precipitation and evapotranspiration. (a) Patterns of groundwater water level and RPE; (b) Patterns of CV of average monthly groundwater level in S2-S7.

## 3.3 Spatiotemporal variations of groundwater level in wet and dry seasons in W2

Overall, GWLs of the wet (i.e., May to October) and the dry (i.e., October to the following April) seasons were characterized by an increasing tendency with upward longitudinal distance, with the upstream sites having higher GWLs than those of the downstream sites due to local topographic settings (Figure 9). Meanwhile, GWLs at the six recording sites exhibited an explicit distinction between wet and dry seasons in that the former had significantly higher GWLs than the latter by an average of 0.22 m for the entire 3-year monitoring period. Their differences in GWLs varied between 0.04 and 0.47 m, with the upstream sites having comparatively small differences than the downstream ones, constituting a general decreasing trend with distance upstream (Figure 9). Such smaller differences in the upstream monitoring sites may be possibly explained by the small incision depths of the adjacent gully, which attenuated runoff and therefore conserved local water within the peat layer. Among wet and dry seasons over the monitoring period, 2018 and 2019 had the highest and the lowest GWLs, respectively, which strongly correlated with precipitation pattern, as described in the proceeding sections.

Figure 9  Mean groundwater levels and their differences between wet and dry seasons from S2 to S7 for the periods of (a) May 2018-April 2019; (b) May 2019-April 2020; (c) May 2020-April 2021.

The presence of precipitation highlights another variational property of GWL fluctuations in the six monitoring sites. Evidently, except S7, all other sites showed similar distinctions of GWL variation between the days with and without recordable rainfall (Figure 10). The temporal patterns of both rainy and non-rain days were highly consistent, with S2-S5 increasing while S6 and S7 decreasing over the study periods. Interestingly, rainy days not only dominated the wet season but also took a significant portion of the dry season, suggesting the subtle variation of GWL as a result of the limited rainfall brought by the westerlies’ southern trough in the winter and possible occurrence of snowfall. In comparison, non-rain days only dominate the falling limb of GWL each year, highlighting the crucial role climatic conditions played in groundwater dynamics. Moreover, the GWLs in rainy days during the wet season had intensively undulating points (Figure 10), which demonstrated the strong precipitation in the summer and the resultant GWL fluctuations. The differences of GWL between non-rainy and rainy days in S2-S7 were 16.8, 16.9, 15.7, 11.0, 9.8, and 5.9 cm, respectively. In summary, the rainy process determined the temporal trajectory of GWL and the gully played less impact on the day-scale variation of GWL.

Figure 10  Groundwater level change and trend over time in non-rain and rain days from S2 to S7 in 2018-2021 in the W2.

# 4. Discussion

## 4.1 Assessment on measurement errors of hydrometeorological data on site

Two peatland watersheds in the core area of the Zoige Basin were successively monitored by using AWLLs for the first time in the absence of meteorological data recorded for the studied watershed. The meteorological data used in this study were obtained at the Hongyuan station (46 km away from the study site), which is the closest meteorological station in local regions (Li et al., 2020). By comparing the meteorological data of China Daily Ground Climate Dataset (V3.0) in the study area and that taken at the Hongyuan station, we found that the errors of monthly precipitation and temperature, at 1-km resolution, were 0.78 mm and 0.4°C, respectively, which accounted for 0.1% of annual precipitation and 13.8% of average annual temperature, respectively (Figure 11a, b). Therefore, asynchronous errors of the meteorological data can be ignored due to the distance between study sites and the meteorological station.

The calculation method of the actual evapotranspiration ETc is the P-M formula proposed by the FAO56 Drainage and Irrigation Manual (Allen et al., 1998), which takes the underlying surface area of various types as the weight factor. This method has been widely used for various river basins and regions around the world (Abtew et al., 2011; Jacobs, 2002; Li et al., 2020; Lott and Hunt, 2001; Mao, 2002). The calculation of water surface evaporation involved in this method mainly considers the E601 evaporation pan data as the standard water surface evaporation amount. Thus, minor errors are inevitable, but the results are close to the actual situation of water surface evaporation. In addition, two small peatland watersheds are surrounded by hillslopes, such that there is likely an uneven absorption of solar radiation, as well as uneven vertical and horizontal distributions of vegetation cover. Although the assumptions of the P-M method cannot be fully met, it may still be feasible to estimate the actual evapotranspiration by the P-M method in two remote peatlands.

This study used AWLLs to monitor GWLchange, of which groundwater level data were regularly downloaded during our field work every summer. Although the data of GWL are reliable, the spatial distribution of AWLLs is still sparse at the watershed scale. In order to examine potential errors of the AWLLs in actual situations, we checked the error between the actual water level and water level monitoring value, which is 2.1 mm (Figure 11c). Thus, the GWL monitoring method in this study is credible.

Figure 11  Evaluation of possible errors of meteorological data caused by the distance  (a) Data error between MWX and precipitation in Hongyuan County Station  (b) Air temperature data error between MWX and Hongyuan County Station (red dotted line is the linear fitting result of data)  (c) Error comparison between monitoring groundwater level and actual water level in S1 of W2.

## 4.2 The impact of the rainfall and gully on groundwater level change

The rainfall data used in the watershed is the hourly precipitation recorded at the Hongyuan Station. Owing to the inconsistency of local weather condition, we observed a lag time between the meteorological station and the monitoring sites. Moreover, it takes some time for the peat layer to seep down water from rainfall, resulting in another lag impact on the GWL. The typical period with large groundwater level change at each observed site was selected and classified in W1 according to the rainfall intensity. Statistical results of the lag time on GWL and rainfall intensity are shown in Figure 12. As the rainfall intensity increases, the lag time decreases accordingly. Moreover, the observed sites far from the gully (P6-P8) have a lag time 18 min shorter than that of the observed site close to the gully (P1-P5). The observed site near the gully can be lateral replenished by the water flow of the gully, and thus shortening the lag time.

The three observed sites (P6-P8) distant from the gully responded more rapidly to rainfall than those sites (P1-P5) close to the gully, and the change of GWL in the former with rainfall had a higher correlation coefficient than that in the latter. Compared with the GWL changes of the eight observed sites (P1-P8) during heavy rain and no rain periods, the GWL of P1 closest to the gully that cuts through the peat layer had a greater hourly average increase rate of 6.8 cm/h, and the other four observed sites (P2-P5) increased at 2.3 cm/h, higher than the three observed sites (P6-P8) away from the gully.

Figure 12  Box diagram of lag duration—rainfall intensity in W1 (a) near the gully (b) far away from the gully.

The reasons for the difference of the spatial change of GWLs are mainly summarized: (1) Whether the water level logger is close to the gully (Figures 13 and 14). P1 and P2 is located in the main gully and tributary gully, so the amplitude and variance of GWL change are different. The mean and std of P1 and P2 is -69.8 mm and 16.3 mm, -47.5 mm and 7.9 mm. Difference 1 shows that P1 and P3-P5 have synchronous response to precipitation, but Difference 2 indicates that GWL in P2 is more stable relative to P3-P5. Average values of P6 and P7 are not impacted by the gully, so their change reflects the GWL regime in the intact peatland. Difference 3 mainly displays the impact of the gully on the GWL change. Difference 4 is much higher than Difference 3, indirectly displaying that the gully significantly decreases the GWL in the gully cutting through the peat. The drainage capacity far away from the gully is less than that of the observed sites near the gully, so the GWL near the gully is not less affected by the rainfall (Hikouei et al., 2023; Li and Gao, 2019). Because part of the water flows into the gully through the surface runoff and the perimeter of the peatland or hillslope, the GWL change is not synchronous to the change of the rainfall (Cooper et al., 2019; Levison et al., 2014).

Figure 13  Comparison of GWL (a) Difference 1 is equal to the difference of average P1 (CT) minus average P3, P4 and P5 (NCT). Difference 2 is equal to the difference of average P2 (CT) minus average P3, P4 and P5 (NCT); (b) Difference 3 is equal to the difference of average P1 minus average P6 and P7 (DFG). Difference 4 is equal to the difference of average P3, P4 and P5 minus average P6 and P7 (DFG).

Figure 14  Correlation between accumulative precipitation and GWL increment in each rainy period in comparison with P1 near the gully and P7 far from the gully in W1.

The reasons for the difference of the spatial change of groundwater level are mainly summarized:

(i) Whether the water level logger is placed close to the gully cutting through the peat layer (e.g., P1), in which the drainage capacity is stronger than that in the gully non-cutting through the peat layer and far from the gully (e.g., P7) (Hikouei et al., 2023; Regan et al., 2019). For example, in the rainy periods, the rate of GWL increment in P1 induced by accumulative precipitation is greater than that in P7, also indicating that the former more sensitive to the rainfall than the latter (Figure 14). Moreover, the GWL near the gully cutting through the peat layer shows a strong distance attenuation effect, i.e., the closer the gully, the greater the GWL drops.

(ii) Local topography has an influence on the GWL. The change of GWL in the depression (P6-P7) is less affected by the rainfall than that in the hillslope (P8) because the terrain elevation around the depression is higher (Cooper et al., 2019; Regan et al., 2019), and only changes significantly when the rainfall is large.

(iii) Whether there is headward incision in the setting position of the AWLLs. If the gully is experiencing an headward incision (P8 and S2), the GWL is likely to change more dramatically because the headward incision is easy to cause the gravity erosion affecting the change of the GWL (Li and Gao, 2019; Li et al., 2019).

## 4.3 Effect of groundwater level variation on water storage change and the implications

Peatland is the main wetland type of the SRYR, in which variations of water storage is a key indicator of the availability of water resources (Jia et al., 2022). For a given peatland watershed, water storage and its change in the intra- and inter-annual scale are primarily affected by fluctuations of GWL. In comparison, the spatiotemporal variation of water storage is related to soil moisture in the dryland mountain (Xu et al., 2022), vegetation coverage in Loess Plateau (Jia et al., 2022; Wang et al., 2021), and climate change in the Dongting Lake wetland (Hu et al., 2020; Hu et al., 2021). For the Zoige Basin, besides the effects of rainfall, evapotranspiration and infiltration, groundwater level in peatlands correlated negatively with gully headward erosion and human activities (e.g., artificial ditches) within the watershed (Li et al., 2018a; Li and Gao, 2019; Li et al., 2015). On the one hand, gully headward erosion a vital hydraulic processes that cuts through peat layers, lowers groundwater levels, increases lateral drainage, and drains peatlands (Li and Gao, 2019; Li and Gao, 2020). On the other hand, there are nearly 1,400 artificial ditches drainage in the Zoige peatland, with a total length of 1,800 km. The increasing hydraulic erosion has a particular impact on the pore flow and groundwater level in the peatland, thereby affecting water storage capacity and reducing water storage amount (Li et al., 2019). Owing to the impact of the declined water storage in the Zoige peatlands, the annual water supply to the Upper Yellow River from the Zoige Basin has also decreased at a rate of about 50 million m3/yr, resulting in a downward trend in recent years in the SRYR (Tian et al., 2019; Wang et al., 2018; Wu et al., 2018b). Thus, the reduction of water storage in the Zoige Basin not only changes the seasonal feature of groundwater level, but also accelerates the degradation of peatlands in the SRYR.

# 5. Conclusions

This study performed an in-situ monitoring work of groundwater level (GWL) in two different peatland watersheds (W1 and W2) of the Zoige Basin in the Source Region of the Yellow River during 2017-2021. We find that the change of GWL in sites distant from the gully tends to be synchronized with the rainfall, whereas the GWL in sites near the gully is affected more by gully drainage. Also, the unsaturation duration near the gully is shorter than that away from the gully because the gully drainage provides a longer water replenishment time for the surrounding peat layer. The GWL near the gully that cuts through the peat layer is lower than those near the gully that does not cut through the peat layer. Because rainfall has a lag impact on the GWL, with the increase of the rainfall intensity, the lag effect gradually decreases, i.e., the lag time far away from the gully about 18 min shorter than that close to the gully in the W1.

Furthermore, we find that the intra-annual change of GWL correlates positively with precipitation change in the W2. The GWL difference of the upstream observation point is lower than that of the downstream. The minimum and maximum values of GWL appear in the month with the lowest and highest precipitation, respectively. The GWL rises with increasing RPE, indicating that rainfall has the major influence on GWL change in the short time scale. The GWL recorded at the monitoring sites exhibit a strong discrepancy between wet and dry seasons in the W2. While both seasons show increasing trends of GWL towards the upstream direction, their differences are smaller in the upstream sites due to the limited gully incision and the stronger groundwater storage within the peat layer. Similarly, the GWL recorded in days with rainfall are significantly higher than days without rainfall. Although rainy days dominate the GWL change in wet and dry seasons, the different rainfall intensity led a stable groundwater level in the dry season but an oscillating pattern in the wet season. Consequently, unraveling the groundwater level variation and gully impacts in peatland watersheds is beneficial for understanding peatland hydrology in the Zoige Basin.

As a critical water source for the Yellow River Basin, SRYR provides a water storage function that is of great significance for wetland protection, water resource regulation and utilization, and enhancement of water storage capacity. Nonetheless, the evolution of GWL-driven water storage in peatlands involves highly complex hydrogeomorphic and ecohydrological processes. Recently, ecological protection and high-quality development of the Yellow River Basin has become a major national development strategy, which demands higher-level requirements for the improvement of water storage in the SRYR. This study provides a primary set of data for evaluating the complex processes embranced in groundwater variation.

Acknowledgements：

This study was supported by the National Natural Science Foundation of China (U2243214, 51979012). We thank Hanyuan Yang, Hanyou Lu, Jie Wen, Hangyi Zhou, and Xupeng Wang for their assistance in field survey during 2017-2021.

References:

Abtew, W., Obeysekera, J. and Iricanin, N., 2011. Pan evaporation and potential evapotranspiration trends in South Florida. Hydrological Processes, 25(6): 958-969.

Allen, R.G., Pereira, L.S., Raes, D. and Smith, M., 1998. Crop Evapotranspiration-Guidelines for Computing Crop Water Requirements.

Chen, H., Yang, G., Peng, C., Zhang, Y., Zhu, D., Zhu, Q., Hu, J., Wang, M., Zhan, W., Zhu, E., Bai, Z., Li, W., Wu, N., Wang, Y., Gao, Y., Tian, J., Kang, X., Zhao, X. and Wu, J., 2014. The carbon stock of alpine peatlands on the Qinghai–Tibetan Plateau during the Holocene and their future fate. Quaternary Science Reviews, 95: 151-158.

Cooper, D.J., Sueltenfuss, J., Oyague, E., Yager, K., Slayback, D., Caballero, E.M.C., Argollo, J. and Mark, B.G., 2019. Drivers of peatland water table dynamics in the central Andes, Bolivia and Peru. Hydrological Processes.

Dixon, S.J., Kettridge, N., Moore, P.A., Devito, K.J., Tilak, A.S., Petrone, R.M., Mendoza, C.A. and Waddington, J.M., 2017. Peat depth as a control on moss water availability under evaporative stress. Hydrological Processes, 31(23): 4107-4121.

Evans, C.D., Peacock, M., Baird, A.J., Artz, R.R.E., Burden, A., Callaghan, N., Chapman, P.J., Cooper, H.M., Coyle, M., Craig, E., Cumming, A., Dixon, S., Gauci, V., Grayson, R.P., Helfter, C., Heppell, C.M., Holden, J., Jones, D.L., Kaduk, J., Levy, P., Matthews, R., McNamara, N.P., Misselbrook, T., Oakley, S., Page, S.E., Rayment, M., Ridley, L.M., Stanley, K.M., Williamson, J.L., Worrall, F. and Morrison, R., 2021. Overriding water table control on managed peatland greenhouse gas emissions. Nature, 593(7860): 548-552.

Fenner, N. and Freeman, C., 2011. Drought-induced carbon loss in peatlands. Nature Geoscience, 4(12): 895-900.

Fossey, M. and Rousseau, A.N., 2016. Assessing the long-term hydrological services provided by wetlands under changing climate conditions: A case study approach of a Canadian watershed. Journal of Hydrology, 541: 1287-1302.

Gunther, A., Barthelmes, A., Huth, V., Joosten, H., Jurasinski, G., Koebsch, F. and Couwenberg, J., 2020. Prompt rewetting of drained peatlands reduces climate warming despite methane emissions. Nature Communications, 11(1): 1644.

Hare, D.K., Boutt, D.F., Clement, W.P., Hatch, C.E., Davenport, G. and Hackman, A., 2017. Hydrogeological controls on spatial patterns of groundwater discharge in peatlands. Hydrology and Earth System Sciences, 21(12): 6031-6048.

Hikouei, I.S., Eshleman, K.N., Saharjo, B.H., Graham, L.L.B., Applegate, G. and Cochrane, M.A., 2023. Using machine learning algorithms to predict groundwater levels in Indonesian tropical peatlands. Science of the Total Environment, 857: 159701.

Holden, J., Evans, M.G., Burt, T.P. and Horton, M., 2006. Impact of land drainage on peatland hydrology. Journal of Environment Quality, 35(5): 1764-78.

Holden, J., Wallage, Z.E., Lane, S.N. and McDonald, A.T., 2011. Water table dynamics in undisturbed, drained and restored blanket peat. Journal of Hydrology, 402(1-2): 103-114.

Hu, W., Li, G., Gao, Z., Jia, G., Wang, Z. and Li, Y., 2020. Assessment of the impact of the Poplar Ecological Retreat Project on water conservation in the Dongting Lake wetland region using the InVEST model. Science of the Total Environment, 733: 139423.

Hu, W., Li, G. and Li, Z., 2021. Spatial and temporal evolution characteristics of the water conservation function and its driving factors in regional lake wetlands—Two types of homogeneous lakes as examples. Ecological Indicators, 130: 108069.

Huang, Y., Ciais, P., Luo, Y., Zhu, D., Wang, Y., Qiu, C., Goll, D.S., Guenet, B., Makowski, D., De Graaf, I., Leifeld, J., Kwon, M.J., Hu, J. and Qu, L., 2021. Tradeoff of CO2 and CH4 emissions from global peatlands under water-table drawdown. Nature Climate Change.

Ishii, Y., Koizumi, K., Fukami, H., Yamamoto, K., Takahashi, H., Limin, S.H., Kusin, K., Usup, A. and Susilo, G.E., 2016. Groundwater in Peatland. Tropical Peatland Ecosystems, Chapter 17. Springer, Japan, 265-279 pp.

Ishikura, K., Yamada, H., Toma, Y., Takakai, F., Morishita, T., Darung, U., Limin, A., Limin, S.H. and Hatano, R., 2017. Effect of groundwater level fluctuation on soil respiration rate of tropical peatland in Central Kalimantan, Indonesia. Soil Science and Plant Nutrition, 63(1): 1-13.

Jacobs, J.M., Mergelsberg Shannon L., Lopera Anderes F., Myers David A., 2002. Evapotranspiration from a wet prairie wetland under drought conditions: Paynes prairie preserve, Florida, USA. Wetlands, 22(2): 374-385.

Jia, G., Hu, W., Zhang, B., Li, G., Shen, S., Gao, Z. and Li, Y., 2022. Assessing impacts of the Ecological Retreat project on water conservation in the Yellow River Basin. Science of the Total Environment, 828: 154483.

Levison, J., Larocque, M., Fournier, V., Gagné, S., Pellerin, S. and Ouellet, M.A., 2014. Dynamics of a headwater system and peatland under current conditions and with climate change. Hydrological Processes, 28(17): 4808-4822.

Li, B., Yu, Z., Liang, Z., Song, K., Li, H., Wang, Y., Zhang, W. and Acharya, K., 2014. Effects of climate variations and human activities on runoff in the Zoige alpine wetland in the eastern Edge of the Tibetan Plateau. Journal of Hydrologic Engineering, 19(5): 1026-1035.

Li, C., Grayson, R., Holden, J. and Li, P., 2018a. Erosion in peatlands: Recent research progress and future directions. Earth-Science Reviews, 185: 870-886.

Li, M., Liang, D., Xia, J., Song, J., Cheng, D., Wu, J., Cao, Y., Sun, H. and Li, Q., 2021. Evaluation of water conservation function of Danjiang River Basin in Qinling Mountains, China based on InVEST model. J Environ Manage, 286: 112212.

Li, Z.W. and Gao, P., 2019. Impact of natural gullies on groundwater hydrology in the Zoige peatland, China. Journal of Hydrology: Regional Studies, 21: 25-39.

Li, Z.W. and Gao, P., 2020. Characterizing spatially variable water table depths in a disturbed Zoige peatland watershed. Journal of Hydro-environment Research, 29: 70-79.

Li, Z.W., Gao, P., Hu, X.Y., Yi, Y.J., Pan, B.Z. and You, Y.C., 2020. Coupled impact of decadal precipitation and evapotranspiration on peatland degradation in the Zoige basin, China. Physical Geography, 41(2): 145-168.

Li, Z.W., Gao, P. and Lu, H.Y., 2019. Dynamic changes of groundwater storage and flows in a disturbed alpine peatland under variable climatic conditions. Journal of Hydrology, 575: 557-568.

Li, Z.W., Gao, P. and You, Y.C., 2018b. Characterizing hydrological connectivity of artificial ditches in Zoige peatlands of Qinghai-Tibet Plateau. Water, 10(10): 1364.

Li, Z.W., Wang, Z.Y., Brierley, G., Nicoll, T., Pan, B.Z. and Li, Y.F., 2015. Shrinkage of the Ruoergai Swamp and changes to landscape connectivity, Qinghai-Tibet Plateau. Catena, 126: 155-163.

Liu, L., Chen, H., JIang, L., Hu, J., Zhan, W., He, Y., Zhu, D., Zhong, Q. and Yang, G., 2018. Water table drawdown reshapes soil physicochemical characteristics in Zoige peatlands. Catena, 170: 119-128.

Lott, R.B. and Hunt, R.J., 2001. Estimating evapotranspiration in natural and constructed wetlands. Wetlands, 21(4): 614-628.

Lu, H.Y., Li, Z.W., Hu, X.Y. and Yu, G.A., 2019. Estimation of runoff change and water storage in Zoige Plateau. Journal of Water Resources & Water Engineering, 30(6): 12-19.

Mao, L.M., Bergman, M.J., Tai, C.C>, 2002. Evapotranspiration measurement and estimation of three wetland environments in the Upper St. Johns River Basin, Florida. Journal of the American Water Resources Association, 38(5): 1271-1285.

Mercer, J.J. and Westbrook, C.J., 2016. Ultrahigh‐resolution mapping of peatland microform using ground‐based structure from motion with multiview stereo. Journal of Geophysical Research: Biogeoscience, 121: 2901-2916.

Price, J.S., Healthwaite, A.L. and Baird, A.J., 2003. Hydrological processes in abandoned and restored peatlands. Weland Ecology and Management, 11: 65-83.

Rahman, M.M., McDermid, G.J., Strack, M. and Lovitt, J., 2017. A new method to map groundwater table in peatlands using Unmanned Aerial Vehicles. Remote Sensing, 9(10): 1057.

Ramchunder, S.J., Brown, L.E. and Holden, J., 2009. Environmental effects of drainage, drain-blocking and prescribed vegetation burning in UK upland peatlands. Progress in Physical Geography: Earth and Environment, 33(1): 49-79.

Ratcliffe, J.L., Peng, H., Nijp, J.J. and Nilsson, M.B., 2021. Lateral expansion of northern peatlands calls into question a 1,055 GtC estimate of carbon storage. Nature Geoscience, 14(7): 468-469.

Regan, S., Flynn, R., Gill, L., Naughton, O. and Johnston, P., 2019. Impacts of groundwater drainage on peatland subsidence and its ecological implications on an Atlantic raised bog. Water Resources Research, 55(7): 6153-6168.

Sheng, Y., Ma, S., Cao, W. and Wu, J., 2020. Spatiotemporal changes of permafrost in the Headwater Area of the Yellow River under a changing climate. Land Degradation & Development, 31(1): 133-152.

Tian, S., Xu, M., Jiang, E., Wang, G., Hu, H. and Liu, X., 2019. Temporal variations of runoff and sediment load in the upper Yellow River, China. Journal of Hydrology, 568: 46-56.

Turetsky, M.R., Benscoter, B., Page, S., Rein, G., van der Werf, G.R. and Watts, A., 2014. Global vulnerability of peatlands to fire and carbon loss. Nature Geoscience, 8(1): 11-14.

Wang, J., Wu, T., Li, Q. and Wang, S., 2021. Quantifying the effect of environmental drivers on water conservation variation in the eastern Loess Plateau, China. Ecological Indicators, 125: 107493.

Wang, T.H., Yang, H.B., Yang, D.W., Qin, Y. and Wang, Y.H., 2018. Quantifying the streamflow response to frozen ground degradation in the source region of the Yellow River within the Budyko framework. Journal of Hydrology, 558: 301-313.

Wilson, L., Wilson, J., Holden, J., Johnstone, I., Armstrong, A. and Morris, M., 2010. Recovery of water tables in Welsh blanket bog after drain blocking: Discharge rates, time scales and the influence of local conditions. Journal of Hydrology, 391(3-4): 377-386.

Wu, C., Chen, W., Cao, C., Tian, R., Liu, D. and Bao, D., 2018a. Diagnosis of wetland ecosystem health in the Zoige Wetland, Sichuan of China. Wetlands, 38(3): 469-484.

Wu, S., Zhou, W., Yan, K. and Zhang, X., 2020. Response of the water conservation fnction to vegetation dynamics in the Qinghai–Tibetan Plateau based on MODIS products. IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, 13: 1675-1686.

Wu, X.L., Zhang, X., Xiang, X.H., Zhang, K., Jin, H.J., Chen, X., Wang, C.H., Shao, Q.Q. and Hua, W.J., 2018b. Changing runoff generation in the source area of the Yellow River: Mechanisms, seasonal patterns and trends. Cold Regions Science and Technology, 155: 58-68.

Xu, H. J., Zhao, C.Y., Wang, X.P., Chen, S.Y., Shan, S.Y., Chen, T. and Qi, X.L, 2022. Spatial differentiation of determinants for water conservation dynamics in a dryland mountain. Journal of Cleaner Production, 362: 132574.

Xue, J., Li, Z., Feng, Q., Gui, J. and Zhang, B., 2022. Spatiotemporal variations of water conservation and its influencing factors in ecological barrier region, Qinghai-Tibet Plateau. Journal of Hydrology: Regional Studies, 42: 101164.

Yao, T., Bolch, T., Chen, D., Gao, J., Immerzeel, W., Piao, S., Su, F., Thompson, L., Wada, Y., Wang, L., Wang, T., Wu, G., Xu, B., Yang, W., Zhang, G. and Zhao, P., 2022. The imbalance of the Asian water tower. Nature Reviews Earth & Environment. 10.1038/s43017-022-00299-4

Young, D.M., Baird, A.J., Morris, P.J. and Holden, J., 2017. Simulating the long-term impacts of drainage and restoration on the ecohydrology of peatlands. Water Resources Research, 53(8): 6510-6522.

Zhang, F., Zeng, C., Wang, G., Wang, L. and Shi, X., 2021. Runoff and sediment yield in relation to precipitation, temperature and glaciers on the Tibetan Plateau. International Soil and Water Conservation Research.

Zhang, S., Hua, D., Meng, X. and Zhang, Y., 2011. Climate change and its driving effect on the runoff in the “Three-River Headwaters” region. Journal of Geographical Sciences, 21(6): 963-978.

Zhang, W., Yi, Y., Song, K., Kimball, J. and Lu, Q., 2016. Hydrological response of alpine wetlands to climate warming in the eastern Tibetan Plateau. Remote Sensing, 8(4): 336.

Zheng, H., Zhang, L., Liu, C., Shao, Q. and Fukushima, Y., 2007. Changes in stream flow regime in headwater catchments of the Yellow River basin since the 1950s. Hydrological Processes, 21(7): 886-893.

Figure caption

Figure 1  Geographic setting of two studied peatland watersheds (W1 and W2). (a) Location of the Zoige Basin (outlined in red) within the Qinghai-Tibet Plateau; (b) Distributions of peatland (gray shaded areas) and major river networks within the Zoige Basin (WR: White River; BR: Black River) and two studied watersheds; (c) Locations of 8 installed AWLLs along the gully G1 in the W1; (d) Locations of 7 installed AWLLs along the gully in the W2.

Figure 2  (a) A photo of the gully with cutting through the peat in W2; (b) A photo of the gully with non-cutting through the peat in W2; (c) Incision depths and groundwater levels of S1-S7 sites in W2 based on the measured data on July 27, 2021; (d) A schematic diagram a photo show the installing method of water level logger.

Figure 3  Variations of precipitation and calculated evapotranspiration at Hongyuan Meteorological Station  (a) Daily values from May 8 to September 18 2017 in W1, (b) Daily values from May 1 2018 to May 31 2021 in W2.

Figure 4  Groundwater level and rainfall of the five monitoring points (P1-P5) near the gully in the W1.

Figure 5  Groundwater level and precipitation of P6, P7, and P8 far away from the gully in the W1.

Figure 6  Groundwater level in day and night scale (a-e) near the gully (P1-P5)  (f-h) far away the gully (P6-P8) (i) Daily precipitation from May 16 to September 16, 2017.

Figure 7  Monthly variations of the mean groundwater level in S2-S7 and precipitation during T1w2-T3w2.

Figure 8 Temporal changes of groundwater level in three different modes and its response to precipitation and evapotranspiration. (a) Patterns of groundwater water level and RPE; (b) Patterns of CV of average monthly groundwater level in S2-S7.

Figure 9  Mean groundwater levels and their differences between wet and dry seasons from S2 to S7 for the periods of (a) May 2018-April 2019; (b) May 2019-April 2020; (c) May 2020-April 2021.

Figure 10  Groundwater level change and trend over time in non-rain and rain days from S2 to S7 in 2018-2021 in the W2.

Figure 11  Evaluation of possible errors of meteorological data caused by the distance  (a) Data error between MWX and precipitation in Hongyuan County Station  (b) Air temperature data error between MWX and Hongyuan County Station (red dotted line is the linear fitting result of data)  (c) Error comparison between monitoring groundwater level and actual water level in S1 of W2.

Figure 12  Box diagram of lag duration—rainfall intensity in W1 (a) near the gully (b) far away from the gully.

Figure 13  Comparison of GWL (a) Difference 1 is equal to the difference of average P1 (CT) minus average P3, P4 and P5 (NCT). Difference 2 is equal to the difference of average P2 (CT) minus average P3, P4 and P5 (NCT); (b) Difference 3 is equal to the difference of average P1 minus average P6 and P7 (DFG). Difference 4 is equal to the difference of average P3, P4 and P5 minus average P6 and P7 (DFG).

Figure 14  Correlation between accumulative precipitation and GWL increment in each rainy period in comparison with P1 near the gully and P7 far from the gully in W1.