Deep Reinforcement Learning Based Algorithm for Symbiotic Radio IoT Throughput Optimization in 6G Network

Gergs M. Salma 1    Samar Shaker Metwaly 2*    E. G. Shehata 3    Ahmed. M. Abd El-Haleem 4

1 Electronic and Communication Engineering                  Abstract

Dept., Minia University, 61511, Minia, Egypt

Internet of Things (IoT) based 6G is expected to revolutionize our

world. various candidate technologies have been proposed to meet

3 Electric Engineering Dept., 61511, Minia                    the requirements of IoT systems based on 6G, symbiotic radio (SR)

University, Minia, Egypt.                                               is one of these technologies. This paper aims to use symbiotic radio

technologies to support the passive internet of things and enhance

4 Electronic and Communication Engineering.               the uplink transmission performance. In SR the IoT tag is parasitic

Dept., Helwan University, 11722, Cairo, Egypt            on the primary transmission shares not only the radio spectrum of

the primary transmission, but also the power, and infrastructure of

Correspondence                                                             the neighbor smartphone primary system which enhances the

Samar Shaker Metwaly, Electronic and                         spectrum and energy efficiency of the system. Then the IoT tags

Communication Engineering Dept.                                 information is sent to cloud for analysis through the Macro base

Helwan University, 11722, Cairo, Egypt.                             station MBS or the wireless access point WAP where the smartphones

Email: samarshaker2904@ h-eng.helwan.edu.eg           are used as a relay to transmit this information to the MBS or WAP.

In this paper two optimization problems are formulated to maximize

the total throughput of the system. First, a problem of achieving the

optimum mode selection of LTE or Wi-Fi Network (MBS or WAP)

by transmitting an expected tags information load from the smartphone

to MBS or WAP aiming to maximize the system throughput. The

matching game algorithm is utilized to solve this problem. Second, a

problem of achieving optimum clustering of tags where, the tags are

divided into virtual clusters and finding which smartphones’ LTE/

Wi-Fi downlink signal all cluster members can ride to maximize the

system throughput. A deep Q- network (DDQN) model is proposed for

solving this optimization problem with low complexity. Simulation results

show that our proposed algorithms increase the total system data rate

by average 90% above the system by using LTE network first and 20%

above the system without using DDQL algorithm. Furthermore, our

proposed algorithms enhance the capacity of the system on the average

by 100% above system using LTE network first without DDQL

algorithm.

Keywords—IoT, DDQL, LTE, Wi-Fi, matching game, backscattering,

symbiotic radio

1. Introduction

Internet of Things (IoT) is considered one of the main applications of the future wireless networks which enables devices to

communicate with each other [1]. IoT devices significantly grow so substantial amounts of spectrum resources and energy are required [2]. It was a challenge for 5G wireless systems to satisfy the requirements of IoT. 5G has introduced some technologies for the IoT but cannot fully satisfy the IoT requirements. So, 6G is a promising generation of wireless communication which is expected to overcome the 5G in enhancing the performance of these systems. Many 6G promising technologies are proposed as artificial intelligence (AI), and symbiotic radio(SR). These technologies are expected to achieve the requirements of the IoT system such as enhancing the system rate, capacity, and energy efficiency where the IoT system has a low downlink data rate of 250 kb/s and an uplink data rate of 15 kb/s [3-5]. AI is a critical and highly valued technology in the future’s 6G where it provides intelligence to wireless networks. AI is used as a tool for system data analysis which helps in making correct decisions automatically [6], [7].

Also, Backscattering (BackCom) is one of the technologies which is used to get better wireless system performance. Backscattering provides spectrum resources and energy for the IoT tags which improve the system spectrum and energy efficiency. Backscattering is presented in the LTE (LScatter) and Wi-Fi (BackFi) frequency bands where the tag’s information is ridden and modulated over continuous wave signals generated by a carrier emitter [8]. Recently, ambient backscatter communication (AmBC) system is used for the passive internet of things which doesn’t need a dedicated carrier emitter. The ambient system enables the backscattering tags to transmit their information over the existing ambient RF signal (e.g. Cellular or Wi-Fi signal) that shares the transmitter and spectrum of the primary signal but it has some decoding troubles at the reader [9]. So, symbiotic radio SR is used for passive internet of things communication which is one of 6G promising technologies. SR provides highly reliable backscattering communications which make up for the disadvantage of the AmBC. In SR the IoT tags not only share the transmitter and spectrum of the primary signal but also share the primary receiver. the primary receiver decodes the signals from the primary transmitter and also the backscattering tags [10]. So, using SR can help significantly in enhancing the performance of IoT system where the mutualism spectrum sharing increase the system spectrum efficiency and the backscattering communication increase the energy efficiency of it and the decoding at the primary receiver by utilizing successive interference cancellation (SIC) can solve the decoding troubles and provide a reliable system. The SIC technique is used to firstly, estimate the primary signal. Then the receiver subtracts this estimated signal from the received signal and finally, detects the tags' backscattered signal.

## 1.1. Related Work

In this section, many researches are introduced which try to improve the IoT performance by maximizing throughput, capacity, and energy efficiency using different 6G techniques. In [11] backscattering (BackCom) is introduced as an emerging technique in communication systems. backscatter technique and tag-power harvest are coupled which enhances the spectrum and energy efficiency of the system. In [12], [13] expose ambient backscatter communication (AMBC) technique that enables the smart tags to communicate by using ambient radio frequency (RF) signals without needing active RF transmission. IoT systems can benefit from this technique to enhance their performance. In [14] AMBC is integrated into the RF-powered underlay CR-NOMA networks which enable the secondary transmitters to utilize the harvested energy for simultaneously data transmission depending on power-domain NOMA. The RF-powered underlay CR-NOMA networks can increase the secondary system throughput. Although, the promising aspect of ambient backscattering success in enhance the system performance it has a detection problem at the reader, where the ambient backscatter system detects the other systems’ signals, which are strangers to the reader, and the channel parameters cannot be estimated by the reader.

Other researches support the symbiotic radio (SR) technique for the communication system. So in [15], [16] SR is introduced and the detection is done at the primary receiver using successive interference cancellation (SIC) which detects the primary signal first and then subtracts this signal from the received signal and decode the backscattered signal. SR can improve the IoT performance more than BackCom and AMBC. In [17] provides an analysis of the downlink of a NOMA-enabled cellular system batched with symbiotic radio then, derives the rate for the symbiotic radio system and formulates an expression of the outage probability of the signal-to-interference and noise ratio. In [18] three practical transmission schemes are proposed with different symbiotic relationships between the backscatter and primary systems. Then The achieved rates for the transmissions of the primary and backscatter systems can be derived for fading state and an optimal power allocation is acquired for the transmission scheme.

## 1.2 Contribution

Our work aims to enhance the total uplink IoT system rate to contribute toward the growing of IoT devices (Massive IoT) which requires better system performance. The problem is proposed to be solved into two optimization problems which are solved in series. In the first problem solution, many-to-one matching game is used to select the network (LTE or Wi-Fi) for information transmission where, the smartphone act as a relay to deliver the decoded IoT tags’ information to the MBS or WAP which maximizes the overall system rate. In the second problem solution, the SR technique is used to enable IoT tags to reflect their information to the smartphones where backscatter their signal with the smart phones and then the IoT tags’ information is decoded on smartphones to be transmitted to the MBS or WAP. IoT tags are divided virtually into optimum clusters by using the DDQL algorithm. Each cluster tags’ information is backscattered on one smartphone through (LTE or Wi-Fi) network. So, the contribution of our paper are:

- A symbiotic radio communication technique is proposed to enhance the throughput of the IoT system.

- Two optimization problem is considered to maximize the total uplink rate of IoT system.

- A Many-to-one matching game algorithm is used to solve the first problem by selecting the network (LTE or                     Wi-Fi) for transmission of the smartphone decoded information which maximize the system rate.

- DDQL algorithm is utilized to solve the second optimization problem by achieving the optimum tags clusters which backscatter on each smartphone signals.

1.3 Organization of This Paper

The rest of this paper is organized as follows. In section 2 We present the proposed system model. In section 3 we formulate the throughput maximization problem of the system. In section 4 we present the matching game algorithm and then explain the details of the proposed algorithm. In section 5 we explain the theoretical description of the double deep reinforcement learning algorithm and the proposed algorithm  details. In section, 6 presents the network parameter. section 7 introduce the parameters of the network. finally, conclude this paper in section 8.

# 2. System Model

Consider a symbiotic radio communication system of K IoT tags which is represented by К ={1,2,.….,  K}, M smart phones are represented by M={1,2,.…., M} and N Wi-Fi access point N= {1,2,.….,N} under coverage of a microcell base station (MBS) as shown in Figure 1. The IoT tags are backscattered on the downlink primary signal from MBS or WAP and reflect their information to the smartphones by using the SR technique to enhance the system performance by increase the spectrum and energy efficiency of the system where it shares the primary signal transmitter, receiver, and the spectrum resource. Then the smartphone is used as a relay that decode the tags information by using SIC technique and transmit it to MBS through LTE network or WAP through the Wi-Fi network to reach the data cloud. The data cloud is an important part of IoT network where it is responsible for the tags information analysis and computing to control the system tags. The IoT tags are distributed virtually into clusters, each cluster tags are allowed to backscatter their information to one of the neighbor smartphones. The optimum cluster is the cluster in which its members can achieve the maximum total rate. Then these tags information is decoded at the smartphone and transmitted through the network of (LTE or Wi-Fi). The WAPs are operated in a frequency range different from that of LTE MBS. Moreover, Orthogonal Frequency Division Multiple Access (OFDMA) is presumed to be utilized by the LTE network for accessing channel while the WAPs use IEEE 802.11 distributed coordination function (DCF) mechanism which is depended on the Carrier Sense Multiple Access with Collision Avoidance (CSMA/CA) protocol for the channel access. The following sections demonstrate

Figure 1. SR for IoT System Model

the received data rate formulas of the IoT tags information at the smartphone and the data rate of the smartphone received information at MBS or WAP based on OFDMA technique for transmission through the LTE network. And OFDM technique for transmission through Wi-Fi network.

## 2.1 Rate Calculation for the  Smartphone

In the first problem solution, the expected detected tags’ information at the smartphone is associated to the MBS through LTE network or to the WAP through the Wi-Fi network based on the maximum data rate achievement. So, the data rate of the tags’ information signal is calculated for purpose of the network selection for the tags’ information transmission.

Figure  2.  Channels model of SR for IoT system

The SNR of the tags’ information signal that is transmitted from the smartphone m to the MBS can be expressed as,

γmlLTE=pml hmlN0                (1)

Where, pml  is the transmitted power from the smartphone to the MBS, hml is the channel gain between the MBS and the smartphone as explained in Figure 2. Then, the physical data rate of the signal can be determined instead of the simple and theoretical form of Shannon by mapping with the SNR (γLTE) as

RmlLTE =   NfB.Nsu.Nn.Nbx .  ECRxTSf × 1Repx               (2)

Where NfB is the number of  frequency blocks in bandwidth B, Nsuis the number of subcarriers in one Block, Nn is the number of each subcarrier’s symbols, TSf is the one subframe duration,  Repx  is the repetition factor. Furthermore, Nbx is the number of each symbol bits and ECRx is the code rate [19], [20].

For WAP association, the SNR of the tags information signal that is transmitted from the smartphone m to the WAP n through the Wi-Fi band (2.4GHZ) can be expressed as follow,

γmnWi-fi=  pmn hmnN0                 (3)

Denote pmn as the transmitting power of the smartphone m to the WAP n. hmn is the channel between smartphone m and WAP n.  A contention extent may be on the Wi-Fi channel which affects the system throughput. This contention is depending on the number of users associated with WAP, in which a collision between them may occur. So each WAP has maximum capacity; based on its characteristics and the technology used; to avoid the collision. The data rate of the tag information signal at WAP is calculated depend on the enhanced 802.11 DCF back-off model [21] to obtain the channel contention and packet collision effect [22] as,

RmnWi-fi=τ(1-τ)Nw D  T+Dτ(1-τ)NwRmnph                                 (4)

Where τ represents the probability of channel contention considering the CW resetting scheme, Nw denotes the total  number of users associated with the WAP, D  is the maximum allowed size of user packets. T  can be denoted as,

T=(1-τ)Nw+1 e+ (1-1-τ)Nw+1  ( TcTS+TDIFS)+

( Nw+1) τ(1-τ)Nw(TRTS+TACK+3TsIFS)            (5)

Where e is the duration of an empty slot time; TRTS ,TcTS,  TDIFS, TsIFS and TACK  denote the duration of the Request to Send (RTS) short frame, Clear to Send (CTS) short frame, DIFS, SIFS and ACK,  respectively. Rwphis the Physical rate of the transmitted tag’s signal which transmits from the smartphone to the WAP.  Rmkph can be calculated as,

Rmnph  =   Nspatial.Nsub.Nb .  CwTS                                 (6)

Where, Nspatial is the streams’ number; Nsub is the total number of data sub-carriers; TS represents the OFDM symbol duration; Nb is the number of bits per symbol and Cw is the coding rate ; that  Nb Cw and are determined by mapping with the measured SNR  (γmnWi-fi) value at the WAP[23].

## 2.2 Rate Calculation for The IoT tags’ Backscattered Information

As mentioned, the IoT tags are backscatter on the downlink primary signal from the MBS to a smartphone through the LTE band. The received backscattered signal at the smartphone can be represented as,

yspL(t)=hlms(t) +ηgmk hlkxts(t) +N0                  (7)

Where, s(t) is the downlink primary signal of the smartphone and x(t) is the tag information signal. As shown in fig 2  hlm is the channel between the RF source (MBS) and the smartphone m. hlk is the channel between the RF source (MBS) and a tag k. gmk is the channel between the smartphone m and the tag k. N0 denotes the additive white Gaussian noise AWGN. η is the reflection coefficient [17], [24]. The SINR γmkL of tag k signal that backscatter the downlink primary signal of smartphone m can be computed as follow,

γmkL=ηplgmkhl,ki=1klm-1ηgmipl,hl,i+N0                          (8)

Where, pl is the power of the downlink RF signal.  i=1klm-1ηgmipl,hl,i represent the interference caused by other tags klm-1which backscatter the same downlink primary signal between the MBS and the smartphone m. The smartphone detects these backscattering signals using the SIC technique. SIC detects the tag's signal after subtracting the primary signal which provides a reliable system.

Also, IoT tags use the Wi-Fi transmissions from the WAP to a Wi-Fi client as the excitation signal to ride it. The tag receives the Wi-Fi signal and then reflects its information to the smartphone. The received signal at the smartphone through the Wi-Fi frequency band can be expressed as,

yspw(t)=hmnw(t) +ηgmkhnkxtw(t) +N0        (9)

Where, w(t) is the primary Wi-Fi transmitted signal,  hmn is the channel between the smartphone m and WAP n, hnk denote the channel between the WAP n and the IoT tag k. SINR of the received tag’s k backscattered information signal at the smartphone m through the Wi-Fi frequency band (2.4 GHz) can be calculated as,

γmkw=ηpwgmkhnki=1knm-1ηgmi pw,hni+N0                            (10)

Where, pw is the power of the WAP transmitted signal.  i=1knm-1ηgmipw,hni represent the interference caused by other tags knm-1 which backscatter the same primary Wi-Fi signal between the WAP n and the smartphone m [25], [26].

The rate of each tag k ∈К at the smartphone m ∈ M can be calculated by using the Shannon Model where the tag rides the primary signal without any control as,

Rmk=B log2(1+γmk)                           (11)

Where, γmk is the SINR of the tag k which may be γm,kL or γm,kw.

3. Problem Formulation

In this section, the problem of the system is formulated to maximize the overall IoT system throughput which is a contribution for massive IoT devices communication. It is represented as two optimization problems. The first problem p1 (12) can be solved by matching the game algorithm to find the optimum mode selection of the network (LTE or Wi-Fi). The second problem P2 (13) is solved by finding the optimal cluster of tags by using the DDQL algorithm. which satisfies the maximum system throughput.

P1:   max  m ∈Mn∈N (RmlLTE+RmnWi-fi)                   (12)

P2:   max  m∈Mk∈KXmk  Rmk                                  (13)

Subject to

C1:  pml   > plth  and   pmn   > pwth ,

C2 :  γlmL> γmkL  and   γmnw    > γmkw ,

∀  k∈ К  ,   m∈M and  n∈N

C3:  Xmk = {0, 1}

Constraint C1 is to confirm that the smartphone is under the coverage of the MBS and WAP. Constraint C2 is to obtain the condition of perfect SIC. Constrain C3 is a binary value that indicates the backscattering indicator. In the following sections, the DDQL and matching game algorithms will be demonstrated.

## 4. Matching Game Algorithm

The matching game is a low-complexity algorithm; it is one of the algorithms that is used to solve the maximization problems to enhance the system's performance. Matching game has many types, one to one matching game in which one agent from one side of matching is matched with one agent on the other side, many to one matching game in which many agents from one side can be matched with one agent on the other side, and many-to-many matching game in which many agents from one side can be matched with other many agents on their side [27-30].

4.1 Theoretical Description

In this paper, we apply the many-to-one matching game to enable each smartphone to determine the optimum selection of the network LTE or Wi-Fi band which transmits the tags information through it to maximize its signal rate. So, the smart mobile constructs a preference list according to the utility function Utm which can be defined as,

Utm=Rm ,                            (14)

Where, Rm is the smartphone mobile rate which may be  RmlLTE or RmnWi-fi. The utility function that the MBS or WAP is to accept the request for the smartphone maximums the overall system rate, which is calculated as:

Utb  =  RT ,                         (15)

Where, RT  is the total rate of the system. The smart mobile request may not be accepted at the MBS or WAP, in this case, the smartphone tries to select the next choice in its preference list.

4.2 The proposed Algorithm 1

In our proposed algorithm initially, the smartphone calculates its utility function; based on (14), and constructs its preference list. Then it tries to select the frequency band (LTE or Wi-Fi) for the tags information transmission by sending a request to the MBS or WAP according to the first choice in its preference list. The request will be accepted or rejected based on (15) and repeats until achieving stable matching Φ.

Algorithm 1: Many –to-One Matching game

- Discover the tags backscattered information signals at each smart phone and calculate its channel gain through the MBS and WAP.

- Construct the preference list of each smart mobile based on its utility fuction Utm on (14).

- Request to select the frequency band for information signal transmission of each smart mobile according to its preference list.

- Construct MBS and WAP  preference list based on (15) and decied accept or reject each mobile request according to it.

- repeat

The rejected request re-apply to the next choice in its preference list.

- Until

All smart mobiles connect with (MBS or WAP) for signal transmission. A stable matching Φ is achieved.

- Calculate the system rate.

5.  Double Deep Reinforcement Learning Algorithm

The reinforcement learning algorithm is a learning process that agents take an action and wait for the environment's reaction. A reward is provided by the environment to the agent for every action and the RL agent selects the right action which maximizes the reward of the new state. Deep reinforcement learning algorithm (DRL) can be classified into three types:

(I) value-based methods which find a policy by learning only the value function (DQL).

(II) policy-based methods which find the policy directly based on the gradient related to the policy.

(III) actor-critic methods that are a hybrid of the previous two types of value-based and policy-based methods[31].

5.1  Theoretical Description

DQL algorithm is proposed to solve our problem P2. DQL is a multi-layered neural network for a given state st outputs where an agent has a parameters θQ at time t.  Firstly, it takes an action at at state st and then observes the environment's immediate reward rt+1with the resulting state st+1, The DQL update equation is :

Q(s, a| θt+1Q ) = Q(s , a | θtQ ) +

ν rt+1+ ξ maa'  x Q(st+1  , a'  | θtQ ) - Q(st  , at| θtQ )                   (16)

= Q(s, a | θtQ ) + ν rt+1+ ξ ma  a'  xQ(st+1   , argmaa'  x Q( st+1  ,a'  | θtQ )| θtQ) -                                  Q(st  , at| θtQ )                                                    (17)

Where, ν is the learning rate. the learning term maxa'  Q(st+1  , argmaxa'  Q( st+1  ,a'  |θtQ )| θtQ)                                                  represent the systematic overestimation of the Q-value. Double DQL is an algorithm is used to reduce the overestimation. In DDQL two networks  Q and Q'   are learned by assigning each experience randomly to update one of the two networks, that there are two parameters θQ  and θQ'  . In each update, one of these parameters is used to obtain the greedy policy and the other to obtain its target value. DDQL reduces the overestimations by decomposing the max operation into selection and evaluation action a'  ,  a that evaluates the greedy policy according to the target network[32], [33]. The DDQL update equation(bellman) can be represented as,

Q(s , a | θt+1Q ) =Q(s , a | θtQ ) + ν rt+1+ ξ ma    a'  x Q(st+1  , argmaa'  xQ( st+1  ,a'  | θtQ'   )| θtQ) -                       Q(st  , at| θtQ )                                                         (18)

The parameter θQ'  of the  Q'  network can be calculated periodically by using the Polyak averaging method based on parameter τ to represent the parameter θQ  of Q network after time step t0 as[34],[35].

θt+t0Q'  = (1- τ) θtQ'  + τ θtQ                                 (19)

Our goal is to solve the maximization problem of the system throughput by designing a DRL system of virtual tags clusters. we optimize these virtual clusters using DDQL algorithm. Fig 3 explains the block diagram of the DDQL Scheme which is used to solve the problem in (12).

Figure  3 . DDQL model

The design includes the tags virtual clusters state s, the reward r which is given by the environment when taking an action a through the  DDQL algorithm training and reply buffer observation of different actions and then randomly sample from the buffer experience. The state s of the IoT tags K is taken as a Vector of SINR value for each tag in its virtual cluster and the reward is the throughput of the system.

5.2 The proposed Algorithm 2

We start by initializing the environment with one MBS, K IoT tags, M smartphones, and N WAP. At the initial state s0 the input of the neural network is a vector of SINR of the IoT tags based on (8), and (10) regardless it backscatters the smartphone signal through LTE or Wi-Fi band. The DDQL agent takes an action at at every time step t of the episode. The environment sends the immediate reward to the agent and the state will transit to a new state st+1 . The neural network updates the parameter θtQ to θt+1Q (18) to calculate the Q(s, a | θt+1Q ) by using bellman equation (18).

Algorithm 2: DDQL algorithm for IoT tags clustering

- Initialization: Discover each IoT tags, smart phones, WAP location coordinates.

- Initialize the Q network Q(s , a | θtQ ) randomly.

- Initialize the  target network Q'

- θQ'  ⇐     θQ

- Initialize replay buffer R

- for episode = 1……; E do

- initializing the Environment state s0

- calculate the SINR for each tag as an input vector of the neural network.

- for t = 1….. ; T do

Select the clustering action at      argmaax Q( st  ,at  )   Execute action a ,the reward  rt and the new state st+1 are observed

Store transition (st ,at , rt  ,st+1) in Buffer R

Sample a random mini-batch of L transitions from the buffer R.

- Training of DDQL

Select a'     argma a x Q'  ( st+1  ,a'  | θtQ'  )

Update the Q based on bellman equation in (17).

Update the DDQL target network if mod (t, p) = 0

end for

end for

6.  Network Parameters

The proposed algorithm results are evaluated based on table 1 parameters. Moreover, the indoor path loss model between tags and MBS, WAP at LTE and Wi-Fi band respectively are represented (20), (21) as,

pLTE  = 128.1+37.6 log(dL )                       (20)

pWi Fi  = 20 log fw+ηw log dw +pf(Mw) -28        (21)

Where dL is the distance between MBS and the IoT tags; dw is the distance between the WAP and the IoT tags; fw is the frequency band of the Wi-Fi;  ηw is the coefficient of distance power loss and is assumed to be 30; pf(Mw) represents the penetration loss factor and is equal to pf (Mw)= Mw+13; Mw denotes the number of walls and is assumed to be 3 [22].

| Simulation parameters | Value |
| --- | --- |
| The cell radius | 500 m |
| IoT bandwidth (LTE) | 180kHZ |
| Number of subcarriers at 3.75KHZ/subcarrier | 48 subcarriers |
| Additive white Gaussian noise | -174 dBm/HZ |
| Max transmit power at LTE (Pmax) | 23 dBm |
| The number of frequency blocks in bandwidth B | 1 block |
| Wi-Fi technology | 802.11g |
| Max transmit power (Pmax) at Wi-Fi | 200 mw |
| Minimum contention window(W) | 32 |
| Slot time | 9 μs |
| DIFS, SIFS | 50,10 μs |
| RTS,CTS | 208,160 bits |
| ACK | 160 bits |
| Data Payload size (D) | 1500 bytes |

Table 1.  simulation parameters of matching game algorithm.

| Simulation parameters | Value |
| --- | --- |
| State s = {s1,.…. sk } | {γ1,.…. γk} |
| Reward r | Rmk=B log2(1+γmk) |
| Discount factor ξ | 0.99 |
| Learning rate ν | 5*10-5 |
| Buffer size | 20000 |
| Episode, Time step | 2000, 500 |

Table 2. Simulation parameters of DDQL algorithm

Table 1 &2 explains the parameters used for the simulation and calculation of the matching game algorithm [22], [30] and DDQL algorithm respectively.

7.   Performance Evaluation

In order to evaluate our proposed algorithms, the performance of the proposed Matching game algorithm which is used as a solution to the first problem of the system is compared with LTE first association technique and Wi-Fi first association technique. Then DDQL algorithm which is proposed to solve the second problem of the system is compared with the system without using DDQL algorithm that the IoT tags backscatter on the nearest smartphone devices. Furthermore, the performance of the system in terms of data rate and capacity is studied at a different number of smartphones to explain the optimum number of smartphones needed for this system.

Figure 4 shows our proposed matching game algorithm which is designed to follow the smartphone utility function for choosing the most suitable network with a specific transmission mode (LTE or Wi-Fi) where an initial smartphone load is proposed to be transmitted through it. Matching game algorithm success in maximizing the total data rate of smartphones by 45% compared with the system using the LTE first and by 12% compared with Wi-Fi first. The figure explains that the system data rate by using matching game is increasing by increasing the number of smartphones which has more information, then it will be saturated as a result of exhausting the spectrum resources. So, the total data rate of the system is saturated regardless of the increase in smartphones and their information.

Figure 4.   Total data rate vs. Number of smartphones

Figure 5 shows that the proposed DDQL algorithm outperforms the backscattering of IoT tags at nearest smartphone by 14% in terms of the total rate at a different number of tags. This is because the DDQL algorithm can find the optimum clusters of backscatter tags for each smartphone which maximize the total data rate of the tags when using only two smart phones at the second phase of the system. As we can see from the figure the total data rate firstly increases by increasing the number of tags and then, it decreases gradually as a result of increasing mutual interference between tags in the same cluster.

Figure 6 illustrates that our proposed algorithm by using the Matching game and DDQL algorithms success in maximizing the system overall data rate compared to the system performance by using the matching game but without the DDQL algorithm, using LTE network first with DDQL algorithm and using LTE network first without DDQL.

Figure 5. The total data rate vs. Number of tags

As shown in the figure our proposed scheme can enhance the total system data rate on average by 90% above the system without any algorithms, 20% above the system without the DDQL algorithm, and 90% above the system with LTE network first. This figure explains the performance of the system at a different number of tags К  and number of smartphones M=4  and Wi-Fi access point  N=2  where the total system data rate increase by increasing the number of tags and then, it saturated according to the achieved data rate of the system with matching game by using a number of smartphones M=4. So, we study the performance at a different number of smartphones in Figure 7.

Figure. 6 The total system data rate vs. the number of tags

The total system data rate of our proposed algorithms (DDQL and Matching game) is illustrated in Figure 7 at different numbers of smartphones to obtain the optimum number of smartphones that are needed for our system. We observe that at a number of smartphones M=4 the system total data rate is increased by an average 90% compared to the system-based number of smartphones M=2 . This is because when smartphones increase the number of tags per smartphone cluster decrease so the achieved rate of each cluster increase as a result of less mutual interference between the cluster tags However, at M=6 and 8 the total system rate is increased slightly by 5%, 8% over the total data rate of the system based number of smartphone M=4 because each smartphone cluster has approximately one or two tags. Based on the aforementioned, the optimum number of smartphones for our system is   M=4 because after 4 smartphones any increase in the number of smartphones has low effect on the total system data rate.

Figure 8 shows the effect of increasing the number of smartphones on the system capacity of IoT tags at the required data rate per IoT tag of 10 Mb/s assuming the number of tags is K=20. This figure explains that with the increasing

Figure 7. The total system data rate vs. the number of tags at different numbers of smartphones.

Figure 8. The capacity of the system vs. the number of smart phones

of the number of smartphones from M=2 to M=4 the capacity of the system greatly increases as a result of fewer cluster tags and subsequently more cluster data rate. After that, the increase in the number of smartphones achieves slight system capacity enhancement. Table III represent the system capacity at different number of smartphones M.

| Number of Smart phones (M) | 2 | 4 | 6 | 8 | 10 | 12 |
| --- | --- | --- | --- | --- | --- | --- |
| Capacity of the System | 9 | 13 | 14 | 15 | 16 | 16 |

Table 3.  Capacity of the system vs. number of smart phones.

Furthermore, the figure illustrates that system capacity of our proposed algorithms by using matching game with DDQL algorithms outperform the capacity of the system with LTE first with DDQL algorithms where it success to enhance the system capacity on average by 100% above system using LTE network first with DDQL algorithm at a number of smartphones M=4. however, At high number of smartphones M=10, 12  the capacity of the system become the same. This because each smartphone cluster has one tag so, DDQL algorithm has no effect in this case.

The achieved data rate per tag at the required IoT tag’s data rate (10 Mb/s) is represented at a different number of tags in Figure 9.

Figure 9. The achieved data rate per tag vs. Number of tags.

we observe that the achieved rate is decreased by increasing the number of tags as a result of more mutual interference in each cluster which reduces the rate per each tag in the cluster. Moreover, the achieved data rate at a number of smartphones M=4 success in achieving the required rate for more tags than the system with smartphone number M=2 and it increase slightly by increasing the number of smartphones M.

8.  Conclusion

In this paper, framework is proposed to jointly optimize the smartphone mode selection of (LTE or Wi-Fi) network which act as a relay for the received backscattered information from IoT tags. the mode selection (LTE or Wi-Fi) problem has been formulated as an optimization problem to maximize the system throughput where many-to-one matching game is used to solve this problem. Also, the IoT tags clustering is optimized which using the SR technique to backscatter on the neighbor smartphone. The IoT tags clustering problem has been formulated as an optimization problem to maximize the system throughput to solve this problem, a DDQL algorithm is proposed to obtain the optimum cluster members. Simulation results show that the proposed scheme has better performance; compared to the system without using DDQL and matching game algorithms. Our proposed algorithm over perform the system using LTE network first without DDQL algorithm on the average by 90% and success to increase the system capacity on average by 100% above system using LTE network first without DDQL algorithm at number of smartphones M=4. Furthermore, the system performance is studied to find the optimum number of smartphone has been needed for the system where we find the optimum number of smartphones which is required for this system is 4 smartphones.

##### References

G. A. Akpakwu, B. J. Silva, G. P. Hancke and A. M. Abu-Mahfouz, "A Survey on 5G Networks for the Internet of Things: Communication Technologies and Challenges," in IEEE Access, vol. 6, pp. 3619-3647, 2018.

A. Al-Fuqaha, M. Guizani, M. Mohammadi, M. Aledhari and M. Ayyash, "Internet of Things: A Survey on   Enabling Technologies, Protocols, and Applications," in IEEE Communications Surveys & Tutorials, vol. 17, no. 4, pp. 2347-2376, Fourthquarter 2015.

Chafii, Marwa, Faouzi Bader, and Jacques Palicot. "Enhancing coverage in narrow band-IoT using machine learning." 2018 IEEE Wireless Communications and Networking Conference (WCNC). IEEE, 2018.

M. Chen, Y. Miao, Y. Hao and K. Hwang, "Narrow Band Internet of Things," in IEEE Access, vol. 5, pp. 20557-20577, 2017.

H. Malik, H. Pervaiz, M. Mahtab Alam, Y. Le Moullec, A. Kuusik and M. Ali Imran, "Radio Resource Management Scheme in NB-IoT Systems," in IEEE Access, vol. 6, pp. 15051-15064, 2018.

Zhao, Yang, et al. "A comprehensive survey of 6g wireless communications." arXiv preprint arXiv:2101.03889 (2020).

Barakat, Basel, et al. "6G Opportunities Arising from Internet of Things Use Cases: A Review Paper." Future Internet 13.6 (2021): 159.

ElMossallamy, Mohamed A., et al. "Backscatter communications over ambient OFDM signals using null subcarriers." 2018 IEEE Global Communications Conference (GLOBECOM). IEEE, 2018.

Boyer, Colby, and Sumit Roy. "—invited paper—backscatter communication and rfid: Coding, energy, and mimo analysis." IEEE Transactions on Communications 62.3 (2013): 770-785.

Y.-C. Liang, Q. Zhang, E. G. Larsson, and G. Y. Li,        “Symbiotic  radio: Cognitive backscattering communications for     future wireless   networks,” IEEE Transactions on Cognitive Communications and  Networking, vol. 6, no. 4, pp. 1242–1255, 2020.

Yang, Changlin, Xiaodong Wang, and Kwan-Wu Chin. "On max–min throughput in backscatter-assisted wirelessly powered IoT." IEEE Internet of Things Journal 7.1 (2019): 137-147.

Van Huynh, Nguyen, et al. "Ambient backscatter communications: A contemporary survey." IEEE Communications surveys & tutorials 20.4 (2018): 2889-2922.

Wang, Gongpu, et al. "Ambient backscatter communication systems: Detection and performance analysis." IEEE Transactions on Communications 64.11 (2016): 4836-4846.

Zhuang, Yuandong, et al. "Optimal resource allocation for RF-powered underlay cognitive radio networks with ambient backscatter communication." IEEE Transactions on Vehicular Technology 69.12 (2020): 15216-15228.

Liang, Ying-Chang, et al. "Symbiotic radio: Cognitive backscattering communications for future wireless networks." IEEE Transactions on Cognitive Communications and Networking 6.4 (2020): 1242-1255.

Liu, Yanyan, Pinyi Ren, and Qinghe Du. "Symbiotic Communication: Concurrent Transmission for Multi-Users Based on Backscatter Communication." 2020 International Conference on Wireless Communications and Signal Processing (WCSP). IEEE, 2020.

Raza, Ali, et al. "A NOMA-enabled cellular symbiotic radio for mMTC." Wireless Personal Communications 122.4 (2022): 3545-3571.

Guo, Huayan, et al. "Cooperative ambient backscatter system: A symbiotic radio paradigm for passive IoT." IEEE Wireless Communications Letters 8.4 (2019): 1191-1194.

Poornima, P., G. Laxminarayana, and D. Srinivas Rao. "Performance Analysis of Channel Capacity and Throughput of LTE Downlink System." International Journal of Computer Networks & Communications (IJCNC) Vol 9.

Anany, Mohamed, Mahmoud M. Elmesalawy, and Ahmed M. Abd El-Haleem. "Matching Game-Based Cell Association in Multi-RAT HetNet Considering Device Requirements." IEEE Internet of Things Journal 6.6 (2019).

Wu, Haitao, et al. "IEEE 802.11 distributed coordination function (DCF): analysis and enhancement." 2002 IEEE International Conference on Communications. Conference Proceedings. ICC 2002 (Cat. No. 02CH37333). Vol. 1. IEEE, 2002.

Elmosilhy, Noha A., Mahmoud M. Elmesalawy, and Ahmed M. Abd Elhaleem. "User association with mode selection in LWA-based multi-RAT HetNet." IEEE Access 7 (2019): 158623-158633.

Anany, Mohamed, Mahmoud M. Elmesalawy, and Ahmed M. Abd El-Haleem. "Matching game-based cell association in multi-rat HetNet considering device requirements." IEEE Internet of Things Journal 6.6 (2019): 9774-9782.

Long, Ruizhe, et al. "Symbiotic radio: A new communication paradigm for passive Internet of Things." IEEE Internet of Things Journal 7.2 (2019): 1350-1363.

Bharadia, Dinesh, et al. "Backfi: High throughput wifi backscatter." ACM SIGCOMM Computer Communication Review 45.4 (2015): 283-296.

Kellogg, Bryce, et al. "Wi-Fi backscatter: Internet connectivity for RF-powered devices." Proceedings of the 2014 ACM Conference on SIGCOMM. 2014.

Bayat, Siavash, et al. "Matching theory: Applications in wireless communications." IEEE Signal Processing Magazine 33.6 (2016).

Gu, Yunan, et al. "Matching theory for future wireless networks: Fundamentals and applications." IEEE Communications Magazine 53.5 (2015).

An, Chengquan, and Yang Liu. "A matching game algorithm for spectrum allocation based on POMDP model." 2011 7th International Conference on Wireless Communications, Networking and Mobile Computing. IEEE, 2011.‏

Metwaly, Samar Shaker, Ahmed M. Abd El-Haleem, and Osama M. El-Ghandour. "NOMA Based Matching Game Algorithm for Narrowband Internet of Things (NB-IoT) System." Ingénierie des Systèmes d Inf. 25.3 (2020): 345-350.

Tan, Fuxiao, Pengfei Yan, and Xinping Guan. "Deep reinforcement learning: from Q-learning to deep Q-learning." International Conference on Neural Information Processing. Springer, Cham, 2017.

Hasselt, Hado. "Double Q-learning." Advances in neural information processing systems 23 (2010).

Van Hasselt, Hado, Arthur Guez, and David Silver. "Deep reinforcement learning with double q-learning." Proceedings of the AAAI conference on artificial intelligence. Vol. 30. No. 1. 2016.

Al-Eryani, Yasser, Mohamed Akrout, and Ekram Hossain. "Multiple access in cell-free networks: Outage performance, dynamic clustering, and deep reinforcement learning-based design." IEEE Journal on Selected Areas in Communications 39.4 (2020): 1028-1042.

Qadir, Zakria, et al. "Towards 6G Internet of Things: Recent advances, use cases, and open challenges." ICT Express (2022)