Hydrological and Hydrogeological Evaluation of Proposed Massive Salty Lagoons in western Dessert in Egypt

Eizeldin, M.Aa,b, Abdel Azeem, H.Sc and El-Didy, S.Md

a  Assistant prof., Civil Engineering Dept., Faculty of Engineering of Mataria, Helwan University.

b Lecturer of Hydraulics, Civil Engineering Dept., Faculty of Engineering, British University in Egypt.

c PhD student, Dept. of Irrigation and Hydraulics, Faculty of Engineering, Cairo University.

d  Prof. of Hydraulics, Dept. of Irrigation and Hydraulics, Faculty of Engineering, Cairo University.
Hydrological and Hydrogeological Evaluation of Proposed Massive Salty Lagoons in western Dessert in Egypt

Hydropower generation has been proposed by utilizing the difference in the water levels between the Mediterranean Sea and the level of accumulated saltwater in the Qattara Depression.

The target of this paper is to represent the study of the accumulated saltwater effect in the Qattara Depression on the Nile Delta drainage and the underneath groundwater reservoir.

The main objectives of the research study are to: a) determine the expected evaporation from the accumulated saline water in the depression, b) study the hydrogeological conditions of the Qattara Depression by identifying the flow direction between the Nile Delta and the depression, after filling the depression, and C) study the effect of lagoon accumulated saline water on the Nile Delta fertility and the underneath groundwater layers.

The study methodology has been performed utilizing the recent tools of analysis that were not available for the previous studies such as; efficient numerical models, geographic information system (GIS), and Digital Elevation Models (DEM). The effect of salinity on evaporation, from the lagoon, was considered. A number of twenty one boreholes, with average depth of 3900 m, were employed to construct the geological stratification. Hydraulic conductivity values and groundwater levels have been collected from the available boreholes and deep exploratory wells drilled by the oil companies. Mass balance of the lagoon salinity was performed, every separate year for a total period of 50 years. Density dependent model (SEAWAT) via MODFLOW has been employed for a duration of 50 years.

The simulation was performed every separate year. It was repeated for different water filling levels (50 m and 60 m below mean sea level). Several well fields are assumed between the filled lagoon and the Nile Delta. The wells screen was assumed in the Moghra, the Limestone, or the deep Sandstone Formations. The results have revealed that the saline lagoon does not affect the Nile Delta drainage. The lagoon accumulated salinity does not reach the Nile Delta. The effect of salinity on the underneath sandstone reservoir is minor due to its upper confining shale layer.

Keywords: Qattara Depression; evaporation from saline water; salinity; density dependent model; mass balance; Nile Delta drainage; saline lagoons.

  - Introduction

The Utilization of the Qattara Depression, in the northern part of the Egyptian western desert, for hydropower generation has been proposed at the beginning of the 20th century. Hydropower generation could be achieved by utilizing the difference in the water levels between the Mediterranean Sea and the Qattara Depression released accumulated water level.

The main objective of this research is to study the hydrological and the hydrogeological conditions of the Qattara Depression, as a case study of massive salty lagoons, and study the effect of filling the Depression with saline water from the Nile Delta drainage and the underneath groundwater reservoir.

The new tools, which were not available more than thirty years ago when the Qattara project was proposed, have been utilized in this study to achieve the research objectives. The tools include numerical efficient models, programs of geographic information system, and digital elevation models. This research intends to accomplish the following objectives:

- Determine the expected evaporation from the accumulated saline water in the Qattara Lagoon,

- Ensuring that filling the lagoon, with saline water, will not reach and affect the Nile Delta fertility, and

- Ensuring that the accumulated saline water, in the lagoon, will not affect the lower sandstone aquifer which is the main source of water in the Western Desert of Egypt.

The Qattara Depression is a desert basin within the Libyan Desert of northwestern Egypt in Matruh Governorate. The depression has a depth of 133 m below sea level, and is the world’s 5th deepest depression. Also, it is the second lowest point in Africa after Lake Assal in Djibouti. The depression covers an area of about 19,500 km² (7,000 square miles). It is 300 km in length and has a maximum width of 145 km.

The eastern part of the depression is shallower and narrower than the western and central parts, and as the northern plateau descends towards the east, the height of the escarpment decreases from west to east. A large proportion about 5800 km2 of the floor of the Qattara Depression is covered by sabkha, which is a mixture of sand and salt. The remaining part of the depression floor is mostly formed of sands, gravels, clays, and limestones (Ball 1933).

The shortest distance between the Mediterranean Sea and the depression (from a point close to El Alamein to the Moghra oasis) is approximately 56 km.

The study area is located in the northern part of the Western Desert of Egypt between longitudes 25º 00 - 31º 00 E and latitudes 27º 00 - 31º 00 N as shown in Figureure-1. It extends from Rosetta branch in the East to the Libyan Border in the West and extends from the Mediterranean Sea in the North to 20 km away from Qattara Depression end in the South.

The main schemes, proposed by various engineers, for the development of the Qattara Depression are presented by (Martino 1973). The various proposals, to convey the Mediterranean Sea water to the depression, are shown in Figure 2.

Figure. 1: Location of Qattara Depression                                   Figure. 2: Different proposals to convey

in the study area                                                            the Mediterranean Sea water to

(World System, UTM_Zone_35N)                                   the Qattara Depression (Martino 1973)

Several studies were performed to study the feasibility of the Qattara project. The basic concept of the project is to discharge the Mediterranean Sea water through an open channel to the depression, and to regulate this discharged water by water turbines, thus utilizing the difference of head between the sea 1evel and the depression water levels for power production. Since the depression is closed from each direction, a vast lake will be formed until its water level reaches 50 or 60 meters below sea level (Lahmeyer et al. 1978).

Ball (1933) made detailed researches to find the most convenient level for the Qattara Lake, bearing in mind the electric power to be generated, the minor transmission losses between the central power plant and the Delta Area, and the lowest cost per kW installed. Having pointed out the necessity of putting at least two-thirds of the length of the conduit in a tunnel due to the height of the land intervening between the sea and the depression, and having proposed to divide the flow into three circular tunnels so that construction could take place in three successive stages. Ball showed that the most convenient solutions were those relating to lakes at 50.0 and 60.0 m below sea level. He left the final selection to be based on the best solution for the geological and topographical conditions. However, it is clear that he preferred the lake with a final level at – 50 m. He suggested that an open channel be constructed for the first 20 km, and that three tunnels with a diameter of 11 m be built to supply the lake at a level of -50 m. The resulting firm power potential would be about 175 MW.

According to the preliminary proposal presented by SWECO (1981), the water will be conducted from the Mediterranean Sea to the Moghra Reservoir, through an intake that consists of a 46 km long canal and two tunnels each 9 km in length and with diameters of about 12 m. From the reservoir, the water will be fed through a hydro-electric power station and then, through a 48 km long canal, to a newly-formed lake in the Qattara Depression. The level of this lake will be 50 m lower than that of the Mediterranean Sea. The SWECO proposal differs in that the canal is located further to the east, so that the section is shorter and located in the formation of the Moghra Reservoir. It is planned that the reservoir will be separated from the Qattara Depression by two earth dykes. The intention was that the reservoir should have a maximum area of about 200 km2. As this reservoir is to function as a regulation storage reservoir for the power station, the level of its surface will vary.

Rizk and Davis (1991) performed a two-dimensional finite-difference model in conjunction with available meteorological, geological, and hydrological data to characterize the present groundwater conditions only in the Moghra aquifer. They mentioned that Qattara Reservoir is saline; the density difference between the proposed reservoir’s water and the Moghra aquifer was considered negligible. A shale layer of 500 m thickness is underlying the Moghra Layer. The southeastern edge of the modeled area, where the Bahariya-Abu Roash uplift and Oligocene basalt flows act as a hydrologic barrier, is a no-flow boundary. The modeled area (170,000 km2), was subdivided into uniform cells of 15 km x 15 km with a finite-difference grid of 16 rows and 20 columns. The evaporation rate was calculated by programming a formula by Harleman et al. (1981) which was supported by investigations at Lake Mead, Nevada-Arizona (Harbeck et al. 1958) and Lake Nasser, southern Egypt (Omar and El-Bakry 1980). Calculated pre-reservoir heads agree with field data and illustrate that the Qattara Depression is a sink for groundwater flow from surrounding regions. The effect of the proposed Qattara reservoir on surrounding aquifers under conditionds of large withdrawals from pumping wells, as well as two-dimensional, density-dependent modeling in cross section would be recommended in future works. The model’s grid in the study is quite coarse, and future work could involve more detailed modeling if sufficient hydrogeologic information becomes available, particularly in areas where the Moghra aquifer is recharged by the Nubian aquifer.

  - Regional Settings of the Study Area

- Description of the Climatological Settings

The meteorology of the Qattara Depression is described as being fairly typical of extremely arid regions with low rainfall, high summer temperature, and a moderately low winter temperature. The yearly rainfall at the Sea coast varies from 110 mm to 190 mm, and decreases to about 20 mm at the northern boundary of the depression. Further south the rainfall is less.

The relative humidity of the depression is, comparatively, high because of the northern winds which carry moisture from the Mediterranean.

- Description of the Topographical Settings

The topography of the Qattara Depression was identified, as shown in Figureure-3, using the Digital Elevation Models (DEM 90 m x 90 m resolution) version 2_1, SRTM Version 3 (also called “SRTM Plus”) released by the National Aeronautics Space Administration (NASA). SRTM DEM is expected to have linear vertical absolute height error of less than 16 m, linear vertical relative height error of less than 10 m, circular absolute geolocation error of less than 20 m, and circular relative geolocation error of less than 15 m (Farr et al. 2007). The DEM, which covered the study area, was downloaded from the website http://dds.cr.usgs.gov/srtm/version2_1/SRTM3/Africa/. The downloaded data is saved in ArcInfo ASCII Grid format using VTBuilder software. The ASCII file has been employed via Global Mapper software to generate the contour levels of the depression after re-projecting it from WGS 1984 geographic coordinates system to Universal Transverse Mercator Zone 35 projection. The last processing procedure to calculate the area of each contour level of the depression is using ArcGIS 10.1 software.

Global Mapper is also used to investigate the topography of bed levels of Qattara Depression. Figureure-4 shows a lateral section (A-A) crossing the depression to investigate its bed level, while Figureure-5 shows a longitudinal section (B-B). Surface area and volume of the accumulated water, for different levels of the lagoon were calculated from DEM. They agree with the values determined by Ball (1933).

The comparison between the calculated water surface area (km2) of the lagoon done by the authors and the one calculated by Ball (1933) is illustrated in Figureure-6; While Figureure-7 shows the comparison between the calculated water volume (km3) of the lagoon done by the authors and the one calculated by Ball (1933).

Figure.3: Topography of Qattara Depression using

Digital Elevation Model, DEM

(World System, UTM_Zone_35N)

Figure.4: Lateral section (A-A') showing                             Figure. 5: Longitudinal section (B-B') showing

the bed level at - 78.43 m                                                     the bed level at - 132.00 m

Figure. 6: Comparison between the accumulated water                  Figure. 7: Comparison between the accumulated water

Surface area (km2) done by the authors                                      Volume (km3) done by the authors and

And the one calculated by Ball (1933)                                           the one calculated by Ball (1933)

- Description of the Stratigraphy (Using Rockworks Model)

Rockworks-15 model has been employed to simulate the current geological conditions of the Qattara Depression area. Rockworks-15 model is a recent version of Rockware. It is an integrated package for geological data management, analysis, and visualization.

The soil stratification of the study area has been investigated and identified by using the existing boreholes that were drilled by the petroleum companies (Lahmeyer et al. 1979). These boreholes give a good view of the general stratigraphy of the aquifer. It is a complex sequence of sandstones, sand, clays, clay stones, and limestones.

A number of twenty-one boreholes and their lithology classification, with average depth of 3900 m, were employed via Rockworks-15 model to construct the geological stratification for the study area. Figureure-8 shows a 3-dimensional view of the existing twenty-one boreholes in the study area, while Figureure-9 shows a 3-dimensional view of the study area stratification with locations of the existing boreholes.

The boreholes were also employed via GIS to represent the geological stratification. Figureure-10 shows a number of thirteen of the existing boreholes in the longitudinal direction, from East to West. By taking longitudinal section (A-A') as shown in Figureure-11, crossing the Qattara Depression, the stratification of these existing boreholes are illustrated and represented as Figureure-12.

Eight of the existing boreholes are drilled in the lateral direction, from North to South direction, as shown in Figureure-13. Figureure-14 shows the lateral section (B-B'), crossing Qattara Depression, while Figureure-15 shows the stratification of Section (B-B') through the existing boreholes in the lateral direction.

Figure. 8: 3-D view of existing boreholes                         Figure. 9: 3-D view of study area stratification with

in the study area                                                                locations of the existing boreholes

Figure. 10: The existing boreholes in the longitudinal               Figure. 11: Location of section (A-A') utilizing the existing

direction (East – West)                                         boreholes in the longitudinal direction (East- West)

Figure.12: Stratification of section (A-A) through                   Figure. 13: The existing boreholes in the lateral direction

the existing boreholes the longitudinal                                                     (North- South)

direction (East- West)

Figure. 14: Section (B-B') through the existing boreholes               Figure. 15: Stratification of Section (B-B') through the

in the lateral Direction (North- South)                                  existing boreholes in the lateral direction

(North- South)

- Description of the Hydrogeology

After applying the Rockworks-15 model in order to identify the soil stratification of the study area, that agrees with the description given by Hellström (1940) and EURCONSULT PACER Consultant (1983), the hydrogeological system of the study area has been subdivided considerably into four layers using the groundwater numerical model (MODFLOW). The four layers from top to bottom are Moghra, limestone, shale, and sandstone as shown in Figureure-16. The average corresponding thickness of these layers is 800, 1000, 200, and 2000 meters, respectively.

There are a number of 35 exploratory wells, drilled by oil companies, scattered nearby the depression (Ezzat 1984). From these exploratory wells, the static groundwater levels were recorded and the values of the hydraulic conductivity were determined. The hydrogeological characteristics of the aquifer layers (hydraulic conductivity and storage coefficient) are illustrated in Table-1 and Table -2, respectively. Figureure-17 shows the observed groundwater levels in meters with respect to the mean sea level. The data of the observed groundwater head have been used in the calibration process of the groundwater model (MODFLOW).

Figure.16: Layers of the study area (World System, UTM)

Table -1: Hydraulic conductivity values for the hydrogeological system (Ezzat, 1984)

| Layer Name | Hydraulic Conductivity Values |  |  |
| --- | --- | --- | --- |
|  | KXX(m/day) | KYY(m/day) | KZZ(m/day) |
| Moghra Layer | 10 | 10 | 5 |
| Limestone Layer | 5 | 5 | 2.5 |
| Shale Layer | 1*10-7 | 1*10-7 | 5*10-8 |
| Sandstone Layer | 1 | 1 | 0.5 |

Table -2: Storage coefficient values for the hydrogeological system (Ezzat, 1984)

| Specific Storage, Ss(m-1) | 0.00001 |
| --- | --- |
| Specific yield, Sy | 0.2 |
| Porosity, n | 0.3 |

Figure. 17: Observed groundwater levels in meters with

respect to the mean sea level (Ezzat 1984)

  - Choice of the Groundwater Numerical Model (Visual MODFLOW)

Visual MODFLOW, a finite difference model, has been employed to simulate the current hydrogeological conditions in the study area. SEAWAT via MODFLOW was used to solve the variable-density flow equation by reformulating the matrix equations in terms of fluid mass rather than fluid volume and by including the appropriate density terms. Fluid density is assumed to be function of the concentration of dissolved constituents.

The SEAWAT program was developed to simulate three-dimensional variable-density, transient ground-water flow in porous media. The source code for SEAWAT was developed by combining MODFLOW and MT3DMS into a single program that solves the coupled flow and solute-transport equations. SEAWAT uses either an explicit or implicit procedure to couple the ground-water flow equation with the solute-transport equation.

  - Methodology of the Study

The flowchart illustrated in Figureure-18 shows the calculations steps of salinity in the Qattara Lagoon, repeated every year for a total period of 50 years to get the piezometric head and the salinity concentration of the groundwater system. According to the flowchart, the salinity in the lagoon is calculated to be used as a boundary conditions (specified concentration) in the groundwater modeling.

By referring to the volume of water contributed by rainfall, water inflow from the sea, inflow of groundwater, evaporation from the accumulated salt water in the lagoon, and outflow of groundwater. The accumulated salt concentration in the lagoon water at t+Δt is calculated.

Figure.18: Flowchart of study steps

Where:

Vo is the volume of the lagoon, Ci+1 is the lagoon salt concentration in the year i+1, VR is the volume of rainfall on the lagoon, Co is the salt concentration of freshwater; Vin is the inflow volume from sea water into the lagoon, Cin is the salt concentration of sea water, CGroundwater is the salt concentration out of the lagoon, Vevapo.is the evaporation volume out of the lagoon (evaporation form saline water), and VGroundwater is the volume recharged by inflowing/outflowing groundwater into/from the lagoon (Fujinawa et al. 2009).

The results of the mass balance in the Qattara Lagoon are huge to be presented in the research as they were performed every separate year for a total period of 50 years where the filling water level of the lagoon was at 50.0 and 60.0 meters below mean sea level.

- Evaporation from Lagoon Saltwater

Meteorological data of Qattara Region are obtained from Water and Climate Atlas (International Water Management Institute IWMI 2009). The climatic conditions that affect the evaporation are introduced from Figureure-19 through to Figureure-23, represent mean monthly rainfall (mm), mean monthly temperature (°C), relative humidity (%), wind speed (m/s), and sunshine hours, respectively. These data are used in calculating the evaporation from freshwater that follows the Penman’s Approach.

Mean monthly precipitation from three different meteorological stations (Wadi El Natrun, Dabaa, and Siwa) are collected and analyzed. The records of rainfall from these stations are from the year 1970 through to the year 2000 as shown in Figureure-24, (Khalil and Eizel-Din 2006). The evaporation rate in mm\day, from fresh water, was calculated, using Penman Equation, for each month for each lagoon level as shown in Figureure-25.

After filling the Qattara Lagoon with seawater, the evaporation rates from saline water in the lagoon have been predicted and calculated by applying Equation-1 whereas the specific gravity of salt water in Qattara Lagoon is less than 1.150, thus the ratio of evaporation rate from saline water to the evaporation from fresh water (ES/EO) is a function of specific gravity as shown in Equation-1 and Figureure-26 (Khalil and Eizel-Din 2006). Where Es and Eo are the evaporation rate from saline water and evaporation from fresh water, respectively.

For each value of water salinity in the lagoon, the corresponding value of specific gravity has been calculated via the following online converter.

(https://www.hamzasreef.com/Contents/Calculators/SalinityConversion.php). This calculator converts between various units of salinity. Relative specific gravity (sg) and conductivity (mS/cm) measurements are inherently temperature-dependent and this calculator follows the prevailing standard of assuming a temperature of 25C/77F. Most salinity measurements in literature follow this convention and most salinity measurement devices correct to this temperature internally.

After converting each value of accumulated water salinity in the lagoon, the resulted relationship between the lagoon water salinity (PPM) and their corresponding values of specific gravity is presented in Figureure-27.

After applying Equation-1, the resulted relationship between the ratio of evaporation from saline water to evaporation from fresh water (ES/EO) and the corresponding specific gravity (S.G.) is presented in Figureure-28. The results fit the adopted equation that has been applied on the Dead Sea. The evaporation increases as by going down due to the increase of temperature.

As a conclusion the higher the lagoon salinity concentrations, the higher the specific gravity values, the smaller evaporation rates from saline water in the Qattara Lagoon.

ESEO=   8.2322 ×S.G.3-32.543 ×S.G.2 +39.826 ×S.G. -14.524                                (1)

(Khalil and Eizel-Din 2006). Should search for another Author

Figure. 19: Mean monthly rainfall (mm)                                   Figure. 20: Mean monthly temperature (C°) 
(International Water Management Institute IWMI 2009)     (International Water Management Institute IWMI 2009)

Figure. 21: Relative humidity (%)                                                     Figure. 22: Wind speed (m/s)

(International Water Management Institute IWMI 2009)             (International Water Management Institute IWMI 2009)

Figure. 23: Sunshine hours (%)                                   Figure. 24: Mean precipitation (mm) from three different

(International Water Management Institute IWMI 2009)               meteorological stations (Khalil and Eizel-Din 2006)

Figure. 25: Evaporation rates from Qattara Lagoon                 Figure. 26: Specific gravity effect on evaporation

for different Lagoon water levels                                           (Khalil and Eizel-Din 2006)

Figure. 27: Relationship between lagoon water salinity            Figure. 28: Relationship between the ratio (ES/EO)

and corresponding values of specific gravity                                      and specific gravity (S.G.)

- Groundwater Simulation for Qattara Lagoon

The first step in MODFLOW modeling, to simulate the hydrogeological conditions of the lagoon, is the conceptual model description. The conceptual model, which helps in adequate understanding of the groundwater flow system, is described as follows:

- The Model Grid Dimensions

An area with dimensions of 647,297 meters, in the east-west direction, and 332,714 meters, in the north-south direction, has been considered in this study. The grid system has been subdivided into 400 rows by 500 columns including 20,000 cells, in plan, and considering the four groundwater layers; the total number of cells is 80,000.

- Initial Heads

From reports prepared by CEDARE (1993), the contour lines map of initial water levels for Moghra, Limestone, Shale, and Sandstone Formations have been adopted and employed via MODFLOW to generate the initial head zones for the Moghra Formation as shown in Figureure-29. Initial head zones for Limestone, Shale, and Sandstone layers are presented in Figureure-30.

Figure. 29: Initial head zones for Moghra Formation               Figure. 30: Initial head zones for Limestone, Shale,

and Sandstone Formations

- Initial Concentration

Values of initial concentration for the Qattara study area have been considered, via the numerical groundwater model, as given in Table-3:-

Table -3: Initial concentration values for the hydrogeological system

| Layer Name | Water salinity (ppm) |
| --- | --- |
| Moghra Layer | 3000 |
| Limestone Layer | 1000 |
| Shale Layer | 1500 |
| Sandstone Layer | 2000 |

- Head Boundary Conditions

Values of boundary conditions for the Qattara Study area have been considered via the numerical groundwater model, as follows:-

- Boundary conditions of constant specified head have been considered on the Northern boundary of the study area (Mediterranean Sea) and the Eastern boundary of the study area (Rosetta branch) with values of 0.00 and 2.50 meters, respectively.

- Boundary conditions for Qattara Depression has been taken -50 meters as first scenario, and – 60 meters for the second scenario.

- Southern boundary condition for Moghra, Limestone, Shale, and Sandstone layers has been considered as a specified boundary condition using the values of initial boundary conditions.

- No flow boundary condition was considered for the Libyan border (West of the study area).

Figureure-31 shows the boundary conditions of the study area including the Qattara Depression.

Figure. 31: Boundary conditions of the study area

- Salt Boundary Conditions

Values of salinity boundary conditions for the Qattara Study area have been considered via the numerical groundwater model, as follows:-

- Boundary condition of specified concentration is considered on the Northern boundary of the study area (Mediterranean Sea) with value 40,000 PPM.

- Specified salinity concentration in the Qattara Lagoon is derived from the mass balance every separate year.

- Model Calibration

Steady state calibration was performed using the Observed groundwater levels in the Qattara study area. The calibration is based on data obtained from observations of the behavior of the aquifer.

Observations of groundwater levels in the Moghra formation were performed during a limited area only as previously shown in Figureure-17. No historical data exist for the whole area. Figureure-32 shows the calibration results for the Qattara Depression area. The model calibration is accepted with correlation coefficient of 0.88 and error of standard deviation of 1.56 m. The calibration related statistical results are demonstrated in Table-4, the calibration results are accepted in spite of the scarcely available records.

Figure.32: Model calibration for observed head versus calculated head

Table 4: Statistical parameters related to the calibration process

| Parameter | Value |
| --- | --- |
| Standard Error of the Estimate | 1.556 (m) |
| Normalized RMS | 19.855 % |
| Correlation Coefficient | 0.879 |

  - Scenarios of the Study and Results

Three scenarios for simulating the hydrogeological conditions and the salinity spread in the modeled area, using density dependent module (SEAWAT) via the numerical model (MODFLOW), were performed. These scenarios and their results are being demonstrated and discussed in details in the following sections.

### First Scenario: - Filling Qattara Depression to a level of 50.0 m (b.m.s.l)

The impact of filling the Qattara Depression, with sea water, on the hydrogeological conditions and the salinity spread is modeled, when the depression water level is 50 meters below mean sea level. The numerical simulation has been performed every separate year for a total operation period of 50 years considering the effect of the accumulated saline water in the depression. The salinity in the depression was calculated from the lagoon mass balance and considered as a boundary condition that increases every year through the simulation due to the evaporation. The salinity results presented in this paper are the final values after 50 years simulation.

The numerical model results showed that there is salinity spread out of the Qattara Depression. Figureure-33 shows limited salinity spread out of the Qattara Lagoon after filling the depression to a level of -50.0 meters after 50 years. The salinity has not reached the Nile Delta, to a distance of 85 km towards the Delta; this result satisfies one of the research objectives.

The salinity spread in the vertical direction has reached the limestone layer underlying the Moghra Formation as shown in the cross section passing the southern part of the lagoon shown in Figureure-34. The salinity has slightly reached the main source of groundwater aquifer (Sandstone Aquifer) which satisfies another objective of this research.

Figure. 33: Salinity spread out of Qattara Lagoon,             Figure. 34: Vertical section shows the salinity spread out                       after filling the depression                                          underneath the depression

to a level of -50.0 m, after 50 years                                           to a level of  - 50.0 m, after 50 years

### Second Scenario: - Filling Qattara Depression to a level of 60.0 m (b.m.s.l)

Studying the effect of filling the Qattara Depression, with sea water, on the hydrogeological conditions and salinity spread is repeated, when the depression water level is at 60 meters below mean sea level. The hydrogeological numerical simulation is implemented every separate year for a total period of 50 years to consider the accumulated saline water in the depression. The salinity in the depression was calculated from the lagoon mass balance and considered as a boundary condition that increases every year through the simulation due to the evaporation. The salinity results presented in this paper are the final values after 50 years simulation.

The study showed that there is salinity spread out of the Qattara Lagoon with salt concentration higher than the values of the scenario of filling the lagoon at - 50 meters. Figureure-35 shows the limited salinity spread out of the Lagoon after filling the depression to a level of -60.0 meters after 50 years. The salinity has not reached the Nile Delta for a distance of 82 km towards the Delta; this result satisfies one of the research objectives. The salinity spread in the vertical direction has reached the limestone layer underlying the Moghra Formation as shown in the cross section passing the southern part of the lagoon as shown in Figureure-36. The salinity, slightly, has reached the main source of groundwater aquifer (Sandstone Aquifer) which satisfies another objective of this research.

Figure. 35: Salinity spread out of Qattara Lagoon,                  Figure. 36: Vertical section shows the salinity spread               after filling the depression                                                         underneath the depression to

to a level of -60.0 m after 50 years                                               a level -60.0 m after 50 years

Third Scenario: - Simulation of well fields between the Qattara Depression and the Nile Delta

Groups of well fields are proposed, between the Qattara Depression and the Nile Delta, to investigate the effect of these wells on the salinity spread either in the groundwater aquifer, or/ and on the Nile Delta. The groups of well fields include a number of 90 wells with pumping rate 2400 m3/day from each well.

Three cases for the location of wells screens were studied; first one was by locating the well screen in Moghra layer, a second one by locating the screen in the limestone layer and a third one in the sandstone layer.

Under the well fields effect, the accumulated salinity does not travel, horizontally, more than 87 km in the sandstone layer outside the lagoon.

Figureure-37 shows a three-dimensional view of the geological aquifers in the region of study. From Figureure-38 through to -40 a 3-dimensional view of layers is represented in case of having wells screen in the Moghra, Limestone, and sandstone formation, respectively. It is shown that there is no effect of the well screen location on the salinity spread in the four layers.

Figureure-41 presents comparison of salinity (PPM) during 50 years between depression filling water levels of 50 m and 60 m (b.m.s.l.). As shown from the Figureure, salinity in the lagoon for water level -60.0 m is more than the corresponding value for water level of -50.0 m.

Figureure-42 presents evaporation rate from the lagoon accumulated saline water (mm\year) during 50 years for depression filling water levels of 50 m and 60 m (b.m.s.l.).

Figure.37: 3-D view of layers without wells                           Figure. 38: 3-D view of layers with wells screen

in the Moghra Formation

Figure. 39: 3-D view of layers with wells                                  Figure. 40: 3-D view of layers with wells screen

screen in the Limestone layer                                                      in the Sandston Layer

Figureure.41: Lagoon salt concentration (PPM) during 50 years             Figure. 42: Evaporation rate from the saline water

for depression filling water levels 50 and 60 m (b.m.s.l)                          in the lagoon during 50 years for depression filling

at levels of 50 and 60 m (b.m.s.l)

  - Conclusion

At the beginning of the 20th century, several studies were performed to deliver the Mediterranean Sea water to the Qattara Depression through a canal or tunnels for hydropower generation by utilizing the difference in the water levels. Main objectives of the current study were to: a) determine the expected evaporation from the accumulated saline water in the Qattara Lagoon, b) study the hydrogeological conditions of Qattara Depression as a drainage lagoon for the Nile Delta, c) study the effect of saltwater lagoon on the Nile Delta, and the groundwater reservoir.

The effect of salinity on evaporation, from the lagoon accumulated saline water, was considered. A number of twenty one boreholes, with average depth of 3900 m, were employed to construct the geological stratification. Hydraulic conductivity values and groundwater levels have been collected from the available boreholes and deep exploratory wells drilled by the oil companies.

Density dependent model (SEAWAT) via MODFLOW has been employed for total duration of fifty years. The numerical simulation has been performed every separate year for a total operation period of 50 years considering the effect of the accumulated saline water in the depression. The salinity in the depression was calculated from the lagoon mass balance and considered as a boundary condition that increases every year in the simulation due to the evaporation.

The simulation has been performed for lagoon water levels of -50.0 m and -60.0 m. Several well fields were assumed between the lagoon and the Nile Delta. The wells screen was assumed in the Moghra, the limestone, or the deep sandstone layers. The main results of the research are:

- Filling the Qattara Depression with sea water up to the absolute level of -50 m or -60 m, as expected to be after the execution of the Qattara Project, does not affect the groundwater conditions and the Nile Delta drainage.

- Accumulated salinity does not spread out towards the Nile Delta maintaining the fertility of the Delta agriculture soil.

- Accumulated salinity slightly spread out vertically down, in the sandstone layer (the main groundwater source in the western desert) because of its upper shale confining layer.

- Accumulated salinity does not travel, horizontally, more than 85 km in the sandstone layer, outside the Qattara Lagoon.

- Considering the groups of well fields, between the Qattara Lagoon and the Nile Delta, the accumulated salinity does not travel, horizontally, more than 87 km in the sandstone layer outside the Lagoon.

- Slight difference in the salinity spread was noticed by locating the well screens in the different layers.

References

Ball, J. (1927) Problems of the Libyan Desert. Geographical Journal”.

Ball, J. (1933) The Qattara Depression of the Libyan Desert and the Possibility of its Utilization for Power Production. George. J 82, 4, 289-314,. London.

Consultant, E. U. R. C. O. N. S. U. L. T. P. A. C. E. R. (1983) Regional Development Plan for New Valley – Groundwater”. Vo/2, Annex B, prepared by Euroconsult/Pacer Consultants. Cairo, Egypt: for Ministry of Development.

Environment, C., Arab Region, D., & Europe. (1993) Regional Strategy for the Utilization of the Nubian Sandstone Aquifer System”. International Fund for Agricultural Development.

Ezzat, M. A. (1983) Qattara Hydro Energy Project Side Effects on Groundwater Aquifers”. Ministry of Electricity. Qattara Hydro and Renewable Energy Project Authority.

Farr, T. G., Rosen, P. A., Caro, E., Crippen, R., Duren, R., Hensley, S., Kobrick, M., et al. (2007) The Shuttle Radar Topography Mission”. Review of Geophysics 45(2).

Fujinawa, K., Iba, T., Fujihara, Y. & Watanabe, T. (2009) Modeling Interaction of Fluid and Salt in an Aquifer/Lagoon system”. In: National Ground Water Association, PP, 35–48.

GMBH, L. I., GMBH, S. C. & GMBH, D. P. U. (1978) Study Qattara-Depression”, in cooperation with Arab Consulting Engineers, Bundesanstalt Fur Geowissenschaften und Rohstoffe, Cer Geonuclear, and Harza Engineering. Ministry of Electricity and Energy- Qattara Project Authority- Arab Republic of Egypt.

GMBH, L. I., GMBH, S. C. & GMBH, D. P. U. (1979) Study Qattara-Depression”, in cooperation with Arab Consulting Engineers, Bundesanstalt Fur Geowissenschaften und Rohstoffe, Cer Geonuclear, and Harza Engineering. Ministry of Electricity and Energy- Qattara Project Authority- Arab Republic of Egypt.

Harbeck, G. E., Kohler, M. A. & Koberg, G. E. (1958) Water loss investigations-Lake Mead studies”. U.S. Geological Survey Professional Paper 298, 100.

Harleman, R. F., Adams, E. E., Salhotra, A. M., Ismail, H., Bazaraa, A. S., El-Fandy, M. G. & Taha, M. F. (1981) Evaporation and salinity studies for the Qattara Depression Project”, Vol. 41.

Hellström, B. (1940) The Subterranean Water in the Libyan Desert. Institution of Hydraulics of the Royal Institute of Technology 34(26), 206–239. Stockholm.

International Water Management Institute. (2009) IWMI Online Climate Summary Service Portal. <http://www.iwmi.cgiar.org/W Atlas/Default.aspx>.

Khalil, M. B. & Eizel-Din, M. A. (2006) Development potential: Evaluation of the hydro- power potential of Egypt’s Qattara Depression”. International Water Power and Dam Construction 58(10), 32–36.

Martino, G. D. (1973) The Qattara Depression”. Naples, Italy: Hydraulics Institute.

Omar, M. H. & El-Bakry, M. M. (1980) Estimation of evaporation from the lake of Aswan High Dam (Lake Nasser) based on measurements over the lake”. Agric. Meteorol V. 23, 293–308.

Sundborg, A. & Nilsson, B. (1986) Qattara Hydrosolar Power Project Environmental Assessment” (UNGI Report 62.). Uppsala, Sweden.194: Uppsala University.

SWECO, P. M. (1981) Qattara Hydro-Solar-Power Project.

Zein, S. R. & Arden, D. D. (1991) Impact of the Proposed Qattara Reservoir on the Moghra Aquifer of Northwestern Egypt”.

http://dds.cr.usgs.gov/srtm/version2_1/SRTM3/Africa/.