Abstract--Fractional slot concentrated winding (FSCW) permanent magnet (PM) machines are taken more attention in low-speed applications because of their great advantages. This paper proposes a new structure for an FSCW double rotor axial flux PM synchronous machine (DRAFPMSM) with counter-rotating rotors. In addition, an analytical formula for the Back EMF waveform of the proposed structure is presented. The accuracy of the formula is investigated by designing a 40 Watts sample machine. The rotors of this machine rotate with speeds of 600 and 428 rpm in opposite directions. The Back EMF of the sample machine is calculated using the suggested formula. In addition, the Finite Element Method (FEM) is used to determine the Back EMF waveform. Comparison of the obtained results confirms that the new formula yields the Back EMF precisely such that the results well match to those of FEM.

An Analytical Formula for Back EMF Waveform of a Counter-rotating Dual-Rotor Non-Slotted Axial Flux Permanent Magnet Synchronous Machine

Reza Mirzahosseini1,*, and Elham Rahimi Namaghi2

1Department of Electrical and Robotic Engineering, Shahrood University of Technology, Shahrood, Iran

2Department of Electrical and Computer Engineering, Semnan University, Semnan, Iran

*Correspondence: reza.mirzahosseini78@gmail.com

Index Terms--AFPM machine; Analytical formula; Back EMF waveform; Counter-rotating; Finite Element Method (FEM)

# Nomenclature

| Do | Outer diameter of the stator core |
| --- | --- |
| Γ | Inner to outer diameter ratio of the stator core |
| pl | Pole pair number of the Low-speed rotor |
| ph | Pole pair number of the High-speed rotor |
| ap | Number of parallel paths of the stator winding |
| g | Length of the air gap |
| lw | Axial length of winding |
| lpm | Thickness of PM |
| Nlayer | Number of stator winding layers |
| NTurnlayer{i} | Number of winding turns in i-th layer |
| θPH | Phase angle: 0 or +2π/3 or –2π/3 for various phases |
| ωmh(t) | High-speed rotor angular speed |
| ωml(t) | Low-speed rotor angular speed |
| n | Number of harmonic orders |
| θm(t) | Instantaneous value of the angle of the rotor |
| K΄n | Fourier coefficient |
| f | Rated frequency |
| Br | PM remanent flux density |
| Hcb | PM coercive force |
| τp | Length of pole pitch at the average diameter |
| µ0 | Permeability of the air |
| α | Pole arc to pole pitch ratio |
| vl | Linear speed of the rotor |
| ra | Average radius of the rotor core |
| Fe | Electromagnetic force |
| Te | Electromagnetic torque |
| Pout | Output power |
| Pin | Input power |
| η | Efficiency |

# Introduction

NOwadays electrical machines are widely employed in numerous industrial applications, such as electric vehicles, fans, elevators, etc. (1,2). Counter-rotating dual-rotor axial flux permanent magnet (CRDRAFPM) motors are one of the types of synchronous motors that have higher power density and reliability in comparison with the single-sided structure (3). CRDRAFPM machines can be used as generators in wind turbine applications (4–6) or as motors in elevators, underwater vehicles, and propulsion ships (7,8). In CRDRAFPM, to produce two fields with different directions, a special arrangement of windings is used. This arrangement of windings is either based on the shifted winding (9,10), or the independent winding model (11). Distributed winding and concentrated winding are two common types of windings that have been used in PM machines. High efficiency, short-end turn, high-torque density, low cogging torque, and high slot fill factor are advantages of concentrated winding in comparison to distributed winding (12–14). Despite these advantages, the use of concentrated windings results in the appearance of additional spatial harmonic components in the magnetomotive force (MMF). Therefore, various low and high-order harmonics appeared in the flux density of the air gap. The existence of these harmonics in PM machines causes magnet losses. In asynchronous machines, these spatial harmonic components produce rotor bar losses and parasitic torque and consequently decrease the average torque of the machine. They cause high torque ripples in synchronous reluctance machines. Besides, the existence of spatial harmonics can cause local saturation, acoustic noise, and vibration. This defect of concentrated winding, i.e., the large amplitude of harmonics can be used in the structures of counter-rotating to produce two different fields. Hassannia has proposed a structure with counter-rotating rotors based on this advantage of the concentrated winding. However, the structure studied by him has a slotted stator, and no analytical formulae for the functional characteristics of the machine have been presented (15). Several papers have been published about the machine's structure with concentrated windings. In (16), advantages, challenges, design process, and analysis of a fractional slot concentrated winding (FSCW) PM machine have been presented. Also, commercial applications of this structure have been studied in this reference. In (17), the previous works done on the optimization of FSCW machines have been presented. Furthermore, the relationships for the winding factor of an FSCW have been given. In (18), a generator with FSCW for use in low-voltage applications in distributed generation has been proposed and designed based on Finite Element (FE) analysis. In (19), a new structure for PM vernier motor with FSCW has been introduced. The induced voltage has been analyzed through FE simulations and compared with analytical calculation results. In addition, besides applying the shifting technique on the stator part, a star/pentagon winding connection has been used. In comparison with the conventional model, the fundamental component of MMF in the new winding structure has been eliminated. For the new winding structure, the winding factor is higher, and the total harmonic distortion (THD) of MMF has been decreased by 2.1% (20). In (21), the influence of the winding layer number on the operation of FSCW interior PM machines has been evaluated. Investigation of the results shows that the high number of layers can improve efficiency and torque density, significantly. In (22), by both methods of the nonlinear magnetic circuit and FEM, the mechanism of reducing magnetic saturation by the FSCW technique has been investigated. A synchronous reluctance machine with FSCW has been introduced, and the stator shifting has been used to eliminate the unwanted MMF harmonics. Also, the comprehensive comparison of the case study machine equipped with FSCW and distributed winding has been presented (23).  In (24), an appropriate winding connection for a PM synchronous machine (PMSM) with FSCW has been presented to decrease the torque ripple. In (25), a comprehensive study on the PMSM with FSCW has been implemented. In this reference, radial and tangential forces, and vibration between the conventional and proposed structures have been compared using FEM.

Our studies show that researchers in the previous papers tried to mitigate the harmonics of MMF. However, these harmonics can be useful for designing a machine with a new structure. In the present paper, a topology for a non-slotted double rotor AFPM machine is introduced that operates based on harmonics components of MMF. This proposed structure has two rotor discs that rotate in opposite directions. The amplitude of the first harmonic of the Back EMF and its higher harmonics have significant influences on the stator current, machine efficiency, and power angle. Therefore, the objective of the present paper is to propose a formula for obtaining the waveform of the Back EMF of the proposed CRDRAFPM machine. For this goal, first, the structure of the CRDRAFPM machine is presented in the next section. Then, Maxwell equations are employed to extract the intended formula. After that, a counter-rotating 40 Watts sample machine is designed, and the Back EMF waveform of this machine is attained by the suggested formula and FEM.

# THE PROPOSED TOPOLOGY

It is evident that the winding MMF of a machine is not pure sinusoidal and contains some harmonic components. Commonly, mitigating the MMF harmonic components is one of the purposes of suggesting a new winding arrangement. Investigating the harmonic analysis of MMF in previously published papers shows that the fifth and seventh harmonics of the MMF often have significant amplitudes. Therefore, decreasing these two dominant harmonics is of interest to machine designers. The fifth and seventh harmonics of the MMF rotate in two different directions. On the other hand, the number of poles in a rotary electrical machine is always even. In this case, it is possible to design a machine with two counter-rotating rotors based on the fifth and seventh harmonics of the MMF. One rotor has 10 poles and is called a ‘High-speed rotor’. The other rotor has 14 poles and is called a ‘Low-speed rotor’. The distributed winding produced harmonics but the generated harmonic components due to the FSCW is usually more significant. Hence, the FSCW is a better candidate to have a machine with high amplitudes of the fifth and seventh harmonics of the MMF. The non-slotted double-rotor Axial Flux Permanent Magnet (AFPM) machine has one stator that is located between two rotor discs. This machine is a suitable choice to design a machine with two different directions of rotation, simultaneously. The topology of the machine is depicted in Fig. 1. In this structure; rotor cores are fabricated from low-carbon steel. The PMs are placed on the surface of the rotor cores. The high silicon–iron alloys are used to fabricate the stator core. A nonmagnetic material is used to avoid entering the magnetic fluxes of one rotor to the other one. The machine has an FSCW type which warped around the stator core, toroidally. The winding arrangement of the machine is shown in Fig. 2. The dot (.) and cross (×) have been used to determine the current direction in each coil. When the stator windings connect to the voltage source and the current flows in the stator windings, two opposite directions fields are produced on the left and right sides of the stator. Therefore, one rotor rotates clockwise, and the other rotor rotates counterclockwise, simultaneously.

Fig. 1. The topology of the CRDRAFPM machine.

Fig. 2. The winding arrangement of the machine.

# the new formula for obtaining the back emf waveform

Knowing the paths of magnetic fluxes in the machine is necessary for extracting a formula for determining the waveform of Back EMF. In PM machines, most of the fluxes in the machine are produced by PMs (26). The magnetic flux lines due to PMs within the machine are shown in Fig. 3. As is evident from Fig. 3, the magnetic fluxes due to PMs of one rotor enter the stator core and pass through it circumferentially. Then these fluxes return to the opposite adjacent poles in the identical rotor. In this case, the magnetic fluxes due to the PMs of one rotor do not enter the other rotor. Therefore, fluxes of PMs of the High-speed rotor induce voltage only in the winding arms of the stator, which are placed in front of the High-speed rotor. A similar analysis exists for the voltage induced in the stator winding arms located opposite the Low-speed rotor. In this case, it is possible to calculate the induced voltages in the arms of stator windings placed opposite the High-speed and Low-speed rotors, independently. After that, the machine Back EMF is attained by algebraic summation of the voltages of the stator winding arms. For this purpose, the algorithm revealed in Fig. 4 is suggested. Based on the Faraday's first law of electromagnetic induction, the induced voltages in the stator winding arms are calculated by derivation from linkage fluxes. Therefore, it is necessary to calculate linkage fluxes before obtaining induced voltages. For this purpose, at the initial step, the magnetic flux density along vertical axes in the winding area is determined. Then the winding linkage fluxes are calculated using magnetic flux density. After that, the induced voltage in each arm of stator windings is calculated by derivation from the linkage fluxes of that arm. Finally, the waveform of the machine Back EMF is determined by summing the induced voltages in the stator windings due to two rotor PMs.

Fig. 3. The paths of magnetic fluxes in the machine

Fig. 4. The suggested algorithm for calculating the waveform of machine Back EMF

A. The induced voltage due to Low-speed rotor PMs

For obtaining a formula for determining the waveform of the Back EMF, the calculations are done at the machine’s average diameter. The part of the two-dimensional model of the machine at the average diameter is depicted in Fig. 5.

Fig. 5. The part of the two-dimensional structure of the machine

Considering the drops of MMF in the cores of the stator and rotor in location x0 by MMFcs(x0) and MMFcr(x0), the PM remanent flux density B'r is calculated as (27):

(1)

In which x0 is a number between 0 and τp/2. The values of B'r for different values of x0 are determined using (1). Then a Fourier series is applied to the values of B'r(x0) as follows (27):

(2)

In which parameters b0, b1, ξ, γ1, b2, and γ2 are determined easily using the technique of curve fitting.

The magnetization vector function M΄y(x) for different values of y in the range of [0, lpm] is obtained as (27):

(3)

In which K'n is calculated as (27):

(4)

The magnetic flux density in the winding zone along the vertical axis for lpm+g<y<lpm+g+lw are obtained as:

(5)

Where, θm(t) denotes the angular location of the middle of pole N relating to the middle of coil A1.

After calculating ByI(x,y,t), the flux linkage of one coil is obtained as:

(6)

In which Δxij and Δyij are the horizontal and vertical distances of the conductor positioned in column j of layer i from the middle of winding, respectively. It should be noted that θPH denotes the phase angle. This parameter for phase a is equal to zero.

The induced voltage in arms A1 of winding of phase a is obtained as:

(7)

The induced voltages in other arms of winding of phase a are calculated using an identical method. However, it should be noted that a shift angle exists between the voltage of each arm. Therefore, the induced voltages in the adjacent arms of winding of phase a are calculated as:

(8)

In which, K is an integer number in the range of [1, 4]. According to Fig. 2, the winding of phase a on the Low-speed side has 8 arms in which 4 arms are shifted 180 degrees related to the other 4 arms. Therefore, the induced voltage in these four arms of winding of phase a is determined as follows:

(9)

The induced voltage in the arms of stator windings on the Low-speed side is obtained as:

(10)

In this case, the induced voltage will be equal to:

(11)

B. The induced voltage due to High-speed rotor PMs

Using a similar procedure, the induced voltages in the stator windings on the High-speed side are obtained. In this case, the induced voltage in arms of winding of phase a on the High-speed side is obtained as:

(12)

C. The Back EMF of the machine

After calculating induced voltages in the arms of each phase coils, it is possible to obtain the Back EMF waveforms of that phase. For this purpose, the waveform of the Back EMF is determined by summing the induced voltages as follows:

(13)

# FINITE ELEMENT EVALUATION

To evaluate the precision of the proposed formulae, numerous machines with different power have been designed and simulated. Here the obtained results of a sample machine are presented. The specifications of this machine are given in Table 1. The materials of each part of the machine are determined in Table 2. The machine has been tested under open circuit and full load operating conditions. In the following, the results obtained from various simulations have been presented.

A. Open circuit conditions

The FE simulations have been used to evaluate the operation of the machine. Fig. 6 shows the distribution of flux density in the machine structure. This flux density distribution has been obtained at open circuit conditions. As is evident, no part of the machine has gone to saturation. The induced voltages in the stator windings due to the magnetic fluxes of PMs of the Low-speed rotor obtained from FE simulation and (11) are illustrated in Fig. 7. In addition, Fig. 8 shows the induced voltages due to the magnetic fluxes of PMs of the High-speed rotor calculated by FE simulation and (12). As is evident, there is a good compromise between the results of the two methods. The Fast Fourier Transform (FFT) algorithm has been used to determine the harmonic components of the attained induced voltages. The obtained results are depicted in Fig. 9. According to this figure, the obtained voltages have small harmonics. To obtain the machine Back EMF, the induced voltages due to the PMs of two rotors are added together. However, the initial position of two rotors is one of the important parameters that affects the waveform of the machine Back EMF. For better investigation, the amplitudes of the fundamental component and total harmonic distortion (THD) of the machine Back EMF waveform for some relative positions of two rotors are given in Table 3. As is clear, the initial location of two rotor discs has a significant influence on the amplitude of the first harmonic and THD of the machine Back EMF. In addition, when the High-speed rotor is shifted 36 degrees related to the Low-speed rotor, the THD of the machine’s Back EMF is minimum. Furthermore, the load angle of the machine depends on the positions of two rotor discs. The output power applied on each rotor affects the rotor position and, consequently load angle. Hence, to avoid the study complexity, it is assumed that the output power of two rotor discs is identical. In this case, the amplitude of the first harmonic of induced voltages of two rotor PMs must be equal. According to Fig. 9, the amplitude of the first harmonic for voltages shown in Fig. 7 and Fig. 8 are equal to 30 volts. The waveforms of the Back EMF attained from FE software and (13) are illustrated in Fig. 10. As is clear, the results of the two methods have a minor difference. For better comparison, the Normalized Root Mean Square Error (NRMSE) procedure has been used to determine the difference between the results. The NRMSE between the waveforms attained from the suggested formula and FE simulation is about 4%. This slight difference approves the high precision of the proposed formula.

Table 1 Specification of the sample machine

| Value | Parameter |
| --- | --- |
| 40 W | Nominal power |
| 60 V | The peak value of terminal voltage |
| 50 Hz | Rated frequency |
| 1 mm | Air gap length |
| 5 mm | Thickness of PM |
| 4.4 mm | Axial length of winding |
| 7 | Number of pole pairs of the Low-speed rotor |
| 5 | Number of pole pairs of the High-speed rotor |
| 6.6 mm | Axial length of the stator core |
| 4.7 mm | Axial length of the rotor core |
| 145.1 mm | Stator Outer diameter |
| 74.9 mm | Stator Inner diameter |
| 1.6 T | Stator yoke flux density |
| 1.6 T | Rotor yoke flux density |

Table 2 Materials of various parts of the machine

| Part | Material |
| --- | --- |
| Stator yoke | 50A470 |
| Rotor yoke | SUS430 |
| Winding | Copper |
| PM | N45H |
| Nonmagnetic material | Aluminium |

Fig. 6. The flux density distribution within the machine under open circuit conditions

Fig. 7. The induced voltage due to 14 pole rotor

Fig. 8. The induced voltage due to 10 pole rotor

(a)

(b)

Fig. 9. The FFT results of induced voltages a) 14 pole rotor b) 10 pole rotor

Table 3 Back EMF THD for some shifting angle

| shifting angle (Degree) | First harmonic amplitude (V) | Back EMF THD (%) |
| --- | --- | --- |
| 28 | 56.9 | 2.92 |
| 30 | 58.5 | 2.34 |
| 32 | 59.64 | 1.69 |
| 34 | 60.33 | 1.025 |
| 36 | 60.5 | 0.65 |
| 38 | 60.32 | 1.05 |
| 40 | 59.64 | 1.71 |
| 42 | 58.49 | 2.37 |
| 44 | 56.89 | 2.94 |
| 46 | 54.87 | 3.38 |
| 48 | 52.43 | 3.66 |
| 50 | 49.59 | 3.74 |

Fig. 10. The waveforms of machine Back EMF attained from simulation and proposed formula

B. Full load conditions

Based on the linear speed and the average radius of the machine, the angular speed of the machine is calculated as follows:

(14)

Also, the output power of the machine is obtained as follows:

(15)

By substituting (14) in (15), we have:

(16)

Therefore, the output power of each rotor is obtained by multiplying the electromagnetic force and linear velocity of the rotor disc.

To evaluate the machine operation under full load conditions, a mechanical load with a power of 20 watts has been applied to each rotor disc. The electromagnetic forces applied to the Low-speed rotor and the High-speed rotor are indicated in Fig. 11. The mean values of linear speeds of the two rotor discs are shown in Fig. 12. With the help of (16) the output power of two rotor discs is calculated. Calculations confirm that the output power of each rotor disc is 20 watts.

By determining the input and output powers, the efficiency is calculated as follows:

(17)

The input power of the machine is shown in Fig. 13. With the help of (17), the efficiency of the machine would be equal to 80%. The obtained results approve that the machine has suitable performance characteristics.

Fig. 11. The electromagnetic forces applied to two rotors

Fig. 12. The linear velocity of two rotors

Fig. 13. The input power

# conclusion

In this paper, a new structure of the FSCW-DRAFPMSM has been introduced. This structure consists of two PM rotors with various numbers of poles and different rotating directions. Since the FEM simulations are time-consuming, some precise analytical formulae for calculating machine Back EMF based on Maxwell's equations have been proposed. The FEM has been used to validate the performance characteristics of the proposed topology and formulae. The obtained results confirm that:

- The new structure can meet the required power while two rotors of the machine rotate in opposite directions with different speeds, simultaneously.

- The difference between the results of the analytical formula and FEM for calculating Back EMF is less than 4%. Therefore, the analytical formula has high accuracy.

- The position of the two rotors has a considerable influence on the Back EMF THD and the amplitude of harmonics.

- The machine Back EMF has a maximum amplitude and minimum THD when two rotor discs are shifted 36 degrees related to each other. In this case, the amplitude of the fundamental component of the Back EMF is about 60.5 V. In addition, the THD of the Back EMF is about 0.65%.

- The efficiency of the sample machine is about 80%. It is possible to attain higher efficiency by optimal design of the machine.

References

1. 	Jahns TM. The expanding role of PM machines in direct-drive applications. In: 2011 International Conference on Electrical Machines and Systems. IEEE; 2011. p. 1–6.

2. 	Piech J. Permanent magnet machines for elevators in super high-rise buildings. Counc Tall Build Urban Habitat CTBUH. 2014;823.

3. 	Mirnikjoo SA, Abbaszadeh K, Abdollahi SE. Multiobjective design optimization of a double-sided flux switching permanent magnet generator for counter-rotating wind turbine applications. IEEE Trans Ind Electron. 2020;68(8):6640–9.

4. 	Kutt F, Blecharz K, Karkosiński D. Axial-flux permanent-magnet dual-rotor generator for a counter-rotating wind turbine. Energies. 2020;13(11):2833.

5. 	Mirnikjoo SA, Asadi F, Abbaszadeh K, Abdollahi SE. Effect of rotor topology on the performance of counter-rotating double-sided flux switching permanent magnet generator. IEEE Trans Energy Convers. 2021;37(1):65–74.

6. 	Ullah W, Khan F, Hussain S. A novel dual rotor permanent magnet flux switching generator for counter rotating wind turbine applications. IEEE Access. 2022;10:16456–67.

7. 	Gulec M, Aydin M. Magnet asymmetry in reduction of cogging torque for integer slot axial flux permanent magnet motors. IET Electr Power Appl. 2014;8(5):189–98.

8. 	Qiu J, Shi C, Jin M, Lin R. Counter-rotating permanent magnet brushless DC motor for underwater propulsion. In: 2006 CES/IEEE 5th International Power Electronics and Motion Control Conference. IEEE; 2006. p. 1–5.

9. 	Caricchi F, Crescimbini F, Santini E. Basic principle and design criteria of axial-flux PM machines having counter-rotating rotors. In: Proceedings of 1994 IEEE Industry Applications Society Annual Meeting. IEEE; 1994. p. 247–53.

10. 	Zhong Y, Huang S, Luo D, Wu X. Characteristics of an axial-flux permanent magnet synchronous machine with contra-rotating rotors under unbalanced load condition from 3-D finite element analysis. CES Trans Electr Mach Syst. 2018;2(2):220–5.

11. 	Liu G, Qiu G, Shi J, Zhang F. Study on counter-rotating dual-rotor permanent magnet motor for underwater vehicle propulsion. IEEE Trans Appl Supercond. 2018;28(3):1–5.

12. 	Cros J, Viarouge P. Synthesis of high performance PM motors with concentrated windings. IEEE Trans energy Convers. 2002;17(2):248–53.

13. 	Magnussen F, Sadarangani C. Winding factors and Joule losses of permanent magnet machines with concentrated windings. In: IEEE International Electric Machines and Drives Conference, 2003 IEMDC’03. IEEE; 2003. p. 333–9.

14. 	Zhu ZQ, Azar Z, Ombach G. Influence of additional air gaps between stator segments on cogging torque of permanent-magnet machines having modular stators. IEEE Trans Magn. 2011;48(6):2049–55.

15. 	Hassannia A. Conceptual design of fractional slot concentrated winding dual-rotor double-speed synchronous motor. IEEE Trans Energy Convers. 2019;35(2):986–93.

16. 	El-Refaie AM. Fractional-slot concentrated-windings synchronous permanent magnet machines: Opportunities and challenges. IEEE Trans Ind Electron. 2009;57(1):107–21.

17. 	Dajaku G, Spas S, Gerling D. Advanced optimization methods for fractional slot concentrated windings. Electr Eng. 2019;101(1):103–20.

18. 	Andrada Gascón P, Blanqué Molina B, Martínez Piera E, Torrent Burgués M, Sánchez López JA, Perat Benavides JI. Fractional-slot permanent magnet synchronous generator for low voltage applications. Electr Eng Electron J. 2013;1(2).

19. 	Kim B. Investigation on slot–pole combinations of a PM vernier motor with fractional-slot concentrated winding configurations. Energies. 2017;10(9):1310.

20. 	Zhao B, Gong J, Tong T, Xu Y, Semail E, Nguyen N-K, et al. A novel five-phase fractional slot concentrated winding with low space harmonic contents. IEEE Trans Magn. 2021;57(6):1–5.

21. 	Reddy PB, El-Refaie AM, Huh K-K. Effect of number of layers on performance of fractional-slot concentrated-windings interior permanent magnet machines. IEEE Trans Power Electron. 2014;30(4):2205–18.

22. 	Gundogdu T, Komurgoz G. Implementation of fractional slot concentrated winding technique to large salient-pole synchronous generators & development with permanent magnets. Electr power Syst Res. 2013;105:57–70.

23. 	Taghavi Araghi SM, Kiyoumarsi A, Mirzaeian Dehkordi B. Synchronous reluctance machines with new type of fractional‐slot concentrated‐windings based on the concept of stator slot shifting. IET Electr Power Appl. 2023;17(1):92–108.

24. 	Li Z, Xia J, Guo Z, Lu B, Ma G. Analysis and Reduction of Pole-Frequency Vibration of Surface Mounted Permanent Magnet Synchronous Machines with Fractional Slot Concentrated Winding Considering the Radial and Tangential Forces. IEEE Trans Transp Electrif. 2022;

25. 	Munteanu A. Six-phase Fractional Slot Concentrated Winding Permanent Magnet Synchronous Motor with Reduced Torque Ripple. In: 2022 International Conference and Exposition on Electrical And Power Engineering (EPE). IEEE; 2022. p. 527–30.

26. 	Park H-J, Jung H-K, Jung S-Y, Chae Y-H, Woo D-K. Field reconstruction method in axial flux permanent magnet motor with overhang structure. IEEE Trans Magn. 2017;53(6):1–4.

27. 	Mirzahosseini R, Darabi A, Assili M. Analytical and experimental analysis of Back EMF waveform of a TORUS-type non-slotted axial flux permanent magnet synchronous machine with shifted rotor. Meas J Int Meas Confed. 2020;156.