A wideband cylindrical conformal microstrip mimo antenna array

Song-Song Qian, Ke-Long Sheng, Song-Zhao Zhou, Xun-Peng Long, Zhi-Yuan Zong and Wen Wu

In this paper, a wideband cylindrical conformal microstrip antenna array employing a proximity-coupled feeding mechanism with a cavity-backed configuration is designed and fabricated. Compared with other conformal microstrip patch antennas by using linear subarrays assembled piecewise, this design uses Teflon instead of traditional dielectric layers, and makes it possible to process the whole conformal array without splicing, obtaining the freedom in unit size and array radius adjustment except ease of manufacturing and assemblage. Combined with the optimization of the cavity size, an array with 44 elements is obtained which has a bandwidth of 40% from 8 to 12GHz and a gain of 16.4 dB.

Introduction: Compared with planar antennas, conformal antennas release the inner space of the platform without destroying the aerodynamic shape, and it has unique superiority in wide-angle coverage, increased radiation aperture, and low antenna radar cross section (RCS) [1]. Most conformal antennas are composed of microstrip patch antennas due to their low profile, simple structure, and ease of conforming to the nonplanar mounting surfaces. However, microstrip patch antennas suffer from the intrinsic property of narrow bandwidth performance as a resonant antenna. Researchers have devised several ways to increase the bandwidth, such as adding stacked patches, designing U-slot patches, utilizing the shorting pins or shorting walls, or changing the feeding mechanism [2]-[6]. Sun et al. first propose a microstrip unit antenna with 40% bandwidth utilizing the proximity-coupled feeding mechanism [7], then extend it to a conformal array with a radius about 500 mm [8]. However, since the array is assembled piece by piece with non-negligible discontinuity and mutual coupling between the element, the bandwidth of the conformal array drops to 27%. At the same time, the overall radius of the array is strictly limited by the size and thickness of the element, otherwise the gap in the splice will affect the overall performance of the array antenna, and this makes conformal array manufacture with small radius difficult.

In this paper, a wideband conformal microstrip array antenna employing a proximity-coupled feeding mechanism is designed. In the design, the radiation patch is printed on a very thin dielectric layer which is stick to the surface of a thicker Teflon layer. Because the thin dielectric layer is easy to bend and the Teflon layer is easily to be manufactured into a conformal structure, then a 4×4 array prototype with a smoothly curved surface is fabricated, which is axially mounted onto a cylindrical surface with a small radius of 100 mm. By optimizing the coupling cavity size [7], an conformal array with impedance bandwidth of 40 % (S11<-10 dB) from 8 to 12 GHz and a gain of 16.4 dB is obtained.

Antenna Design: Considering the design efficiency, the plane element with the required bandwidth is designed first. Fig.1 presents the geometry of the antenna unit cell.

(a)                                           (b)

Fig 1 Structure of the antenna unit cell. (a) 3-D view. (b) Side view.

As shown in Fig.1, the element is comprised of three substrate layers and a cavity-backed ground plane.  Layer 1 is a 0.127 mm thin layer of Rogers 5880 dielectric substrate (), which is easy to bend, and the radiation patch is printed on it. Layer 2 is a 3 mm Teflon dielectric substrate layer () to ensure conformal manufacture. The feed line is printed on the top of Layer 3 (Rogers 5880, h3=0.508 mm). A 4 mm aluminum plate is attached to the bottom of the dielectric layers, holding the 50Ω coaxial connector, and the coax probe is vertically connected to the feed line.

Fig 2 Simulated reflection coefficient of the plane antenna element.

Layer 2 is designed relatively thick to lower the Q value and broaden the bandwidth, however, the patch will be under-coupled since the antenna layer is too thick. To alleviate this contradiction, turning the solid GND into a GND with a narrow cavity is a viable method [7]. The width (Wc), length (Lc), and depth (Hc) of the cavity will influence the coupling, resonance, and input impedance of the antenna. Besides, the overlap area between the patch and the feed line (Wm*Lm1) will affect the coupling as well. The optimized antenna element parameters are listed in Table 1. As shown in Fig.2, the simulated bandwidth (S11<-10dB) is from 7.92-12.07 GHz (41.5%), covering the X-band. The simulated radiation patterns of the antenna element at 8 and 12 GHz are demonstrated in Fig.3. Overall, the cross-polarization level is lower than -20 dB in the E- and H-plane over the operating band. However, considering the antenna layer is thick, the surface wave could be excited more effectively at higher frequencies, and meanwhile, the vertical plane structure is asymmetric, leading to the slight asymmetric pattern in E-plane at 12 GHz [7].

(a)                                         (b)

Fig 3 Simulated realized gain patterns of the plain antenna element in the E- and H-planes at (a) 8GHz and (b) 12GHz.

Table 1. Parameters for the antenna element

| Lp | Wp | Lm | Wm | Lm1 | Wc | Lc | Hc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.7 | 6.7 | 13.5 | 1.7 | 5.05 | 5 | 14 | 3 |

Unit: mm

Compared with conformal arrays constituted by separate pieces, the designed array with a completely smooth curved surface can introduce parasitic structures to mitigate the effects caused by mutual coupling and curved shape. A cylindrically conformal array is constructed using the 4×4 designed elements, as shown in Fig.4. Its cross-section is a sector with a radius of 100 mm and a central angle of about 57.3°. To move grating lobes out of the visible region, the interelement spacing (d) can be evaluated by , where  is the element number () and we choose the operating wavelength at the highest frequency (). Thus, d should be less than 21.8 mm. The optimized parameters are listed in Table 2.

Fig 4 3-D model of the 4×4 cylindrically conformal array. The top view of the element is shown on the right. Each element is numbered from 1 to 16.

Table 2. Parameters for the antenna array

| Lp | Wp | Lm | Wm | Lm1 | Wc | Lc | Hc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.7 | 6.7 | 12.5 | 1.8 | 4.65 | 6 | 13 | 3 |

Unit: mm

Fig. 5 presents the realized gain patterns of the 4×4 array in the E-plane at 8 and 12 GHz. The orthogonal polarization coupling (between main- and cross-polarization) was found to be below -25 dB over the operating band. However, different from planer arrays, the elements in a curved conformal array have misaligned patterns and maximum directivities, which leads to a slight uplift of the side lobes, especially at higher frequencies (rising to -7 dB at 12 GHz in this case) [9]-[10].

(a)                                         (b)

Fig 5 Simulated realized gain pattern of the conformal array in the E-plane (red line) and H-plane (blue line) at (a) 8GHz and (b) 12GHz.

Fabrication and Measurements: A 4×4 cylindrical conformal array prototype was fabricated and assembled, as shown in Fig. 6. Layer2 and GND are processed into curved planes by lathe, and Layer 1 and Layer 3, the thin PCB layers, are mounted on them. The whole structure is fixed by screws. In this case, we do not need to approximate the smooth surface by several planar facets.

(a)                                         (b)

Fig 6 (a) Fabricated 4 × 4 cylindrically conformal array prototype. (b) Measurement setup.

Fig.7 (a) shows the reflection coefficients of the conformal array, and the reflection coefficients (S11) are typically less than -10 dB over the operating band except for a few frequency points. The measured mutual coupling between two neighboring elements of the center element is shown in Fig.7 (b), and the maximum mutual coupling is -16.6 dB.

(a)                                           (b)

Fig 7 (a) Measured reflection coefficients of the conformal array. Four units on the diagonal line were chosen to represent the conformal array. (b) Measured mutual couplings of the center element.

Fig. 8 presents the measured and simulated radiation patterns of the 4×4 conformal array at frequencies of 8, 10 and 12 GHz. The measured orthogonal polarization coupling is below -25 dB over the operating band. Considering the effects of the cylindrical conformal structure, the side lobes are found to increase to -7.8 dB at 12 GHz.

Fig 8 Realized gain patterns of the conformal array in the E- and H-planes at 8, 10 and 12 GHz. Measured patterns (dotted line) are compared against simulations (solid line).

The designed conformal microstrip array is compared with some published conformal arrays based on patch antennas, as summarized in Table 3. The bandwidth and impedance matching are better than those reported in [8][11]-[13].

Table 3. Compare the proposed antenna with related works

| Reference | Number of elements | Typical bandwidth | Typical Gain (dB) | minimum radius of curvature | Splicing or not |
| --- | --- | --- | --- | --- | --- |
| [8] | 48×4 | 27.6% (8.54-11.28 GHz) | 12.28* @10.9GHz | 500 | yes |
| [11] | 24×4 | 13% (8.9-10.2 GHz) | 23 @9.4GHz | 83 | yes |
| [12] | 2×8 | 20% (7.16-8.76 GHz) | 6.33 @ 8.76GHz | - | yes |
| [13] | 2×2 | 3.67% (9.47-9.83) | 10.6 @ 9.65GHz | 70 | not |
| This work | 4×4 | 40% (8-12 GHz） | 16.4 @ 10GHz | 100 | not |

* 1×4 subarray

Conclusion: A wideband microstrip conformal array based on a proximity-coupled feeding mechanism is presented in this work, and the GND is dug with a cavity to improve the radiation and coupling efficiency. The whole structure of the conformal array has a smooth curved surface, which can avoid the impact caused by discontinuous splicing. At the same time, because of the integrated conformal structure, the spacing between the elements can be adjusted freely to reduce the coupling.  The proposed design is easy to fabricate, and the performance of antenna can be ensured easily.

Song-Song Qian, Ke-Long Sheng,  Song-Zhao Zhou, Xun-Peng Long, Zhi-Yuan Zong and Wen Wu (Key Laboratory of Near-Range RF Sensing ICs & Microsystems (NJUST), Ministry of Education, School of Electronic and Optical Engineering, Nanjing University of Science and Technology, Nanjing 210094, China)

E-mail: qianss@njust.edu.cn

References

- R. J. Mailloux, Phased Array Antenna Handbook, 2nd ed. Boston, MA: Artech House, 2005, pp. 185.

- A. Panther, A. Petosa, M. G. Stubbs and K. Kautio: A wideband array of stacked patch antennas using embedded air cavities in LTCC. IEEE Microwave and Wireless Components Letters. 15(12), 916-918 (2005).

- S. Weigand, G. H. Huff, K. H. Pan and J. T. Bernhard: Analysis and design of broad-band single-layer rectangular U-slot microstrip patch antennas. IEEE Transactions on Antennas and Propagation. 51(3), 457-468 (2003)

- S. Wi, Y. Lee and J. Yook: Wideband Microstrip Patch Antenna With U-Shaped Parasitic Elements. IEEE Transactions on Antennas and Propagation. 55(4),1196-1199 (2007)

- A. K. Shackelford, Kai-Fong Lee and K. M. Luk: Design of small-size wide-bandwidth microstrip-patch antennas. IEEE Antennas and Propagation Magazine. 45(1), 75-83 (2003)

- R. B. Waterhouse: Microstrip Patch Antennas: A Designer’s Guide. Boston, MA: Kluwer Academic, 2003

- D. Sun and L. You: A Broadband Impedance Matching Method for Proximity-Coupled Microstrip Antenna. IEEE Transactions on Antennas and Propagation. 58(4), 1392-1397 (2010)

- D. Sun, W. Dou and L. You: Application of Novel Cavity-Backed Proximity-Coupled Microstrip Patch Antenna to Design Broadband Conformal Phased Array. IEEE Antennas and Wireless Propagation Letters. vol. 9, 1010-1013 (2010)

- F. Zhao: Analysis and Synthesis Study of Conformal Phased Antenna Array and Experiment. Ph.D. dissertation, Electronics Science and Technology, School of National University of Defense Technology, Hunan, P. R. China, 2012.

- F. Rigobello, G. Mansutti, M. S. Khan and A. Capobianco: Pattern recovering of conformal antenna array for strongly deformed surfaces. In 2017 11th European Conference on Antennas and Propagation (EUCAP). 869-871 (2017)

- P. Knott: Antenna design and beamforming for a conformal antenna array demonstrator. In IEEE Aerospace Conference, Big Sky, MT, USA (2006)

- Y. Yan, Y. Jiao and W. Zhang: Design of an X-band Antenna on Airborne Craft with Omnidirection Radiation. In 12th International Symposium on Antennas, Propagation and EM Theory (ISAPE), Hangzhou, China (2018)

- B. A. Nunna and V. K. Kothapudi: Design and Analysis of Single Layer C/X-band Conformal Shared Aperture Antenna Array for Spaceborne SAR Applications. In IEEE Microwaves, Antennas, and Propagation Conference (MAPCON), Bangalore, India (2022)