Dynamic Restoration Electricity Price Optimization Method to Enhance the Resilience of Distribution Networks with MMG

Hongkun Wang1, Yujie Gao1, Hong Zhang*1, Dongmei Yan1, Hongwei Li1

1 College of Mechanical and Electrical Engineering, Shihezi University, Shihezi, 832000, China

* Corresponding author：zhanghong1802@163.com; No. 221, North 4th Road, Shihezi City, Xinjiang

Author Contributions: Hongkun Wang supervised the study, coordinated the investigations, and checked the manuscript’s logical structure. Yujie Gao validated the simulation and data curation. Hong Zhang wrote this article, designed the optimization methods, drew the figures, and performed the simulation as well as the calculus analysis. Dongmei Yan reviewed the writing and editing. Hongwei Li provided practical suggestions and evaluated the feasibility of the application. All authors have read and agreed to the published version of the manuscript.

Funding：This work was supported by Corps Key Laboratory Open Project of Modern Agricultural Machinery (XDNJ2022002)；Shihezi University Independent Project (ZZZC2022003;ZZZC202107)

Conflict of Interest statement：The authors declare no conflict of interest.

Permission to reproduce materials from other sources：Not applicable.

Data Availability statement：Not applicable.

Abstract: Resilience is one of the main features of smart distribution network, and a microgrid access to the distribution network provides an effective way to improve resilience. Microgrid and distribution network belong to different interests, so it is necessary to use price leverage to actively guide microgrids and flexible resources within microgrid to participate in post-disaster restoration of distribution network and enhance the resilience. Firstly, this paper proposes a dynamic restoration electricity price response mechanism for distribution network after extreme disasters and constructs a power response model for loads and electric vehicles within the microgrid. Secondly, the optimal scheduling model of distribution network with multiple-microgrid is proposed to improve the restoration rate of critical loads(RRCL) in the distribution network. Single microgrid achieves the largest microgrid revenue and restoration contribution, and multi-microgrid uses the power headroom index to optimize the dynamic restoration electricity price to achieve the smallest power purchase cost of distribution network. Finally, the optimal scheduling method for resilience enhancement of distribution networks with MMG considering dynamic restoration electricity price response mechanism is validated by dual microgrid access to IEEE 33-node distribution system. The simulation results show that the proposed optimization method effectively improves the RRCL of distribution network.

Key word: multi-microgrids; resilience; dynamic electricity price; restoration rate of critical loads; optimization scheduling

Nomenclature

| PHIM | the PHI of MMG |
| --- | --- |
|  | the critical load expectation value of NPF |
|  | the total load power at time t of the mth microgrid |
|  | the interruptible load power |
|  | the emergency regulation load power |
|  | the actual load demand power |
|  | the critical load power |
| , | the control variables of interruptible load and emergency regulation load |
|  | the charge satisfaction indices |
|  | the discharge satisfaction indices |
| tia | the EV arrival time |
| tid | the EV departure time |
|  | the EV maximum charge/discharge response start time |
|  | the initial EV charge/discharge capacity |
|  | the maximum allowable charge/discharge capacity of the EV battery |
|  | the desired charge/discharge capacity |
|  | the minimum allowable charge/discharge capacity |
|  | the capacity of charging/discharging the EV at time t |
|  | the charging power of EV at time t |
| , | the charging/discharging efficiency |
|  | the maximum allowable charging/discharging power of EV |
|  | the total revenue of the microgrid m |
|  | the revenue of electricity sales |
|  | the controllable load compensation cost |
|  | the microgas turbine fuel cost |
|  | the contribution rate for CRCL of the microgrid m to the passive microgrid |
|  | the time-of-use price for microgrid |
|  | the contracted compensation tariff for interruptible load |
|  | the compensation price for emergency regulated load |
|  | the dynamic RP for distribution network NPF |
|  | the MG incoming power |
|  | the distribution network NPF expected power |
| T0,Td | the distribution network fault start time and fault end time |
|  | the natural gas price |
|  | the low calorific value of natural gas |
|  | the start-up state parameter of gas turbine |
|  | the start-up cost of gas turbine |
|  | the dispatch time |
|  | the gas turbine slope rate |
|  | the gas turbine climb rate |
|  | the electricity price during the restoration period |
|  | the basic electricity price |
|  | the highest electricity price |
|  | the restoration promotion electricity price |
|  | the power headroom index under the basic electricity price |
|  | the power headroom index under the highest electricity price |

- Introduction

The safe and reliable operation of the power system is one of the important guarantees for industrial production and social life in modern society. However, the frequent occurrence of extreme natural disasters have caused large-scale and widespread power disruptions, resulting in huge social and economic losses and bringing unprecedented challenges for the power system. According to statistics, about 80% of major power outages are caused by natural disasters, and distribution network accidents account for about 80% to 90%, which highlights the lack of resilience and effective measures to deal with extreme disasters for the distribution network. With the development of distributed energy, the distribution system gradually becomes energy utilization center, and the modern distribution system gradually becomes large and complex physical network system, so the resilience enhancement of the distribution system becomes more and more urgent and important.

At this stage, scholars at home and abroad have researched and achieved certain results on power system resilience. The literature [1] summarizes the views of scholars and proposes that resilience refers to the ability of the power system to prevent, resist, and quickly restore load in response to extreme events with low probability-high loss. The literature [2] reviews the definition of resilient power grids and its key characteristics, and proposes that resilience is the core characteristic. The literature [3] proposes that the integrated energy system is an important development direction to enhance the resilience of distribution networks. In terms of resilience enhancement, prevention, resilience and recovery measures are mainly focused on the prevention and preparation phase, resistance and infiltration phase and rapid recovery phase when the power system is hit by extreme disasters. The literature [4-5] propose an optimal allocation method for mobile energy storage to enhance the resilience of the distribution network, which effectively improves the ability of the distribution network to withstand disasters. The literature [6] proposes an integrated resilience enhancement strategy to achieve resilience enhancement of the distribution network in the presence of climatic hazards. The literature [7] considers the relevance of multiple uncertainties about natural disasters and proposes a resilience enhancement investment plan for distribution networks based on probabilistic values. The literature [8] constructs a planning framework based on deep reinforcement learning, and enhancing the long-term resilience of distribution systems through reinforcement strategies. The literature [9] proposes an integrated planning framework to improve the combined resilience of distribution network and water distribution network to withstand hurricanes. In the literature [10], an optimal configuration method of gas turbine and gas storage system in an electric-gas interconnected system is proposed, and analyzes the resilience level of the system under different scenarios.

Coordinated optimal scheduling of distributed generators and energy storage can achieve rapid post-disaster restoration and reduce the interruption time of power loads and power. The literature [11] proposes a joint planning-attack-network reconfiguration optimization model of DG to enhance the resilience of the distribution network. In the literature [12], a hierarchical management strategy is proposed to improve the anti-risk ability and post-disaster resilience of the distribution network considering the coordination of regional multi-energy systems. Based on the constructed probability model of low-frequency and high-loss events, the literature [13] proposes an optimal scheduling method under extreme events with renewable energy generators participating in the day-ahead energy and spinning reserve market, which explores a new way to improve the resilience of the distribution network from the perspective of the electricity market. In literature [14], it considers emergency operation decisions and restoration decisions that can minimize the loss of power system load during restoration. In literature [15], it proposes a two-stage approach to enhance the resilience of the distribution network by reconfiguring the intentional islanding of the distribution network considering of extreme events. With the continuous development of microgrid technology, the study of microgrid participation in enhancing the resilience of distribution networks is becoming a new direction of interest. The literature [16] proposes a resilience enhancement model of distribution networks with the participation of micro-energy grids, and establishes a rolling management scheme for cold, heat, and electric loads under natural disasters, which achieves effective support of resilience of distribution networks by micro-energy grids. The literature [17] proposes a differentiated operation model of distribution networks with microgrids, which is used to enhance the post-disaster restoration of distribution networks.

All of the above-mentioned literatures all focus on enhancing the resilience of distribution network from technical measures such as infrastructure reinforcement, plan and optimization of energy storage and distributed power sources, without considering the power market mechanism. This paper provides a new way to explore the power market mechanism to improve the resilience of distribution networks with multiple microgrids through the optimization of dynamic electricity price. Firstly, it constructs a dynamic restoration electricity price response mechanism containing the linkage between electricity price and power headroom index after extreme disasters in microgrid distribution networks, and establish a power response model for load and electric vehicles within the microgrid to characterize the response of electricity price. Secondly, coordinated optimal scheduling model of MMG considering dynamic restoration electricity price response mechanism to enhance RRCL of distribution network is proposed. The single microgrid optimization model achieves the maximum revenue and restoration contribution of distribution network, and transmits the grid response power from the microgrid to the distribution network. Construction of a three-step dynamic restoration tariff optimization model for distribution networks containing microgrids to achieve multi-microgrid optimization with maximum power purchase cost for restoration of critical loads in distribution networks. Finally, the proposed optimal scheduling method considering restoration electricity price response mechanism is validated by dual microgrid access to IEEE 33-node distribution system, and the simulation results show that the method can effectively improve the RRCL of the distribution network.

- MMG system after a distribution network disaster

  - Multi-microgrid structure after distribution network disasters due to faults

The distribution network is attacked by extreme natural disasters, resulting in multiple failures in the distribution network and making it is extremely easy to form isolated areas of power loss. Therefore, the MMG structure in the recovery phase after an extreme disaster is not exactly the same as the MMG structure in the normal operation phase. After an extreme disaster, the distribution network is cut into a number of island networks by faulty lines or equipment, such networks do not have ability to supply power, temporarily called the network of power failure (NPF). If the NPF is connected to other MGs in the distribution network after an extreme natural disaster, a certain amount of power can be provided to restore power of critical loads in the NPF when the power in the MGs is sufficient, which enhances the RRCL of the distribution network. The formed MMG structure due to faults in the post-disaster recovery phase of the distribution network is shown in Figure 1.

Fig.1  MMG structure in recovery phase of the distribution network

The MMG structure of the distribution network after an extreme disaster can be regarded as a two-layer agent structure. The distribution network layer mainly considers NPF, which is part or all of the distribution network. In this paper, it is assumed that the distribution network does not lose the coordinated control function of NPF under extreme disasters, only fails to realize primary power transmission, and also assumed that the disaster resistance strength of microgrid is higher than that of distribution network. If the actual microgrid is damaged, the support of the microgrid to the resilience of the distribution network can be disregarded, which does not affect the validity of the proposed method. In order to fully utilize the effect of MG on the resilience enhancement of the distribution network, the resilience of critical loads are generally given priority[16]. In this paper, the load resilience of NPF only considers the critical loads. MG agent is able to coordinate with the distribution network and its own internal sources and loads, and maximize the benefits.

  - Electricity price response mechanism for enhancing resilience of the distribution network

In order to enhance the RRCL of NPF, the post-fault resilience enhancement of the distribution system with MMG needs to fully consider the size relationship of the power deficit between grid-connected power available from each MG and load demand of NPF, and it is called the power headroom index(PHI), which is conducive to set dynamic restoration electricity price of NPF. Therefore, the PHI of MMG is established to describe the size relationship between the two, and its expression is shown in equation (1).

If , the MMG feed-in power meets the NPF critical load recovery, if , the MMG feed-in power does not meet the NPF critical load recovery, and the distribution grid restoration electricity price should be increased to motivate and guide the controllable loads, EVs, and gas turbines to actively participate in post-disaster critical load restoration in the NPF.

Therefore, the MMG participates in coordinated operation of the distribution network resilience enhancement needs to be considered in the following 3 aspects.

(1) The optimal scheduling of single MG system needs to consider the operating characteristics and economy of DG, load and EV to meet the safe and economic operation of MG.

(2) Considering the limitation of MG’s internal power supply during island operation, NPF only considers the recovery of critical loads. At the same time, to balance the interests of multiple parties in the distribution network and MG, it is necessary to formulate a reasonable dynamic restoration electricity price mechanism, and the price mechanism is used to guide multiple resources in MG to participate in the restoration of critical loads in the distribution network.

(3) Through a prior agreement, the microgrid transmits information to the distribution network during the post-disaster recovery phase of the distribution network, and the distribution network develops a unified dispatch plan.

During the post-disaster recovery period, a linkage mechanism between electricity price and power headroom index is adopted, the distribution network provides electricity price incentives to the MG, and the MG responds with power to the restoration electricity price of the distribution network. Distribution network optimization to restore tariffs and maximize power headroom index demand, MG intelligent coordination system to optimize the dispatch of response resources within the MG. Under the dynamic electricity price response mechanism, coordinated and orderly power transmission among different interests is ensured, and the resilience of the distribution network is improved. The general steps are as follows.

Step 1: Active distribution network transmits NPF's critical load power headroom index and dynamic tariffs to its intelligent coordination system.

Step 2: Distribution network intelligent coordination system transmits restoration electricity price and power demand to MMG according to the network connection relationship.

Step 3: MG responds to the dynamic restoration electricity price of the distribution network, optimizes the response power of each resource within MG, and transmits it to the distribution network coordination system.

Step 4: The distribution network makes a judgment based on the feed-in response power of MG and the critical load power headroom index of NPF, whether it is necessary to adjust the recovery price, if so, it returns to Step 2, if not, the restoration price response mechanism ends.

The MMG dynamic restoration electricity price mechanism to enhance the resilience of the distribution network is different from the tariff mechanism in normal operation state. It does not require the power generation cost and electricity price information with MGs, nor a centralized market trading platform, and can be of multiple correspondences. That is, one MG corresponds to one NPF and multiple MGs correspond to one NPF, which is suitable for the recovery of critical loads within the NPF during extreme disaster emergencies.

  - Power response model for load and electric vehicles in microgrid

2.3.1 MG load power response model

The MG load is divided into critical load and controllable load. The critical load is not involved in market dispatching and priority needs to be given to ensure power supply of the critical load. The controllable loads are divided into Interruptible loads (IL) and Emergency regulation loads (ERL), where the interruptible loads execute the contract electricity price and the emergency regulation loads assume the power balance in the islanded operation mode of the microgrid. The microgrid load power response model is shown in equations (2) and (3).

(2)

(3)

2.3.2 Electric Vehicle Response Model

As a special electric load, electric vehicles (EV) can effectively guide electric vehicles to charge and discharge in an orderly manner through a reasonable and scientific electricity price mechanism, and realize friendly interaction with the microgrid. In the post-disaster recovery stage of the distribution network, if the market demand increases and the power purchase price of the NPF of the distribution network is reasonable, EVs can choose the appropriate time to charge and discharge.

EV charging and discharging response is mainly reflected in response time and response power, with the response characteristics of charging power and charging time, which can participate in the critical load recovery of NPF in the peak power demand and tariff stage of NPF to obtain the maximum benefit. However, the scheduling response of EVs to the microgrid has autonomous behavioral characteristics, and the power response model of EVs sets the user charging and discharging satisfaction index[18], and the satisfaction index of the whole microgrid can be distributed according to a certain probability. The probability distribution of satisfaction is not as the focus of this paper, and it can be assumed that when the NPF's power purchase price satisfies the user's requirements, the EVs with capacity constraint, arrival time and departure time constraints are involved in the power response. The expressions are shown in Eqs. (4)-(10).

(4)

(5)

(6)

(7)

(8)

(9)

(10)

Equation (5) is the charge/discharge mode constraint, equation (6) is the charge/discharge capacity constraint, equation (7) is the charge/discharge time constraint, and equation (9) is the maximum charge/discharge time.

- Optimal scheduling model for resilience enhancement of distribution networks with multiple microgrids considering dynamic restoration price mechanism

  - Optimal scheduling model of single microgrid

3.1.1 Objective function

A single MG system consists of wind turbines, photovoltaics, energy storages, micro gas turbines, electric vehicles and loads. During a distribution network fault, the single MG receives the dynamic restoration price and load demand from the distribution network, controls and optimizes the controllable resources within the MG, and meets the objectives of maximizing microgrid revenue and distribution network restoration contribution. MG revenues include distribution network sales revenues, electric vehicle charging and discharging costs, interruptible load contract costs, adjustable load compensation costs, and micro gas turbine operation costs. The operation costs of distribution generation and energy storage are ignored due to the relatively short distribution network fault time. The objective function is shown in equation (11).

(11)

Among them,

(12)

(13)

(14)

(15)

(16)

3.1.2 Constraints

The constraint equations of the single microgrid are shown in equation (17) to (25).

(a) Power balance constraint equation

(17)

(b) Interruptible load constraint equation

(18)

(c) Emergency regulation load constraint equation

(19)

(d) Energy storage constraint equation

(20)

(21)

(22)

(e) Microgrid contact line power constraint

(23)

(f) Micro Gas Turbine Constraint

(24)

(25)

In addition to the above constraint equations, it is also included the load balance equations (1) and (2), and the EV constraint equations (3) to (8).

3.1.3 Single microgrid solution method

The proposed single MG optimal scheduling model is a mixed integer linear programming (MILP) model, which can be solved by CPLEX.

  - Multi-microgrid dynamic restoration electricity price optimization model

3.2.1 A three-step restoration electricity price optimization model considering resilience enhancement of distribution networks

Time-of-use (TOU) electricity price are determined by peak valley load difference, which is not suitable for load restoration after extreme disaster failures in the distribution network. Establishing the reasonable dynamic restoration electricity price during a distribution network disaster is conducive to motivating and guiding MG to actively participate in post-fault distribution network critical load restoration. Considering the power headroom index of MMG and the interests of both MG and the distribution network, the dynamic restoration electricity price implements a three-step electricity price mechanism, including basic electricity price, highest electricity price, and restoration promotion electricity price. The basic electricity price is the TOU electricity price formulated during normal operation of the distribution network with the MMG. If the PHI of the microgrid meets the critical load demand of the distribution network under the power response based on the basic electricity price, the restored electricity price is the Time-of-use electricity price. The highest electricity price is the maximum electricity price that the distribution network can bear after the disaster. If the PHI of the microgrid still cannot meet the restoration demand of the critical loads of the distribution network under the power response based on the highest electricity price, the restoration electricity price does not need to be optimized and is directly assigned to the highest electricity price. The restoration promotion electricity price is between the basic electricity price and the highest electricity price, which is to meet the critical loads recovery of NPF, and to motivate and guide MMG to actively participate in the resilience enhancement electricity price of distribution networks.

The objective of the three-step dynamic restoration electricity price optimization model considering the resilience enhancement of the distribution network is to achieve the minimum power purchase cost of the distribution network, and the objective function is shown in equation (26).

(26)

(27)

The constraint equation is the CRCL constraint, as shown in equation (28).

(28)

3.2.2 The Algorithm for solving the three-stage dynamic restoration electricity price model.

The steps of the three-step dynamic restoration electricity price mechanism during the restoration period of the distribution network are the four steps in section 2.2. The dynamic restoration electricity price is solved by the dichotomous method and single microgrid optimization model, the solution steps are as follows.

Step 1: Input the original parameters ,, the convergence value of the electricity value, the value of the dichotomous times K, , set the original search interval [,] of the dichotomous.

Step 2: Transmission of and, load power demand from the distribution network to each MG.

Step 3: Each microgrid is optimized separately with electricity price of  and  to determine the feed-in power.

Step 4: If , then , and execute step 11, if , then step 6.

Step 5: If , then , and execute step 12;

Step 6: Calculate the average electricity value in the search interval, each microgrid responds to dynamic restoration electricity price, execute the objective function (11) to optimize the single microgrid, and transmit the feed-in power, and calculate the PHI index value of equation (26).

Step 7: If, determine the dynamic restoration electricity price and jump to Step 12, otherwise execute Step 10.

Step 8: Calculate the value of , if it is less than , then execute Step 7, if it is greater than , then execute Step 9.

Step 9: Determine whether k reaches the value of K. If , then execute Step 10, if k>K, then execute Step 11.

Step 10: If , update the electricity price search interval to [,] and execute Step 6, and if , update the tariff search interval to [,] and execute Step 6.

Step 11: If , sort  in order from largest to smallest and selectaccordingly, ensuring that ,.

Step 12: End.

The solution flow of the model is shown in Figure 2.

Fig.2  Three-stage dynamic restoration electricity price optimization process

- Example analysis

  - Test system and simulation scenario

4.1.1 Test System

An IEEE 33-node distribution network with two microgrids as a test system, with energy storage, wind turbines, photovoltaics, gas turbines and electric vehicles within the microgrid. Typical daily load and fluctuation curves of PV and wind turbine for two microgrids are shown in Figure 3. Assume the same shape of PV and wind turbine output curves for both microgrids. MG1 is equipped with 600kW photovoltaics, 400kW wind turbines, 115kW gas turbines, 150kW/500kWh energy storage and 20 electric vehicles. The rated power of each electric vehicle is 30kW, the interruptible load ratio is 30%, and the emergency regulation load ratio is 40%. MG2 is equipped with 500kW photovoltaics, 800kW wind turbines, 115kW gas turbines, 200kW/600kWh energy storage and 20 electric vehicles. The interruptible load ratio is 30%, and the emergency regulation load ratio is 30%. The electric vehicle is a household electric vehicle and the travel pattern is referred to the literature [19] statistical results. The relevant parameters of the micro gas turbine are given in the literature [20].

The structure of the IEEE 33-node distribution network with 2 microgrids is shown in Figure 4. The critical load nodes are distributed in nodes 9, 12, 20, 22, and 27, totaling 360kW. The time-of-use electricity price and contract electricity price during the normal operation of the system are shown in Table 1, which is used as the basic electricity price, and the highest electricity price is set as 1.5 times of the peak electricity price. The conversion value of natural gas is 0.099Nm3/kWh, and the gas price is 3.79/Nm3.

Fig.3  The load and DG output power profile

Table 1 The time-of-use electricity price and contract electricity price (¥/kWh)

| Stage | 22:15-6:30 | 6:45-10:00 | 10:15-11:30 | 11:45-17:00 | 17:15-20:45 | 21:00-22:00 |
| --- | --- | --- | --- | --- | --- | --- |
| TU | 0.3 | 0.55 | 0.3 | 0.55 | 1 | 0.55 |
| IL | 0.35 | 0.61 | 0.35 | 0.61 | 1.06 | 0.61 |
| ERL | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 | 1.5 |

Fig.4  The diagram of the modified IEEE 33-bus network

4.1.2 Simulation scene

Assume that the time of the distribution network fault is 6:00 a.m., the distribution network completes the post-disaster fault repair after 6 hours. In this paper, three fault scenarios are set to analyze the impact of MG on the resilience improvement of distribution network NPF under different electricity price mechanisms.

Scenario 1: Extreme disasters cause line break faults on 2 lines within the distribution network, that is. fault points 2 and 3. After a fault, two NPFs are formed, MG1 is connected to NPF1, and MG2 is connected to NPF2. Electric vehicles in the microgrid are charged in response, and the resilience of MGs to critical loads in the NPFs of the distribution network under the time-of-use electricity price mechanism and the restoration electricity price response mechanism is analyzed.

Scenario 2: The fault is the same as Scenario 1, but the EVs in the microgrid responds to charge and discharge.

Scenario 3: Extreme disasters cause faults in the upper level of the distribution network, that is. fault point 1. After the fault, the entire distribution network is NPF 3, which forms a multi-microgrid structure with MG1 and MG2.

  - Analysis of scenario simulation results

4.2.1 Analysis of distribution network resilience improvement results in scenario 1

In the fault situation of Scenario 1, the disconnection fault forms two independent MMG systems, namely, NPF1 and MG1 form MMG1, and NPF2 and MG2 form MMG2. The distribution network optimizes restoration electricity price, and MG1 and MG2 conduct internal optimization to respond to the electricity price. Since the power output and load of MG1 and MG2 are different in size and timing, the load demand of the distribution network during the fault period is also different, so the response of MG to the load demand of the distribution network and the restoration electricity price is different, and the role of resilience improvement different. The scheduling results and PHI index under the two electricity price mechanisms of MG1 and MG2 are shown in Figure 5, Figure 6 and Figure 7 respectively.

Fig.5  Scenario 1 MG1 load responds to the scheduling results of two electricity prices

Fig.6  Scenario 1 MG2 load responds to the sched-uling results of two electricity prices

During the fault islanding period from 6:00 to 9:15, MG1 does not respond to the critical load power demand of NPF1 under the time-of-use electricity price, the feed-in power is 0, and the RRCL of NPF1 is 0. The main reason is that the distributed power output of MG1 cannot meet the load demand of the microgrid itself, especially during the period from 6:00 to 7:45. In order to meet the power balance, all IL and Part of the ERL responds to the scheduling of MG1. Moreover, the electricity price of NPF during this period is the time-of-use electricity price of normal operation, which is far lower than the contract and penalty electricity price of microgrid load power response, and the PHI index is -100% at this time. During the period from 9:30 to 11:00, the distributed PV output increases while the load demand decreases, The surplus power of the microgrid is connected to the grid to support the critical load power supply of NPF 1, and the PHI index is 0%. Between 11:15 and 11:45, the PHI index was -75%. The critical load restoration rate RRCL for NPF1 during this fault restoration period improved to 41.96%, but most of the critical loads were still not restored to power.

Fig.7  Scenario 1 MG1 and MG2 responds to the PHI of two electricity prices

The resilience (RRCL) improvement of MG 2 to the critical load of NPF 2 was 77.12%, much higher than the improvement result of MG 1. The contribution of MG2 to the recovery of critical loads at NPF2 during the period 6:00 to 7:45 is only 0.0401 kW at 6:00 and 0 at other times, with a PHI index of -100%, for the same reason as for MG1, where the power supply within the microgrid cannot meet the power demand of the loads.

The results of the response time-sharing tariff on NPF1 and NPF2 resilience enhancement for the two microgrids of MG1 and MG2 in Scenario 1 illustrate that the DG output is able to meet the power demand of NPF critical loads when it is sufficient under the time-sharing tariff mechanism, such as MG2 during 8:00 to 12:00, where the critical load RRCL of NPF2 reaches 100% and the PHI index reaches up to 350%. Once the DG output of the microgrid is insufficient, the dispatchable resources within the microgrid are not effectively involved in the post-disaster fault recovery of the NPF and cannot provide sufficient online power to meet the recovery of critical loads in the distribution network.

From Figure 5, it can be found that the restoration tariff reaches a maximum value of 1.5¥/kWh between 6:00 and 7:45, as the gas turbine output and IL response power in MG1 reach their maximum values at this time, and the ERL also responds to the NPF1 power demand. During the three time periods of 8:00 to 9:15 and 10:15, and 10:45 to 11:30, the IL of the microgrid meets the critical load power demand of NPF1 and the restoration tariff is the contracted tariff of the IL. For the rest of the time, the residual power of the microgrid power output meets the power demand of the critical load, and the restoration tariff is a time-of-use tariff. The feed-in power of MG1 under the dynamic restoration tariff mechanism is shown in Figure 5, and the RRCL of the restoration tariff mechanism NPF1 is 100%, which has an improvement of 58.04% compared to the time-sharing tariff mechanism.

From Figure 6, we can find that the IL response power meets the critical load demand of NPF2 between 6:00 and 7:45, and the restoration tariff is the contracted tariff of IL, and the restoration tariff is the same as the time-sharing tariff at other times. The feed-in power of MG2 under the restoration tariff mechanism is shown in the red solid line in Figure 6. The RRCL of NPF1 under the dynamic restoration tariff mechanism is 100%, which has an improvement of 23.88% compared to the time-of-use tariff mechanism.

In the scenario 1, the PHI index reaches 0% and above at all moments under the dynamic restoration tariff mechanism, satisfying critical load restoration and boosting the RRCL of NPF1 and NPF2 to 100%, which is a significant enhancement relative to the time-of-use tariff mechanism. At the same time, the dynamic restoration tariffs of the two MMGs differ significantly. It shows that adopting only the time-of-use tariff during normal operation cannot achieve full restoration of critical loads in the distribution network, and different MMG systems need to adopt different dynamic restoration tariffs to achieve restoration of critical loads in the distribution network.

4.2.2 Analysis of distribution network resilience improvement results in scenario 2

During the resilience of critical loads in the distribution network, The EV uses a charge response mode that responds to charging power and time. Since the failure occurred between 6:00 a.m. and 12:00 p.m., when most of the home EVs had finished charging or were about to finish, the power response of the EVs was small, but in terms of time response, the EV charging time response was staggered from 6:00 to 7:45 a.m., when the MG power supply was underpowered. The results of charging response and discharging response of EV in MG1 under scenario 1 and scenario 2 are shown in Figure 8.

Fig.8  MG 1 EV response results for scenario 1 and 2

As shown in Figure 8, EVs were discharged to respond during the time from 6:00 to 7:45 when the MG1 power supply was underpowered. 4 EVs participated in the charging time response and 11 EVs participated in the charging and discharging response to meet the charging and discharging time and SOC constraints, and the discharging capacity reached 462kWh. Similarly, it can be found that 3 EVs were involved in the charging time response and 11 EVs were involved in the charging and discharging response in Figure 8, with a discharging capacity of 406 kWh. The results illustrate that EVs can respond to the post-disaster fault recovery of the distribution network and enhance the resilience of the distribution network and microgrid in the case of insufficient power of the microgrid. Obviously, it is that the EV charging and discharging response is significant.

Under the dynamic resilience price mechanism, the critical load RRCL of the distribution network is raised to 100% in both Scenario 1 and Scenario 2, but the economic costs for the distribution network are different. the comparison results of the dynamic resilience price between Scenario 1 and Scenario 2 are shown in Figure 9. For MG1, the EV discharge response during the 6:00 to 7:45 hours when the power supply is underpowered reduces the ERL response power by 462 kW, and reduces the dynamic resilience price of the distribution network, as shown by the black dashed line in Figure 9. For MG2, it is not as significant as MG1 in reducing the dynamic restoration tariff, but it is able to reduce the response power of IL and ensure the reliability of power supply to the microgrid load.

Fig.9  Dynamic restoration of electricity price com-parison in scenario 1 and 2

4.2.3 Analysis of distribution network resilience improvement results in scenario 3

The composition of the multi-microgrid system in Scenario 3 is different from that of Scenarios 1 and 2. MG and NPF in Scenarios 1 and 2 have a one-to-one relationship, while the MMG structure of two microgrids and one NPF is formed in Scenario 3. Under scenario 3, MG1 and MG2 coordinate to meet the demand of critical loads in the NPF3 distribution network, and the dynamic resilience price mechanism introduces a microgrid power headroom index to coordinate the power supply and demand balance between MG and NPF.

The optimization results of the dynamic resilience price for the distribution network in Scenario 3 are shown in Figure 10. MG1 and MG2 also have a distribution network RRCL of 0 during the 6:00 to 7:00 hours under the time-sharing tariff mechanism due to insufficient power output from their own power sources. Under the time-of-use electricity price mechanism, MG1 and MG2 also have the problem that the RRCL of the distribution network is 0 during the hours of 6:00 to 7:00 due to insufficient power output from their own power sources. Under the dynamic resilience price mechanism, MG1 and MG2 respectively respond to the power demand of critical loads in the distribution network with an RRCL of 100%, which further verifies the dynamic resilience price mechanism can effectively improve the distribution network resilience in the distribution system with multiple microgrids.

Fig.10  Scenario 3 Dynamic recovery price optimization results of distribution network

- Conclusions

This paper proposes an optimal scheduling method for resilience enhancement of distribution networks with MMG considering dynamic resilience price response mechanism. The introduction of dynamic resilience price response mechanism and PHI index can actively guide distributed power sources and controllable loads such as IL, ERL, EV and gas turbines in the microgrid to actively participate in distribution network fault recovery, which enhances the RRCL of critical loads in the distribution network during disasters. The two-tier dynamic price response mechanism of the distribution network with MMG can coordinate the interests of multiple parties, while the microgrid operation alone requires only the on-grid power under the dynamic restoration price, and optimization of dynamic resilience price based on PHI index to meet the power demand of critical loads in the distribution network. The dynamic resilience price mechanism can effectively improve the resilience of critical loads in the distribution network compared to the time-of-use price. The simulation results show that the method is reasonable and effective, and has practical guidance in fault recovery of distribution networks with multiple microgrids.

- References

[1] BIE Zhaohong, LIN Chaofan, LI Gengfeng, et al. Development and Prospect of Resilient Power System in the Context of Energy Transitio[J]. Proceedings of the CSEE, 2020, 40(09): 2735-2745.

[2] RUAN Qiantu, XIE Wei, XU Yin et al. Concept and Key Features of Resilient Power Grids[J]. Proceedings of the CSEE, 2020,40(21):6773-6784.

[3] WANG Shouxiang, LIU Qi, ZHAO Qianyu, et al. Connotation Analysis and Prospect of Distribution networks Elasticity[J]. Automation of Electric Power Systems, 2021,45(09):1-9.

[4] ZHANG Lu, HUANG Rui, WANG Zhaoqi, et al. Optimal Configuration Strategy of Mobile Energy Storage in Distribution networks Considering Balance Between Resilience and Economy [J]. Automation of Electric Power Systems, 2020, 44(21): 23-31.

[5] WANG Shouxiang, WANG Lin, WANG Hongkun, et al.Optimal Restoration strategy of distribution networks to enhance resilience under earthquake disaster [J]. Automation of Electric Power Systems, 2020, 32(06): 28-35.

[6] Ma S, Li S, Wang Z, et al. Resilience-Oriented Design of Distribution Systems[J]. IEEE Transactions on Power Systems, 2019, 34(4): 2882-2891.

[7] Gautam, P, Piya, P, Karki. Resilience Assessment of Distribution Systems Integrated With Distributed Energy Resources [J]. IEEE TRANSACTIONS ON SUSTAINABLE ENERGY, 2021, 12(1): 338-348.

[8] Dehghani, NL; Jeddi, AB, Shafieezadeh, A. Intelligent hurricane resilience enhancement of power distribution systems via deep reinforcement learning [J]. APPLIED ENERGY, 285.

[9] Najafi J, Peiravi A, Anvari-Moghaddam A, et al. Resilience improvement planning of power-water distribution systems with multiple micro-grids against hurricanes using clean strategies[J]. Journal of Cleaner Production, 2019, 223(6): 109-126.

[10] HUANG Gechao, LI Gengfeng XIAO Yao. et al. Optimal Placement for Integrated Electricity and Gas Energy System Considering Natural Disasters and Man-made Attacks[J]. Power System Technology, 2021,45(03):959-970.

[11] Hong kun Wang, Shou xiang Wang, Lu Yu, et al. A novel planning-attack‐reconfiguration method for enhancing resilience of distribution systems considering the whole process of resiliency[J]. International Transactions on Electrical Energy Systems, 2019(6).

[12] LIAN Yiqing, GUO Zuogang, MA Xiyuan, et al. Elastic scheduling of integrated energy system in gas-electricity coupling park considering hot standby[J], Automation of Electric Power Systems, 2019,31(11):115-121.

[13] Qi Shixiong, Wang Xiuli, Li Xue, et al. Enhancing Integrated Energy Distribution System Resilience through a Hierarchical Management Strategy in District Multi-Energy Systems[J]. SUSTAINABILITY, 2021, 11(15): 1-20.

[14] Khaloie H, Abdollahi A, Rashidinejad M, et al. Risk-based probabilistic-possibilistic self-scheduling considering high-impact low-probability events uncertainty[J]. International Journal of Electrical Power & Energy Systems, 2022, 110: 598-612.

[15] Oboudi M H, Mohammadi M, Rastegar M. Resilience-oriented intentional islanding of reconfigurable distribution power systems[J]. Journal of Modern Power Systems and Clean Energy, 2022, 7(4): 741-752.

[16] GE Shaoyun, ZHANG Chenghao, LIU Hong, et al. Resilience Enhancement Strategy for Distribution networks Considering Supporting Role of Micro Energy Grid [J]. Power System Technology, 2020, 43(07):2306-2317.

[17] PENG Hanmei, WANG Xiaohao, WEI Ning, et al. micro-grid Differentiated Recovery Operation for Enhancing Distribution System Resilience [J]. Power System Technology, 2021, 43(07): 2328-2335.

[18] XU Mengyao, AI Xiaomeng, FANG Jiakun et al. Two-stage Stochastic Optimal Scheduling Model for Joint Regulation of EV and Thermal Units Considering Users Enthusiasm[J]. Power System Technology, 2022, 46(06): 2033-2041.

[19] SUN Han, CHEN Zonghai, WU Ji. Online energy dispatch strategy for residential micro-grid considering the uncertainty of electric vehicle [J]. Power System Technology, 2021, 43(07): 2544-2551.

[20] DONG Shuai, WANG Chengfu, XU Shijie, et al. Day-ahead optimal scheduling of electricity-gas-heat integrated energy system considering dynamic characteristics of networkss [J]. Automation of Electric Power Systems, 2020, 42(13): 12-19.