1. Title of the manuscript

An Online Distributed Optimization Model Solving the Time-Coupled Conundrum by the Lyapunov Drift Plus Penalty Method

2. All the authors' complete names

Molin An, Tianguang Lu, Xueshan Han, and Zhaohao Ding

3.   Departmental & institutional affiliations of the authors

Molin An 1, Tianguang Lu 1*, Xueshan Han 1, and Zhaohao Ding 2

1 School of Electrical Engineering, Shandong University, No.17923 Jingshi Road, Jinan, China

2 State Key Laboratory of Alternate Electrical Power System with Renewable Energy Sources, School of Electrical and Electronic Engineering, North China Electric Power University, No.2 Beinong Road, Beijing, China

4.    Postal and E-mail address of the corresponding author

tlu@sdu.edu.cn

School of Electrical Engineering, Shandong University, No.17923 Jingshi Road, Jinan, China

5.  Conflict of Interest statement

The authors declare that we have no conflicts of interest in this research.

6.  Funding information (Please mention "Funding: None" if you have not received any support for your research)

Funding: State Key Laboratory of Alternate Electrical Power System with Renewable Energy Sources: LAPS23012.

7.  Data Availability statement.

The data that support the findings of this study are available from the corresponding author upon reasonable request.

8.   Credit contribution statement

Molin An: Methodology; Software; Writing – original draft

Tianguang Lu*: Writing – review & editing

Xueshan Han: Writing – review & editing

Zhaohao Ding: Writing – review & editing

An Online Distributed Optimization Model Solving the Time-Coupled Conundrum by the Lyapunov Drift Plus Penalty Method

Molin An 1, Tianguang Lu 1*, Xueshan Han 1, and Zhaohao Ding 2

1 School of Electrical Engineering, Shandong University, No.17923 Jingshi Road, Jinan, China

2 State Key Laboratory of Alternate Electrical Power System with Renewable Energy Sources, School of Electrical and Electronic Engineering, North China Electric Power University, No.2 Beinong Road, Beijing, China

*tlu@sdu.edu.cn

Abstract: As the proportion of distributed energy resources (DER) continues to grow, it is imperative to enhance the economic and safe operation of the active distribution network (ADN). This paper presents an online distributed optimization model, in which loads and the active distribution network (ADN) operators transfer signals and perform local calculations. In order to significantly reduce the complexity of the model, a novel linear power flow (LPF) model that can be updated online is employed. Furthermore, the high proportion of DER access presents a time-coupled conundrum, thereby creating difficulty in finding the optimal solution. In this study, the Lyapunov drift plus penalty method (LDPPM) is employed to address this challenge by transforming the long-term optimization problem into a current-time one. Additionally, to enhance the calculation efficiency, the iterative optimization algorithm is regarded as a control process and is controlled with the PID controller to form a PID-Lagrange algorithm. The proposed PID-Lagrange algorithm has a clearly defined parameter tuning strategy that aligns with control theory, resulting in a significant reduction in operating costs by 30.27% in the case study. The results of the performance analysis demonstrate the superiority of the proposed method.

Keywords: active distribution network; linear power flow; online distributed optimization; time-coupled; Lyapunov drift plus penalty method.

- Introduction

The extensive incorporation of DER in the ADN presents a significant challenge to the optimization and dispatching processes. The operation of the ADN is contingent upon the implementation of optimal power flow, which is essential for maintaining an economical, safe, and reliable operational state. In general, the existing and commonly used centralized operation and control pattern in the ADN encompasses two operational layers: the physical layer and the cyber layer. The ADN operator collates all the requisite data to solve the optimization problem and transmits instructions to control equipment, leveraging a diverse array of sensor and communication infrastructures [1]. However, as the proportion of DER access increases, centralized algorithms may encounter significant mathematical challenges in solving large-scale, multi-variable problems or long-term scale problems. This could significantly increase the calculation difficulty and negatively impact the accuracy [2]. Conversely, distributed algorithms are gaining in popularity due to the potential advantages they offer in terms of efficiency, privacy, and security compared to centralized algorithms [3]. A distributed algorithm decomposes a centralized problem into sub-problems, thereby enabling rapid parallel calculation [4]. Furthermore, it offers the advantage of data privacy, as part of the calculations is conducted locally and the local information is not accessible to the ADN operator. However, the centralized algorithm has high requirements regarding the quality and speed of data transmission. Any data misalignment, loss, or delay will result in errors or failure [1]. In conclusion, the number of DER and loads in the ADN is large and widely distributed, and data privacy is significant. Therefore, the distributed algorithm is ideally suited for application in the ADN. The main factors affecting the algorithm performance are model complexity, calculation accuracy, and speed, respectively.

Firstly, the power flow model represents a significant constraint in the description of the nonlinear relationship between the parameters of the power system [5]. However, the nonlinearity gives rise to nonconvexity [6], an NP-hard challenge [7], and poor convergence [8]. The considerable number of variables in the optimization problem, in conjunction with its nonconvex nature, significantly increases the burden [9]. It is therefore evident that the LPF model is of crucial importance in order to accomplish the aforementioned tasks in an efficient and accurate manner. The model can be classified into two distinct categories: model-driven and data-driven methods [10]. Over the past decade, there has been a gradual replacement of model-driven LPF models with data-driven LPF models [11]. This is due to the fact that data-driven models do not rely on network model parameters [12]. Nevertheless, the extant LPF models are beset with numerous deficiencies, including the onerousness of offline updates and the considerable data burden [13]. For this reason, this paper applies a new LPF model that can be updated online, with the aim of greatly reducing complexity and improving efficiency.

Secondly, the high proportion of DER introduces entirely novel constraints within the ADN. The time-coupled nature of energy storage (ES) represents a significant challenge, as its capacity and power are inherently coupled over time. This is typically addressed through the application of dynamic programming techniques. Nevertheless, this approach results in intricate solutions that diminish overall efficiency [14]. The LDPPM addresses the aforementioned issue by employing the current system state in a manner that does not rely on a priori statistical knowledge [15]. Recently, it has been applied to demand response [16], energy management [17], vehicle-to-grid control [18, 19], and ES control [20, 21]. Accordingly, this paper employs the LDPPM to address the time-coupled conundrum of ES.

Finally, the efficiency of distributed optimization is also affected by the iterative calculation speed. The distributed calculation is performed in an iterative manner with restricted information interaction and local calculation, with the objective of ensuring the maximum protection of user privacy [22]. Accordingly, the number of iterative calculations is of significant consequence with respect to the calculation cost. The most commonly utilized iterative calculation is the gradient method. However, the optimal parameters are challenging to obtain and difficult to adjust in practical applications. For instance, the calculation of the momentum algorithm's step coefficients is intricate, and the convergence is slow in certain exceptional circumstances [23]. In Nesterov’s accelerated gradient algorithm, the hyperparameter is typically set to 0.9, although this remains a conservative suggestion rather than an optimal choice [24]. As a result, the absence of a transparent parameter tuning strategy hinders the enhancement of efficiency in engineering applications. For this reason, the iterative calculation is regarded as a control process in this paper, and a PID controller is employed to regulate the calculation process. The objective is to significantly enhance the calculation speed while providing a rational parameter tuning strategy. This represents a significant enhancement in the efficiency and performance of the calculation process in engineering applications.

Compared with the existing distributed optimization methods, the proposed method exhibits a number of advantages which are detailed in Table 1. The contributions of this paper are also multifaceted as follows:

1. Introducing a new LPF model that facilitates online parameter updates after a single initialization significantly simplifies the optimization model and enhances efficiency;

2. The application of the LDPPM to resolve the time-coupling conundrum in ES, transforming a complex long-term optimization problem into a more tractable momentary optimization, while maintaining a balance between global optimality and efficiency;

3. The conceptualization of the iterative optimization process as a control process, which is effectively managed by a PID controller, resulting in a clear parameter tuning strategy and a substantial reduction in operating costs;

4. The development of a distributed optimization model that empowers users to perform local computations and engage in limited information exchange with ADN operators, fostering a collaborative optimization paradigm.

In addition, several valuable prospects in this research field have been proposed:

1. Expand the PID-Lagrange algorithm to more complex optimization problems in application, and verify the applicability of the method;

2. Study how to extend the model to scenarios with random loads and renewable energy output, and verify the robustness of the method;

3. Expand the proposed model to consider scenarios with more types of DER, such as PV, ES, and demand response, and conduct more comprehensive optimization analysis.

- Theoretical analysis

  - The LDPPM

The access of DER in the ADN, while providing initiative and adjustable capability, creates model complexity and a time-coupled conundrum. The time-coupled property of ES is defined as the phenomenon whereby the charging and discharging state and capacity at a subsequent time are influenced by the preceding time. In this paper, the LDPPM is employed to address this issue, thereby simplifying the optimization problem as follows.

In light of the time-coupled nature of ES, it is imperative for the ADN operator to consider optimization at long-time scales in order to achieve global optimality. To this end, an optimization problem is formulated as follows:

(1)

(2)

(3)

(4)

(5)

(6)

(7)

(8)

Table 1 Comparison of the proposed method with existing studies

| Literature | Methodology | Voltage control | ES | Time-coupled | Time scale | Power flow | Parameter strategy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [25] | A distributed algorithm based on relaxed ADMM for energy resources coordination | × | √ | × | Short | Nonlinear | × |
| [26] | Distributed data-driven optimization for voltage regulation in distribution systems | √ | √ | × | Short | Nonlinear | × |
| [27] | Non-cooperative and cooperative optimisation of battery es system for energy management in multi-microgrid | × | √ | √ | Long | × | × |
| [28] | Distributed model predictive control strategy based on block-wised alternating direction multiplier method for microgrid clusters | × | √ | √ | Long | Linear | × |
| [29] | Distributed online prediction optimization algorithm for distributed energy resources considering the multi-periods optimal operation | √ | √ | × | Long | Nonlinear | × |
| [30] | Fairness-incorporated online feedback optimization for real-time distribution grid management | √ | × | × | Short | Linear | × |
| [31] | Dynamic weighted-gradient descent method with smoothing momentum for distributed energy management of multi-microgrids systems | √ | × | × | Long | × | × |
| [32] | Distributed robust optimization of distribution network incorporating novel battery charging and swapping station | √ | √ | × | Long | Linear | × |
| This paper | An online distributed optimization for the active distribution network based on a novel linear power flow model considering time-coupled conundrum | √ | √ | √ | Long | Linear | √ |

where  denotes the node ,  represents the time,  and  denote the active and reactive power,  denotes the charging power of the ES,  denotes the ES capacity,  is the value of the voltage , and  is the PV capacity.

The cornerstone of the optimization problem is the power flow, which serves as the basis for determining the complexity, efficiency, and accuracy. The distributed optimization model proposed in this paper is available for rapid online calculation, necessitating a balance between accuracy and speed in the power flow calculation. As previously stated in the Introduction, the LPF model's generalization ability presents a challenge. In other words, it is not an optimal use of time and resources for the LPF model to update the model by retraining. Accordingly, the optimal choice is an LPF model that is capable of updating its parameters in an online manner. This paper presents a novel LPF model that requires only one initial training phase. Consequently, the parameters are updated online, thereby obviating the necessity for retraining. This significantly simplifies the model and enhances its efficiency, which are discussed in depth in [33] along with the utilization procedure.

The ES constraints are time-coupled, and the current decisions will have an impact on those made in subsequent optimizations. Consequently, the optimization process must be forward-looking, necessitating the acquisition of global information. Nevertheless, the operation of the ADN, in particular the DER, is inherently stochastic. As a result, it is not possible for the ADN operator to know in advance all the information that will be relevant in the following period. Consequently, numerous methodologies are founded upon forecasting techniques. However, such methods necessitate a high degree of prediction accuracy, and there is a significant possibility that the actual situation will deviate from the prediction. The expenditure incurred on prediction is not offset by the resulting accuracy; thus, an efficient method is sought to achieve a mutually beneficial outcome between the dispatching cost and the optimization objective. The LDDPM is a widely utilized technique in the domain of information and signal processing, offering an efficient solution to queuing problems and its applications extend to time-coupled conundrums. The method does not rely on prediction information and is capable of achieving the relative optimality of the objective over long-time scales by optimizing the objective in the current moment. This approach offers a mutually beneficial solution that simultaneously reduces costs and ensures accuracy.

The time-coupled constraints of ES are relaxed as follows:

(9)

(10)

(11)

(12)

According to the LDDPM [15], the ES constraint is finally converted to:

(13)

(14)

(15)

It implies that the ES capacity is mean rate stable. In other words, it requests that the ES cannot be charged or discharged all the time. Otherwise, the resulting value will not be equal to zero. This represents a relaxed ES capacity limit. By employing the LDDPM, it is possible to make this condition equivalent to the original ES capacity constraint [15].

The aforementioned optimization problem can be transformed into an optimization problem related only to the current moment. It has been demonstrated that the optimization problem initially posed can be achieved by calculating the following problem [15].

(16)

(17)

  - PID-Lagrange Algorithm Based on Control Theory

This paper employs control theory as a means of enhancing efficiency. The distributed optimization method employed in the ADN is founded upon iterative calculation, which is regarded as a control process. The computed value is continuously iterated from the initial value to the optimal value. From the perspective of control theory, any process with initial and objective values can be controlled to enhance performance, whether in terms of efficiency or accuracy. Accordingly, the iterative optimization calculation process is regarded as a control process, and a new PID-Lagrange algorithm is formed using the PID control.

For the convenience of the derivation in this section, we denote the variable , ,  by  and simplify the problem to the standard form:

(18)

(19)

(20)

Assumption 1: The objective function  is convex. The equality constraint is affine as  and there exists a feasible value  such that . The inequality constraint is convex as  and there exists a feasible value  such that .

Assumption 2: , , and  are continuously differentiable.

Under Assumption 1-2, a unique optimal value exists for problem . Let  denote the Lagrange multiplier associated with the equality constraint , and  denote the Lagrange multiplier associated with the inequality constraint .  and  are the vectors of  and .

The Lagrange function is:

(21)

The Lagrange algorithm is:

(22)

(23)

(24)

where ,  and  are step coefficients,  represents the gradient, and  represents the number of iterations.

The iterative optimization algorithm is a control process to keep the calculated value getting close to the optimal value from the initial value. Therefore, the control object is the optimal value, but it cannot be known and is just what needs to be found. Nevertheless, the KKT conditions elucidate the attributes of the optimal value, thereby disclosing the control object in an alternative format. In consequence, the controlled objects are the KKT conditions.

Consider the KKT conditions of  as the control object:

(25)

(26)

(27)

(28)

(29)

where superscript * indicates the optimal value.

The errors are defined as:

(30)

(31)

(32)

The iterative calculation is a discrete process. Therefore, the discrete incremental PID controller is used:

(33)

where , is the PID output.

Based on the error definitions and the PID controller, the following PID-Lagrange algorithm is proposed:

(34)

(35)

Fig. 1.  Flowchart of the proposed PID-Lagrange algorithm.

(36)

where superscripts P, I, and D indicate proportional, integral, and differential coefficient.

The control system block diagram of the PID-Lagrange algorithm is illustrated in Fig. 1. The three inputs in each PID controller suggest that the PID-Lagrange algorithm improve the performance by increasing the historical data, input, and the order of control system. The PID controller provides a framework for interpreting the parameters in a manner consistent with the principles of control theory. Consequently, the PID-Lagrange algorithm offers a more straightforward approach to setting and adjusting parameters in accordance with the established rules of control theory.

In the following discussion, the PID-Lagrange algorithm is applied for analysing the performance by establishing the state equation. The proposed PID-Lagrange algorithm can be converted into a discrete state equation:

(37)

(38)

where  is the system matrix and  is the constant matrix.

The system matrix is expressed in the following form:

(39)

Theorem 1: The necessary and sufficient condition for the system  to be Asymptotic Stability in the Large at equilibrium state  is that the characteristic roots of  are all in the open unit disk.

It should be noted that the criterion given in Theorem 1 is based on the equilibrium state . But in actual optimization calculation, the optimal value is not always the origin . In this case, the optimal value can be transformed to the origin through coordinate transformation to satisfy the criteria. The coordinate transformation of the variables in the linear system only changes , but not . Thus, coordinate transformation does not affect the system stability judgment. Therefore, there is the following conclusion:

Conclusion 1: The necessary and sufficient condition for the iterative calculation to converge to the optimal value is that the characteristic roots of  are all in the open unit disk.

  - The Distributed Optimization Method

The economic benefits need to be considered in the ADN optimization. Therefore, assuming that the economic cost is , and the user side optimization is as follows:

(40)

(41)

where  and  represent operation signals by the ADN operator, and  represents the power constraint.

The optimal solution is defined as:

(42)

where  and represent function symbols, and  represents the meaning of the definition.

It also needs to consider the safety and stability of the ADN.  represents the voltage objective and the voltage deviation between the voltage and the standard nominal voltage  is minimized, which can be expressed by .

Based on the above assumption and definition, an optimization problem of the ADN with optimal comprehensive objectives can be constructed based on the model proposed in the Section 2.1:

(43)

(44)

(45)

where  represents a coefficient of balance. Operation signals are formed into , and ,  are formed into .

The above optimization problem is solved by the PID-Lagrange algorithm. Then, a mode of collaborative calculation between the users and the ADN operator is performed.

After receiving signal , the load is regulated:

(46)

The ADN operator only needs to collect the voltage information and calculate an operation signal:

(47)

(48)

(49)

(50)

where  and  should be ensured,  and  represent Lagrange multipliers corresponding to inequality constraints, which can be understood as intermediate variables required to calculate the operation signal. The stop criterion of this algorithm is the difference between the two iterations is less than a pre-set threshold, or a pre-set maximum number of iterations reaches.

Fig. 2 graphically illustrates the proposed method.

- Performance Analysis

  - The PID-Lagrange Algorithm

This calculation example aims to verify that Conclusion 1 can determine the convergence.

(51)

(52)

Fig. 2.  The flowchart of the proposed method.

Fig. 3.  The convergence of the PID-Lagrange algorithm. (A) The parameters setting. (B) The curve of calculation results. (C) The characteristic roots of the system matrix. (1) Damped oscillation. (2) Continuous oscillation. (3) Divergence. (4) The comparison between the Lagrange algorithm and the PID-Lagrange algorithm.

There are several reasons for using this simple optimization problem as an example. First, we simply want to prove whether the proposed convergence theorem can be explained in terms of control theory. Therefore, a clear graphical proof and categorized discussion is needed to show the three cases of convergence, oscillation and non-convergence. Second, the power system optimization problem presented in this paper cannot be analyzed graphically with the above discussion. Because it contains dozens of nodes, resulting in a large number of variables and constraints that cannot be represented. Therefore, we have chosen this example optimization problem to demonstrate that the three cases of our proposed theory are correct in practice. The power system optimization problem is then used to verify the superiority rather than the convergence of the PID-Lagrange algorithm.

Fig. 4.  The network structures.

Assuming that the initial value is  and , and the threshold for stopping calculating is . The calculation process and the characteristic roots of  are observed using the PID-Lagrange algorithm with different parameters settings in Fig. 3.

When the parameters in Fig. 3(A1) are set, the iterative calculation in Fig. 3(B1) converges, and the characteristic roots of the system matrix  shown in Fig. 3(C1) are all located in the open unit disk, hence the system is stable.

When the parameters in Fig. 3(A2) are set, the iterative calculation in Fig. 3(B2) continuously oscillates, and the characteristic roots of the system matrix  shown in Fig. 3(C2) are partially located on the open unit disk, hence the system continuously oscillates.

When the parameters in Fig. 3(A3) are set, the iterative calculation in Fig. 3(B3) diverges, and the characteristic roots of the system matrix  shown in Fig. 3(C3) are partly outside the open unit disk, hence the system is unstable.

The Lagrange algorithm and the PID-Lagrange algorithm are used to calculate for comparison. As illustrated in Fig. 3(A4)-(C4), the PID-Lagrange algorithm enhances the convergence velocity and optimizes the shape attributes of the calculated curve.

  - Distributed Dispatching Model

The actual data of the 33-node ADN is used for simulation to verify the performance advantages of the proposed method. Assuming that an objective function is . The upper and lower limits of the voltage constraint are 1.05 and 0.95. The parameters , and . The max charge and discharge power of ES is 0.05MW, and the capacity is 0.2MW·h. Fig. 4 is a diagram of the network structure.

Fig. 5.  The PV and load data.

To demonstrate the process of data preparation for the simulation, Node 7 is selected as an illustrative example, showcasing the PV and load curves in Fig. 5. The remaining PV nodes exhibit a comparable pattern to that depicted in Fig. 5, given their identical positioning in the same lighting and environmental conditions. The load dynamics throughout the day also demonstrate a striking resemblance, as a considerable number of nodes precludes the possibility of plotting each curve individually. To focus the analysis on a representative subset of data, we selected the 4:00 to 20:00 interval, removing periods with minimal loads and no PV power, with a 60-second sampling interval.

Fig. 6.  The box whisker diagram of voltages.

The dispatching results are demonstrated in the next figures. In Fig. 6, all the node voltages are within the specified range of 0.95-1.05. This indicates that the proposed method can effectively control the voltage within a safe, reasonable and stable range. It provides a strong guarantee for the economic, safe, and efficient operation of the ADN.

The result in Fig.7 and Fig.8 is achieved by the flexible regulation of ES by the LDPPM, which achieves good cooperation with the PV. The method can dispatch the PV and ES without knowing the future prediction value, using the appropriate strategy to cooperate, and finally achieve the optimization within one day. Consistent with the theoretical

Fig. 7.  The PV abandonment.

analysis, the ES capacity is mean rate stable. In other words, the ES capacity does not increase indefinitely over time, and charging and discharging form a balance. The reason for this balance is the penalty taken by the LDPPM, which is closely related to the value of S. The next analysis will still use the above case but set different values of S to show the influence of S on the ES action strategy.

Fig. 8.  The ES capacity.

In Fig. 9, the ES operation strategy becomes more conservative as S increases, which is consistent with the analysis of the principle of the LDPPM. In practical applications, the ES operation strategy can be controlled by adjusting the value of S to ensure that the ES capacity is mean rate stable. The above case study validates the effectiveness and practicality of the proposed method in this paper.

Finally, we will verify the superiority of the PID-Lagrange algorithm by setting different calculation

Fig. 9.  The influence of S on ES operation strategies.

parameters. The parameters of the original Lagrange algorithm are set as: . The parameters of the PID-Lagrange algorithm are set as:

, ,.

In the analysis, the differentiation parameter is set to 0 because typically, in practical applications, the PI control performs well enough to satisfy the operational requirements. The comparison results verify it well. The convergence judgment condition is that the difference between the objective functions of two iterations is less than 0.0001.

Calculation results are compared in Table 2 by analyzing the number of iterative calculations in the above 960 dispatches. Compared to the original Lagrange algorithm, the PID-Lagrange algorithm requires a fewer number of

Table 2 Comparison of the efficiency

| Index | Lagrange algorithm | PID-Lagrange algorithm | Improvement |
| --- | --- | --- | --- |
| Upper quartile | 30 | 22 | 26.67% |
| Mean | 22.1656 | 15.1542 | 30.27% |
| Median | 15 | 6 | 60.00% |
| Lower quartile | 12 | 4 | 66.67% |

calculations. The PID-Lagrange algorithm reduces the average number of calculations by 30.27%. It shows that the PID-Lagrange algorithm can increase the calculation speed and efficiency, thereby reducing the operation cost.

The above performance analysis demonstrates the superiority of the proposed PID-Lagrange algorithm as well as the LDPPM, respectively. It shows that the proposed method is well suited for economical and safe operation and can facilitate the management of the ADN.

- Conclusion

In this paper, an online distributed optimization for the ADN is proposed. To reduce the complexity of the optimization problem, a new LPF model is used, which effectively improves the calculation efficiency. Further, the high proportion of DER access poses a time-coupled conundrum. Based on the control theory, the LDPPM method is used to tackle this challenge, which simplifies the optimization problem with long time scales and high complexity into the optimization problem at the current time. It not only ensures the accuracy of the optimization but also further improves the efficiency, which is a win-win situation. However, it suffers from the same difficulties as existing optimization algorithms in the lack of a clear strategy for parameter setting and tuning. To further overcome this, an iterative optimization algorithm is considered as the control process in this paper. The PID controller is used to improve the calculation process to achieve better performance. Consequently, an improved PID-Lagrange algorithm is proposed. Finally, based on the above innovative approach, a distributed model of dispatching is proposed. It is enabled to allow users to perform local calculations and exchange only limited information with the dispatching operator. The interaction and collaborative optimization are accomplished by both sides using the exchanged signals. The analysis shows that the LDPPM can form a good cooperation with PV and load by adjusting the conservative and aggressive preferences of ES action strategies, and finally achieve the objective of optimal dispatching. Further, the superiority of the proposed PID-Lagrange algorithm is also demonstrated and the efficiency in the case study is improved by 30.27%.

- References

- Patari, N., Venkataramanan, V., Srivastava, A., Molzahn, D.K., Li, N., Annaswamy, A.: Distributed Optimization in Distribution Systems: Use Cases, Limitations, and Research Needs. Ieee T Power Syst 37(5),3469-81 (2022). https://doi.org/10.1109/Tpwrs.2021.3132348

- Molzahn, D.K., Dorfler, F., Sandberg, H., Low, S.H., Chakrabarti, S., Baldick, R., Lavaei, J.: A Survey of Distributed Optimization and Control Algorithms for Electric Power Systems. Ieee T Smart Grid 8(6),2941-62 (2017). https://doi.org/10.1109/Tsg.2017.2720471

- Wang, Y.M., Wang, S.X., Wu, L.: Distributed optimization approaches for emerging power systems operation: A review. Electr Pow Syst Res 144,127-35 (2017). https://doi.org/10.1016/j.epsr.2016.11.025

- Wang, Y.N., Yemula, P., Bose, A.: Decentralized Communication and Control Systems for Power System Operation. Ieee T Smart Grid 6(2),885-93 (2015). https://doi.org/10.1109/Tsg.2014.2363192

- Yang, Z.F., Zhong, H.W., Xia, Q., Kang, C.Q.: A novel network model for optimal power flow with reactive power and network losses. Electr Pow Syst Res 144,63-71 (2017). https://doi.org/10.1016/j.epsr.2016.11.009

- Bai, X.Q., Wei, H., Fujisawa, K., Wang, Y.: Semidefinite programming for optimal power flow problems. Int J Elec Power 30(6-7),383-92 (2008). https://doi.org/10.1016/j.ijepes.2007.12.003

- Lehmann, K., Grastien, A., Van Hentenryck, P.: AC-Feasibility on Tree Networks is NP-Hard. Ieee T Power Syst 31(1),798-801 (2016). https://doi.org/10.1109/Tpwrs.2015.2407363

- Long, J.R., Yang, Z.F., Zhao, J.B., Yu, J.: Modular Linear Power Flow Model Against Large Fluctuations. Ieee T Power Syst 39(1),402-15 (2024). https://doi.org/10.1109/Tpwrs.2023.3243636

- Kargarian, A., Fu, Y., Wu, H.Y.: Chance-Constrained System of Systems Based Operation of Power Systems. Ieee T Power Syst 31(5),3404-13 (2016). https://doi.org/10.1109/Tpwrs.2015.2499275

- Tan, Y., Chen, Y.Y., Li, Y., Cao, Y.J.: Linearizing Power Flow Model: A Hybrid Physical Model-Driven and Data-Driven Approach. Ieee T Power Syst 35(3),2475-8 (2020). https://doi.org/10.1109/Tpwrs.2020.2975455

- Chen, Y.C., Wang, J.H., Domínguez-García, A.D., Sauer, P.W.: Measurement-Based Estimation of the Power Flow Jacobian Matrix. Ieee T Smart Grid 7(5),2507-15 (2016). https://doi.org/10.1109/Tsg.2015.2502484

- Liu, Y.T., Li, Z.S., Zhou, Y.: Data-Driven-Aided Linear Three-Phase Power Flow Model for Distribution Power Systems. Ieee T Power Syst 37(4),2783-95 (2022). https://doi.org/10.1109/Tpwrs.2021.3130301

- Liu, Y.T., Li, Z.S., Zhao, J.B.: Robust Data-Driven Linear Power Flow Model With Probability Constrained Worst-Case Errors. Ieee T Power Syst 37(5),4113-6 (2022). https://doi.org/10.1109/Tpwrs.2022.3189543

- Neely, M.J., Tehrani, A.S., Dimakis, A.G.: Efficient Algorithms for Renewable Energy Allocation to Delay Tolerant Consumers. Int Conf Smart Grid,549-54 (2010).

- Neely, M. Stochastic Network Optimization with Application to Communication and Queueing Systems 2010.

- Zheng, L., Cai, L.: A Distributed Demand Response Control Strategy Using Lyapunov Optimization. Ieee T Smart Grid 5(4),2075-83 (2014). https://doi.org/10.1109/Tsg.2014.2313347

- Guo, Y.X., Pan, M., Fang, Y.G., Khargonekar, P.P.: Decentralized Coordination of Energy Utilization for Residential Households in the Smart Grid. Ieee T Smart Grid 4(3),1341-50 (2013). https://doi.org/10.1109/Tsg.2013.2268581

- Sun, S., Dong, M., Liang, B.: Real-Time Welfare-Maximizing Regulation Allocation in Dynamic Aggregator-EVs System. Ieee T Smart Grid 5(3),1397-409 (2014). https://doi.org/10.1109/Tsg.2014.2300040

- Jin, C.R., Sheng, X., Ghosh, P.: Optimized Electric Vehicle Charging With Intermittent Renewable Energy Sources. Ieee J-Stsp 8(6),1063-72 (2014). https://doi.org/10.1109/Jstsp.2014.2336624

- Sun, S., Dong, M., Liang, B.: Real-Time Power Balancing in Electric Grids With Distributed Storage. Ieee J-Stsp 8(6),1167-81 (2014). https://doi.org/10.1109/Jstsp.2014.2333499

- Lakshminarayana, S., Quek, T.Q.S., Poor, H.V.: Cooperation and Storage Tradeoffs in Power Grids With Renewable Energy Resources. Ieee J Sel Area Comm 32(7),1386-97 (2014). https://doi.org/10.1109/Jsac.2014.2332093

- Romvary, J.J., Ferro, G., Haider, R., Annaswamy, A.M.: A Proximal Atomic Coordination Algorithm for Distributed Optimization. Ieee T Automat Contr 67(2),646-61 (2022). https://doi.org/10.1109/Tac.2021.3053907

- Abrudan, T.E., Eriksson, J., Koivunen, V.: Steepest descent algorithms for optimization under unitary matrix constraint. Ieee T Signal Proces 56(3),1134-47 (2008). https://doi.org/10.1109/Tsp.2007.908999

- Nesterov, Y., Pardalos, P.M., Hearn, D.W., SpringerLink. Introductory Lectures on Convex Optimization : A Basic Course. 1st 2004. ed. New York, NY: Springer; 2004.

- Liu, H.L., Shi, Y.W., Wang, Z., Ran, L., Lu, Q.G., Li, H.Q.: A distributed algorithm based on relaxed ADMM for energy resources coordination. Int J Elec Power 135 (2022). https://doi.org/10.1016/j.ijepes.2021.107482

- Hong, T.Q., Zhang, Y.C., Liu, J.Z., Zhao, D.B., Xiong, J.: Distributed Data-Driven Optimization for Voltage Regulation in Distribution Systems. Ieee T Power Syst 39(1),1263-73 (2024). https://doi.org/10.1109/Tpwrs.2023.3242868

- Liu, X.F., Gao, B.T., Zhu, Z.Y., Tang, Y.: Non-cooperative and cooperative optimization of battery energy storage system for energy management in multi-microgrid. Iet Gener Transm Dis 12(10),2369-77 (2018). https://doi.org/10.1049/iet-gtd.2017.0401

- Zhu, P.C., Liu, Z.Y., Sun, K., Wang, L., Hu, P.F., Zhu, N.X., Jiang, D.Z.: Distributed model predictive control strategy based on block-wised alternating direction multiplier method for microgird clusters. Iet Gener Transm Dis 16(22),4630-9 (2022). https://doi.org/10.1049/gtd2.12627

- Zhu, X.X., Hou, X.C., Li, J.H., Yan, G.G., Li, C.P., Wang, D.B.: Distributed online prediction optimization algorithm for distributed energy resources considering the multi-periods optimal operation. Appl Energ 348 (2023). https://doi.org/10.1016/j.apenergy.2023.121612

- Zhan, S., Morren, J., van den Akker, W., van der Molen, A., Paterakis, N.G., Slootweg, J.G.: Fairness-Incorporated Online Feedback Optimization for Real-Time Distribution Grid Management. Ieee T Smart Grid 15(2),1792-806 (2024). https://doi.org/10.1109/Tsg.2023.3315481

- Bai, C., Li, Q., Zheng, X.Z., Yin, X.P., Tan, Y.J.: Dynamic Weighted-Gradient Descent Method With Smoothing Momentum for Distributed Energy Management of Multi-Microgrids Systems. Ieee T Smart Grid 14(6),4152-68 (2023). https://doi.org/10.1109/Tsg.2023.3253886

- Zhao, X.Q., Xu, Q.S., Yang, Y.B., Zhou, G.: Distributed distributionally robust optimization of distribution network incorporating novel battery charging and swapping station. Int J Elec Power 155 (2024). https://doi.org/10.1016/j.ijepes.2023.109643

- An, M.L., Lu, T.G., Han, X.S.: An online updated linear power flow model based on regression learning. Iet Gener Transm Dis 18(10),2006-19 (2024). https://doi.org/10.1049/gtd2.13170