Data-Model Hybrid Driven Dynamic Optimal Voltage Control for AC/DC Hybrid Distribution Network

LU Xiaoxing1，XIAO Xiaolong1，ZHANGE Chenyu1，GUO Ning1，WU Fan1，GUO Jiahao1

1 State Grid Jiangsu Electric Power Co., Ltd. Electric Power Science Research Institute，Nanjing, 211103，China

Abstract: With the development of power electronics technology, AC/DC hybrid distribution networks have gradually become an important form of distribution networks in the future and have attracted widespread attention. The high proportion access of distributed new energy represented by distributed photovoltaic (PV) brings great challenges to the safe and stable operation of AC/DC hybrid distribution networks, meanwhile the randomness and fluctuation of PVs’ power output put forward higher requirements for the real-time and dynamic response of the AC/DC hybrid distribution network optimal voltage control. Firstly, the Q-V voltage control model for PV in AC network and P-V voltage control model for PV in DC network are established. Then, the dynamic optimal voltage control model of AC/DC hybrid distribution network is constructed. After that, the data-driven and model-driven control strategies are combined, the voltage control controller is trained through the Multi-agent Gradient Descent Strategy (MADDPG) to solve the optimal adjustment action value of the PV droop parameter in the real-time state, so as to realize the online dynamic voltage control of AC/DC hybrid distribution network. Finally, based on the IEEE33 node system, an analysis example of AC/DC distribution network is built, and the effectiveness and accuracy of the hybrid data and model driven voltage control strategy proposed in this paper are verified by simulation calculations.

- Introduction

Building a new power system that can accommodate high proportions of renewable energy is a key measure to promote China's goals of carbon neutrality and carbon peak. The high proportion of new energy integration poses significant challenges to the safe and stable operation of the system. As distributed generation and power electronics technologies develop, hybrid AC/DC distribution networks are emerging[1,2]. These networks connect the AC and DC sides using Voltage Source Converters (VSCs). They offer advantages such as flexible control of power flow, achieving load transfer between multiple regions, and optimizing power supply methods. As a result, they are gradually becoming an important form of future distribution networks [3,4].

The strong randomness of new energy generation and the low inertia of power electronic equipment pose significant challenges to the safe and stable operation of hybrid AC/DC distribution networks[5]. These networks face issues such as severe voltage fluctuations and prominent dynamic characteristics. Voltage control technology has become one of the key technologies for the operation of distribution networks. Current typical voltage control methods include centralized optimization, distributed control, and local control[6,7]. Literature [8] proposes a centralized voltage control method for distribution networks, which pre-regulates transformer tap changers and capacitor banks. During the day, it uses distributed power inverters to mitigate source-load fluctuations. Centralized optimization can achieve global optimal coordination, but it relies on a central control center. As the network scale increases, communication and control difficulties also rise. To address the poor scalability and high communication requirements of traditional centralized methods, existing literature has explored distributed approaches. For example, literature [9] uses the coupled branch method to partition hybrid AC/DC distribution networks, thereby decomposing the original voltage control problem into multiple sub-region problems. This approach achieves distributed solving using the Alternating Direction Method of Multipliers (ADMM). Compared to centralized optimization methods, distributed voltage control can reduce computational and communication burdens. However, it often focuses on steady-state optimization and lacks dynamic response capabilities in decision-making[10]. Literature[11] proposed the droop control method based on Q-V and Q-P, which controls distributed power inverters using local measurement information, effectively mitigating frequent voltage fluctuations. However, local control strategies have notable drawbacks; they cannot adapt to changes in grid structure, and different controllers lack communication and global observability, leading to decisions that lack global optimality[12]. The aforementioned three voltage control methods are model-based approaches, essentially relying on the construction and solution of models. However, local control strategies have the advantage of rapid response, enabling real-time control of distribution networks, although they do not achieve global optimality. Centralized and distributed optimization methods can reach global optimality, but due to high computational and communication demands, they struggle to achieve real-time control in distribution networks. Additionally, model-based methods require accurate knowledge of the parameters and topology of the distribution network. For low-voltage distribution networks with limited perception and difficulty in obtaining complete models, the practical application value of these methods is limited[13].

In recent years, with the advancement of digital grid construction and the rapid development of data mining and machine learning technologies, data-driven techniques have been increasingly applied in the operation and control of distribution networks[14]. Data-driven methods typically extract valuable information from massive historical operational data for learning, thereby obtaining control strategies with generalization capabilities. Deep reinforcement learning (DRL) implemented based on Markov games is a typical data-driven method[15], where a trained deep neural network (DNN) can respond within milliseconds through feedforward computation. Literature [16] proposes a data-driven active-reactive power coordinated optimization method for distribution networks, which implements the centralized training-distributed execution architecture to achieve the coordinated optimization of photovoltaic inverters and energy storage systems. Literature [17] proposes a data-model hybrid-driven voltage control method for distribution networks, combining primal-dual deep deterministic policy gradient and optimal power flow models. By utilizing the Lagrangian function, the constrained DRL training problem is transformed into an unconstrained problem, achieving good results. Literature [18] proposes a distributed optimization scheduling method based on multi-agent deep reinforcement learning, which can adapt to source-load uncertainties and provide real-time optimized scheduling strategies under conditions of random fluctuations in sources and loads. Literature [19] proposes a model-free method for voltage regulation in three-phase unbalanced distribution networks when system parameters and topology are unknown. This method learns the relationship between voltages, active power injections, and reactive power injections at each node through a surrogate model. Then, using the rewards provided by the surrogate model, it learns the voltage regulation strategy from historical data through a deep reinforcement learning algorithm.

Although data-driven methods can effectively balance global optimality and dynamic response capabilities in decision-making, they require high-quality and comprehensive data. The data used for training decisions often does not cover all possible operating scenarios in distribution networks. When the data quality or completeness is insufficient, DRL faces issues such as convergence difficulties, inadequate flexibility, and poor decision-making performance[20], which limits the application of data-driven methods in the operation of hybrid AC/DC distribution networks. Meanwhile, both Q-V droop control and P-V droop control are effective simulations of the power-voltage control characteristics of photovoltaic inverters. Their control characteristics conform to the actual operating rules of distribution networks. Combining data-driven and model-based control can significantly enhance the stability and effectiveness of control methods. Based on the above analysis, this paper proposes a data-model hybrid-driven dynamic voltage control method for hybrid AC/DC distribution networks. This method combines multi-agent deep reinforcement learning (MA-DRL) with traditional droop control. It utilizes data-driven techniques to optimize the parameters of local Q-V droop control and Q-P droop control. Consequently, it formulates control decisions according to the dynamic changes in system operating conditions, achieving dynamic voltage control with global optimality in hybrid AC/DC distribution networks. This improves the economic, safe, and reliable operation of distribution networks.

- Distributed Dynamic Voltage Control Framework for AC-DC Hybrid Distribution Networks

This paper constructs a distributed optimization framework for AC-DC hybrid distribution networks, which decomposes the original network into multiple AC subnetworks and DC subnetworks. Each subnetwork is controlled by corresponding agents, with information exchange between adjacent agents to ensure consistent and synergistic distributed control decisions and global optimal control, as shown in Figure 1. Based on this control architecture, this paper combines the MA-DRL (Multi-Agent Deep Reinforcement Learning) with global real-time optimization capabilities and Q-V and P-V droop control with good stability. A data-model joint-driven real-time dynamic voltage control strategy is proposed. This control strategy is based on the global optimal operation target of the distribution network, integrating system topology, network parameters, load demand, and other information. It takes into account the output patterns and dynamic fluctuation characteristics of distributed new energy sources, combining Q-V/P-V droop control of photovoltaic inverters. A real-time dynamic voltage control model for AC-DC distribution networks is constructed, and the MA-DRL is utilized to optimize and solve the parameters of the droop control, thereby achieving dynamic optimization control of the distribution network voltage.

Fig.1. Voltage Control Framework for AC/DC Hybrid Distribution Network

- Dynamical Voltage Control Model for AC-DC Hybrid Distribution Networks

  - Objective Function

The dynamic voltage control method for the AC-DC hybrid distribution network presented in this paper aims to minimize system power loss. The objective function for the power loss is shown below:

|  |  | (1) |
| --- | --- | --- |

Where, and are the voltages at AC nodes and , respectively. and  are the voltages at DC nodes and, respectively.and  represent the resistance and reactance of AC branch , respectively. represents the resistance of DC branch.

  - Constraints

- AC Power Flow Constraints

|  |  | (2) |
| --- | --- | --- |

Where, andare the active and reactive power injected at AC node , respectively. and  are the active and reactive power injected by the VSC converter station at the AC node, respectively.  and are the active and reactive power of the load, respectively. and are the voltage magnitudes at AC nodes and, respectively. is the number of AC nodes. and are the real and imaginary parts of the elements in the -th row and -th column of the admittance matrix of the AC distribution network, respectively.  is the phase angle difference between AC node   and node   .

- DC Power Flow Constraints

|  |  | (3) |
| --- | --- | --- |

Where,  is the active power injected at DC node .  is the active power injected by the VSC converter station at the DC node. is the active power injection from the photovoltaic  system into the DC node. is the active power load at the DC node. and  are the voltages at DC nodes  and , respectively. is the number of DC nodes. represents the element in the -th row and -th column of the conductance matrix of the DC distribution network.

- VSC Constraints

Fig.2 VSC equivalent circuit diagram

VSC is a key device in the AC-DC hybrid distribution network, connecting the AC side network with the DC side network. Its structure is shown in Figure 2. In this figure: and  represent the active and reactive power injected at the AC port , respectively. The AC voltage on the AC side of port  is denoted as . The AC voltage on the converter side of port is denoted as . represents the equivalent loss of the converter at port .  and represent the fundamental frequency reactance of the converter at port  and the fundamental frequency reactance of the AC filter, respectively. Based on this equivalent circuit diagram, the VSC model is simplified to obtain its mathematical model of operation, as follows:

|  |  | (4) |
| --- | --- | --- |
|  |  | (5) |
|  |  | (6) |

Where,and are the active and reactive power injected from the AC side to the VSC, respectively. is the active power output on the DC side. is the internal reactive power of the VSC.  and are the internal voltage of the VSC and the DC side voltage, respectively. is the DC voltage utilization rate. is the modulation index of the VSC, where . and  are the upper limits of the active and reactive power input to the AC side of the VSC, respectively.

- Safety Operation Constraints

The safety operation constraints of the AC-DC hybrid distribution network include node voltage constraints and line current flow constraints. Their mathematical models are shown as follows:

|  |  | (7) |
| --- | --- | --- |

Where,  andare the upper and lower limits of the AC voltage node, respectively.  and  are the upper and lower limits of the DC voltage node, respectively.  and are the active and reactive power of AC line, respectively.  is the active power of DC line . is the upper limit of the current flow for AC line . is the upper limit of the current flow for AC line .

  - Parameter-Adjustable Inverter Droop Control Model

- Inverter Q-V Droop Control Model

(a) Conventional Q-V Droop Control

(b) Parameter-Adjustable Q-V Droop Control

Fig.3. The Q-V droop control models

As shown in Figure 3, Figure 3(a) represents the conventional droop control model, which utilizes the sensitivity relationship between voltage and reactive power to regulate the voltage. This method has advantages such as rapid response and simplicity. However, due to its focus on regulating voltages that exceed limits, it has a significant voltage dead zone. The control strategy only activates when the voltage exceeds this dead zone, leading to insufficient flexibility. Furthermore, the droop parameters of this type of control method are often pre-set and difficult to adaptively adjust according to the network parameters, system structure, and fluctuations in renewable energy in AC-DC hybrid distribution networks. Consequently, it is challenging to fully utilize the regulation capability of the remaining capacity of PV inverters. As the proportion of distributed renewable energy increases, the impact of this lack of flexibility becomes more pronounced.

Given this, this paper proposes a parameter-adjustable Q-V droop control method. By flexibly selecting the droop control intercept based on the actual operating conditions of the distribution network, it can better leverage the flexible regulation capability of the inverter. Its mathematical model is shown as follows:

|  |  |  |  |  |  |  | (8) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | (9) |  |  |  |
|  |  |  |  |  | (10) |  |

Where,  is the set of buses in the AC subnetwork m where PV is installed., , and  represent the active power, reactive power, and apparent power injected by PV at node  at time , respectively. and  represent the upper and lower limits of the reactive power provided by PV, respectively. is the slope of the Q-V droop curve.denotes the voltage magnitude at node  at time . denotes the droop curve intercept of PV at node  at time . is the optimal droop curve intercept.  is the number of system nodes.  is the sensitivity of the voltage magnitude at node  to the injected reactive power.

- Inverter P-V Droop Control Model

(a) Conventional P-V Droop Control

(b) Parameter-Adjustable P-V Droop Control

Fig.4. The P-V droop control models

On the DC distribution network side, photovoltaic (PV) inverters are effective resources for voltage regulation. P-V droop control is an important technique to fully leverage the regulation capabilities of these inverters. The conventional P-V droop control is illustrated in Figure 4(a). Similar to the traditional Q-V droop control, the conventional P-V droop control also suffers from dead zones and lacks flexibility, failing to achieve error-free regulation during voltage adjustments. Following the same research approach, this paper proposes a parameter-adjustable P-V droop control method, as shown in Figure 4(b). Its mathematical model is as follows:

|  |  | (11) |
| --- | --- | --- |
|  |  | (12) |
|  |  | (13) |

Where, is the set of buses in the DC subnetwork  where PV is installed.and  represent the active power and apparent power injected by PV at DC node  at time , respectively.  is the slope of the P-V droop curve. denotes the voltage magnitude at node  at time. denotes the droop curve intercept of PV at node at time .  is the optimal droop curve intercept. is the number of system nodes.  is the sensitivity of the voltage magnitude at node  to the injected active power.

Equations (8) and (11) describe the behavior patterns of PV reactive power and active power, respectively. Equations (9) and (12) indicate that the droop slope is determined by the number of nodes and the system's sensitivity. In this paper,  and  are set as adjustable parameters, with the goal of minimizing system operation losses. Through model construction and solving, the optimal values of these adjustable parameters suitable for the current distribution network can be obtained.

- Solving Strategy Based on Deep Reinforcement Learning Markov Decision

  - Representation for Dynamic Voltage Control Model

In the real-time voltage control problem, the AC-DC hybrid distribution network can be considered as the environment, and the central controller that controls the PV devices is defined as an agent with the ability to interact with the environment[22-24]. After the AC-DC hybrid distribution network is partitioned, each region is controlled by a central controller. Each central controller is responsible for adjusting the positions of the Q-V droop curves of all inverters within the region to change the reactive power output and regulate the voltage. By treating the controllers as agents, the local observations  within the region are transmitted as inputs to the agents, which then generate dispatch commands and send them back to the inverters. Thus, the real-time voltage control problem can be described as a decentralized partially observable Markov decision process (Dec-POMDP) among multiple agents[25]. More specifically, a Dec-POMDP can be defined as a tuple:

|  |  | (14) |
| --- | --- | --- |

Where,  represents the set of agents, with one agent in each region;  represents the state of the AC/DC hybrid distribution network at time step  ;  is the set of regions ;  is the action set of all agents, ,where  represents the action taken by the agent in area .  represents the number of PV inverters in region . is the policy set, and  is the strategy adopted by the agent when taking action;  is the joint observation set of all agents, , where  is the partial observation of the m-th agent, which consists of the feature vector of all buses in region .

is the immediate reward at each decision step. The goal of the model is to reduce network losses; therefore, the immediate reward of the agent at time  is .  is the discount factor, which can be used to balance the importance between short-term rewards and long-term rewards."

The environment, which is based on the actual system configuration and the power flow equations in (2), (3), (4), and (5), models a hybrid AC/DC distribution system. At each time step, the hybrid AC/DC distribution network is in a specific operational state. A joint observation value is generated by the global state, from which each agent observes its partial components. The local observation consists of the necessary information from all the buses within the region.

|  |  | (15) |
| --- | --- | --- |

Where, is the feature vector of node  at time within the region ,  is the number of nodes in region , and  consists of voltage magnitude, active and reactive power of local loads, and active and reactive power output from PV inverters, as shown below:

|  |  | (16) |
| --- | --- | --- |

  - MADDPG Algorithm

The MADDPG algorithm is an extension of the Deep Deterministic Policy Gradient algorithm in multi-agent systems. In this approach, multiple agents not only share global state information but also their individual action information, effectively addressing the non-stationary problem in multi-agent scenarios[26]. Therefore, this paper uses the MADDPG method, as shown in the figure, to solve the Dec-POMDP model constructed based on the real-time voltage control problem. The MADDPG employs an independent policy-critic architecture for each agent, optimizing both networks involved in the real-time voltage control problem simultaneously.

The policy network maps the current state  to the required output  based on the current policy network parameters . The critic network maps the state and action tuple  to the expected cumulative reward, with its input being the actions  taken by each agent based on policy network  and the environmental information . Its output is the  value for the -th agent. They train each other to improve the value function's ability to better predict outcomes and enhance learning effectiveness and stability during training by considering the strategy information of other agents.

Fig.5. The structure of the MADDPG

1)  Value Function

The value function  is the expected cumulative reward when taking actionin state under policy . The parameters of the action value function can be optimized by minimizing the following loss function:

|  |  | (17) |
| --- | --- | --- |
|  |  | (18) |

2)  Policy Function

The function of the policy network is to map the current state to an action. The parameters  of the policy function  should be updated in the direction of its gradient, that is:

|  |  | (19) |
| --- | --- | --- |

The parameters of the policy network are adjusted in the direction of maximizing Q values, where Q values represent the expected cumulative rewards that the agent obtains in a given state.

3) Target Network

To improve the stability during the training process, the MADDPG algorithm introduces target critic functions  and target actor functions  to compute the target values . By introducing the target network, equation (19) can be rewritten as equation (20).

|  |  | (20) |
| --- | --- | --- |

The parameters of the target actor function  and the target critic function are updated through soft updates: , where .

4) Experience Replay Pool

During the training process of the MADDPG algorithm, the correlation between data points leads to significant variance in parameter updates, thereby affecting the learning performance. To improve the training effect, an experience replay mechanism is used to store the experience samples accumulated during the interaction between agents and the environment and randomly sample these experiences for model training. This helps reduce the correlation between sequential data and enhances the model's training efficiency and effectiveness.

In the MADDPG algorithm, there are two sets of parameters to be optimized: the critic network parameters  and the policy network parameters . The training process is illustrated in Table 1.

Table1 Algorithm flow

Algorithm Flow:

Hyperparameters: Soft update factor, reward discount factor . For each agent : Initialize the replay buffer , initialize the critic network parameters , policy network parameters , target network , . Initialize the target network parameters , . Repeat (for each episode): Initialize the state of the hybrid AC/DC distribution network. Repeat (for each time step): For each agent : Based on the current policy and state , observe , choose action and execute all agents’ actions , obtaining new state and reward . For each agent , store the transition datainto. For each agent : Randomly sample a batch of transition data from . For each sampled transition data Use the target network to calculate the target value according to Equation (21). Update the critic network according to Equation (18). Update the policy network according to Equation (20). For each agent : Perform soft updates of the target network parameters: Until the environment ends the episode. Repeat until the termination condition is met.

- Case Analysis

  - Case Introduction

This case study is a modified hybrid AC/DC distribution network based on the IEEE 33 system, with photovoltaic (PV) units added to validate the effectiveness of the proposed model-data joint-driven dynamic voltage control. The system is divided into two sub-regions based on the location of the voltage-source converters (VSCs), as shown in Figure 6. The system reference voltage is 10 kV, with the voltage safety range set to [0.95, 1.05] p.u. The parameters of the PV units and VSCs are detailed in Tables 2 and 3. PV6 and PV7 connected to the DC distribution grid, can regulate active power, while the remaining PV units connected to the AC distribution grid can regulate reactive power. The power factor (p.u.) prediction curves for the PV units and total load are shown in Figure 7. Most of the time within a 24-hour period, the PV output cannot meet the load demand and requires power supply from the external grid. Only during certain periods from 12:00 to 13:00, the PV output fully meets the load demand, resulting in a situation where the DC distribution grid sends power back to the external grid.

Fig.6. IEEE 33-bus AC-DC hybrid distribution system.

Table2 Parameters of PV

| Equipment | Parameters | Connection Nodes |
| --- | --- | --- |
| PV | Capacity：600 kVA | AC：10、12、14、15、18、22、25 DC：5、7 |

Table3 Parameters of VSC

| Resistance/ | Reactance/ | /MVA |
| --- | --- | --- |
| 0.0082 | 0.0035 | 1.1 |

Fig.7. Factor of PV Active Power and Active Load

  - Case Analysis

The initial parameter and variable settings for the MADDPG algorithm used in this article are shown in Table 4.

Table4 Setting of MADDPG algorithm parameters

| Algorithm Name | MADDPG |
| --- | --- |
| Value Network | 128-128 |
| Value Parameter Learning Rate | 1e-4 |
| Strategy Network | 128-128 |
| Actor Parameter Learning Rate | 1e-3 |
| Discount Factor | 0.99 |
| Soft Update Factor | 0.01 |
| Activation Function | ReLU |
| Action Noise |  |

In the MADDPG control strategy adopted in this paper, to ensure the effectiveness of voltage correction control and power loss optimization control, the weight coefficients for the voltage deviation or power loss optimization parts in the objective function are set to be much higher than those for the control cost part. The convergence process of the loss function during the training of the reactive voltage controller in the hybrid AC/DC distribution network is shown in Figure 8. As can be seen from the curve in the figure, the decline in the loss function is significant in the early stages of training. After reaching a certain stage, the rate of decline gradually decreases and becomes more stable. After 800 iterations, the loss function during the controller training process basically reaches convergence.

Fig.8. Convergence Process of Loss Function During Training of Voltage Controller in AC/DC Hybrid Distribution System

After training, the hybrid AC/DC distribution network voltage optimization controller conducts 24-hour voltage optimization control on the IEEE 33 system. The control decisions and control effects for each device are analyzed as follows:

- Photovoltaic Unit Decision-Making

For photovoltaic units connected to the AC distribution grid, their reactive power decisions over 24 hours are shown in Figure 9. For photovoltaic units connected to the DC distribution grid, their active power decisions over 24 hours are shown in Figure 10.

Fig.9. PV Reactive Power Output Decision

Fig.10. PV Active Power Output Decision

- VSC Device Decision-Making

The active power decisions for VSCs connected to the DC distribution grid over 24 hours are shown in Figure 11. It can be observed that the VSC devices connected to the hybrid AC/DC distribution network satisfy the safety and stability operation constraints during the participation in voltage optimization control.

Fig.11. Active Power Exchange for VSC

- Voltage Distribution and Power Loss

After real-time decision-making by the hybrid AC/DC distribution network voltage optimization controller, the 24-hour voltage distribution in the IEEE 33 system is shown in Figure 12.It can be observed that the node voltages in both the optimized AC and DC distribution grids remain within the safe operating range of the grid, with no voltage violations. This effectively reduces voltage deviations, enhances grid operation safety, and improves power supply quality. Additionally, the network losses in the AC side distribution grid have been reduced by 959.2034 kW, and the network losses in the DC side distribution grid have been reduced by 97.3295 kW. These improvements effectively enhance the energy transmission efficiency of the grid, improving the reliability and economic performance of the distribution network.

Fig.12(a). Power Profile for AC sub-network

Fig.12(b). Power Profile for DC sub-network

- Conclusion

This paper proposes a novel data-model hybrid-driven voltage optimization control method for hybrid AC/DC distribution networks. The feasibility of the proposed voltage optimization control method is verified using an adjusted IEEE 33-node test system as a case study. By adjusting the output power of distributed photovoltaic sources connected in the hybrid AC/DC distribution network, the active power losses in the system can be reduced while ensuring that the node voltages meet the safety and stability requirements. This improves the overall system's operating status, thereby enhancing the reliability and economic performance of the hybrid AC/DC distribution network. In future research, to better utilize the support capability of distributed power sources and energy storage devices in the hybrid AC/DC distribution network for node voltages, the coordination among various adjustable resources can be improved based on the regulation characteristics and time scales of different adjustable resources. This will address the requirements for adjustable ranges, response speeds, and regulation costs under different application scenarios, further enhancing the safety, stability, and economic performance of the hybrid AC/DC distribution network.

- Acknowledgments

This work was supported by the State Grid Corporation of China (Grant No. 5700-202355295A-1-1-ZN).

- References

- Haytham M A Ahmed, Magdy M A Salama. Energy management of AC-DC hybrid distribution system considering network reconfiguration[J]. IEEE Transactions on Power Systems, 2019, 34(06):4583-4593.

- Haytham M A Ahmed, Ayman B Eltantawy, Magdy M A Salama. A planning approach for the network configuration of AC-DC hybrid distribution systems[J]. IEEE Transactions on Smart Grid, 2018, 09(03):2203-2213.

- Qianwen Xu, Tianyang Zhao, Yan Su，et al. A distribution and robust energy management system for networked hybrid AC/DC micro-grids[J]. IEEE Transactions on Smart Grid, 2020, 11(04): 3497-3507.

- Hong Liu, Zhanfeng Deng, Xialin Li, et al. The averaged-value model of a flexible power electronics based substation in hybrid AC/DC distribution system[J]. CESS Journal of Power and Energy System, 2022, 08(02): 452-463.

- Yang Fu, Zhiquan Zhang, Zhenkun Li, et al. Energy management for hybrid AC/DC distribution system with microgrid clusters using non-cooperative game theory and robust optimization[J]. IEEE Transactions on Smart Grid, 2020, 11(02):1511-1525.

- Hanyan Huang, Shiyi Zhang, Lijun Zhang, et al. Exploiting the operational flexibility of wind integrated hybrid AC/DC power systems[J]. IEEE Transactions on Power Systems, 2021, 36(01): 819-825.

- Zhaoyu Wang, Jianhui Wang, Bokan Chen, et al. MPC-based voltage/var optimization for distribution circuits with distributed generators and exponential load models[J]. IEEE Transactions on Smart Grid, 2014, 5(5): 2412-2420.

- Xuexue Zhang, Sanyang Liu, Nannan Zhao. An accelerated algorithm for distributed optimization with Barzilai-Borwein step sizes[J]. Signal Processing, 2023, 202: 108748.

- Xianzhuo Sun, Jing Qiu, Junhua Zhao. Real-Time Volt/Var Control in Active Distribution Networks With Data-Driven Partition Method[J]. IEEE Transactions on Power Systems, 2021, 36(3):2448–2461.

- Bo Zhao, Zhicheng Xu, Chen Xu, et al. Network partition-based zonal voltage control for distribution networks with distributed PV systems[J]. IEEE Transactions on Smart Grid, 2017, 9(5): 4087-4098.

- Attar M, Homaee O, Falaghi H, et al. A novel strategy for optimal placement of locally controlled voltage regulators in traditional distribution systems[J]. International Journal of Electrical Power & Energy Systems, 2018, 96: 11-22.

- Delong Zhang; Jianlin Li; Dong Hui. Coordinated control for voltage regulation of distribution network voltage regulation by distributed energy storage systems[J]. Protection and Control of Modern Power Systems, 2018, 3(1): 8.

- Min Tan, Chuanjia Han, Xiaoshun Zhang, et al. Hierarchically correlated equilibrium Q-learning for multi-area decentralized collaborative reactive power optimization[J]. CSEE Journal of Power and Energy Systems, 2016, 2(3):65–72.

- Ruan J, Fan G, Zhu Y, et al. Super-Resolution Perception Assisted Spatiotemporal Graph Deep Learning against False Data Injection Attacks in Smart Grid[J]. IEEE Transactions on Smart Grid, 2023.

- Zidong Zhang, Dongxia Zhang, Qiu, Robert C. 	Deep reinforcement learning for power system applications: An overview[J]. CSEE Journal of Power and Energy Systems, 2019, 6(1): 213-225.

- D. Cao, J. Zhao, J. Hu, et al. Physics-Informed Graphical Representation-Enabled Deep Reinforcement Learning for Robust Distribution System Voltage Control[J]. IEEE Transactions on Smart Grid, 2024,6(3): 233-246.

- D. Cao, J. Zhao, W. Hu, et al. Model-free voltage control of active distribution system with PVs using surrogate model-based deep reinforcement learning[J].Applied Energy, 2022, 47(13):451-454.

- Shenyi Wang, Jiajun Duan, Di Shi, et al. A Data-driven Multi-agent Autonomous Voltage Control Framework Using Deep Reinforcement Learning[J]. IEEE Transactions on Power Systems, 2020, 35(6): 4644-4654.

- Liu H, Zhang C, Chai Q, et al. Robust regional coordination of inverter-based Volt/Var control via multi-agent deep reinforcement learning. IEEE Transactions on Smart Grid 2021;12 (6):5420-5433.

- Toubeau J F, BB Zad, Hupez M, et al. Deep Reinforcement Learning-Based Voltage Control to Deal with Model Uncertainties in Distribution Networks[J]. Energies, 2020, 13(15): 3928-3942.