A Multi-Objective Interval Optimization Approach to Expansion Planning of Active Distribution System with Distributed Internet Data Centers and Renewable Energy Resources

Yuying Zhang1, Chen Liang1, Han Wang1, Jiayi Zhang1,*, Bo Zeng1 and Wenxia Liu1

1School of Electrical and Electronic Engineering, North China Electric Power University, Beijing 102206, China;

2CHN Energy New Energy Technology Research Institute Co. Ltd, Beijing 102209, China.

*Corresponding Author: Jiayi Zhang. Email: zhangjiayi1009@163.com

Abstract: With the development of the digital economy, the power demand for data centers (DCs) is rising rapidly, which presents a challenge to the economic and low-carbon operation of the future distribution network. To this end, this paper fully considers the multiple flexibility of DC and its impact on the distribution network, and establishes a collaborative planning model of DC and distribution network. In this model, the interval method is  utilized to capture the inherent uncertainties within the system (such as the renewable energy source (RES) output, wholesale market price, power load demand, carbon emission factor and workloads), and the planning model is transformed into a multi-objective optimization problem with interval parameters problem (IMOP) to minimize economic cost and carbon emission. On this basis, an interval multi-objective optimization evolutionary algorithm based on decomposition (IMOEA/D) is proposed to solve the IMOP and obtain the Pareto optimal solution while retaining all the uncertainty information. Finally, an improved IEEE 33-node distribution network is utilized as an example for simulation and analysis to confirm the efficacy of the proposed approach.

Key words: Data Centers; Renewable Energy Sources; Multi-Objective Optimization; Carbon Emission Reduction; Interval Optimization

1. Introduction

In recent years, with the increasing integration of emerging technologies such as artificial intelligence, cloud computing, and big data, the demand for data computing services has increased significantly. As a hub for storing massive data and undertaking computing tasks, the scale of the data center (DC) will continue to expand. Data centers' energy usage is predicted to account for around 2% of total worldwide electricity consumption, with power demand expected to expand at a 15%-20% annual rate[1].  Therefore, if the DC load is supplied based on China 's current power grid structure (fossil energy power generation accounts for about 70 %), it will not only have an impact on the safety and economy of power system operation, but it will also emit a significant quantity of carbon dioxide from power generation, which is detrimental to achieving the carbon neutrality target [2].

In order to deal with the above problems, existing research has explored to develop the IT energy-saving technology from the equipment level., such as optimization of heat dissipation and cooling system[4], intelligent power management[5], dynamic resource management [6], etc. However, due to economic and technical limitations, it is difficult to solve the above problems only relying on the device level.

On the other hand, some studies, starting from the system level, develop the flexibility of DC and make the interaction of DC and distribution network (DN), to promote the overall operation efficiency. Reference [7] proposed a flexible scheduling strategy for DC based on the spatio-temporal adjustment potential of workloads, resulting in a significant enhancement in the energy efficiency of the DN. In [8], the flexible scheduling strategy of workloads is proposed to the DC with electric energy storage device, renewable energy and water cooling system, so as to reduce the operating cost. Reference[9]adopts the time-space scheduling method of load to maximize the advantages of renewable energy, realize low-carbon and cost-effective operation, and improve the utilization efficiency of renewable energy. Aiming at the low-carbon development of power system, literature[11][12] proposes a joint dispatching model integrating wind and solar power system with DC, which can significantly enhance the absorption and utilization of renewable energy while reducing operating costs by deeply exploring the temporal and spatial distribution of workloads.

Although the above literatures have made great contributions to the energy saving and emission reduction of data centers, they are all studied from the perspective of operating. However, in practical, the system benefits of different DC and distribution network capacity allocation schemes are significantly different, so to obtain the optimal integration, the coordination of the two needs to be fully considered in the planning stage. Reference[14]proposes an optimal planning approach for distributed DC-microgrids to promote economic and environmental performance. Reference[15]proposed a location and capacity optimization planning scheme for DC and battery energy storage system to improve system service quality. Reference[16]proposed an o planning framework of regional electric-thermal DC integrated system considering wind power uncertainty to actively cope with wind power uncertainty and improve system flexibility. Reference [17] proposed a flexible uncertainty set robust multi-objective optimization model based on affine tunable strategy for DC microgrids to reduce the operating cost and the redundancy level of computing resources.

The above results lay a crucial foundation for the research on the coordination and integration of DC and DN, however, most of the existing studies ignore the uncertainty in the system [3][5][7][10][12][15], or simply use stochastic optimization (SO)[6], robust optimization (RO)[16][17], which has an important impact on the operation. However, if the uncertainty is not considered, it may lead to unstable system energy consumption, unbalanced power load, insufficient utilization of renewable energy, thus reducing the overall reliability and energy efficiency. In addition, RO is based on worst-case analysis, which suffers from being too conservative; Besides, SO needs to know the likelihood distribution of the uncertain factors in advance to generate scenarios, which is difficult to execute when the uncertain parameter distribution is unknown.

In order to integrate the advantages of both methods, interval optimization (IO) methods have been developed and applied to uncertain problem. Compared with the traditional stochastic optimization (SO), IO represents the uncertainty in the form of interval variables, which is defined by the lower and upper boundaries to describe the range of the associated data set, which is simpler to get in the case of long-term planning. And, IO strives to minimize the interval of the objective function, avoiding overly conservative decisions. This feature makes IO more practical and adaptable in the face of uncertain optimization problems, and provides more flexible options for decision makers. Due to these unique advantages of IO, it is frequently utilized in the field of power system research. [].

Despite the growing number of studies on IO, it is still a challenging problem to solve the mathematical formulation of interval symbols efficiently. At present, the most commonly used method to solve the IO problem is to remove interval variables and transform it into a deterministic problem by using deterministic conversion. For example, reference [18] replaces the interval optimization objective with the optimal objective, and replaces the interval inequality constraint with the deterministic inequality constraint of the interval's maximum range, so as to transform the model into a deterministic problem. Reference [19][20] proposed the order relation and probability degree of interval number to transform the IO model into a deterministic optimization problem.

In addition, it will be more difficult to solve the interval optimization problem with multi-objective compared to the single objective. Reference [21] reformulated the multi-objective optimization problem with interval parameters problem (IMOP) by  considering  the midpoint of the interval objective as the optimization objective. Reference [22] expands all interval numbers and linearizes the model, and equivalently transforms the multi-objective optimization problem into a single-objective interval fuzzy programming problem to obtain the optimal value interval of each decision variable.

However, the above IO solution methods mainly deal with interval symbols through deterministic conversion, which inevitably leads to the omission of some essential information during the decision-making process. In IO, because it is the interval of the target that needs to be optimized rather than the specific scenario, the loss of this key information may hinder the solutions from resisting uncertainties in possible realizations.

To address the knowledge gaps mentioned above, this paper constructs a collaborative planning model of DC and distribution network to advance the low-carbon development of distribution network with DC, where the multiple flexibility of the DC and its impact on distribution network is fully considered. Given that interval optimization is adopted to address the uncertainties in the system (e.g., the RES output, wholesale market price, power load demand, carbon emission factor, and workloads), the planning model is reformulated as a multi-objective optimization problem with interval parameters problem (IMOP) to minimize economic costs and carbon emissions. In addition, to address the limitations of the current approaches to solving the problem, an interval multi-objective optimization evolutionary algorithm based on decomposition (IMOEA/D) is proposed. Differing from the previous literature, which resorts to deterministic conversion, the planning scheme may be made robustly immune to uncertainties by using the suggested IMOEA/D approach, which preserves all uncertain information throughout the solution process and yields the Pareto optimum solution. Finally, to validate the effectiveness of the proposed method, the improved IEEE-33 node distribution network is utilized as a case study to verify the effectiveness of the proposed planning model and algorithm.

The primary contributions of this paper are as follows:

(1) Aiming at the efficient integration of DC and distribution network, this paper proposes a collaborative planning framework for DC and DN, where the operational flexibility of DC is captured (mainly from the spatio-temporal distribution of workloads, thermodynamic characteristics of DC building). Considering the uncertainties existed in the system, the planning framework is reformulated as a multi-objective optimization problem with interval parameter problem to co-optimize economic and environmental performances.

(2) Interval optimization is implemented to solve the uncertainty of RES outputs, wholesale market prices, electrical load demands, carbon emission factor, data workloads, which is represented by the lower and upper bounds of the relevant  datasets.

(3) An IMOEA/D is developed to deal with the proposed IMOP. In contrast to the conventional deterministic conversion metho, IMOEA/D method could effectively retain the whole uncertain characteristics, has better risk aversion ability and robustness to uncertainty.

The remainder of this paper is arranged as follows. Section 2 introduces the model framework. Section 3 describes the DC model. The fourth section proposes a DN and DC collaborative planning model based on IMOP. The fifth section introduces the multi-objective evolutionary algorithm based on decomposition. The sixth section proves the effectiveness of the proposed model and algorithm through case studies. Finally, the conclusion is presented in the seventh section.

2. Problem description

2.1 System structure

The typical equipment composition of a data center (DC) is shown in Figures 2 (a). Inside the DC, the electrical devices encompass primarily IT equipment, notably physical servers, alongside auxiliary devices, including lighting systems. During the operation of DC facilities, servers, lighting would generate heat dissipation. Given the precise temperature requirements of IT equipment, DCs are required to have air conditioning equipment(ACE) in place to fulfill their cooling needs, thereby ensuring that the internal environmental temperature is always maintained within a safe range. In addition, a front proxy is included in the DC, which is tasked with monitoring the usage of computational resources in real-time and allocating workloads to designated DCs to optimize processing with the intent of reducing expenses and enhancing environmental performance.

Based on the data characteristics, the workloads in a DC can generally be categorized into two types: delayable workloads (DW) and real-time workloads (RW). DW refers to workloads with a fixed amount of tasks within a certain time range but with flexible and variable processing times. Common examples of DW in a DC include big data offline analysis and other batch processing loads. RW denotes tasks that have set processing times but can be dynamically relocated across various DC locations based on necessity. Common instances of RW include streaming of entertainment content, web surfing, audio and video communication, and electronic commerce activities.

Figure 2-1: Schematic diagram of DC-Power distribution network interaction system.

2.2 Model framework

In the context of the deep informatization in power systems, the energy provider will be more inclined to engage in developing its DCs (i.e, private data centers dedicated solely to communication with the distribution network) and employ advanced communication technologies to enhance the operation of the distribution network. Therefore, the DC and DN are operated as an integrated system, where DCs are used for DN data analysis, and DN supply reliable electricity for DCs. Consequently, in this study, it's posited that the DSO oversees both the investment in and operation of DC and renewable energy sources (RES).

The integrated system with the DN and DCs is shown in Figures 2 (b). One can note that the DN, incorporating DC, primarily encompasses three elements: the network's distribution lines, the DCs themselves, and RES. The DN is linked to the upper-tier transmission grid via distribution transformers, offering power supply services to users within the DN, encompassing DCs, through both the acquisition of electricity from external markets and distributed power production.

In this new scenario, the DN is tasked with devising the system's best configuration plan to reduce economic costs. However, the integration of DC without proper coordination can lead to increased carbon emissions from power generation. Hence, environmental considerations must also be factored into the model. This requirement leads to the formulation of the proposed model as a multi-objective optimization model.

As aforementioned, the decision-making problem is pertinent to multidimensional uncertainties which mainly arise from RESs output, wholesale market prices, power demands, workloads and carbon emission coefficients. In this study, interval optimization is used to address the above the uncertainties. Therefore, the uncertain variables are represented using a collection of predicted confidence intervals, where each interval is defined by the upper and lower limits of uncertain variables, based on forecast values and statistical data collected over a year.

In summary, the problem is structured as a multi-objective optimization model incorporating interval variables (IMOP), which is difficult to solve. As such, we develop an IMOEA/D approach to solve the proposed IMOP problem, which could preserve all uncertain information in the solution process, so as to obtain the planning solution defending the possible risk.

3. DC modeling

3.1 Workloads processing model

3.1.1 Delay-tolerant workloads (DW)

Considering the nature of DW requirements, the DW characteristic is represented as follows:

(1)

(2)

(3)

(4)

Equation (1) delineates the DW capacity ratio. Equation (2) ensures the data demand is balanced before and after a response is made. Equation (3) defines the constraint characteristic of time-shifting. Equation (4) sets forth the constraint on the total data load demand transfer.

3.1.2 Real-time workloads

For the scheduling control of RW, the relevant constraints are expressed as follows:

(5)

(6)

Equation (5) represents the capacity ratio of RW. Equation (6) ensures the balance of RW demand transfer.

Based on Equations (17) to (26), considering spatial and temporal distribution of workloads, the aggregate workload required to be processed by the DCs within each time frame can be articulated in the following manner:

(7)

3.1.3 Quality of service requirement

Indeed, when handling workloads, one must account for inherent transmission and queuing latencies. To ensure that DCs maintain an acceptable quality-of-service (QoS), it is imperative that the aggregate of these transmission and queuing latencies remain below the workloads' maximum permissible delay threshold. To simplify the problem-solving process, this document presumes that all servers within the DCs are of a uniform model, and workloads are distributed equally across servers that are operational. Therefore, based on M/M/1 queue theory, the QoS of workloads can be determined as follows:

(8)

(9)

(10)

Equation (8) calculates the queue waiting time based on M/M/1 queue theory. Equation (9) calculates the data processing time, which is inversely proportional to the server processing speed. Equation (10) represents the data workload delay duration constraint.

3.2 Thermodynamic Characteristics

3.2.1 Server heat removal

As DC servers are typically located inside building structures, drawing from the thermal storage properties of the structure and the principles of energy conservation, the comprehensive heat balance equation for the data center can be formulated as follows:

(11)

3.2.2 Heat exchange power through the walls:

The power of heat exchange across the walls correlates with the surface area of the DC's building and the temperature difference between its interior and exterior.It is typically determined by the following equation:

(12)

3.2.3 Heat dissipation power from servers:

When servers are in operational status, their heat dissipation power can be calculated by multiplying their electrical power consumption by the heat dissipation coefficient :

(13)

3.2.4 Heat dissipation power from auxiliary equipment:

The heat dissipation power from auxiliary devices, like interior lighting and power distribution systems, encompasses both no-load and operational losses. This can be quantified through the following calculation method:

(14)

(15)

Taking into account that the DC building is typically a windowless enclosed structure, the internal heat is influenced by the combined effects of , , , . This can be articulated in the manner described below:

(16)

(17)

Equation (16) represents the thermal balance inside DC. Equation (18) represents the heat exchange inside and outside DC.

3.2.5 Climatic Temperature Limits:

To guarantee the secure and consistent functioning of devices within the DC, controlling the indoor temperature and its fluctuations within a specified limit is essential. Hence:

(18)

(19)

Equation (18) represents the temperature constraint for DC. Equation (19) represents the temperature change constraint for DC.

3.3 Power Usage Attributes

The power usage of a DC primarily consists of three main components: computing servers, cooling systems, and various support equipment. Usually, the electricity used by computing servers and cooling systems together represents more than 90% of the entire energy expenditure of the DC. Hence, the overall energy consumption of the DC while operational can be formulated as::

(20)

3.3.1 Server energy consumptions

The power usage of servers is mainly influenced by the quantity of active servers and the intensity of the workload within the given time frame. For simplicity, this paper assumes that all servers in the DC are the same, and the workload at any given time is evenly distributed among all active servers. The characteristics are expressed as follows:

(21)

(22)

(23)

Equation (21) represents the formula for calculating the power usage of a single server. Equation (23) defines the limitations on the maximum quantity of servers that can be activated within the DC. Equation (24) imposes constraints on the processor usage levels of the servers.

3.3.2 ACE energy consumption

The operation characteristics of the air conditioning system can be expressed as follows:

(24)

(25)

Equation (25) calculates the air conditioning power, which is related to the cooling power and energy efficiency coefficient. Equation (26) represents the air conditioning installation capacity constraints.

4. A collaborative planning model for DN and DC based on IMOP

4.1 Objective function

4.1.1 Minimization of economic cost

The primary goal of the suggested IMOP planning framework is to reduce financial expenditures, encompassing both the annualized costs of capital investment and the expenses incurred from operations as delineated below:

(26)

The investment cost  corresponds to the initial capital costs that associated with RES, DN and DC’s equipment installation, which is presented as:

(27)

The operational cost of the system  includes the purchased electricity cost, RES curtailment penalty, and the associated costs generated by DC participation in demand response:

(28)

(29)

(30)

(31)

Equation (30) represents the electricity purchase cost, which is computed as the product of market price and the quantity of electricity purchased. The RES curtailment penalty cost, as described in Equation (32), is derived based on the penalty term and the amount of curtailed RES. Equation (33) is the related cost incurred by DC's participation in demand response.

4.1.2 Minimization of emissions

In practice, within DN where the share of fossil fuel energy sources on the generation side is comparatively high, the establishment of an energy-demanding DC will lead to significant carbon emissions. As such, minimizing the system's carbon emissions is considered as the second objective:

(32)

4.2 Constraints

4.2.1 Planning stage

During the planning phase, given the limitations of the overall investment budget and available space, it becomes essential to apply restrictions on the maximum capacity of different types of equipment that can be installed within the system. Equations (36) through (38) specify the restrictions on the expansion capacity of transformers, the choices for extending distribution lines, and the allowable installation capacity for RES and DC equipment, such as servers and ACE, respectively.

(33)

(34)

(35)

4.2.2 Operation stage

(1) RES

The electricity generation by RES units is limited by their utmost installed capacity and the capacity factor, which accounts for the impact of weather conditions:

(36)

(37)

(2) DN

Nodal voltage limitations:

To guarantee that the voltage quality within the distribution network adheres to predefined standards, it's essential to impose restrictions on the voltage variations at every node during its operation, as specified by:

(38)

Nodal power balance:

The outgoing power from each node in the distribution network should be equal to the incoming power, expressed as:

(39)

Electricity distribution limitations in the energy network:

The operation of the DN must adhere to power flow limitations within the power grid, which are illustrated as:

(40)

(41)

Line current constraints:

The active/reactive power transmitted via each line within the distribution network must not surpass its designated current capacity, articulated as:

(42)

Purchase power capacity constraints:

Restricted by the capacity of the distribution transformer, the electricity acquired from the main power grid during each time interval must meet the ensuing constraints:

(43)

4.2.3 Uncertain variables

As aforementioned, the decision-making problem towards synergetic DC-DN planning is subject to multiple uncertainties which mainly arise from RES outputs, bulk market prices, carbon footprint factor, electricity consumption requirements, data workloads. In this study, We employ an interval-based strategy to encompass the effects of the aforementioned uncertainties in our planning model:

(44)

(45)

(46)

(47)

(48)

5. Solution methodology

The overall expression of the proposed collaborative planning is outlined as follows:

(49)

(50)

(51)

(52)

In this study, X represents an n-dimensional vector comprising the planning variables; Y denotes an m-dimensional vector that encompasses the operational variables; U is a q-dimensional vector containing uncertain variables. Equation (50) is linked to Equations (29) and (35), aiming at the concurrent minimization of economic costs and carbon emissions; Equation (51) is related to Equations (36)-(38) and serves to frame planning restrictions; Equation (52) aligns with Equations (1)-(27), (39)-(49) for the formulation of operational limitations. Equation (53) is associated with Equation (28) for the interval-based representation of uncertain variables.

Given the interval presentation in the model, which complicates the solution process, an IMOEA/D method within the MOEA/D framework is suggested for optimizing the coordinated planning of DCs and DN. It is able to provide the operator with a series of solutions with interval objectives, which could be a reference to make decisions based on their preferences regarding economic factors, environmental concerns, and uncertainties regarding risk.

In this section, we initially provide an overview of the MOEA/D framework. Following that, we delve into the detailed development of the proposed IMOEA/D approach, which is designed to effectively solve the IMOP model.

5.1 Evolutionary algorithm for multi-objective optimization using decomposition

MOEA/D is a framework that introduces the decomposition method commonly employed in traditional mathematical programming into evolutionary algorithms to solve MOPs. It generates an approximate Pareto frontier by explicitly breaking down a MOP into multiple scalar optimization subproblems. These subproblems are optimized concurrently and cooperatively by evolving the solution population through an evolutionary algorithm.

There are several methods for decomposing a MOP into several scalar subproblems in the MOEA/D, including methods such as the weighted sum approach, the Tchebycheff approach, and the boundary intersection approach. Since the Tchebycheff approach could find the pareto optimal solution in both convex and nonconvex pareto frontier, it is mainly discussed in this paper, formulated as follows:

(53)

(54)

(55)

where m represents the number of objective functions, z={z1,z2,…,zm} denotes the reference point, and {r1,r2,…,rm} is the weight vector. Ξ symbolizes the feasible RESion. For each index i=1,…,m, both the reference point and weight vector must comply with constraints (55)-(56). The MOEA/D approach aims to minimize all N subproblems in a single simultaneous run. Each subproblem is primarily optimized by leveraging information from its neighboring subproblems. Consequently, MOEA/D demonstrates reduced computational complexity in contrast to alternative multi-objective optimization algorithms.

5.2 Interval multi-objective optimization evolutionary algorithm based on decomposition (IMOEA/D)

While the MOEA/D framework exhibits enhanced performance in solving MOPs, it cannot directly address the proposed planning problem with interval formulation, as the objective functions in this case are represented by intervals. To address these challenges, the IMOEA/D approach is developed for the effective resolution of the collaborative planning of DC and DN, which is capable of retaining all uncertain information. The essence of the IMOEA/D approach comprises two key components:  interval-valued chebyshev scalar function, and ensemble strategy for assessing interval objectives. The specifics of each component are elaborated upon in the subsequent sections.

5.2.1 Interval-valued chebyshev scalar function

Given that the objective function of the proposed multi-objective optimization problem with interval parameters (IMOP) is interval-based, the Pareto front is characterized by geometric shapes such as rectangles or hypercubes, as shown in Fig 4. To adequately represent uncertainties, we define a reference hypercube, which is constructed from two vertices: each vertex represents the minimum values of the higher and lower bounds and lower boundaries of the interval Pareto front for each objective. These vertices are referred to as the higher and lower bounds points, respectively. As depicted in Fig 4, Z={Z1,Z2,Z1,Z2} constitutes the reference hypercube, where Z=Z1,Z2 and Z=Z1,Z2 are the higher and lower bounds points, respectively.

Drawing inspiration from the MOEA/D framework, we propose the Interval-valued chebyshev scalar function (as indicated in Equation 53), formulated in accordance with the reference hypercube. This approach transforms the IMOP problem into several scalar IO subproblems by modifying the weight vector ri(x). During each iteration of the algorithm, there is a simultaneous effort to minimize all these IO subproblems, which in turn updates the population.

(56)

where gte(x|rx,Z) is Tchebycheff aggregate functions with interval parameters; Z is the reference hypercube; Zi is the i-th component in the reference cuboid.

Fig 5-1. The interval Pareto front with reference hypercube.

5.2.2 Ensemble strategy for assessing interval objectives

As the Interval-valued chebyshev scalar function plays a critical role in updating the population, it becomes necessary to compare two individuals with interval objectives to ascertain the superior one. Interval partial ordering relationships (IPORs) have been suggested in previous studies to facilitate the comparison of interval numbers, and these are summarized in Appendix A2. Given the variability in results when using different IPORs in the algorithm, the choice of these relations significantly influences the outcomes of the collaborative planning. To address this, we have designed an ensemble strategy that leverages the seven types of IPORs, which aims to provide a comprehensive comparison of interval-valued objectives, thereby mitigating the subjective decision in the selection of interval order relations. The specifics of this strategy are outlined below.

Assume that it should compare two candidate individuals xi and xj, and their Tchebycheff aggregate function value with interval parameters are AI and BI. When any an interval order relation is employed to compare intervals AI and BI, the comparison results may be the following three cases:

(1) AI is smaller than BI, represented as AI<INBI;

(2) BI is smaller than AI, represented as BI<INAI;

(3) AI and BI cannot be compared, denoted as AI||INBI

In our approach, we utilize seven distinct types of IPORs outlined in Appendix A2 to compare intervals AI and BI, thereby deriving their comparative results. We then assess these comparisons based on frequency, which refers to the number of times each specific comparison outcome occurs across the seven IPORs. The outcome that appears most frequently is ultimately chosen as the definitive comparison result between the two interval numbers. This method ensures a comprehensive assessment to reduce the influence of any single interval order relation on the final decision.

5.2.3 Implementation of IMOEA/D into the collaborative planning problem

The procedure of the proposed IMOEA/D to determine the location and capacity of RES, renovation of distribution lines, the site and size of DCs and the optimal operating strategy are illustrated in Fig 5-2. Detailed steps of the algorithm are described as follows:

Step 1: Input initial parameters of IMOEA/D such as the maximum iteration count for Gen, the neighborhood size T, a uniformly distributed weight vectors {r1,r2,…,rm}, the population size N

Step 2: Input the basic parameters of DCs, RES, DN.

Step 3: Calculate the Euclidean distances between two weight vectors, work out the T closest weight vectors ri to each weight vector and generate the neighborhood matrix BN×T

Step 4: Generate an initial population of RES, DN, DCs investment solutions randomly

Step 5: Initialize reference hypercube Z.

Step 6 For each individual, randomly select three individuals x1,x2,x3 from their neighborhood, and then generate a new solution y

Step 7: Apply Gaussian mutation on y to produce the new solution y'

Step 8: Update the reference hypercube.

Step 9: Update neighboring solutions according to interval parameters ensemble comparative scheme

Step 10: Output the optimal population if it reaches the maximal number of iterations.

Fig 5-2. Flowchart of the proposed IMOEA/D.

6. Case study

6.1 Teat system and parameters

To affirm the effectiveness of the proposed method delineated in this paper, a case study is carried out utilizing the modified IEEE-33 node distribution network as an illustration.As depicted in Figure 6-1, this system comprises 32 load nodes and 1 substation node, with a system rated voltage of 12.66 kV. The system is divided into four regions, with each region intended to host a DC for meeting the data demands of users within that region, with assumed installation locations being {13,18},{19,22},{23,25},{26,33}.RESs considered in this paper are wind turbine generators, with assumed installation locations in the system being {8,9,13,18,19,22,23,25,26,30,33}.

In our DC simulation, we set the indoor temperature limits to 5-25°C for optimal hardware performance and energy efficiency. For workload management, workloads are assigned a maximum delay of 500ms []. The DW to RW ratio is configured at 4:1, prioritizing DW to maximize energy cost savings while meeting real-time data demands. The system workloads are determined based on [], with an uncertain range of ±10%, as depicted in Fig. 6-3.

The technical and financial characteristics of different apparatus within the system are outlined in Table 3. The enlargement expenditure of the substation is fixed at 1.5 million RMB/MVA,while parameters related to DC can be found in Table 4. The values of expansion of the power grid lines are specified in Table 5.

Fig.6-1   Topological structure of the concerned system

In the test scenario, we assume that the maximum allowable number of units for each potential busbar of RES is 10. We determine the projected load factor for RES units based on actual wind speed data recorded in Beijing. Besides, considering the predictive accuracy of existing wind power forecasting technology over a one-year period, we set the fluctuation range to ±20%, as illustrated in Fig. 6-2(a).

The projected electrical load is derived from a real DN in Beijing. The uncertain boundaries for electrical load are set at ±20%, based on the annual fluctuations in load demand, as depicted in Fig. 6-2(b). Additionally, the electricity price is presented in Fig. 6-2(c), with the predicted prices derived from [1]. The predicted carbon footprint factor is set to 0.9, with their uncertainty limit set to ±10%[].

Fig. 6-2 Interval forecasts of ambiguous parameters: (a) RES load factor (b) Electrical load (c) electricity price

Figure. 6-3 Data Load Prediction Mean Curve

Tab.1 Parameters of system equipment

| Equipment | Parameters |  |
| --- | --- | --- |
|  | Technical | Economic |
| Wind Turbine | =100kW | =7000¥/kW |
|  | =10 |  |
|  | =6% |  |
|  | =20 |  |
| Server | =600W | =15200¥/unit |
|  | =300W |  |
|  | =500 |  |
|  | =1500 |  |
|  | =8% |  |
|  | =20 |  |
| Air Conditioner | =50kW | =835¥/kW |
|  | =4 |  |
|  | =10 |  |
|  | =8% |  |
|  | =20 |  |

Tab.2 Setting of DC parameters

| Parameters | Values | Parameters | Values |
| --- | --- | --- | --- |
|  | 1.2 kg/m³ |  | 1.09 W/(m2·k) |
|  | 1000 J/(kg·℃) |  | 1000m2 |
|  | 0.9 |  | 0.1¥/Gbps |

Tab.3 Parameters of feeders

| Distribution line model | Resistivity (Ω/km) | Reactance (Ω/km) | Current Rating (A) | Construction Cost (10⁴ ¥ /km) |
| --- | --- | --- | --- | --- |
| 1 | 0.076 | 0.069 | 410 | 60 |
| 2 | 0.046 | 0.066 | 713 | 80 |
| 3 | 0.035 | 0.06 | 1025 | 100 |

6.2 Experiments Results

To elucidate the benefits brought about by the implementation of collaborative planning for DC and DN, this section conducts a comparative analysis under three scenarios.

Case-I: Independent planning for DC and DN. In this scenario, it is assumed that DC locations are fixed, and DCs do not interact with DN. DC's equipment capacity is planned solely, followed by determining the expansion plan for the distribution grid based on the DC load demand.

Case-II: Collaborative planning for DC and DN, but DCs do not participate in demand response. It is assumed that all DCs do not control workloads scheduling and utilize thermal inertia.

Case-III: The proposed planning model.

Utilizing the suggested methodology, the interval-oriented Pareto fronts from each scenario are depicted in Figure 6-4. In contrast to traditional Pareto fronts, which is consisted of points, the interval-oriented Pareto front is characterized by a range of rectangles. These rectangles are plotted along two axes: the horizontal axis denotes the anticipated economic expenses, and the vertical axis signifies the system's carbon emissions. In addition, the length of each rectangles illustrates the range of variation in the system's economic costs, whereas the width indicates the degree of variability in carbon emissions. In practice, the optimal planning solution can be selected by decision-makers using the interval Pareto front depicted in Figure 6-4, based on their preference for system benefits and tolerance for uncertainty and risk.

Figure 6-4 Generated interval Pareto fronts across various scenarios.

It can be seen a conflict is between the overall expenses and carbon footprints, where indicates minimizing carbon emissions requires enhancing the RES planning capacity and utilizing demand response in DCs and vice versa. In this research, we adopt an interval TOPSIS method to ascertain the ultimate compromise solution from the interval Pareto front. For simplicity, we assume equal weightage for both economic and environmental objectives. The benefits from different planning frameworks are displayed Table 6, and the planning solutions is provided in Table 7.

Tab.4 System benefits under different scenarios

|  | Investment cost (10⁴¥) |  |  |  |  | System operation (10⁴¥) | Total cost (10⁴¥) | Carbon emissions (10⁴t) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case | Distribution line | Transformer | Server | Air Conditioner | Wind Turbine |  |  |  |
| I | 84.28 | 31.94 | 506.49 | 10.78 | 241.60 | (1631.98, 2446.46) | (2507.07,3321.55) | (3.45,4.05) |
| II | 63.64 | 32.00 | 506.49 | 10.78 | 234.78 | (1451.12, 2176.68) | (2298.81,3024.37) | (3.00,3.60) |
| III | 37.84 | 9.62 | 270.99 | 6.46 | 216.72 | (1308.82, 1963.22) | (1855.72,2510.13) | (2.40，2.85) |

Tab.5 Optimal planning outcomes across diverse scenarios

|  | Distribution line (From-To Nodes) |  |  | WT/ Unit (Node) | Server/Unit （Node） | ACE/ Unit （Node） |
| --- | --- | --- | --- | --- | --- | --- |
|  | Model 1 | Model 2 | Model 3 |  |  |  |
| I | 6-7/7-8/8-9/3-23/23-24/ 6-26/26-27/27-28/28-29 | 3-4/4-5/5-6 | 1-2/2-3 | 6(8),10 (13),10 (19), 10(25),10 (26),10(30) | 956(18),967(22), 964(25),967(33) | 3(18),4(22), 4(25),4(33)、 |
| II | 6-7/7-8/8-9/ 3-23/6-26 | 3-4/4-5/5-6 | 1-2/2-3 | 7(9),10 (13), 9(19), 10 (23),10 (26) | 956(13),967(19), 964(23),967(26) | 3(13),4(19), 4(23),4(26) |
| III | 4-5/5-6/6-7 | 2-3/3-4 | 1-2 | 9(18), 8(22), 10 (25), 9 (33) | 470(13),460(19), 460(25),680(26) | 2(13),2(19), 2(25),3(26) |

The initial comparative analysis between Cases-I and II affirms the enhanced effectiveness of the proposed collaborative planning approach for DN. Case-I, with its higher RES capacity of 4000 kW, leads to increased economic costs and carbon emissions when compared to Case-II’s 3900 kW capacity, confirming our forecast that suboptimally positioned DC stations heighten network congestion and energy inefficiency. In contrast, Case-II's integrated planning promotes a synergistic RES and DC deployment, carefully select the best method to combine DC, distribution lines, and RES by evaluating their joint impact on DN operations. Such a coordinated approach not only streamlines the integration of RES but also enhances the overall energy consumption efficiency, leading to a cooperative benefit that includes efficient RES utilization and a tangible reduction in carbon emissions.

Furthermore, analyzing Case-II and III highlights the impact of exploiting the demand response of DCs. Table 6 reveals that Case-III offers significantly higher economic and emission advantages compared to Case-II. Additionally, the reduced RES capacity in Case-III, as compared to Case-II, signifies a stronger interaction between the DC and the DN. This is achieved by optimizing the distribution of data loads over time and space, minimizing power consumption in DCs, and enhancing load curves, which collectively lead to a decrease in the required capacity configuration for servers, ACE, and related equipment. Besides, by fully harnessing the demand response potential of DC resources, the system can maximize the utilization of RES and lessen reliance on external market electricity (i.e., electricity with high carbon emission characteristics). However, Case-II fails to efficiently manage the temporal and spatial demand of the DC workloads, resulting in heightened power consumption, particularly during peak-load periods and times of low renewable energy output. To meet the demand during these periods, the operator in Case-II is forced to invest in a larger renewable energy capacity and purchase additional electricity from external markets.

6.3

6.3.1 Performance of DC-flexibility

In this study, considering the characteristics of spatiotemporal scheduling of workloads and thermodynamic characteristics, the operational flexibility of DCs was taken into account. To elucidate how the impact of flexibility, a comparative analysis was conducted by defining four cases:

Case-a: For the purpose of flexibility, only DW is considered and it is presumed that the internal temperature of the DC is kept at 20°C.

Case-b: For the purpose of flexibility, both DW and RW are considered for participation in demand response. In this case, thermal inertia is not taken into account, and it is assumed that the internal temperature of the DC is maintained at 20 °C.

Case-c: Both DW and RW are not considered for participation in demand response. The acceptable temperature range is defined with a lower limit of 5 °C and an upper limit of 25 °C.

Case-d: Both DW and RW are considered for participation in demand response, with thermal inertia taken into account. The acceptable temperature range is defined with a lower limit of 5 °C and an upper limit of 25 °C.

To delve deeper into how DC flexibility enhances system benefits, Figures 6-5 and 6-6 illustrate the actual wind power output and server scheduling scenarios from Case a to b.

Figure 6-5 shows that when the DC does not participate in DR, as presented in Case-II, there is a poor match between wind power output and DC power consumption, leading to wind curtailment in certain periods and thus poorer system low-carbon benefits. However, when considering DC flexibility, (i.e., Cases a and b), the utilization rate of RES experiences a notable increase. Moreover, comparison across the three scenarios demonstrates that system benefits are highest when the data center engages in both temporal and spatial DR, as seen in Case b., compared to temporal DR alone (i.e., Case a).

Figure 6-5 Wind power forecasted output and actual output

Figure 6-6 Number of powered-on servers and DC power (a) Case-II (b) Case-a (c) Case-b

The comparison between Figure 6-6 (a) and (b) shows that in Case-a, the existing load during 08:00-17:00 is deferred to 18:00-24:00, coinciding with high wind power generation periods, thus reducing the wind curtailment rate from 11.66% to 9.1%. However, since DW only changes the timing of energy use, wind curtailment still occurs during 01:00-05:00. In contrast, Case-b, by scheduling workloads fully utilizes the DC’s flexible operational attributes in both temporal and spatial aspects. The comparison between Figure 6-6(b) and (c) indicates that to enhance the consumption efficiency of renewable energy during 00:00-05:00, under Case-b, the quantity of servers operating in DC1 and DC2 rises. This indicates a shift of more workloads to regions 1 and 2 for processing, thereby enhancing both economic and low-carbon advantages.

Figure 6-7 DC temperature and ACE power (a) Case-b (b) Case-c (d) Case-d

Figure 6-7 depicts the influence of thermal inertia on indoor temperature fluctuations. Without considering thermal inertia, the indoor temperature remains constant at the desired 20°C without any variations. However, when thermal inertia is taken into account, the temperature in Case-d can fluctuate within an acceptable range, allowing for flexible operation of the CRAC system to reduce energy consumption for cooling. Hence, the thermal inertia of data centers can be leveraged to alleviate stringent constraints on indoor temperature by establishing an acceptable temperature range, thus rendering the cooling process more adaptable and providing flexibility support for DN.

6.3.2 Effectiveness of the interval optimization

This study employs an interval-based approach to deal with the uncertainties associated with system planning. In order to verify the effectiveness of the proposed approach, we conduct a comparative study to compare the proposed IO method with respect to three other state-of-the-art uncertain modeling, namely, DO, RO, and SO. Specifically, for the DO case, all uncertain parameters are assumed to be fixed at their midpoint values. While in the RO, the polyhedral uncertainty set is used to describe the uncertain variables. For the SO method, The Monte Carlo simulation method is employed to generate random sample sequences following specific distributions. It's worth noting that, in the absence of prior information, we utilized several widely used distribution functions, including uniform distribution, normal distribution, and truncated Gaussian distribution.

The calculated excepted performance of these methods is presented in the first five columns of Table 6. As observed, The performance of the system planning, based on the SO model, heavily relies on the distribution information of the provided data. Moreover, the benefits vary significantly under different distribution conditions. Consequently, the outcomes derived from the SO model are not universally applicable, posing significant challenges to collaborative planning between DC and DN in unfamiliar data regions. In contrast,, the IO model represents uncertain variables as interval numbers, where each interval number is denoted by a lower and a higher bound (i.e., the variation of the corresponding datasets), which is easy to obtain in the planning setting. Therefore, compared to the SO approach, the planning based on the IO in this paper has better applicability. Besides, as can be seen, the planning based on the DO realizes better excepted benefits than that on IO; while the planning performance through RO obtains the worst expected benefits.

To demonstrate the advantage of the IO approach under uncertain risk, the performances are tested. Specifically, to simulate the impact of uncertain factors, it is assumed that uncertain variables follow the uniform distribution in the interval of their predicted values, and then 1000 samples are randomly generated using the Monte Carlo simulation method. Finally, the optimal operation problems are solved with the samples to determine their average operation costs. The results are shown in the last three columns of Table 8.

As seen, although the solution on DO is expected to have a more satisfactory performance, its actual operation costs and emissions are much large. This indicates that the actual performance derived from the DO is less favorable than the proposed solution. Furthermore, although the mean actual operational expenditure of the planning by RO is lower, its investment cost is much higher, which makes the total cost to be higher.

The above results demonstrate that, since RO focuses on the worst scenario only, thus the results are relatively conservative. As for the proposed IO method, it maintains all uncertain information in the solution process and optimizes the interval of objective functions instead of the worst case. Therefore, the result is more economical than this obtained by the RO and has better robustness and reliability than this under the DO.

Table.6 Comparison based on different uncertain optimization approaches.

| Approaches |  | Investment cost (10⁴¥) | Expected operating cost (10⁴¥) | Expected total cost (10⁴¥) | Expected carbon emissions (10⁴t) | Average actual operating cost (10⁴¥) | Average actual economic cost (10⁴t) | Average carbon emissions (10⁴t) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SO | Normal distribution | 619.68 | 1913.81 | 2533.49 | 2.79 | - | - | - |
|  | Uniform distribution | 488.69 | 1616.81 | 2105.5 | 2.69 | - | - | - |
|  | Gaussian truncated distribution | 560.06 | 1758.91 | 2318.97 | 2.75 | - | - | - |
| DO |  | 461.26 | 1481.12 | 1942.38 | 2.46 | 528.44 | 2475.2 | 2.87 |
| RO |  | 651.03 | 2113.51 | 2764.54 | 3.12 | 315.44 | 2417.1 | 2.58 |
| IO |  | 546.91 | (1308.82, 1963.22) | (1855.72,2510.13) | (2.40,2.85) | 366.52 | 2284.1 | 2.61 |

6.3.3 Performance on proposed algorithm

Compared to existing work, this paper introduces a novel solution algorithm, IMOEA/D, to address the proposed IMOP problem. To demonstrate the positive aspects of the proposed IMOEA/D, we compared the results with two deterministic conversion based IO solutions previously employed in earlier work.

S1: Optimizing only the midpoint of the interval-valued objective function, which is consistent with reference [].

S2: Utilizing a weighted-based approach for the collaborative optimization of both the midpoint and deviation of the interval-valued objective function, which is consistent with reference [].

S3: The proposed IMOEA/D.

The relevant statistical results are shown in Table 10 and Figure 6-8. It is observed that Scenario S3 incurs the lowest average cost, followed by S2, while S1 has the highest average expenditure. This underscores the significant impact of how interval uncertainty is managed in the IMOP model on the ultimate decision-making process. Despite S1 seeming to be the most cost-effective on average, due to the potential underutilization of uncertain information, such a choice might not be truly effective or optimal in practice. Hence, the performance of these methods is compared through sampling analysis outlined in Section 4.3.3. A total of 400 scenarios were randomly generated to represent realizations of uncertain variables. Subsequently, the performance of these solutions in the presence of uncertainty was evaluated by applying the planning solutions to the generated operational scenarios and computing the system's metrics. The relevant statistical outcomes are presented in Table 10 and Figure 6-8.

Table.7 Comparison of system performances with different solution approaches.

|  | Investment cost (104 ¥) | Expected performances |  |  | Actual performances |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Operation cost (104 ¥) | Total cost (104 ¥) | Carbon emissions (104 t) | Average value |  |  | Standard deviation |  |
|  |  |  |  |  | Operation cost (104 ¥) | Total cost (104 ¥) | Carbon emissions (104 t) | Total cost (104 ¥) | Carbon emissions (t) |
| S1 | 495.29 | 909.20 | 1857.84 | 2.40 | 1894.88 | 2615.2 | 2.4 | 533.4 | 950.1 |
| S2 | 523.01 | 1061.20 | 2038.33 | 2.55 | 1671.95 | 2511.6 | 2.1 | 324.1 | 1948.7 |
| S3 | 546.91 | (1308.82, 1963.22) | (1855.72,2510.13) | (2.40,2.85) | 1367.96 | 2246.69 | 2.70 | 136.5 | 4342 |

As shown in Figure 6-8 and the last five columns of Table 10, it can be observed that although the deterministic conversion based planning results with better excepted economic and environmental performance, the actual benefits would be poor when uncertainty is involved. Moreover, the actual benefits of scenarios S1 and S2 differ more substantially, indicating that the deterministic conversion based planning solutions perform poorly in uncertainties.

Figure 6-8 Comparison of system operation performances: (a) Economic cost, (b) Carbon emissions.

The findings suggest that deterministic conversion-based planning, due to its failure to fully account for the influence of uncertain factors, may not ensure optimality when certain parameters deviate from predicted values. Conversely, planning with IMOEA/D effectively retains the features of uncertainty, leading to lower actual performance and standard deviation compared to deterministic conversion-based methods. This underscores its superior risk-averse capabilities and robustness against uncertainty.

7. Conclusion

This paper introduces an IMOP-based framework for the collaborative planning of DCs and DN within the context of a low-carbon DN. The proposed model considers the spatiotemporal effects of workloads and thermal-related characteristics of DCs, effectively capturing their potential flexibility. It explicitly incorporates the impact of system uncertainties through an interval optimization approach. To address the proposed problem effectively, an evolutionary algorithm named IMOEA/D is developed. A case study on the modified IEEE 33-bus system was conducted, resulting in the following conclusions:

a): The collaborative planning of DCs and DN can bring synergistic benefits to DNs. Compared to independent planning, this approach can enhance the utilization of RES and encourage the participation of DCs in demand response. This integrated strategy not only optimizes resource allocation but also promotes a more efficient and responsive distribution system, effectively utilizing the potential of DC in reducing carbon emissions.

b): The coordinated energy management of DC and DN can significantly improve the efficiency of renewable energy utilization by flexibly utilizing the spatiotemporal characteristics of workloads. By aligning the energy consumption of DC with the period of high renewable energy generation, the wind curtailment rate can be reduced. In addition, incorporating thermal inertia into the management of DC allows for more flexible cooling strategies, which helps save energy and provides additional support for the efficiency of DN.

c): Compared with DO, RO, and SO, IO methods are more effective for system planning under uncertain conditions. The IO method considers uncertain variables as interval numbers with easily obtainable upper and lower limits, which has better applicability and strikes a balance between cost and reliability.

d): The proposed IMOEA/D maintains robustness against uncertainty, effectively preserving the characteristics of uncertain parameters, and therefore delivers more reliable and optimal planning solutions with lower actual costs and standard deviations.

Reference

- Koot M, Wijnhoven F. Usage impact on data center electricity needs: A system dynamic forecasting model[J]. Applied Energy, 2021, 291: 116798.

- Ting Y, Han J, Yucheng H. Study on carbon neutrality regulation method of interconnected multi-datacenter based on spatio-temporal dual-dimensional computing load migration[J]. Proceedings of the CSEE, 2022, 42(1): 164-177.

- Huang P, Copertaro B, Zhang X, et al. A review of data centers as prosumers in district energy systems: Renewable energy integration and waste heat reuse for district heating[J]. Applied energy, 2020, 258: 114109.

- Kwon S, Ntaimo L, Gautam N. Demand response in data centers: Integration of server provisioning and power procurement[J]. IEEE transactions on Smart Grid, 2018, 10(5): 4928-4938.

- Sang Bingyu, Wang Deshun, Yang Bo, et al. Collaborative optimization configuration of photovoltaic-energy storage based on economy in an Internet data center[J]. Power System Protection and Control, 2020, 48(17): 131-138.

- Niu T, Hu B, Xie K, et al. Spacial coordination between data centers and power system considering uncertainties of both source and load sides[J]. International Journal of Electrical Power & Energy Systems, 2021, 124: 106358.

- Wen Z, Liu J. A optimal scheduling method for hybrid wind-solar-hydro power generation system with data center in demand side[J]. Power Syst. Technol, 2019, 43(7): 2449-2459.

- Qi W, Li J, Liu Y, et al. Planning of distributed internet data center microgrids[J]. IEEE Transactions on Smart Grid, 2017, 10(1): 762-771.

- Guo C, Luo F, Cai Z, et al. Integrated planning of internet data centers and battery energy storage systems in smart grids[J]. Applied Energy, 2021, 281: 116093.

- Xu Y P, Liu R H, Tang L Y, et al. Risk-averse multi-objective optimization of multi-energy microgrids integrated with power-to-hydrogen technology, electric vehicles and data center under a hybrid robust-stochastic technique[J]. Sustainable Cities and Society, 2022, 79: 103699.

- Vafamehr A, Khodayar M E, Manshadi S D, et al. A framework for expansion planning of data centers in electricity and data networks under uncertainty[J]. IEEE Transactions on Smart Grid, 2017, 10(1): 305-316.

- Bahrami S, Wong V W S, Huang J. Data center demand response in deregulated electricity markets[J]. IEEE Transactions on Smart Grid, 2018, 10(3): 2820-2832.

- Chen M, Gao C, Shahidehpour M, et al. Internet data center load modeling for demand response considering the coupling of multiple regulation methods[J]. IEEE Transactions on Smart Grid, 2020, 12(3): 2060-2076.

- Wan T, Tao Y, Qiu J, et al. Internet data centers participating in electricity network transition considering carbon-oriented demand response[J]. Applied Energy, 2023, 329: 120305.

- Cao F, Wang Y, Zhu F, et al. UPS node-based workload management for data centers considering flexible service requirements[J]. IEEE Transactions on Industry Applications, 2019, 55(6): 5533-5542.

- Wang Z, Wu F, Li Y, et al. Day-ahead dispatch approach for cascaded hydropower-photovoltaic complementary system based on two-stage robust optimization[J]. Energy, 2023, 265: 126145.

- Chen B, Che Y, Zheng Z, et al. Multi-objective robust optimal bidding strategy for a data center operator based on bi-level optimization[J]. Energy, 2023, 269: 126761.

- Shi R, Li S, Zhang P, et al. Integration of renewable energy sources and electric vehicles in V2G network with adjustable robust optimization[J]. Renewable Energy, 2020, 153: 1067-1080.

- Lian Y, Li Y, Zhao Y, et al. Robust multi-objective optimization for islanded data center microgrid operations[J]. Applied Energy, 2023, 330: 120344.

- Li X, Wang W, Wang H. A novel bi-level robust game model to optimize a regionally integrated energy system with large-scale centralized renewable-energy sources in Western China[J]. Energy, 2021, 228: 120513.

- Liu W, Wang T, Wang S, et al. Day-ahead robust optimal dispatching method for urban power grids containing high proportion of renewable energy[J]. Process Safety and Environmental Protection, 2023, 178: 715-727.

- Yuan H, Feng K, Li W, et al. Multi-objective optimization of virtual energy hub plant integrated with data center and plug-in electric vehicles under a mixed robust-stochastic model[J]. Journal of Cleaner Production, 2022, 363: 132365.

- Zeng L, Xu J, Wang Y, et al. Day-ahead interval optimization of combined cooling and power microgrid based on interval measurement[J]. Energy and Buildings, 2022, 273: 112383.

- Zeng L, Xu J, Wu M, et al. Day-ahead interval optimization for CCHP system considering uncertainty of wind power and PV[J]. International Journal of Electrical Power & Energy Systems, 2022, 138: 107895.

- Jiang T, Li X, Kou X, et al. Available transfer capability evaluation in electricity-dominated integrated hybrid energy systems with uncertain wind power: An interval optimization solution[J]. Applied Energy, 2022, 314: 119001.

- Dong Y, Zhang H, Ma P, et al. A hybrid robust-interval optimization approach for integrated energy systems planning under uncertainties[J]. Energy, 2023, 274: 127267.

- Wang S, Yuan S. Interval optimization for integrated electrical and natural-gas systems with power to gas considering uncertainties[J]. International Journal of Electrical Power & Energy Systems, 2020, 119: 105906.

- Yang X, Wang X, Deng Y, et al. Integrated energy system scheduling model based on non-complete interval multi-objective fuzzy optimization[J]. Renewable Energy, 2023, 218: 119289.

- Rahimi M, Ardakani F J, Olatujoye O, et al. Two-stage interval scheduling of virtual power plant in day-ahead and real-time markets considering compressed air energy storage wind turbine[J]. Journal of Energy Storage, 2022, 45: 103599.

- Li Y, Wang P, Gooi H B, et al. Multi-objective optimal dispatch of microgrid under uncertainties via interval optimization[J]. IEEE Transactions on Smart Grid, 2017, 10(2): 2046-2058

Author Contributions: The authors confirm contribution to the paper as follows: study conception and design: Yuying Zhang, Chen Liang; data collection: Han Wang, Jiayi Zhang; analysis and interpretation of results: Yuying Zhang, Bo Zeng, Wenxia Liu; draft manuscript preparation: Yuying Zhang. All authors reviewed the results and approved the final version of the manuscript.

Funding Statement: Funding for this research has been provided by t the National Natural Science Foundation of China (52177082) and Beijing Nova Program (20220484007).