Submission Template for IET Research Journal Papers

Physics-Informed Fault Classification and Anomaly Detection in Wind Energy Systems using Deep CNN and Adaptive Elite PSO-XGBoost

Chun-Yao Lee 1*, Edu Daryl C. Maceren 2

1 Department of Electrical Engineering, National Taiwan University of Science and Technology, Taipei City,   Taiwan

2 Department of Electrical Engineering, Chung Yuan Christian University, Taoyuan City, Taiwan

*cylee@mail.ntust.edu.tw

Abstract: Wind energy systems require fault diagnosis that identifies faults despite data inconsistencies. This study addresses challenges in supervisory control and data acquisition (SCADA) systems for monitoring wind turbine conditions from imbalanced data representation and error vulnerability. It examines the efficacy of Adaptive Elite-Particle Swarm Optimization (AEPSO)-tuned Extreme Gradient Boosting (XGBoost) on an imbalanced SCADA dataset for wind turbine fault classification. The methodology integrates the resampled SCADA dataset with t-distributed Stochastic Neighbour Embedding represented deep learning features. Employing AEPSO-XGBoost classifier trained on merged SCADA and deep learning data from a physics-informed deep convolutional neural network forms the basis of the fault classification model. The AEPSO-XGBoost regressor is validated across three distinct rear bearing temperature datasets, facilitating parameter optimization and model robustness. Also, this study explores supervised and unsupervised anomaly detection models using PDCNN and AEPSO-XGBoost with rear-bearing temperature data. Findings exhibit substantial fault classification and prediction enhancements by merging resampled SCADA data with deep learning features. Moreover, results show that applying AEPSO-XGBoost can significantly improve anomaly detection metrics. Through AEPSO-XGBoost's efficacy in enhancing fault prediction within imbalanced SCADA datasets, the study proposes an integrated framework for fault classification and anomaly detection as an innovative predictive maintenance system for wind energy systems.

- Introduction

Recently, wind energy has emerged as a prominent choice of stakeholders for clean electricity generation, making substantial contributions to global efforts toward sustainable renewable energy systems [1]. In most wind electrical power generation facilities, supervisory control and data acquisition (SCADA) systems play a fundamental role in the operation and control. However, the SCADA data collected often exhibit significant imbalances due to unexpected irregularities during the operation and control [2]. Additionally, the inherent unpredictability of working environments for wind turbines can worsen the recorded SCADA data, which may lead to frequent faults and escalate maintenance costs [3]. Despite their importance, SCADA systems also encounter several challenges that can impede their effectiveness in detecting faults and anomalies accurately during maintenance operations. One such challenge is imbalanced data representation within SCADA datasets, where identified fault classes may be erroneous due to human factors and sensor failures, leading to biased model predictions. Thus, SCADA systems are susceptible to inaccuracies, which may compromise the reliability of fault classification and detection for fault diagnosis.

Some recent advancements in machine learning have introduced new ideas for enhancing the predictive maintenance of wind energy systems. One of these is the integration of a physics-informed approach model, which holds promising potential for revolutionizing fault detection and classification processes [4-6]. However, in our research, we introduce an innovative method for fault diagnosis that utilizes a physics-informed Deep Convolutional Neural Network (PDCNN) in conjunction with an Adaptive Elite-Particle Swarm Optimization (AEPSO)-tuned Extreme Gradient Boosting (XGBoost) classifier and regressor. This integrated approach addresses the inherent limitations of conventional SCADA systems by effectively using resampled SCADA datasets combined with deep learning features to improve fault classification and anomaly detection in wind energy systems.

Initially, in our study, we performed standard preprocessing; we resampled the SCADA data using the Synthetic Minority Oversampling technique (SMOTE) and Near-miss Undersampling to obtain a balanced dataset. Then, we use t-distributed stochastic neighbor embedding (t-SNE) to analyze our resampled data. The t-SNE is a non-linear dimensionality reduction technique that is particularly effective for visualizing high-dimensional data in a low-dimensional space. It works by converting similarities between data points into joint probabilities and then minimizing the Kullback-Leibler divergence between these probabilities in both the high-dimensional and low-dimensional spaces [7]. This process allows t-SNE to preserve the local structure of the data, making it effective at capturing complex and non-linear relationships that linear methods like PCA might not detect. Also, in our research, we employ t-SNE as a tool for feature representation to improve the classification accuracy of neural network-based classifiers [8]. Our integration of t-SNE is intended to represent deep learning features extracted from PDCNN, providing a beneficial method for examining and reducing the dimensionality of non-linear data.

The concept of physics-informed deep convolutional neural networks (PDCNNs) for fault classification represents an advanced approach in the field, particularly in applications such as bearing fault detection [9-11]. These networks are designed to integrate physical principles into the learning and prediction stages, thereby improving the interpretability and accuracy of the models. A notable study in [9] introduced a physics-informed deep learning approach for bearing fault detection. Their method combines a threshold model, which categorizes the health of bearings based on established physical fault characteristics, with a deep convolutional neural network (DCNN). For prediction, DCNN is tasked with autonomously extracting deep learning features from the input data. Their approach was also validated using data collected from bearings under field and laboratory conditions, proving its potential effectiveness in real-world applications [9]. Another research proposed a physics-based convolutional neural network (PCNN) for fault diagnosis of rolling element bearings by Sadoughi and Hu [10]. Their approach modified a convolutional neural network to incorporate useful information from physical knowledge about bearings and their fault characteristics. The PCNN utilized spectral kurtosis and envelope analysis to extract sidebands from raw sensor signals and feed fault characteristics into the CNN model. The PCNN approach demonstrated the capability to monitor multiple bearings simultaneously and detect faults in these specified bearings.

In the context of motor fault diagnosis, several studies about machine learning techniques for optimization have emerged as practical tools for enhancing fault classification and detection in wind energy systems [12-14]. In our study, we combine AEPSO-XGBoost with PDCNN-extracted features to form the basis of our fault classification model, which is trained on merged SCADA data and deep learning representation. We also investigate utilizing both supervised and unsupervised anomaly detection models using PDCNN and AEPSO-XGBoost, focusing on rear-bearing temperature data—a critical indicator of turbine health. Optimization is crucial in various domains, from production to maintenance applications, where finding the optimal solution to complex problems is vital. In wind energy systems, optimization techniques help enhance wind turbines' performance, reliability, and efficiency. Among the different variants of PSO for optimizing wind energy systems, the Adaptive Elite Particle Swarm Optimization (AEPSO) stands out as a promising approach due to its adaptability, efficiency, and robustness [15-16]. In this research, we explore the utilization of AEPSO to optimize XGBoost parameters in the context of wind energy systems with resampled (synthetic) data.

The motivation for this study is based on the critical need to enhance the reliability and efficiency of wind energy systems through advanced predictive maintenance techniques. Faults in wind turbine components, such as the rear bearing, can lead to significant operational disruptions and financial losses. Due to some computational constraints, traditional fault detection methods often have difficulties with the imbalanced nature of SCADA datasets and the complex dynamics of wind turbine operations [17-19]. This research seeks to overcome these challenges by introducing a physics-informed methodology and adaptive gradient boosting algorithm, offering a more accurate and robust framework for fault classification and anomaly detection. Moreover, physics-informed DCNNs present a promising direction for fault classification, utilizing physis-based gathered data to enhance both the learning process and the accuracy of predictions. This approach can lead to more accurate and interpretable models, which is crucial for practical fault detection applications. By integrating the adaptability of AEPSO with the robustness of XGBoost, we seek to develop a fault classification and detection framework that not only addresses imbalanced data representation but also enhances the accuracy and efficiency of fault classification and anomaly detection. AEPSO, as an adaptive optimization algorithm, offers the capability to fine-tune the parameters of the XGBoost classifier, thereby optimizing its performance to the specific characteristics of wind energy system SCADA data.

In our study, we aim to explore and evaluate the effectiveness of AEPSO-tuned XGBoost in addressing the challenges associated with fault classification and detection in wind energy systems using the Synthetic Minority Oversampling Technique (SMOTE) and Near-miss Undersampling to resample the imbalanced SCADA data. This goal arises from the compelling need within the renewable energy generation sector to develop robust predictive maintenance strategies to identify and mitigate faults in wind turbines caused by certain irregularities during operation. Through comprehensive testing and validation across multiple datasets, we demonstrate the effectiveness of our proposed approach in enhancing fault prediction accuracy and anomaly detection performance. Our findings emphasize the importance of integrating domain knowledge into machine learning models and highlight the potential of adaptive optimization techniques in improving the robustness and reliability of fault classification and detection systems in wind energy applications. The contributions of this research are the following:

1.	The visualization of extracted deep learning features using t-SNE from resampled SCADA data derived from SMOTE and Near-miss Undersampling.

2. Implementing a fusion technique for a resampled wind energy system SCADA dataset and using PDCNN to extract deep learning features and improve fault classification accuracy.

3.	The utilization of optimized XGBoost classifier and regressor parameters with anomaly detection using adaptive elite-PSO to improve fault classification and anomaly detection performance metrics.

- Physics-informed Deep Convolutional Neural Network and AEPSO-XGBoost

2.1. Physics-informed Deep Convolutional Neural Network

Deep convolutional neural network (DCNN) is an effective machine learning tool that can be utilized as a supervised or unsupervised algorithm to process particular data and has been proven to provide robust performance in classifying faults in electrical machines; also, DCNN has emerged as one of the most effective tools in fault detection. The interconnection of multiple layers of convolutional filters and pooling operators allows it to learn and capture important patterns and extract valuable features of complex data [20]. The DCNN can learn by executing its forward propagation and backpropagation using an optimization algorithm to adjust its parameters and obtain the desired outcome. Its iterative process training includes updating the convolutional neural network weights multiple times or epochs based on the gradient of the loss function with respect to the weight. In this study, the output of a 1D convolutional neural network layer can be expressed as:

| Zl[i]=k=0N-1Xl-1i-k*Wl[k]+bl |  | (1) |
| --- | --- | --- |

where N is the size of the convolutional kernel, l denotes the hidden layer, Zl[i] represents the ith element of the output layer, Xl-1, Wl, and bl are input arrays, weights, and biases, respectively. The pooling layer in a deep 1D convolutional neural network is expressed as:

| Xl+1i=max Xli×s+j, 0 <j<p |  | (2) |
| --- | --- | --- |

where Xl represents the input activation sequence at layer l, Xl+1 is the output activation sequence after the max pooling. Also, j is the iteration index during the forward propagation within the pooling window; the p and s are pooling window size and stride, respectively. However, the implementing a pooling layer in convolution neural network can be an effective strategy to mitigate overfitting [21].

The DCNN is proven to extract adaptive features from sequential data such as audio signals; thus, it can learn patterns in complex data, leading to better performance [22]. Due to the characteristics of certain data, convergence may not always be a guarantee. Thus, an appropriate backpropagation can be employed. In this study, we used the Adam optimizer to execute stochastic gradient descent for 1D-DCNN containing the input layer, two convolutional layers with pooling layers, the softmax layer, and the output layer. The softmax layer can enable the DCNN to attain a normalized output based on the probability distribution range. We use rectified linear unit as the activation function to execute the learning process, and the softmax layer is implemented to determine the output values in the output layer of the 1D-DCNN.

In recent years, integrating physics-informed neural network knowledge into deep learning architectures has gained significant attention due to its potential to enhance model performance and interpretability [23]. One approach that has emerged recently is the Physics-Informed Deep Convolutional Neural Network (PDCNN). PDCNN represents a novel paradigm in deep learning feature extraction by incorporating physics-based constraints into convolutional neural networks (CNNs), thereby maintaining them with enhanced robustness, interpretability, and generalization capabilities [24].

The PDCNN lies the physics-based loss functions, designed to encode domain-specific knowledge or constraints into the learning process. In this study, we integrated a categegorical cross-entropy loss function and physics loss function in equation (3), where λ is a corresponding weight equal to 0.1. The categorical cross-entropy loss function combines a traditional mean squared-error term which is a physics-based loss function, thereby encouraging the model to learn features that adhere to the data distribution.

| Loss=-i=1Nyilogyi+λ×1Ni=1Nyi-yi2 | (3) |
| --- | --- |

The physics loss function uses the temporal structure of the data by computing the differences between consecutive predictions. By squaring these differences and taking their mean, the loss function penalizes deviations from expected temporal behaviors, effectively enforcing smoothness constraints on the learned features [25]. As a result, this integration of physics-based constraints not only enhances the model's ability to capture temporal dependencies but also facilitates better generalization to unseen data by promoting more coherent feature representations. By incorporating physics-based constraints directly into the loss function, PDCNN can capture underlying patterns and relationships in the data that may not be apparent from the data alone [26]. This approach improves the model's predictive performance and enhances its interpretability, as the learned features are aligned with known physical principles.

In this study, the Tensorflow Keras library is utilized to construct the deep PDCNN and extract deep learning features from SCADA dataset. The extraction process entails retraining the neural network using the Sequential Model within the Tensorflow Keras library. The resulting extracted features, represented as t-SNE components, will be combined with resampled data derived from the imbalanced wind turbine SCADA dataset, serving as input parameters for the subsequent utilization in the PSO-XGBoost algorithm.

Furthermore, PDCNN represents an improved approach to feature extraction in deep learning, offering a unique fusion of domain knowledge and data-driven learning. By integrating physics-based constraints into the learning process it enables the development of models that are not only highly accurate but also interpretable and robust. As the field of deep learning continues to evolve, PDCNN stands as a promising innovation, offering new avenues for advancing our understanding of complex datasets and phenomena.

2.2. Extreme Gradient Boosting

Extreme Gradient Boosting (XGBoost) is a highly regarded machine learning algorithm in the realm of supervised learning, initially introduced by Chen et al. [27]. Recognized for its outstanding performance in structured and tabular data tasks, XGBoost has earned a reputation as a cutting-edge approach, evidenced by its successes in various machine learning competitions. As an ensemble method, it aggregates predictions from numerous weak learners, typically decision trees, thus creating a more accurate and robust model. Its versatility and effectiveness are underscored by its widespread application in motor fault classification models [28], highlighting its practicality  in different electric motor applications.

The mathematical formulation of XGBoost can be derived from a dataset D=Xi,yi consisting of n examples and m features, where Xi∈ Rm and yi∈R. Here, Xi encompasses the characteristic parameters of the ith sample, encapsulating its unique features. Its components Fi,xi1, xi2,…, xis delineate fault types, with xi representing the operational parameters of the dataset entries. The predicted output from the ensemble model is defined as:

| yi=k=1Kfk(Xi), fk∈F, | (4) |
| --- | --- |

where K denotes the number of trees and fk represents a function in the functional space.

The primary objective is to facilitate the learning of function f by minimizing the objective function:

| O=i=1nl(yi,yi)+k=1KΩ(fk) | (5) |
| --- | --- |

The first term, which includesl(yi,yi), represents the training loss function, while the second term denotes the regularization term containingΩ(fk). The training loss function is assessed to gauge the predictive capability of XGBoost in identifying fault types, while the regularization term serves to mitigate overfitting. Utilizing Taylor's second-order approximation, equation (4) can be transformed into its iterative form as:

| O(t)=i=1n[l(yi,yi(t-1))+giftXi+ | (6) |
| --- | --- |
| 12hift2(Xi)]+ Ω(ft) |  |

In the objective function, the term ft(Xi) is incorporated to optimize the performance of XGBoost. Here, gi=∂t-1(yi-yit-1)l and hi=∂t-12(yi-yit-1)l represent the first- and second-order gradient statistics, respectively. Furthermore, the objective function can be expressed without the constants as:

| Ot=i=1n[giftXi+12hift2(Xi)] + Ω(ft) | (7) |
| --- | --- |

The tree prediction function is denoted as ftX is defined as:

| ftX=ωqX, ω∈RT, q: Rd →1,2…,T. | (8) |
| --- | --- |

where ω represents a vector score of individual leaves within the tree structure; q signifies a functional mapping that assigns each data point to its corresponding leaf, while T denotes the total count of leaves. The process of determining the tree's prediction entails retrieving the score ω associated with the specific leaf to which X belongs, utilizing the mapping function q. The second term within the objective function of XGBoost is the regularization function designed to mitigate the risk of overfitting, defined as:

| Ωft=γT+12λi=1Tωj2. | (9) |
| --- | --- |

The iterative form of the objective function can be expanded by combining equations (7) and (9):

| Ot=j=1T[(i∈Ijgi)ωj+12(i∈Ijhi+λ)ωj2] + γT | (10) |
| --- | --- |

where Ij={i|qXi=j} denotes the set of indices of all data points assigned to the jth leaf. The objective function can be further simplified by defining Gj=i∈Ijgi and Hj=i∈Ijhi:

| Ot=i=1n[Gjωj+12Hj+λωj2] + γT. | (11) |
| --- | --- |

The optimum value of   ωj can be determined by obtaining the gradient of the objective function from equation (11). Consequently, the reduced form of the objective function is expressed as:

| O(q) =-12j=1TGj2Hj+λ+γT. | (12) |
| --- | --- |

where γ and λ are parameters used to control the degree of regularization. The reduced form of the objective function serves as a scoring metric for qX and assists in evaluating the suitability of the tree structure for classification. However, determining all possible q tree structures is infeasible. Therefore, a strategy introduced in reference [29], known as a greedy algorithm, is employed. This algorithm begins with a single leaf and iteratively adds branches to the existing tree. The process involves splitting a leaf into two (left and right), resulting in a revised expression of the objective function as:

| Osplit=12GL2HL+λ+GR2HR+λ-GL+GR2HL+HR+λ+γ. | (13) |
| --- | --- |

In practical applications, equation (13) represents the formulation used to execute the XGBoost algorithm iteratively until the optimal score is achieved.

Furthermore, in this study, we aim to leverage XGBoost as both a classifier and regressor, leveraging its algorithmic enhancements enabled by regularization techniques to mitigate overfitting and effectively manage sparse input data. Additionally, its integration of a cross-validation tool further bolsters its advantages. Consequently, XGBoost is widely regarded as a superior machine learning algorithm, making it the preferred choice among both researchers and industry professionals [29]. Thus, the objective of this research is to establish the superior classification and regression metrics offered by XGBoost. Moreover, we intend to utilize the XGBoost algorithm following a comparative analysis of the testing accuracy of various methods for wind turbine fault classification and prediction with anomaly detection. Its adaptability, robustness, controllability, and flexibility position it favorably for application to diverse datasets.

2.3. Extreme Gradient Boosting Optimization using AEPSO

Particle Swarm Optimization (PSO) is a heuristic mathematical algorithm designed to find the optimal solution for a given objective function. It operates by maintaining a population of particles, where each particle represents a potential solution and undergoes iterative updates by traversing within a limited search space [15]. The position and velocity of each particle p are dynamically adjusted according to the following equation:

| vt+1=wt+vpt+C1×r1t×Pbestt-Xpt +C2×r2t(Gbestt-Xpt) | (14) |
| --- | --- |
| Xpt+1=Xpt+vpt+1 | (15) |

where the vectors Xptand vpt represent the position and velocity, respectively, of each particle p within a limited search space population. The parameter inertiawt holds significant importance in the algorithm, dictating how previous updates influence subsequent iterations and ultimately convergence [30-31]. Acceleration coefficients C1 and C2 , along with randomly generated numbers r1t and r2t ranging between 0 and 1, contribute to the particle movement. Pbestt denotes the best particle at a given iteration, while Gbestt signifies the globally optimal position within the search space.

In our study, we utilize the Adaptive-Elite Particle Swarm Optimization (AEPSO) algorithm, which performs several critical tasks during its operation: determining the inertia weight, conducting particle mean search, implementing pruning, and managing particle cloning [32]. These processes are vital for maintaining the stability of the iterative procedure. Unlike employing a fixed random number, the algorithm employs an adaptive method to determine the optimal value of the inertia weight, employing a sigmoid mapping equation. As a result, the modification of the inertia weight equation, as articulated in equation (14), is as follows:

| wt=11+1.5e-1.5f(tT) ∈[0.9, 1.2] ∀f ∈[0, 1] | (16) |
| --- | --- |
| f=Gbestt-XptPbestt-Xpt | (17) |

where t denotes the iteration index, and T represents the maximum iteration count. It is important to note that a previous study in [30] has highlighted the viability of employing a different range for the inertia weight (0.4 to 0.9), which has shown to yield commendable results. Additionally, improving the adaptive mechanism of the PSO involves strategies aimed at manipulating the values of C1 and C2 in equation (14). Moreover, modifying the expression for particle velocity in AEPSO involves computing particle means and standard deviations between particles throughout the iterative process:

| vt+1=wt×vpt+C1t×r1t×Pbestt-Xpt+C2t× r2tGbestt-Xpt+C3tr3tGbestt-Xmt | (18) |
| --- | --- |

where a new learning factor C3 (not exceeding four) is introduced:

| C3t=4-C1t-C2t | (19) |
| --- | --- |

And the values of C₁ and C₂ can be determined by:

| C1t=C2t=1+11+1.5e1.5α | (20) |
| --- | --- |

In this context, α=Gbest1Gbestt signifies a ratio between the current and initial global bests. The iterative convergence process gradually diminishes C3 to zero utilizing equations (19) and (20), eventually simplifying equation (18). The concluding term in equation (18) facilitates the coordination between global and local search efforts.

Finally, the pruning phase involves removing distant particles within the local search, as depicted in Fig. 1. Assessing the standard deviation σt of distances between particle pairs determines the range of particle positions for the pruning process. The equation below governs the determination of the number of particles for pruning:

| nt=3-2C3t. | (21) |
| --- | --- |

The values of C3t gradually decrease from approximately 1.48 to 0, while nt increases from 0.04 to 3 [15]. Pruning occurs by eliminating particles outside the range equation, Xpt+1±ntσt.

The fitness function used to measure the performance of AEPSO for tuning the XGboost’s parameters is the mean squared error function. The loss function is defined as:

| Lerror=-1Ni=1Nyi-yi2 | (22) |
| --- | --- |

Fig. 1 Particle Pruning and cloning.

- Proposed Method

This section presents our methodology for diagnosing faults in a 3-MW direct-drive wind turbine, utilizing a 1-D physics-informed deep convolutional neural network (PDCNN) in conjunction with an adaptive elite-PSO XGBoost classifier. Given the inherent imbalance of the wind turbine dataset, ensuring balanced data becomes crucial to avoid biased models favoring the majority fault class. To address this, we adopt the Synthetic Majority Oversampling Technique (SMOTE) and NearMiss Undersampling techniques before employing the PDCNN and the XGBoost classifier to ensure reliable performance. Subsequently, a DCNN algorithm extracts t-SNE-represented deep learning features from the preprocessed dataset and then utilizes AEPSO-XGBoost. Merging the features extracted from oversampled and undersampled datasets may reduce inherent in imbalanced data. Notably, the original imbalanced dataset is not included in the resampled data to mitigate potential overfitting risks [9]. The resampling process involves applying the preferred and undersampling methods to the imbalanced dataset.

The primary aim of the PDCNN is to categorize various fault types based on deep learning features extracted from the imbalanced dataset, thereby enhancing classification performance when combined with resampled SCADA datasets. Subsequently, the AEPSO-XGBoost classifier is utilized to classify labeled faults, avoiding overfitting, determining feature importance, and identifying critical features for fault prediction and anomaly detection. The workflow for fault prediction with anomaly detection is depicted in Fig. 2. Based on the feature importance score, the data chosen for fault prediction is partitioned into training, validation, and testing sets to assess the overall performance of the proposed AEPSO-XGBoost regressor. During training, the optimal parameters of the proposed XGBoost regressor are determined using AEPSO, while the validation set evaluates the regression model's performance. Evaluation metrics, including Mean Absolute Error (MAE), Mean Squared Error, Root-Mean Squared Error (RMSE), and coefficient of determination (R2), are utilized for this purpose. Subsequently, the testing dataset further evaluates the performance of the AEPSO-XGBoost regressor.

Additionally, we intend to incorporate PDCNN as it efficiently captures essential attributes within the dataset. Specifically, we will utilize features extracted by the PDCNN to classify faults and detect anomalies based on predefined criteria, enhancing our understanding of potential deviations from expected patterns in the data. Our proposed integrated framework is designed mainly for fault classification and anomaly detection within the SCADA data of wind energy systems, aiming to offer a robust solution to meet the essential requirements of predictive maintenance. With adaptability to imbalanced SCADA data, the framework demonstrates versatility and efficacy. Its integrated approach facilitates seamless utilization of evolving technological advancements, ensuring long-term reliability and relevance in the dynamic environment of wind energy systems. Furthermore, the proposed method allows customization, allowing stakeholders to modify the solution to their specific operational needs. Moreover, the framework's scalability ensures its applicability across various wind turbine configurations and environmental conditions.

3.1. SCADA Dataset Exploratory Data Analysis

To evaluate the efficacy of our proposed fault prediction method, we utilized a publicly available imbalanced dataset sourced from a 3-MW direct-drive wind turbine system near southern Ireland's coastal area. This dataset comprises SCADA logs from May 2014 to April 2015, consisting of three main components: operational, status, and fault data. The operational data encompasses 47 variables, recorded at 10-minute intervals, and 37,225 recorded data.

However, wind power generation facilities often experience irregularities and deviations from normal operations due to dynamic weather conditions and potential system defects, resulting in abnormal status codes. Accurate categorization of these status codes is vital for effectively monitoring and maintaining wind turbine systems. Notably, the plot of average real power generated versus wind speed in

the SCADA dataset reveals data points associated with distinct faults lying within the range of normal conditions. This observation suggests anomalies within the dataset, highlighting the importance of graphical representations for visualizing wind turbine system conditions.

While direct fault identification can be complex and unreliable, graphical representations offer insights into turbine conditions. For instance, high generator temperatures may indicate gearbox damage or bearing misalignment. Interestingly, the rear bearing temperature data analysis may indicate that generator faults exhibit a lower temperature range than normal conditions. Given the critical role of the rear bearing in supporting rotor rotation and ensuring operational efficiency, our study focuses on anomaly and fault detection in this component. Proper monitoring of rear bearing temperature is essential to prevent potentially catastrophic failures within the wind turbine system. Timely detection of temperature variations enables maintenance personnel to address issues promptly, such as lubrication adjustments or component replacements, thereby preventing further damage and minimizing costly repairs. Effective monitoring and analysis of temperature fluctuations ultimately contribute to sustaining optimal wind turbine performance and sustainability.

3.2  Data Acquisition and Preprocessing

In the initial phase, we conducted data cleaning and integrated three components of the wind turbines' SCADA dataset: operational SCADA data, status data, and fault data. The identified fault types include 43 Generator Faults (GF), 254 Feeder Faults (FF), 174 Excitation Faults (EF), 20 Mains Faults (MF), 62 Air-cool Faults (AF), and 48,581 No Faults (NF). To address the dataset's imbalance, we combined SMOTE with Near-miss Undersampling to acquire a balanced dataset. Specifically, GF corresponds to thermal issues in the generator, FF relates to cabling issues, EF pertains to the generator's excitation system, MF represents the main electricity supply for control systems, and NF denotes fault-free operation.

3.3 Imbalanced Dataset Resampling

The imbalanced nature of the SCADA dataset presents challenges in accurately classifying and detecting faults. The primary concern arises from the model's bias towards the majority class (fault-free or normal turbine operation) due to the significant class imbalance. Consequently, the model may prioritize learning characteristics of the majority class over

Fig. 2  Proposed fault classification and anomaly detection method

fault-related patterns, increasing the risk of overfitting and reducing performance on unseen data.

We employed complementary techniques to create a balanced dataset to mitigate these challenges. SMOTE and NearMiss undersampling were utilized to achieve a balanced representation of both minority (fault instances) and majority classes (normal operation instances). SMOTE involved duplicating instances from the minority class to ensure robust learning, while Near-miss Undersampling removed instances from the majority class to prevent bias towards it.

The basis behind choosing SMOTE and Near-miss Undersampling is their simplicity and effectiveness. However, carefully monitoring the resulting class distribution was essential to avoid introducing noise or losing critical information due to unsymmetrical resampling. This approach aimed to address the class imbalance and preserve dataset integrity.

3.4 Learning Feature Extraction using PDCNN

We employ 1D physics-informed deep convolutional neural networks (PDCNNs) for wind turbine fault classification because they efficiently extract useful features from complex datasets. This preference is rooted in the proven capability of 1D deep CNNs to capture complex patterns inherent in SCADA data, aligning well with the complexities of wind turbine fault classification tasks. By effectively learning important features, distinguishing indirect variations, and modeling sequential dependencies, the architecture of 1D deep CNNs is well-suited for such tasks. Consequently, the extracted features are represented as deep learning features, visualized using t-Distributed Stochastic Neighbor Embedding (t-SNE), to augment the SCADA data and classify wind turbines without overfitting.

The t-SNE technique serves as a valuable tool for reducing data dimensionality and facilitating the visualization of high-dimensional datasets in a lower-dimensional space. Its ability to capture both global and local data structures comprehensively aids in categorizing different aspects within the dataset, enhancing classification accuracy. By identifying similarities even among distant data points, t-SNE retains the global structure of the data, revealing underlying patterns effectively.

In our study, we utilize t-SNE to generate a low-dimensional representation of the balanced wind turbine SCADA data, as depicted in Fig. 3. The t-SNE algorithm plays a pivotal role in categorizing various fault types based on the derived t-SNE components. These components, obtained from the SCADA data, serve as input features for the subsequent PDCNN, which extracts deep learning features from the datasets represented by these t-SNE components. These extracted features, illustrated in Fig. 3, are then utilized as new features, combined with the resampled balanced SCADA data.

Integrating resampled SCADA data with t-SNE representations as deep learning features is crucial for our fault diagnosis model, considering the synthetic data samples in the resampled dataset. While random oversampling and undersampling are commonly employed techniques to address class imbalance, combining them with other methods enhances the balance and reliability of the training set. Therefore, augmenting the resampled SCADA data with t-SNE components from PDCNNs may help to mitigate overfitting and information loss, ensuring a balanced and representative training set. This approach facilitates effective learning of patterns from both minority and majority class instances, leading to improved model performance on unseen data.

3.5 Feature Fusion for Fault Classification

Feature fusion plays a crucial role in enhancing data representation and improving classification accuracy by combining deep learning features with SCADA data. This merging of features from diverse sources creates a comprehensive representation of the data, capturing essential information and making the machine learning model more resilient to input variations. Moreover, feature fusion is instrumental in domain adaptation, facilitating transfer - Pretrained t-SNE plot

(b) PDCNN-trained t-SNE plot

Fig. 3 t-SNE representation of deep learning  features

learning by fusing features from both source and target domains.

Our study aims to combine the balanced wind turbine SCADA dataset with deep learning features extracted by the PDCNN. These deep learning features offer valuable insights into patterns and relationships within the data, and they are trained on extensive datasets to ensure their reliability in wind turbine fault classification. By fusing SCADA data with deep learning features, we aim to enhance the accuracy of our fault classification model, especially in the presence of noise, outliers, and data fluctuations. Additionally, incorporating deep learning features enriches the data representation, capturing crucial information comprehensively. In our fault classification model, we intend to employ t-SNE to reduce redundancy and dimensionality within the features.

The merged SCADA dataset with deep learning features forms a new wind turbine dataset for fault classification. Subsequently, as our fault classifier, we utilize the XGBoost algorithm with optimized parameters obtained using AEPSO. This practical feature fusion allows our fault classification model to capture complex patterns and fundamental data characteristics effectively, resulting in significantly improved performance in fault classification for wind turbine systems.

3.6 Feature Importance using AEPSO-XGBoost

The fusion of resampled SCADA data with t-SNE-represented deep learning features constitutes the main idea of our fault prediction model, demonstrating enhancement of performance metrics for fault classification and offering valuable insights into feature importance for fault prediction within our regression model. Moreover, the strategic integration of resampled SCADA data with t-SNE-represented deep learning features is a pivotal aspect of our fault prediction framework, optimizing model training, enhancing classification performance, and facilitating feature importance analysis, thus reinforcing the overall effectiveness of the regression model for fault prediction in wind turbine systems.

Assessing the feature importance scores of all dataset variables is paramount in predictive modeling. Leveraging the feature importance technique enables us to gain a deeper understanding of the data, extract insights into the model, and establish a foundation for selecting optimal features. Furthermore, it significantly enhances the efficiency and effectiveness of the model in addressing the given problem.

However, given the focus of our study on anomaly detection, we aim to identify the dataset feature most conducive to fault prediction. To this end, we utilize the feature importance technique to pinpoint the feature with the highest influence (score) in predicting wind turbine faults, as determined by the AEPSO-XGBoost classifier using the SCADA dataset. Utilizing XGBoost is particularly advantageous due to its effectiveness in determining the importance of features. Additionally, we apply XGBoost with optimized parameters obtained through AEPSO to identify the dataset reliably features with the highest importance scores. The XGBoost parameters will be optimized using AEPSO based on the framework shown in Fig. 4.

.

3.7 Fault Prediction with Anomaly Detection

Fault prediction using the XGBoost regressor stands as a practical and robust application of machine learning, geared towards anticipating imminent faults or irregularities in a system with high accuracy. By utilizing various operational parameters (features) from the dataset, this predictive model assesses the possibility of faults occurring within the system. Through the inclusion of predictions from multiple weak learners (decision trees), the XGBoost regressor constructs a resilient and accurate regression model.

In our study, the XGBoost regressor undergoes training and optimization using AEPSO based on the original imbalanced SCADA data, which encompasses historical system operation data and recorded fault instances. Given the significant feature importance score of rear bearing temperature (the target variable), our focus lies on fault prediction concerning this variable. Accordingly, we split this variable into train, validation, and test sets. The training data facilitates the optimization of the XGBoost regressor, while the validation data evaluates the regression model's accuracy using metrics such as Root Mean Squared Error (RMSE), Mean

Squared Error (MSE), Mean Absolute Error (MAE), and Coefficient of Determination (R2). Furthermore, the effectiveness of the proposed regression model is validated using the test data.

Following fault classification, we proceed with anomaly detection, a critical aspect of fault diagnosis. To detect anomalies, we employ a physics-informed deep convolutional neural network. The PDCNN obtains the trend of the given data

and computes the residuals between the predicted and actual data. Then, the standard deviation of the residuals is calculated to understand the spread or variability of the residuals. Our anomaly threshold is set based on a specified three standard deviations (of the three data splits) from the mean of the residuals.

- Results

4.1  Fault Classification

To resample the imbalance in the wind turbine SCADA data, we applied Synthetic Minority Over-sampling Technique (SMOTE) and Near-miss undersampling methods. Subsequently, deep learning feature extraction was performed using a physics-informed deep convolutional neural network (PDCNN). The features extracted were then visualized using t-distributed Stochastic Neighbor Embedding (t-SNE), as illustrated in Fig. 3. These extracted features were merged with the resampled SCADA dataset, resulting in a newly derived dataset where all identified fault classes were represented proportionally. Specifically, this dataset comprised six classes, each containing 300 samples, totaling 1800 data instances. For the training set, we utilized 70% of the resampled data and 30% for testing to verify the proposed fault classification method.

Fig. 4 Fault classification based on AEPSO-XGBoost

Accomplishing the classification of detected faults utilizing the obtained dataset comprising deep-learning feature characteristics has been achieved to a satisfactory extent. Within the examination, we assess the efficacy of the approach through various metrics, including accuracy (A), precision (P), recall (R), and F1-score (F1). Below are the formulas representing the classification metrics of the proposed method:

| A=TP+TNTP+TN+FP+FN | (23) |
| --- | --- |
| P=TPTP+FP | (24) |
| R=TPTP+FN | (25) |
| F1=2P×RP+R | (26) |

where TP, TN, FP, and FN are the true positive, true negative, false positive, and false negative, respectively.

Table 1 AEPSO-XGBoost Classifier Perfomance Metrics

| Method | Accuracy | Precision | Recall | F1-Score |
| --- | --- | --- | --- | --- |
| SVM | 0.663 | 0.657 | 0.663 | 0.645 |
| KNN | 0.868 | 0.872 | 0.868 | 0.867 |
| Random Forest | 0.908 | 0.912 | 0.908 | 0.910 |
| PDCNN | 0.983 | 0.982 | 0.983 | 0.983 |
| PDCNN+XGB | 0.962 | 0.962 | 0.962 | 0.963 |
| PDCNN+PSO-XGB | 0.980 | 0.978 | 0.980 | 0.977 |
| Proposed | 0.985 | 0.968 | 0.985 | 0.985 |

Fig. 5 illustrates that the integration of a physics-informed deep convolutional neural network significantly enhances classification accuracy. In particular, the testing accuracy notably rises to 0.983, as depicted in Table 1. It is worth noting that employing XGBoost with randomly adjusted parameters could potentially undermine the exceptional performance achieved by the PDCNN. Therefore, meticulous parameter tuning of XGBoost is imperative. Nonetheless, the incorporation of PDCNN consistently yields notable improvements in classification accuracy.

Moreover, integrating physics-informed deep learning features with the XGBoost classifier significantly boosts accuracy, emphasizing the potential of this feature extraction approach. Crucially, our proposed fault classification method attains the highest accuracy within this investigation. This accomplishment is primarily based on optimizing XGBoost parameters through Adaptive Elite-Particle Swarm Optimization (AEPSO). These discoveries also emphasize the essential role of parameter optimization and deep learning feature extraction via physics-informed neural network techniques in achieving superior outcomes.

4.2  Fault Prediction and Anomaly Detection

Our proposed regression model is developed and evaluated using a SCADA dataset from a wind turbine system, focusing specifically on the rear bearing temperature data entries. The training phase involves optimizing XGBoost parameters using the training data, while the validation phase assesses regression performance metrics, including Root Mean Squared Error (RMSE), Mean Squared Error (MSE),

- Training and validation data prediction

- Testing data prediction

Fig. 6 Rear-bearing temperature prediction comparison

Mean Absolute Error (MAE), and Coefficient of Determination (R2), using the validation data. Additionally, the robustness of the proposed regression model is validated using separate testing data. The data division into training, validation, and testing sets follows the recommended train-test segmentation outlined in [33]. Table 2 presents the data segmentation, and the comparison of predicted

Fig. 5 Comparison of classification accuracy of different methods using the resampled SCADA dataset

rear bearing fault temperatures using the testing data split is depicted in Fig. 6.

Throughout the validation process, we implemented several measures to ensure the robustness of the AEPSO-XGBoost regressor. Firstly, the SCADA data underwent rigorous preprocessing to eliminate any insignificant outliers. Secondly, we analytically identified the important feature as the basis for fault prediction. Thirdly, the model underwent training using cross-validation to mitigate the risk of overfitting. Lastly, the parameters of XGBoost were fine-tuned using PSO to optimize performance metrics for both wind energy system fault classification and detection.

Table 3 AEPSO-XGBoost regressor performance using training and validation data segmentation

| Method | MAE | MSE | RMSE | R2 |
| --- | --- | --- | --- | --- |
| Proposed | 0.0738 | 0.0123 | 0.1108 | 0.9996 |
| XGBoost | 0.3713 | 0.2490 | 0.4990 | 0.9924 |
| Naive Bayesian | 0.3357 | 0.3815 | 0.6176 | 0.9884 |
| KNN | 0.5099 | 1.0799 | 1.0392 | 0.9672 |
| SVM Regression | 2.399 | 12.467 | 3.5309 | 0.6212 |
| Logistic Reg. | 1.9141 | 10.078 | 3.1747 | 0.6938 |

Table 4 AEPSO-XGBoost regressor performance using testing data segmentation

| Method | MAE | MSE | RMSE | R2 |
| --- | --- | --- | --- | --- |
| Proposed | 0.0861 | 0.0667 | 0.2582 | 0.9901 |
| XGBoost | 0.3343 | 0.2552 | 0.5051 | 0.9623 |
| Naive Bayesian | 0.0128 | 0.0707 | 0.5659 | 0.9895 |
| KNN | 0.5504 | 1.1075 | 1.0524 | 0.8363 |
| SVM Regression | 1.5268 | 3.8554 | 1.9635 | 0.4302 |
| Logistic Reg. | 1.3942 | 4.0260 | 2.0065 | 0.4050 |

Table 2 Data segmentation

|  | Start | End | Number of Rows | Split Percentage (%) |
| --- | --- | --- | --- | --- |
| Training | 05/01/2014 | 10/15/2014 | 28327 | 58.362 |
| Validation | 10/16/2014 | 12/31/2014 | 11064 | 22.795 |
| Testing | 01/31/2015 | 05/01/2015 | 9146 | 18.843 |
| Total | 05/01/2014 | 05/01/2015 | 48537 | 100 |

The validation process was executed to ensure the robustness and reliability of the AEPSO-XGBoost regressor by incorporating three distinct rear-bearing temperature datasets. This multiple split of the datasets allows for the verification of the model across diverse scenarios, thereby enhancing its generalization capability and providing a comprehensive assessment of its performance.

The datasets were temporally partitioned into training and validation sets to demonstrate the real-world applicability

of the proposed regression model. This segmentation ensures that the model is assessed on unseen data, thus mimicking its performance in practical scenarios. Another crucial aspect of validating the performance of our proposed regression model

involves optimizing the parameters of XGBoost based on the training data split. Utilizing PSO facilitated the fine-tuning of XGBoost's parameters through an iterative process. This iterative approach ensured rigorous optimization of the model across all data splits of the wind turbine rear bearing temperature data, with the aim of achieving robustness and high predictive accuracy. These aspects were also evaluated using metrics such as MAE, MSE, RMSE, and R2.

It is evident that the proposed regression model, along with its counterparts, demonstrates satisfactory predictions of rear bearing temperature. However, the inclusion of Support Vector Machine (SVM) yielded inadequate regression results. To assess the performance of our proposed prediction method, we compared it with other regression models employed in this study, using the rear turbine temperature validation data. Our proposed regression method showcases superior performance in terms of Mean Squared Error (MSE), Mean Absolute Error (MAE), Root Mean Square Error (RMSE), and R2, as presented in Table 3 and Table 4. Specifically, our proposed prediction method achieved the lowest values for MSE, MAE, and RMSE, indicating its effectiveness and prediction accuracy. Notably, our proposed prediction method also attains the highest coefficient of determination, R2, highlighting its superior fit to the rear bearing temperature data. Additionally, the results reveal that Naïve Bayesian can serve as an effective regression tool, as it achieves a 2.75% higher R2 compared to the XGBoost regressor.

The integration of AEPSO and XGBoost plays a crucial role in augmenting the fault prediction model, encompassing

- Predicted rear-bearing temperature mean and standard deviation

- detected anomalies

Fig. 7 Anomaly detection using PDCNN and AEPSO-XGBoost

Fig. 8 Actual faults from the SCADA data

both fault classification and fault detection aspects. This combination enables the finding of optimal or near-optimal parameter values, thereby fostering the development of a model finely tuned to the unique characteristics of the SCADA dataset. Consequently, this refinement process leads to an increased accuracy, robustness, and generalization performance of the model.

The iterative mechanism inherent in AEPSO ensures a comprehensive exploration of the parameter space, facilitating the identification of XGBoost parameter values that maximize the effectiveness of the fault prediction model across diverse operational scenarios and conditions. Through this iterative optimization process, the fault prediction model becomes proficient at capturing complex patterns and distinctions present in the data, thereby enhancing its predictive capabilities and adaptability to varying real-world conditions.

In this study, we incorporate a physics-informed deep CNN to detect anomalies by capturing the trend of the original rear-bearing temperature data. Subsequently, we plan to utilize the predicted rear bearing temperature obtained through AEPSO-XGBoost in conjunction with the actual SCADA data to facilitate anomaly detection. Anomalies are identified based on the standard deviation of the calculated residuals between the absolute values of the predicted and actual rear-bearing temperature data. Any predicted rear bearing temperature values deemed anomalous are plotted outside predetermined threshold values. Through this approach, we can correlate the detected anomalies, as illustrated in Fig. 7, the detected anomalies based on the predicted rear-bearing temperature data, and in Fig. 8, showing the actual faults. This enables us to gain insights into potential deviations from expected operational patterns and behavior within the system.

The proposed fault prediction model, when applied to actual SCADA data, serves to detect anomalies that may indicate unexpected behavior within the wind turbine system. The PDCNN model successfully identifies 2741 anomalies, while the SCADA dataset records 553 faults. In this study, the anomaly detection model achieves an accuracy score of 83.72%. Although the precision score is 87%, indicating that the model effectively identifies anomalies, it also suggests instances where 'No Fault' data is erroneously labeled as anomalies. Similarly, the recall score and F1 score, at 84% and 82% respectively, indicate the model's ability to identify actual anomalies, but also highlight cases where true faults are overlooked. These classification scores collectively provide insights into the performance of the anomaly detection model, showing its effectiveness in detecting anomalies while also acknowledging areas for potential improvement, particularly in minimizing false positives and enhancing fault detection sensitivity.

Upon using AEPSO-XGBoost, the anomaly detection result yields an impressive accuracy score of 96.5%, signifying the successful identification of anomalies using the proposed method. Moreover, achieving recall and F1 scores of 97.2% and 95.8% respectively highlights the model's efficacy in identifying genuine anomalies and capturing fault instances accurately. These results emphasize the capability of the anomaly detection model to distinguish true faults through the integrated PDCNN and AEPSO-XGBoost approach, thereby demonstrating its potential for real-world application in fault detection and system monitoring.

These two models complement each other synergistically in fault detection. The supervised AEPSO-XGBoost model identifies known fault patterns with high precision, while the unsupervised PDCNN model enhances system resilience by capturing anomalies that may not conform to predefined fault signatures. Combining both supervised and unsupervised approaches provides a comprehensive and adaptive fault detection system that addresses both well-established fault conditions and emerging anomalies in rear-bearing temperature data [34]. This dual-model strategy ensures a more robust and versatile fault detection framework, enhancing the reliability and accuracy of predictive maintenance system for wind energy applications.

- Conclusion

In this study, we showcase the efficacy of employing a physics-informed deep convolutional neural network and XGBoost, optimized through adaptive elite-particle swarm optimization (AEPSO), for the classification of wind energy system faults and the detection of anomalies based on rear bearing temperature data extracted from imbalanced SCADA datasets. Our classification methodology integrates two distinct feature sets: the resampled SCADA dataset, employing SMOTE and Near-miss undersampling techniques (including deep learning features represented by t-SNE). The AEPSO-XGBoost classifier is trained on a combination of resampled SCADA data and deep learning features extracted by the physics-informed deep convolutional neural network. Also, we conduct fault prediction model training and testing based on three data splits of rear bearing temperature: training, validation, and testing, with XGBoost regressor's optimal parameters determined through training dataset analysis, and robustness assessment through validation and testing datasets. Anomaly detection is successfully implemented utilizing a physics-informed deep CNN and AEPSO-XGBoost regressor, using the trend of rear bearing temperature data.

Our findings prove the enhancement of wind turbine system fault classification and prediction performance metrics achieved by optimizing XGBoost parameters and fusing resampled SCADA data with extracted deep learning features. Benchmarking against different datasets further validates the effectiveness of our proposed classifier for fault diagnosis. Furthermore, the performance metrics of anomaly detection are shown to be further enhanced with AEPSO-XGBoost. Notably, our proposed fault classification and anomaly detection approach addresses the challenge of achieving high-performance metrics in imbalanced SCADA data, eliminating the need for random or trial-and-error parameter tuning approaches for XGBoost. Finally, the proposed fault classification and anomaly detection prediction methodology can be a potential for integration into a reliable predictive maintenance and prognosis architecture utilizing real-time SCADA data in future endeavors for a reliable and robust electric power generation facility.

- References

A. Elia, M. Taylor, B. Ó Gallachóir, and F. Rogan, “Wind turbine cost reduction: A detailed bottom-up analysis of innovation drivers,” Energy Policy, vol. 147, p. 111912, 2020. doi:10.1016/j.enpol.2020.111912.

“Advantages and challenges of wind energy,” Energy.gov, https://www.energy.gov/eere/wind/advantages-and-challenges-wind-energy (accessed Dec. 6, 2023).

Q. He, Y. Pang, G. Jiang, and P. Xie, “A spatio-temporal multiscale neural network approach for wind turbine fault diagnosis with imbalanced SCADA data,” IEEE Transactions on Industrial Informatics, vol. 17, no. 10, pp. 6875–6884, 2021. doi:10.1109/tii.2020.3041114.

Moya, B., Badías, A., González, D., Chinesta, F., Cueto, E.: ‘Computational sensing, understanding, and reasoning: An artificial intelligence approach to physics-informed world modeling’Archives of Computational Methods in Engineering, 2023.

Zhou, Q., Tang, J.: ‘Gearbox fault detection via physics-informed parallel deep learning model architecture’Volume 11: 2023 International Power Transmission and Gearing Conference (PTG), 2023.

Zeng, Y., Pou, J., Jie, H., et al.: ‘A physics-informed pattern recognition method for open-circuit fault detection of inverters under unexpected conditions’IECON 2023- 49th Annual Conference of the IEEE Industrial Electronics Society, 2023.

Hamid, Y., Sugumaran, M.: ‘A T-sne based non linear dimension reduction for network intrusion detection’International Journal of Information Technology, 2019, 12, (1), pp. 125–134.

S. Mukherjee, “T-SNE based feature extraction technique for multi-layer Perceptron neural network classifier,” 2017 International Conference on Intelligent Computing, Instrumentation and Control Technologies (ICICICT), 2017. doi:10.1109/icicict1.2017.8342641.

Shen, S., Lu, H., Sadoughi, M., et al.: ‘A physics-informed deep learning approach for bearing fault detection’Engineering Applications of Artificial Intelligence, 2021, 103, p. 104295.

Sadoughi, M., Hu, C.: ‘Physics-based convolutional neural network for fault diagnosis of rolling element bearings’IEEE Sensors Journal, 2019, 19, (11), pp. 4181–4192.

Parziale, M., Lomazzi, L., Giglio, M., Cadini, F.: ‘Physics-informed neural networks for the condition monitoring of rotating shafts’Sensors, 2023, 24, (1), p. 207.

Ogaili, A.A., Hamzah, M.N., Jaber, A.A.: ‘Enhanced fault detection of wind turbine using extreme gradient boosting technique based on nonstationary vibration analysis’Journal of Failure Analysis and Prevention, 2024.

Waqas Khan, P., Byun, Y.-C.: ‘Multi-fault detection and classification of wind turbines using stacking classifier’Sensors, 2022, 22, (18), p. 6955.

Attouri, K., Dhibi, K., Mansouri, M., Hajji, M., Bouzrara, K., Nounou, H.: ‘Enhanced fault diagnosis of wind energy conversion systems using ensemble learning based on sine cosine algorithm’Journal of Engineering and Applied Science, 2023, 70, (1).

Zhang, L.: ‘Research on intelligent asynchronous adaptive model of wind energy storage based on particle swarm optimization’2022 IEEE 4th Eurasia Conference on IOT, Communication and Engineering (ECICE), 2022.

Wang, X., Wang, X., Li, L., Zheng, Y., Zhou, L., Liu, Y.: ‘Reactive power optimization for wind power system based on adaptive weights flight adjustment particle swarm optimization’Lecture Notes in Electrical Engineering, 2013, pp. 1047–1053.

Velandia-Cardenas, C., Vidal, Y., Pozo, F.: ‘Wind turbine fault detection using highly imbalanced real SCADA data’Energies, 2021, 14, (6), p. 1728.

Simani, S., Castaldi, P.: ‘Intelligent fault diagnosis techniques applied to an offshore wind turbine system’Applied Sciences, 2019, 9, (4), p. 783.

Cong, Z., Wang, J., Li, S., Ma, L., Yu, P.: ‘Fault detection and diagnosis of multi-faults in turbine gearbox of wind farms’2021 International Conference on Applications and Techniques in Cyber Intelligence, 2021, pp. 192–198.

Magar, R., Ghule, L., Li, J., Zhao, Y., Farimani, A.B.: ‘FaultNet: A deep convolutional neural network for Bearing Fault Classification’IEEE Access, 2021, 9, pp. 25189–25199.

Brunton, S.L., Kutz, J.N., Morrison, J.B.: Chapter 6. In: Data‐Driven Science and Engineering Machine Learning, Dynamical Systems, and Control, pp. 212–216. Cambridge University Press, Cambridge (2019).

1Rayhan Ahmed, Md., Islam, S., Muzahidul Islam, A.K.M., Shatabda, S.: ‘An ensemble 1D-CNN-LSTM-GRU model with data augmentation for speech emotion recognition’Expert Systems with Applications, 2023, 218, p. 119633.

Mohamad, T.H., Abbasi, A., Kappaganthu, K., Nataraj, C.: ‘On extraction, ranking and selection of data-driven and physics-informed features for Bearing Fault Diagnostics’Knowledge-Based Systems, 2023, 276, p. 110744.

Zhang, Z., Yan, X., Liu, P., Zhang, K., Han, R., Wang, S.: ‘A physics-informed convolutional neural network for the simulation and prediction of two-phase Darcy flows in heterogeneous porous media’Journal of Computational Physics, 2023, 477, p. 111919.

Ihunde, T.A., Olorode, O.: ‘Application of physics informed neural networks to compositional modeling’Journal of Petroleum Science and Engineering, 2022, 211, p. 110175.

Johnson, R., Boubrahimi, S.F., Bahri, O., Hamdi, S.M. (2023). Physics-Informed Neural Networks for Solar Wind Prediction. In: Rousseau, JJ., Kapralos, B. (eds) Pattern Recognition, Computer Vision, and Image Processing. ICPR 2022 International Workshops and Challenges. ICPR 2022. Lecture Notes in Computer Science, vol 13645. Springer, Cham. https://doi.org/10.1007/978-3-031-37731-0_21.

Jiang, H., et al.: Network intrusion detection based on PSO‐XGBoost model. IEEE Access 8, 58392–58401 (2020). https://doi.org/10.1109/ access.2020.2982418.

Xie, J., et al.: A novel bearing fault classification method based on XGBoost: the fusion of deep learning‐based features and empirical features. IEEE Trans. Instrum. Meas. 70, 1–9 (2021). https://doi.org/10. 1109/tim.2020.3042315.

D. A. Dias Júnior et al., “Automatic method for classifying COVID-19 patients based on chest X-ray images, using deep features and PSO-optimized XGBoost,” Expert Systems with Applications, vol. 183, p. 115452, 2021. doi:10.1016/j.eswa.2021.115452

Zhan, Z.‐H., et al.: Adaptive particle swarm optimization. IEEE Trans. Syst. Man Cybern. B Cybern. 39(6), 1362–1381 (2009). https://doi.org/10.1109/tsmcb.2009.2015956.

Zaman, H.R., Gharehchopogh, F.S.: An improved particle swarm optimization with backtracking search optimization algorithm for solving continuous optimization problems. Eng. Comput. 38(S4), 2797–2831 (2021). https://doi.org/10.1007/s00366-021-01431-6.

Hong, Y.‐Y., et al.: A novel adaptive elite‐based particle swarm optimization applied to var optimization in electric power systems. Math. Probl. Eng. 2014, 1–14 (2014). https://doi.org/10.1155/2014/761403.

Brownlee, J.: ‘Train-test split for Evaluating Machine Learning Algorithms’, https://machinelearningmastery.com/train-test-split-for-evaluating-machine-learning-algorithms/, accessed February 2024

V. Wilmet, S. Verma, T. Redl, H. Sandaker, and Z. Li, "A Comparison of Supervised and Unsupervised Deep Learning Methods for Anomaly Detection in Images," arXiv:2107.09204, 2021.