An enhanced YOLOv8-based bolt detection algorithm for transmission line

Guoxiang Hua1,3，Huai Zhang2，Chen Huang2，Moji Pan2，Jiyuan Yan3,*，Haisen Zhao1

1. School of Electrical and Electronic Engineering, North China Electric Power University, Beijing, China

2. School of Automation, Nanjing University of Information Science & Technology, Nanjing，China

3. School of Automation, Wuxi University, Wuxi, China

Correspondence：Jiyuan Yan，School of Automation, Wuxi University, Wuxi, China

Email: wuxi_yjy@cwxu.edu.cn

Funding information: National Key Laboratory of New Energy Power System

Grant/Award Number: LAPS23014

Abstract: The current overhead work robot for transmission line faces issues related to its compact structure and high target localization requirements. To address these challenges, this paper proposes a lightweight bolt detection algorithm based on improved YOLOv8 (You Only Look Once v8) model. Firstly, the C2f module in the feature extraction network is integrated with the Self-Calibrated Convolution (SCConv) module, and the model is streamlined by reducing spatial and channel redundancies of the network through the SRU and CUR mechanisms in the module. Secondly, the P2 Small Object Detection Layer is introduced into the neck structure and the BiFPN network structure is incorporated to enhance the bidirectional connection paths, thereby promoting the upward and downward propagation of features. It improves the accuracy of the network for bolt-small target detection. The experimental results show that, compared to the original YOLOv8 model, the proposed algorithm demonstrates superior performance on a self-collected dataset. In this paper, the mAP accuracy is improved by 9.9%, while the number of model parameters and the model size is reduced by 0.973×106 and 1.7MB, respectively. The improved algorithm improves the accuracy of the bolt detection while reducing the computation complexity to achieve more lightweight model.

Keywords: object detection; YOLOv8; BiFPN; SCConv

# 1. Introduction

As society progresses and the demand for electricity continues to rise, the distribution of transmission lines has become increasingly widespread [1-3]. These lines predominantly utilize bolted connections, with the loosening or detachment of bolts being a primary cause of transmission line failures. Traditionally, the inspection and maintenance of transmission lines have required specialized personnel to perform on-site checks. However, manual live-line maintenance at high altitudes involves working in outdoor environments and interacting with high-voltage equipment, which presents significant safety risks. As China’s power grid transitions into a new era of smart technology, conventional manual inspection methods are gradually being replaced by advanced approaches that employ live-line working robots for remote inspection and fault detection. In this context, a wide array of image recognition and processing techniques is increasingly being utilized in power line inspection scenarios [4,5].

Figure 1 Transmission Line Fitting Bolts

In recent years, the rapid advancement of deep learning has led to the widespread application of machine vision-based image detection technologies across various fields. Addressing the specific image detection requirements for transmission line maintenance, several studies have proposed innovative approaches. For instance, literature [6] introduces a comprehensive improvement strategy for detecting insulators and cross-scale fracture defects in transmission lines. This approach enhances the perception of insulator and defect regions by refining the loss function, constructing a backbone network, and integrating an attention module. Literature [7] presents a bird nest defect detection model for transmission lines that combines domain knowledge with an occlusion reasoning network. This model uses feature-level random erasure within the occlusion reasoning network to simulate real-world scenarios where bird nests occlude transmission line components, thereby enhancing the model's ability to detect occluded targets. Additionally, literature [8] proposes YOLOu-Quasi-ProtoPNet, designed for the detection and classification of faulty insulators. This model leverages an optimized quasi-ProtoPNet to improve the classification accuracy of faulty insulators. Furthermore, literature [9] develops a neural network (NN) model aimed at classifying contaminated insulators, significantly boosting the accuracy of such classifications. While these research methods improve the precision of target detection in transmission lines by enhancing traditional neural network models, they also increase computational complexity, which poses challenges for model deployment.

To address the challenges of model deployment and limited computational power, researchers have extensively explored model lightweighting techniques. These efforts primarily focus on model compression strategies, including parameter pruning, low-rank decomposition, and knowledge distillation [10-12]. For instance, literature [13] integrates Ghost convolution and model pruning into the YOLOv5 framework, achieving significant lightweighting. In another study, literature [14] introduces FSS-YOLO, a lightweight model based on YOLOv5n, designed for detecting rail fastener screws. This model replaces the C3 module in the backbone and neck with the C3Fast module, effectively reducing the number of parameters (Params) and floating-point operations (FLOPs). However, while these lightweight models successfully decrease computational requirements, they often exhibit reduced capabilities in feature extraction and fusion. Consequently, in complex outdoor environments, these methods not only reduce model parameters but also compromise the performance of small target detection.

To reconcile the trade-off between bolt target detection performance and model lightweight, this paper proposes enhancements to the YOLOv8 model. Key improvements include:

(1) Integration of the ScConv convolution module with the C2f module in the YOLOv8 feature extraction network, reducing model redundancy in spatial and channel dimensions via SRU and CRU modules to minimize model parameters.

(2) Enhancement of the Neck layer connection using the BiFPN network architecture, facilitating bidirectional feature adjustment and adaptive feature refinement. The introduction of the P2 small target detection layer enhances focus on small targets, thereby improving bolt detection accuracy and reducing misdetection rates.

(3) Conducting ablation comparison experiments on a self-collected dataset to validate the performance enhancements of the proposed algorithm.

# 2 Architecture of improved YOLOv8n network

YOLOv8 represents the latest advancement in the YOLO (You Only Look Once) series for target detection, introduced by Ultralytics in January 2023. It continues the core principle of treating target detection as a single regression problem, predicting both the target’s class and bounding box in a single forward pass [15]. The YOLOv8 series includes five variants ranging from smaller to larger models: v8n, v8s, v8m, v8l, and v8x. As model size increases, so does accuracy, accommodating diverse requirements for tasks such as target detection, image classification, instance segmentation, and keypoint detection.

This paper selects the YOLOv8n model due to hardware constraints on the robot device, prioritizing small size and high accuracy. The model architecture comprises three main networks: Backbone, Neck, and Head.

(1) The Backbone network focuses on feature extraction, leveraging the CSP concept. It replaces the C3 module with the C2f module from YOLOv5 while retaining the SPPF module, ensuring fixed vector sizes across different scales.

(2) The Neck network handles feature fusion and adopts the C2f module to replace the original C3 module. Inspired by PAN-FPN, it eliminates the 1×1 convolutions in up-sampling, directly operating on features outputted from various Backbone stages.

(3) The Head network adopts an Anchor-Free approach, departing from YOLOv5’s Anchor-Based method to independently handle parameters for classification and regression tasks, mitigating issues like leakage and misdetection from the original YOLOv8.

Figure 2 Structure of the improved YOLOv8 model network

The structure of the improved model is shown in Figure 2, which mainly consists of basic modules such as ConvModule, C2f, improved C2f, SPPF, Concat, and Upsample.

The original YOLOv8 is prone to missed detections and false detections due to the small pixel footprint of targets in the low-altitude steel frame bolt dataset, dense distribution, and the influence of background illumination. Therefore, this paper improves the Backbone and Neck networks. The Backbone section incorporates the C2f module into the CVPR2023 SCConv module. Models embedded with the SCConv module can significantly reduce complexity and computational cost. This results in a significant reduction in model parameters and redundant features, achieving better performance while maintaining the model's integrity. The Neck network improves the BiFPN network structure by adding bidirectionall propagation between different layers on top of the original FPN structure. In addition to the original top-down connections, bottom-up connections are introduced, allowing the network to fully utilize feature information across lower and higher levels. Cross-scale information fusion is added on top of feature fusion to capture target information at different scales. The P2 small target detection layer is also introduced. The P3 layer is fused with the P2 layer through up-sampling and convolution module processing. Parameter sharing is realized between each layer to reduce the network's parameter count, and the feature fusion weights between each layer are adjusted adaptively to improve the model’s generalization ability.

The performance of object detection and semantic segmentation tasks is improved through bidirectional connectivity, adaptive feature adjustment, and modularization. The introduction of the P2 small target detection layer allows the model to detect and localize small targets more accurately, thereby enhancing the accuracy of bolt small target detection.

## 2.1 Basic modules of the network

This paper focuses on lightweight improvements to YOLOv8, extending its Conv, SPPF, Concat, and Upsample modules within the improvement network. The Conv Module is composed of Conv2D, BatchNorm2D, and SiLU, as shown in Figure 3. Here, the parameters k, s, p, and c represent the convolution kernel size, convolution compensation, padding size, and the number of output channels of the convolutional layer, respectively. The SPPF module performs a spatial pyramid pooling step, which generates fixed-length outputs from inputs of different sizes and merges multi-scale feature maps through pooling operations The composition of the module is shown in Figure 4.

| Figure 3 ConvModule Module Structure | Figure 4 SPPF Module Structure |
| --- | --- |

2.2 Backbone Network Improvements

In this paper, the improved C2f_ScConv is used to replace the C2f module in the Backbone network. The structure of the improved C2f_ScConv is shown in Figure 5. The improved module effectively limits the feature redundancy, which not only reduces the number of model parameters and FLOPs but also enhances the capability of feature representation.

Figure 5 C2f_ScConv Module Structure

The C2f_ScConv improvement module integrates the ScConv module with the original Bottleneck module within the C2f module. Additionally, it incorporates the ScConv module between the two convolution modules in the feedforward channel of the original system module. The Bottleneck module’s output feature map undergoes spatial and channel reconstruction via ScConv, reducing redundancy and producing a streamlined feature map. This fused C2f_ScConv module effectively reduces redundant features, thereby enhancing model performance for lightweight improvements. The structure of the SCConv module is shown in Figure 6. It mainly consists of a Spatial Reconstruction Unit (SRU) and a Channel Reconstruction Unit (CRU) [16-17].

### 2.2.1 SRU

The SRU, a sequence model akin to a simpler and faster form of recurrent neural network compared to LSTM [18], consists of two operations: Separate and Reconstruct. The input feature map is separated from less spatially informative feature maps through group normalization, Sigmoid activation, and threshold setting. Each parameter of the feature map X follows the order of batch axis, channel axis, spatial height, and width axis order, and undergoes group normalization for input feature normalization.

In the GN (group normalization) layer, the variance of the batch and channel space pixels is measured using the trainable parameter γ. The correlation weight Wγ can be calculated using equation (1):

|  |  | (1) |
| --- | --- | --- |

After group normalization, the obtained correlation weights are mapped to the range (0,1) by a Sigmoid function and gated using a threshold. The weights above the threshold are set to 1 and the weights below the threshold are set to 0. The obtained weights are set to W1 and W2 respectively. Multiply the feature X with W1 and W2 element by element to obtain the feature with rich information X1W and the feature with less information X2W. After that, the two features with different information are reconstructed by element-by-element summing to obtain more informative features and save space. Finally, the reconstructed features X1W, X2W are concatenated to obtain a spatially reconstructed feature map.

|  |  | (2) |
| --- | --- | --- |

whereis the element-by-element multiplication, is the element-by-element addition, and is the Concat module.

Figure 6 ScConv Module Structure

### 2.2.2 CRU

The CRU structure can further reduce the redundancy of the feature map in the channels, which mainly includes three operations Split, Transform, and Fuse. Firstly, the spatially reconstructed feature map XW is subjected to a split operation, which divides the C channels of the feature map into αC, 1-αC parts. Subsequently, the two channels after segmentation are compressed using the 11 convolution module and the compressed channel features are divided into two parts,Xup and Xlow. In the Transform stage, Xup is used as the up-conversion input, and rich information features are extracted using Group-wise Convolution (GWC) and Point-wise Convolution (PWC). Xlow is the down-converted input for PWC extraction.

|  |  | (3) |
| --- | --- | --- |
|  |  | (4) |

where MG is the learning weights of GWC. MP1 and MP2 are the learning weights of PWC for up and down conversion channels.

After that, the global spatial information is collected by Fuse operation on the output features  Y1 and Y2 of the up and down conversion channels by Pooling operation, and the importance vectors β1 and β2 of the global information about the up and down parts are generated using the channel attention. Finally, the output channel reconstruction features are represented as Compared with the traditional convolution module, all the parameters in the ScConv module are located in the Transform stage, so the number of parameters involved in the computation is greatly reduced.

## 2.3 Neck network improvements

BiFPN (Bi-directional Feature Pyramid Network) serves as a neural network architecture designed for tasks such as target detection and semantic segmentation to enhance the performance of the Feature Pyramid Network (FPN) [19-20]. The traditional Neck connection layer in YOLOv8 utilizes the FPN architecture [21], as depicted in Figure 7. However, FPN’s top-down information flow sometimes fails to fully integrate lower-level features with higher-level semantic information, thereby limiting the model’s ability to detect smaller targets. Additionally, adjustments between feature maps at different scales through up-sampling and down-sampling may lead to redundant computations and increased computational costs.

Figure 7 FPN Module Structure

To address these challenges, this paper adopts the BiFPN network structure in the Neck portion for algorithmic improvement, as illustrated in Figure 8. The BiFPN structure introduces a bidirectional information flow path that effectively integration feature information across different scales and levels.

Figure 8 BiFPN Module Structure

In the improved Neck layer of this paper, the small target detection layer P2 is introduced, and the Backbone outputs P2, P3, P4, and P5 are fed into the BiFPN network structure to fuse the input multi-scale feature maps. By introducing the P2 small target detection layer, the model’s capability to detect small targets is enhanced, leading to an improvement in overall performance. This reduction in false alarms and omissions is particularly beneficial in dense bolt detection scenarios. The network architecture reduces redundant computations between feature maps of different scales by sharing feature computations, thereby enhancing computational efficiency. Furthermore, the BiFPN network, with its reduced number of parameters and computational complexity, facilitates practical deployment on robot vision platforms.

# 3 Results

## 3.1 Experimental environment

The dataset utilized in this study consists of images depicting bolts situated on steel frames at a substation construction site. These images were captured using a drone to photograph the construction of steel frames. The dataset comprises a total of 371 original images, showcasing bolts in various real-world conditions: 191 images under bright lighting, 116 under low lighting, and 64 under normal lighting. Each image has a resolution of 3904 × 2928 pixels. To enhance the dataset, we applied augmentation techniques including image rotation, flipping, random cropping, and noise addition, resulting in an augmented dataset containing 1084 images, which adequately meets the training requirements of our model. The augmented dataset was manually annotated using labelImg software and subsequently divided into a training set and a validation set in an 8:2 ratio. The bolts in the self-collected dataset are situated in an outdoor, high-altitude environment, where the background is significantly affected by environmental factors. Furthermore, variations in bolt density distribution and shooting angles add complexity to bolt detection on steel frames. Thus, our self-collected dataset of steel frame bolts is particularly suited for simulating real conditions at a substation construction site, thereby enhancing the practical relevance of our model improvement algorithm applied to this dataset.

Experimental evaluations of our algorithm were conducted on a computer equipped with a Core i5-12490F CPU (2.5 GHz, 10 cores), an NVIDIA GeForce RTX 4060Ti GPU (16 GB VRAM), and 32 GB of RAM, within a Python 3.11.4 programming environment. Specific experimental parameters for training are detailed in Table 1.

Table 1 Training parameter settings

| Parameters | settings | Parameters | settings |
| --- | --- | --- | --- |
| batch | 32 | lr0 | 0.01 |
| epochs | 500 | lrf | 0.01 |
| imgsz | 640 | box | 7.5 |
| workers | 16 | cls | 0.5 |
| weight_decay | 0.0005 | dfl | 1.5 |

All models were trained with consistent settings: an input image size of 640 pixels, a batch size of 32, and a uniform training duration of 500 epochs. An early-stopping mechanism was implemented, halting training if model performance did not improve within approximately 50 epochs, ensuring efficient model convergence.

## 3.2 Improved model training and evaluation metrics

This paper introduces improvements to the model architecture. Firstly, the SCConv module is integrated into the C2f module within the Backbone network. Secondly, the enhanced BiFPN serves as the connection layer structure in the Neck network. During experimentation, the initial learning rate is set to 0.01 with a momentum factor of 0.937. Figure 9 illustrates the training process of the enhanced model, which focuses on the classification detection of dense bolts. In the plot of Figure 9, cls_loss represents the confidence level of bolt target localization, while box_loss and dfl_loss denote the accuracy of detection frame prediction in bolt detection tasks. The val/box_loss and val/dfl_loss in Figure 9 show the performance of the three loss functions on the validation set during the training phase. The figures indicate that the proposed algorithmic model is in a favorable training condition, exhibiting smooth curves without signs of overfitting.

Figure 9 Improving the training process of the YOLOv8 model

In this paper, Precision, Recall, and MAP@0.5 are used as the evaluation metrics of network performance [22]. Precision is the ratio of the number of correctly predicted pixels in the resultant map to the number of pixels in the resultant map with all the outlines detected, which is given by (5):

|  |  | (5) |
| --- | --- | --- |

In the above equation, TP is the number of correctly identified pixel points in the extraction result, and FP is the number of incorrectly identified pixel points in the extraction result.

Recall is the ratio of the number of correctly predicted pixel points to the number of all contour pixel points in the image,which is given by (6):

|  |  | (6) |
| --- | --- | --- |

FN is the number of missed pixel points in the extracted result.

A higher precision rate indicates that the algorithm has a lower error rate for bolt detection, while a higher recall rate means that fewer bolts are correctly detected but missed, while MAP@0.5 balance the two performances better and reflect the overall performance of the model.

## 3.3 Contrast experiment

### 3.3.1 Compared with other methods

The experiments in this paper specifically address the detection of dense bolts on steel frames in substations using self-acquired datasets. Considerations include real-time requirements, computational resource constraints, target scale adaptability, and simplification of the engineering process for robotic systems. Robotic systems necessitate rapid and accurate target detection, thus demanding high real-time performance. The YOLO algorithm is renowned for its efficient one-shot forward propagation design, making it suitable for applications with stringent real-time demands. Moreover, YOLO employs an end-to-end training approach, simplifying detection from raw images to results, thereby reducing complexity in training and deployment, and lowering development and maintenance costs for robotic systems. Additionally, YOLO’s grid-based approach enhances adaptability to targets of various scales, enabling effective detection across different sizes, particularly beneficial for detecting small targets. Consequently, this paper focuses on comparative experiments involving various iterations of the YOLO algorithm series. The results, as presented in Table 2, indicate that the YOLOv8n model achieves the highest recognition accuracy among the tested algorithms and offers superior lightweight characteristics within its series. Therefore, YOLOv8 is selected as the base model for further enhancement in this study.

Table 2 Comparative experiments of different algorithms

| Model |  | P/% | R/% | mAP/% | Params/M |
| --- | --- | --- | --- | --- | --- |
| YOLOv4 |  | 84.6 | 80.6 | 84.1 | 27.0 |
| YOLOv5n |  | 87.4 | 83.2 | 86.6 | 14.4 |
| YOLOv6 |  | 86.3 | 78.4 | 84.3 | 26.5 |
| YOLOv7 |  | 85.4 | 81.8 | 85.2 | 21.2 |
| YOLOv8n |  | 87.7 | 84.2 | 87.8 | 6.01 |

### 3.3.2 Compared of ablation experiments

In this paper, the YOLOv8n model is improved, and the results of each improved model are statistically analyzed. The basic model is integrated with different attention mechanism modules, and the optimal model improvement method is selected through experiments.

Table 3 Comparative ablation experiments

| Experiments | ScConv | CA | GAM | BiFPN | P(%) | R(%) | mAP(%) | Model Size(MB) | Patameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No.1 |  |  |  |  | 87.7 | 84.2 | 87.8 | 6.01 | 3005843 |
| No.2 | √ |  |  |  | 90.1 | 86.5 | 89.7 | 5.62 | 2813315 |
| No.3 |  | √ |  |  | 88.7 | 86.0 | 88.6 | 6.11 | 3017563 |
| No.4 |  |  | √ |  | 90.5 | 87.0 | 90.9 | 9.5 | 4645523 |
| No.5 |  |  |  | √ | 90.2 | 87.4 | 91.2 | 4.71 | 2225620 |
| No.6 | √ |  |  | √ | 96.3 | 94.7 | 97.7 | 4.28 | 2033092 |

To evaluate the detection performance and model efficiency, metrics such as Precision(P), Recall(R), mAP, Model Size, and Parameters were employed, as presented in Table 3. Experiment 2 integrated the ScConv module into the original Backbone network, reducing the model parameter count by 1.87×105 while increasing mAP by 1.9%. This indicates that the simple structure of the ScConv module does not compromise feature extraction and balances network computational load and detection performance.

Experiment 3 uses the CA attention module [23] to replace the C2f module of Experiment 1; Experiment 4 uses the GAM module [24] to replace the C2f module of Experiment 1. Results showed that Experiment 3 improvements in P, R, and mAP by 1.0%, 1.8%, and 0.8%; and Experiment 4 improved in P, R, and mAP by 2.8%, 2.8%, and 3.1%. However, in terms of the improved model size and the number of parameters, Experiment 3 increases by 0.1MB and 1.17×104, and Experiment 4 increases by 3.49MB and 1.64×106.

Experiment 5 enhanced the Neck network of Experiment 1 by adopting the BiFPN architecture and introducing the P2 small object detection layer, which increased feature learning channels and enabled parameter sharing during training. This led to improvements in P, R, and mAP by 2.5%, 3.2%, and 3.4%, respectively, with a reduction of 7.8×105 parameters.

Based on comparisons from Experiments 2-4, Experiment 6 synchronized improvements from Experiments 2 and 5 to enhance both the Backbone and Neck sections. This strengthened the network’s learning of shallow features, enhancing the detection of small objects and achieving model lightweight by removing redundancy and reducing model parameters. Experiment 6 achieved a P of 96.3%, an R of 94.7%, and increased mAP by 9.9%, with reductions in model size and parameter count by 1.7MB and 0.973×106 respectively. The efficacy of improvements using the SCConv module and BiFPN network architecture was illustrated through ablation studies comparing the original and enhanced models, depicted in Figure 10.

| Figure 10 Ablation experiment diagram |  |
| --- | --- |

## 3.4 Visualization of test results

Figure 11 depicts diagrams illustrating the effect of the original and enhanced models after training under identical experimental conditions, focusing on the density distribution of steel frame bolts in a high-altitude outdoor environment with strong light irradiation.

Figure 11(a) presents the detection image using the base YOLOv8 model. Figure 11(b) exhibits the improved model’s detection graph with the BiFPN network architecture integrated, demonstrating enhanced detection accuracy and reducing the likelihood of misidentification, particularly in detecting nuts compared to the original model. Figure 11(c) incorporates the ScConv module, which contributes to a reduction in misidentifications and omissions, albeit with less pronounced effects. Figure 11(d) showcases the detection improvement achieved by the algorithm proposed in this paper, highlighting substantial enhancements in addressing misidentifications and missed detections.

According to Figure 12, although the model represented in this paper exhibits improvements in detecting small target bolts, it still encounters challenges in detecting very small bolts, indicating some issues with missed detections. This is attributed to insufficient sample data of small-sized bolts during dataset collection, underscoring the need for gathering more data on small-sized bolts in future endeavors to address practical challenges more effectively.

| (a) YOLOv8 | (b)YOLOv8_BiFPN |
| --- | --- |
| (c)YOLOv8_ScConv | (d) Ours |
| Figure 11 Comparison of detection results 1 |  |
| (a) YOLOv8 | (b)YOLOv8_BiFPN |
| (c)YOLOv8_ScConv | (d)Ours |
| Figure 11 Comparison of detection results 2 |  |

# 4 Conclusion

This paper presents a novel lightweight bolt detection algorithm based on YOLOv8. The proposed method integrates the SCConv module from CVPR2023 into the feature extraction network, employing a spatial reconstruction unit that utilizes a separation-reconstruction strategy to mitigate spatial redundancy. Additionally, the channel reconstruction unit applies a segmentation-transformation-fusion strategy, effectively minimizing channel redundancy, thereby facilitating the lightweight nature of the model. To addresse the challenges of dense bolt distribution and small target sizes on steel frames, the network structure of the model's Neck connection layer has been refined to incorporate a BiFPN architecture. Furthermore, the introduction of a P2 small target detection layer has significantly enhanced the detection capabilities for densely packed bolts.

The experimental results indicate that the proposed algorithm achieved a Precision (P) of 96.3% and a Recall (R) of 94.7% on a self-collected dataset of steel frame bolts. Furthermore, the mAP increased by 9.9%. The model size and the number of parameters were reduced by 28.3% and 0.973×106, respectively. These improvements demonstrate that the enhanced algorithm not only increases detection accuracy but also reduces the computational complexity, making the model more lightweight compared to the original version. Compared with other models, the algorithm presented in this paper performs effectively on bolts on steel frames affected by lighting in outdoor high-altitude environments, making it more suitable for deployment on small devices.

AUTHOR CONTRIBUTIONS

Guoxiang Hua: Writing-original draft; writing-review and editing; funding acquisition; resources. Huai Zhang: Writing-original draft; writing-review and editing. Chen Huang: Data curation; software. Moji Pan: Investigation; validation. Jiyuan Yan: Project administration; supervision; resources. Haisen Zhao: Conceptualization; methodology; supervision.

ACKNOWLEDGEMENTS

This work was supported by the National Key Laboratory of New Energy Power System (LAPS23014).

CONFLICT OF INTEREST STATEMENT

The authors declare no conflicts of interest.

DATA AVAILABILITY STATEMENT

The data that support the findings of this study are available from the corresponding author upon reasonable request.

REFERENCES:

[1]Bianchi A R ,Ferraz F H ,Gonçalves S R , et al.A synthetic high-voltage power line insulator images dataset[J].Data in Brief,2024,55110688-110688.

[2]Amiri H M ,Pourgholi M ,Hashjin M N , et al.Monitoring UAV status and detecting insulator faults in transmission lines with a new classifier based on aggregation votes between neural networks by interval type-2 TSK fuzzy system[J].Soft Computing,2024,(prepublish):1-34.

[3]Ahsan M ,Baharom R N M ,Zainal Z , et al.Measuring and simulation of magnetic field generated by high voltage overhead transmission lines[J].Results in Engineering,2024,23102688-102688.

[4]Ma M ,Wang C ,Li Y , et al.Intelligent acceptance check for towers of overhead transmission line based on point clouds[J].IET Generation, Transmission & Distribution,2023,17(22):5074-5089.

[5]Bocheng L ,Wen L ,Liang Z , et al.Ranging Algorithm of UAV Inspection of Transmission Line Based on Monocular Vision and SURF Method[J].Journal of Physics: Conference Series,2023,2452(1):

[6]Wang Q ,Li Y ,Cui S , et al.Enhanced recognition of insulator defects on power transmission lines via proposal-based detection model with integrated improvement methods[J].Engineering Applications of Artificial Intelligence,2024,136(PB):109078-109078.

[7]Dong N ,Zhang W ,Chen Z , et al.Bird's nest defect detection of transmission lines based on domain knowledge and occlusion reasoning[J].IET Generation, Transmission & Distribution,2023,17(22):4946-4957.

[8]Stefenon F S ,Singh G ,Souza J B , et al.Optimized hybrid YOLOu‐Quasi‐ProtoPNet for insulators classification[J].IET Generation, Transmission & Distribution,2023,17(15):3501-3511.

[9] Stefenon S , Corso M P , Nied A ,et al.Classification of insulators using neural network based on computer vision[J].IET Generation, Transmission & Distribution, 2021.DOI:10.1049/gtd2.12353.

[10]Miroslav R ,Elif G ,Jaroslav H , et al.New pruning tests for the branch-and-prune framework for interval parametric linear systems[J].Soft Computing,2022,27(18):12897-12912.

[11]Yang J ,Zhu X ,Bulat A , et al.Knowledge Distillation Meets Open-Set Semi-supervised Learning[J].International Journal of Computer Vision,2024,(prepublish):1-20.

[12]Wang X ,Wang Y ,Pan Z , et al.Unsupervised anomaly detection and localization via bidirectional knowledge distillation[J].Neural Computing and Applications,2024,(prepublish):1-16.

[13]Liu L ,Huang K ,Bai Y , et al.Real-time detection model of electrical work safety belt based on lightweight improved YOLOv5[J].Journal of Real-Time Image Processing,2024,21(4):151-151.

[14] Cai Y ,He M ,Tao Q , et al.Fast Rail Fastener Screw Detection for Vision-Based Fastener Screw Maintenance Robot Using Deep Learning[J].Applied Sciences,2024,14(9):

[15] Reis D, Kupec J, Hong J, et al. Real-Time Flying Object Detection with YOLOv8[J]. ar Xiv:2305. 09972v1, 2023.

[16] Li, Jiafeng, Ying Wen, and Lianghua He. "Scconv: spatial and channel reconstruction convolution for feature redundancy." Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2023.

[17] Li, Xue, Huibo Zhou, and Ming Zhao. "Transformer-based cascade networks with spatial and channel reconstruction convolution for deepfake detection." Mathematical Biosciences and Engineering 21.3 (2024): 4142-4164.

[18] Dechen Y ,Boyang L ,Hengchang L , et al.Remaining useful life prediction of roller bearings based on improved 1D-CNN and simple recurrent unit[J].Measurement,2021,175

[19] Chen, Jun, et al. "Effective feature fusion network in BIFPN for small object detection." 2021 IEEE international conference on image processing (ICIP). IEEE, 2021.

[20] Zha, Wenting, et al. "ENGD-BiFPN: A remote sensing object detection model based on grouped deformable convolution for power transmission towers." Multimedia Tools and Applications 82.29 (2023): 45585-45604.

[21] Ghiasi, Golnaz, Tsung-Yi Lin, and Quoc V. Le. "Nas-fpn: Learning scalable feature pyramid architecture for object detection." Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2019.

[22] XU Jinfang, LUO Xiaolong, JlANG Weidong et al. Extracting accurate building outlines from 3D point clouds considering local features [J]. Bulletin of Surveying and Mapping, 2023(08):51-56.

[23] Shen, Lingyun, Baihe Lang, and Zhengxun Song."CA-YOLO: Model optimization for remote sensing image object detection." IEEE Access (2023).

[24] Ll Shenghui, LlXiaofei, SONG Zhanghan et al. Multi-scale SAR lmage Detection Algorithm for Ships Based on lmproved YOLOv5 [J]. Journal of Data Acquisition and Processing, 2024,39(01):120-131.