Adaptive focusing multi-scale feature network for pinning defect detection in transmission lines

Guoxiang Hua1,3    Moji Pan2    Shuzhe Yin2    Jiyuan Yan3,*    Yehcheng Chen4    Haisen Zhao1

1 State Key Laboratory of Alternate Electrical Power System with Renewable Energy Sources, North China Electric Power University, Beijing, China

2 School of Automation, Nanjing University of Information Science and Technology, Nanjing, China

3 School of Automation, Wuxi University, Wuxi, China

4 Department of computer science, University of California Davis, CA, USA

Correspondence

Jiyuan Yan.

Email: wuxi_yjy@cwxu.edu.cn

*These authors contributed equally to this work.

Funding information

Natural Science Foundation, Grant/Award Number: 521770410.

Abstract

With the development of smart grid, transmission line UAV intelligent inspection technology has been widely used. Pin defect detection is a common task in the intelligent inspection process, but due to the small size of the pin bolts in the inspection image, it is difficult for the existing detection algorithms to accurately recognize the pin defects in the complex background. In this paper, an Adaptive Focusing Multi-scale Feature Network (AFMFNet) is proposed. First, the Path-Interleaved Deformation Convolution (PIDC) is proposed to further enhance the feature extraction ability for the irregular pose of pin bolt. Second, the Small Target Enhanced Pyramid (STEP) is constructed. It realizes the effective fusion of multi-scale features of small targets through the differentiated processing between different layers and the global perception capability granted by CSP_OmniKernel. Finally, the improved Wise-MPDIoU loss function is utilized to improve the convergence speed and regression accuracy of the model. AFMFNet enhances detection accuracy for normal and defective pins by 7.5% and 13.4% respectively compared to the baseline model, achieving a reasoning speed of 141.2 f/s on PC, meeting real-time detection needs. Its robustness is verified in complex scenarios, offering a new intelligent approach for transmission line inspection.

- INTRODUCTION

As a key fastening element of transmission lines, pin bolts are used to connect various components securely. However, most pin bolts are exposed for a long time and prone to missing pins, leading to power system failure. It is necessary to carry out regular inspection and maintenance of transmission lines to ensure the safe and stable operation of the power grid. With the development of UAV technology, intelligent inspection has gradually become the mainstream way of transmission line inspection[1], [2] because of its high security and low cost, and efficient and accurate detection and analysis of these images has become the key to intelligent inspection.

Currently, defect detection methods applied to inspection images can be broadly categorized into two types: digital image processing and deep learning. Before the rise of deep learning, the defect detection of key components of transmission lines was mainly been realized by traditional image processing methods, using wavelet transform, image segmentation, and other techniques for identification and localization. However, there are many limitations in practice, poor recognition of small targets in complex scenes, and the detection speed can’t meet real-time [3].

In recent years, with the development of deep learning in the field of target detection, its technical frameworks can be broadly categorized into two groups: one is the two-stage detection algorithms based on region suggestion, such as Fast R-CNN [4],[5], Mask-RCNN [6], and so on. And the other is the regression-based one-stage detection algorithms, such as SSD [7], YOLO [8], etc. As these techniques continue to mature, researchers have begun to apply them to solve the field of transmission line defect detection [9], [10], [11].

Tao [12] et al. proposed an insulator localization and defect detection technique based on cascaded CNN. And data enhancement was utilized to alleviate the problem of insufficient defect samples and obtain high accuracy detection results. However, the generalization ability of their method in complex scenarios still needs to be verified. Gou [13]et al, on the other hand, proposed an improved Mask-RCNN model. It enhances the recognition rate and segmentation accuracy of the algorithm for insulator defects in complex environments by introducing a convolutional attention module, using global intersection-parallel ratio and Tversky loss calculation. However, the model size and detection speed are yet to be optimized. However, compared with larger detection targets such as insulator defects, pin bolts are more difficult to detect due to their smaller size. Moreover, changes in the aerial photography angle and installation position will directly cause the pin bolt defects to show a variety of deformations. The complexity of the shooting environment and scene further aggravates the difficulty of pin bolt defect detection.

To solve the problem of accurate recognition of small targets such as pin defects, Li [14] et al targeted the design of PinFPN feature fusion structure for small targets on top of Fast R-CNN model, which makes the recognition accuracy of pin defects improved, but the overall parameter amount of the model is too large. Zhao [15] et al. proposed an Automatic Visual Shape Clustering Method. This method focuses on the extraction of small target pin features to improve the detection accuracy. However, it ignores the effect of multiple complex environments on the model. Xiao [16] et al. proposed a cascaded CNN detection method. The method takes into account the small target of pin defects and the complexity of the background problem, but lacks the sacrifice of detection speed to improve the accuracy of identification. Luo [17] et al. proposed a UBDDM model based on ResNet-50, which improves the network's ability to extract features of small targets of pins by introducing a hybrid attention mechanism and multi-scale feature fusion. However, the two-stage algorithm is difficult to meet the demand of real-time detection.

Aiming at the above problems and combining the features of aerial pin-bolt images and the lightweight advantage of the YOLO series of algorithms, AFMFNet is proposed. The main contributions of this paper are as follows:

(1) In the feature extraction stage, Path-Interlaced Deformable Convolution (PIDC) is proposed. It is obtained by reconstructing DCNv2 through the Path-Interlaced Coordinate Attention (PICA) mechanism. And a new feature extraction module C2f_PIDC is constructed by utilizing PIDC to more flexibly and adaptively extract features of irregular targets.

(2) In the feature fusion stage, a Small Target Enhance Pyramid is proposed for small target detection tasks. It subverts the traditional way of adding a P2 detection layer and fuses multi-scale feature information from global to local across stages by CSP_ OmniKernel.

(3) To improve the convergence speed and regression accuracy of the model, the Wise-MPDIoU loss function is used instead of the CIoU loss function of the original model.

- RELATED WORK

YOLOv8, as a collector of the YOLO series, introduces a series of innovations based on previous algorithms to enhance model performance. The algorithm not only has the advantages of lightweight and high-speed computing, but can also be deployed on a variety of hardware platforms, making it extremely suitable for transmission line defect inspection scenarios.

Due to the characteristics of lower resolution, more noise, and the small number of defective pin samples in aerial images, a complex network will lead to overfitting, so YOLOv8n is chosen as the baseline model in this paper. The YOLOv8n algorithm mainly consists of Backbone, Neck and Head. Backbone is improved from Darknet-53 and is mainly responsible for feature extraction, including standard convolution, C2f module and SPPF module. The Neck component primarily handles the integration of feature data extracted by the backbone network, constructs the feature pyramid, and facilitates cross-layer connections via the PAFPN architecture. The Head section employs a Decoupled-Head design, distinguishing between classification and regression tasks, and determines positive and negative samples based on weighted classification and regression scores.

- METHODOLOGY

Despite the excellent performance of YOLOv8n, there is a large difference in its detection effect in the complex shooting environment of transmission lines, especially for irregular small-target defect targets, the detection accuracy still needs to be improved. Therefore, the AFMFNet model is proposed.

FIGURE 1. Structure of AFMFNet

AFMFNet firstly proposes C2f_PIDC in the Backbone phase for irregular detection targets. Specifically, DCNv2 is reconstructed using the Path-Interlaced Coordinate Attention (PICA) mechanism, and the reconstructed convolution PIDC replaces the traditional convolution in C2f. Secondly, a Small Target Enhancement Pyramid is proposed for the small target detection task. Finally, the MPDIoU loss function is optimized through the idea of Wise-IoU and replaces the original CIoU loss function. The network structure is shown in Fig. 1, in which the improvement modules are marked with different colors respectively.

  - Path-Interlaced Deformation Convolution

Aiming at the transmission line pinning images with the problems of irregular shooting angle and irregular defect features. The traditional feature extraction C2f module realizes rich feature interaction and transformation through segmentation and multi-path design. However, it relies more on fixed receptive fields for feature capture, and this fixed structure of feature extraction is very unsatisfactory for irregular targets with variable angles. To solve the appealing problem, we propose Path-Interlaced Deformation Convolution (PIDC), which reconfigures DCNv2 by Path-Interlaced Coordinate Attention (PICA) to dynamically adjust the receptive field for adaptive extraction of irregular features.

3.1.1	Deformable Convolution Networks v2

By adding learnable offsets and modulation scalars to each convolutional sample point, DCNv2 [18] enables the convolutional kernel to adaptively adjust to the target region, compute the true sampling locations and assign weights to the true sampling locations. DCNv2 employs sparse sampling offsets to localize the regions with irregular defect information and accurately captures these defective regions while ensuring that the effective information can be precisely extracted through modulation scalars. Compared with standard convolutional operations, DCNv2 not only detects the full range of irregular defect features, but also significantly improves computational efficiency through sparse sampling, weight sharing and modulation strategies.

Let the input feature be x, the output feature be y, and the output position be p0. Take 3×3 convolution for example for each output y(p0), it needs to be sampled sequentially at the grid positions where K is 9. These 9 positions are obtained by uniformly diffusing the center point x(p0) in all directions, where the diffused positions are denoted as . The formulas for the traditional convolutional CNN and DCNv2 are given below:

|  | (1) |
| --- | --- |
|  | (2) |

Where wk and pk denote the weight and ore-specified offset for the k-th location, respectively. and are the learnable offset and modulation scalar for the k-th location, respectively.

3.1.2	Path-Interlaced Coordinate Attention

From the above, the key to DCNv2 lies in introducing learnable offsets and modulation scalars for each convolutional kernel to adapt to changes in the shape and position of the detected target. However, since the original offsets and modulation scalars of DCNv2 are generated by only one convolution operation of the original feature layer, the ability to generate offsets and modulation scalars at this time is relatively weak and the accuracy of the offsets is poor, thus affecting the performance of DCNv2 itself. To solve this problem, a new PICA mechanism is proposed based on Coordinate Attention [19], and the structure is shown in Fig. 2.

PICA performs a global pooling decomposition of the height and width directions of the input feature maps, and generates a pair of directionally aware feature maps by aggregating features along the two spatial directions. This significantly enhances the network's ability to accurately localize thearget location. Subsequently, they are spliced and constructed using a 3×1 convolutional transform to produce the spatial information feature map F1. To further refine the spatial features, F1 is split into width coefficients X and height coefficients Y in the spatial dimension by Split.

In particular, to enhance the channel information of the feature map, PICA incorporates a new weight assignment channel compared to Coordinate Attention. This channel performs global average pooling directly on the input features, capturing the feature map from a global perspective and generating a feature representation with a wide range of sensory fields. In addition, a new branch is created, which generates the spatial information feature map F2 by doing feature transformation using 1×1 convolution and Sigmoid function on the feature map F1. F2 is split into width-attention weight X_weight and height-attention weight Y_weight and multiplied by the feature coefficients X and Y correspondingly, which realizes the fine-tuning of the channel information and enhances the feature expression's relevance.

To avoid the isolation of the new channel feature information, PICA also fuses the global information channel with the spatial information feature map F2 with direction awareness, which promotes the deep complementarity of the information of different channels. PICA comprehensively considers the importance of the three dimensions of the feature map C, H, and W, which further strengthens the attention to the essential features and enhances the feature extraction capability of the network.

3.1.3	C2f_PIDC

To enhance the ability of DCNV2 to generate offsets and modulation scalars, the PICA mechanism is utilized to reconstruct DCNV2, and the new Convolution Networks-PIDC is shown in Fig. 3. PIDC performs a convolution operation on the input feature map, immediately followed by the PICA mechanism in order to obtain a bias domain with a channel number of 2N. Where the PICA mechanism enhances the generation of offsets and modulation scalars as well as the offset accuracy by extracting multi-channel feature information. Finally, PIDC utilizes Deformable Roi Pooling to convert each region of the input feature map into a fixed-size feature to extract key features and output the feature map.

| FIGURE 2. Structure of PICA attention mechanism | FIGURE 3. Schematic diagram of PIDC |
| --- | --- |

FIGURE 4. Structure of C2f_PIDC

We further utilize PIDC to reconstruct the feature extraction module C2f, as shown in Fig. 4, replacing all the standard convolutions in C2f with PIDC to enhance the feature extraction effect on irregular targets.

  - Small Target Enhance Pyramid

When dealing with small target detection tasks such as pin defects, the PAFPN in YOLO only considers the feature information of the C3, C4, and C5 layers and ignores the shallow detailed information. The more common strategy is to increase the P2 detection layer to improve the detection ability of small targets, but at the same time, it will make the overall computation of the model larger and reduce the inference speed. At the same time, PAFPN is more inclined to fuse the same scale feature maps, and lacks the integration of multi-scale feature information in different resolutions. To solve the above problems, the new feature pyramid Small Target Enhancement Pyramid (STEP), which is effective for small targets, is proposed, as shown in Fig. 5.

STEP is mainly designed for the utilization of shallow detail feature information and the interactive fusion between multi-scale feature information.

Detailed features: We use SPD-Conv [20] to downsample the C2 layer, extract detailed features rich in small target information, and interact features with other layers. This strategy not only circumvents the loss of detail information in the shallow feature layer during the traditional downsampling operation, but also skillfully avoids the need for adding new detection heads.

Multi-scale features: To address the need for fusion between multi-scale feature information, we propose CSP-OmniKernel. This module enhances the STEP multi-scale representation learning capability through the OmniKernel Module and utilizes the concept of CSPNet [21] to integrate multi-scale feature information across phases, in order to empower the network's global sensing capability.

FIGURE 5. Schematic diagram of Small Target Enhance Pyramid

3.2.1	SPD-Conv

Existing convolutional neural networks usually use convolution or max pooling with a step size of 2 for downsampling. However, due to the characteristics of low-resolution aerial images of transmission lines and small targets of pin bolts, this downsampling method leads to the loss of detailed information. Therefore, SPD-Conv is introduced to replace the original downsampling module when extracting small target information from the C2 feature layer.

SPD-Conv consists of a space-to-depth conversion layer and a 1×1 convolutional CBS module. When the feature map size is S×S×C1, SPD-Conv can specify the downsampling parameter scale to slice out a series of subfeature maps, as shown in Fig. 6. Downsampling using SPD-Conv can maximize the extraction of detailed information from pin small targets in the C2 feature layer.

FIGURE 6. Schematic diagram of SPD-Conv(scale=2) module

| (a) CSP_OmniKernel | (b) OmniKernel Moduel | (c) DCAM | (d) FSAM |
| --- | --- | --- | --- |
| FIGURE 7. Structure diagram of CSP_OmniKernel module |  |  |  |

2.2	CSP_OmniKernel

The CSP_OmniKernel provides the network with both global sensing capability and improved model computation performance through the cross-stage connectivity feature as well as multi-scale representation learning capability. The input of CSP_OmniKernel is divided into two parts according to the divided channel, one part is processed by the standard convolution and integrates the multi-scale features through the OmniKernel Module. The other part is used as the residual connection, as shown in Fig. 7(a).

One of them, the OmniKernel Module organizes Global Branch, Large Branch, and Local Branch in parallel in order to efficiently learn global-to-local feature representations, as shown in Fig. 7(b).

Large Branch: Its design aims to use deep convolutions with different shapes and large kernel sizes, to provide multi-grained receptive fields. In this branch, a deep convolution with low computational cost and k×k kernel size is introduced to maximize the sensory fields. Meanwhile, two deep convolutions are deployed in parallel with kernel sizes of 1×k and k×1, respectively, to capture richer contextual information. In order to determine the convolutional kernel size applicable to the model, we conducted experiments in STEP by gradually increasing the kernel size, and the experimental results are shown in Fig. 8.

As we increase the kernel size from 3 to 63, the GFLOPs and Parameters increase gradually, but the Precision and mAP50 metrics reach their best near K=31. To get better performance, we finally choose kernel size of 31 in the Local branch.

Global Branch: Since the input training image is 3×640×640, but the size of the defective pin is very small. In the inference phase, the input image is much larger than the size of the target to be inspected, and a 31×31 kernel cannot fully cover the global domain. Therefore, the global branch realizes the global perceptual field through a Dual-domain Channel Attention Module (DCAM) and a Frequency-based Spatial Attention Module (FSAM).

As shown in Fig. 7(c), DCAM first performs Frequency Channel Attention (FCA) to the input featuresas:

|  | (3) |
| --- | --- |

whereandare fast Fourier transformer and its inverse operation, respectively;represents the element-wise multiplication operation. With Fourier processing, the global features can be efficiently refined according to the spectral convolution theorem.

The features obtained from the FCA are fed into the SCA and the final DCAM is represented as:

|  | (4) |
| --- | --- |

However, DCAM can only enhance the coarse-grained dual-domain features of the channel. Therefore, the FSAM is applied in the spatial dimension to compensate for the shortcomings of fine-grained features, as shown in Fig. 7(d). The specific expression is as follows:

|  | (5) |
| --- | --- |

In summary, it is known that the global branch makes full use of the frequency domain information after the Fourier transformer to perceive the global content.

Local Branch: It mainly utilizes point-wise depth-wise convolution to supplement local information.

FIGURE 8. Experimental results for different convolutional kernel sizes

  - Optimization of loss function

The YOLOv8n algorithm uses the CIoU loss function [22] , to compute the coordinate regression loss, whose expression is shown in (6).

|  | (6) |
| --- | --- |

Where b and bgt represent the prediction frame and the background real frame, respectively;is the Euclidean distance between them; c is the diagonal length of the smallest outer rectangle, which can cover both the prediction frame and the real frame; and v are the evaluation parameters and the aspect ratio balance coefficients.

However, the CIoU loss function is vague in the definition of aspect ratio and ignores the positive and negative sample imbalance, which leads to a lower convergence speed of the model. Therefore, Wise-MPDIoU is used here to replace CIoU.

MPDIoU [23] improves the algorithm by directly minimizing the vertex distance between the predicted frame and the real frame, which detects and localizes small targets more accurately and shows better robustness. Its calculation formula is as follows:

|  | (7) |
| --- | --- |

Where  denotes the upper-left and lower-right corners of the predicted and real boxes, respectively; and w and h denote the width and height of the bounding box.

To improve the convergence speed of MPDIoU, the idea of Wise-IoU is borrowed to improve MPDIoU by weighting the region between the prediction frame and the real frame, and the Wise-MPDIoU loss function is constructed with the following formula:

|  | (8) |
| --- | --- |

Where bi denotes the coordinates of the i-th target frame, gi denotes the coordinates of the true labeled frame of the i-th target, and wi denotes the weight value.

- EXPERIMENTS RESULTS AND DISCUSSION

4.1	Datasets and Experimental setup

Bolts used in transmission lines can be categorized into two main types: bolts with pins and bolts without pins. Specifically, bolts with pins have pins that prevent the nut from falling off, as shown in Fig. 9 (b). It is worth noting that the phenomenon of pin dislodgement is the most common defective problem of this type of bolts. And a defective bolt with a dislodged pin is very similar to a normal bolt without a pin, as shown in Fig. 9 (a) and (c).

| (a) Bolts without pins |  | (b) Bolts with pins (normal) |  | (c) Bolts with pins (defective) |  |
| --- | --- | --- | --- | --- | --- |
| FIGURE 9. Presentation of three types of bolts |  |  |  |  |  |
| FIGURE 10. Complex environment sample example |  |  |  |

According to the conclusion obtained from the tests conducted in the literature [24] for different sample classifications: a balanced distribution of samples can improve the accuracy of the algorithm's detection, especially the defect recognition rate of fixed-point retake images is the highest. To circumvent the model overfitting that may result from the dataset, this paper deliberately avoids the scene categorization and similarity concentration of the sample labels when processing the dataset.

A total of 972 inspection images of different scenes under complex environment were collected by aerial photography from a drone. Some samples are shown in Fig 10. It was subsequently expanded to 2312 samples by adjusting the brightness, randomly flipping, cropping and splicing. The samples were randomly divided into training set (1618), validation set (231) and test set (463) according to the ratio of 7:1:2. The normal pins and defective pins in the samples were labeled using LabelImg, and 10064 normal pins were labeled as “1” and 2534 defective pins were labeled as “0”.

The hardware configuration of the experimental platform in this paper: CPU is Intel Cool i5-13600KF; GPU is NVIDIA 4060Ti (16G); RAM is 64G (6400MHz). The software environment of the experimental platform: the operating system is Win10; the driver environment is CUDA11.7.

The experimental code was improved based on version 8.2.50 of ultralytics, using the SGD gradient descent optimizer with Mosaic enhancement of the dataset. The rest of the hyperparameter settings are shown in Tab. 1.

TABLE 1. Hyper parameter configuration table

| Parameter name | Parameter value |
| --- | --- |
| Image Size | 640640 |
| Epoch | 550 |
| Batch Size | 16 |
| Learning Rate | 0.01 |
| Momentum | 0.937 |

4.2	Evaluation metrics

In order to accurately assess the detection performance of the model, the mean average precision (mAP), Parameters and the speed of inference (FPS) are chosen as the evaluation metrics in this experiment, and their formulas are as follows:

|  | (9) |
| --- | --- |
|  | (10) |
|  | (11) |

Where n denotes the number of categories, AP denotes the accuracy of each category; frameNum denotes the total number of detected images, elapsedTime denotes the time required for detection; TP and FP are true and false positive examples, and FN is a false negative example.

In the next experiments, the APs of 'Normal Pins' and 'Defective pins' were denoted as APN and APD, respectively. The IoU threshold of mAP was taken as 0.5.

4.3	Experimental comparison

4.3.1	PIDC validation experiment

In order to verify the effectiveness of the PIDC for feature extraction improvement, the traditional convolution was replaced with DCNv2 and PIDC in the original C2f module, respectively. The results of the experiments are shown in Tab. 2. Furthermore, to visualize features more intuitively, Fig. 11 analyzes the attention distribution changes in the model's neck with each C2f module improvement. Where Fig. 11 (a) is the original figure used for the experiment. And Fig. 11 (b), (c), (d) are the effect graphs of feature extraction performed by C2f, C2f_DCNv2, and C2f_PIDC modules, respectively.

From Tab. 2, the introduction of DCNv2 in the C2f module instead of the traditional convolution increases APN and APD by 1.3% and 3.2%, respectively, and improves mAP by 2.2%. On this basis, PIDC is proposed for further optimization of DCNv2. The detection accuracy of C2f_PIDC is again significantly improved compared to the C2f_DCNv2 module. The mAP is improved by 1.9, and the APN and APD are increased by 0.6% and 3.3%.

Fig. 11 (b) and (c) show that the C2f_DCNv2 module, compared to the original C2f module, enables the convolutional kernel to dynamically adjust the target region through the learnable offsets and modulation scalars, further covering the attention region over the target to be examined. Fig. 11 (c), (d) show that the incorporation of the PICA attention mechanism into DCNv2 significantly enhances the ability of DCNv2 to generate offsets and modulation scalars, allowing attention to be better focused on irregular defective pinning regions.

TABLE 2. Comparative experimental results of different C2f modules

|  | APN | APD | mAP | Params (M) | FPS (f/s) |
| --- | --- | --- | --- | --- | --- |
| C2f | 73.7 | 57.6 | 65.7 | 3.0 | 172.4 |
| C2f_DCNv2 | 75.0 | 60.8 | 67.9 | 3.0 | 149.0 |
| C2f_PIDC | 75.6 | 64.1 | 69.8 | 3.1 | 141.2 |

FIGURE 11. Heat map of attention distribution of C2f nodes. From left to right are the (a) original figure, (b) C2f, (c) C2f_DCNV2 and (d) C2f_PIDC.

4.3.2	Loss function comparison experiment

To assess the impact of various loss functions, we used CIoU in YOLOv8n as a baseline and compared it with other loss functions. Tab. 3 shows the results, while Fig. 12 plots the convergence curves for bounding box regression loss.

From the data in Tab. 3, the EIoU loss function does not achieve an overall improvement in detection accuracy. The WIoU loss function also has a limited effect on the improvement of detection accuracy, with the mAP rising by only 0.5%. In contrast, the MPDIoU and Wise-MPDIoU loss functions have a significant improvement effect on the detection accuracy, in which the Wise-MPDIoU loss function makes the mAP of the model go up by 2.4%. As for the two indicators, Parameters and FPS, there is no large fluctuation between the loss functions.

TABLE 3. Comparative experimental results of different loss functions

|  | APN | APD | mAP | Params (M) | FPS (f/s) |
| --- | --- | --- | --- | --- | --- |
| CIoU | 73.7 | 57.6 | 65.7 | 3.0 | 172.4 |
| EIoU [26] | 74.3 | 57.3 | 65.8 | 3.0 | 172.3 |
| WIoU [25] | 74.4 | 58.0 | 66.2 | 3.0 | 174.1 |
| MPDIoU | 76.6 | 58.3 | 67.4 | 3.0 | 167.3 |
| Wise-MPDIoU | 74.7 | 61.6 | 68.1 | 3.0 | 169.7 |

FIGURE 12. Comparison of boundary box regression loss convergence

As shown in Fig. 12, Wise-MPDIoU loss function converges faster with the lowest loss, enhancing edge regression accuracy while balancing detection speed and accuracy. In summary, Wise-MPDIoU suits improving the original model.

4.3.3	Mainstream algorithm comparison experiment

To verify the effectiveness of the algorithm improvement in this paper, the popular target detection models of CNN and Transformer frameworks in recent years , and the newer improved algorithms SPD (2022)[20], Gold-YOLO (2023)[27] and Hyper-YOLO (2024)[28] are compared, and the experimental results are shown in Tab 4.

TABLE 4. Comparative Experimental Results of Different Loss Functions

| Module | APN | APD | mAP | Params (M) | FPS (f/s) |
| --- | --- | --- | --- | --- | --- |
| ResNet50 | 58.6 | 44.7 | 51.7 | 25.6 | 74.5 |
| VGG-16 | 72.5 | 53.9 | 63.2 | 138.3 | 28.5 |
| ViT-B | 74.9 | 67.3 | 71.1 | 85.8 | 52.2 |
| YOLOv5s | 67.3 | 55.2 | 61.3 | 7.2 | 150.4 |
| YOLOv8n | 73.7 | 57.6 | 65.7 | 3.0 | 172.4 |
| YOLOv8s | 75.7 | 63.8 | 69.8 | 11.2 | 142.0 |
| SPD | 74.9 | 66.3 | 70.6 | 4.7 | 122.7 |
| Gold-YOLO | 74.3 | 62.9 | 68.6 | 6.0 | 139.5 |
| Hyper-YOLO | 75.8 | 58.1 | 67.0 | 3.9 | 123.5 |
| AFMFNet | 81.2 | 71.0 | 76.1 | 3.4 | 141.2 |

Comparative experimental results show that the two-stage detection algorithm based on ResNet50 and VGG-16 architectures is much lower than the single-stage YOLO series of detection algorithms in terms of comprehensive performance metrics. It is worth noting that although the ViT model exhibits excellent detection accuracy (71.1%), the amount of its model parameters is too large. In contrast, the AFMFNet proposed in this paper achieves 76.1% mAP, a 5 percentage point improvement over the ViT model, while maintaining the lightweight advantage. Compared with the popular improved algorithms SPD, Gold-YOLO and Hyper-YOLO, the mAP of AFMFNet is also improved by 5.5%, 7.5% and 9.1%, respectively.

In addition, AFMFNet has a smaller number of parameters compared to other network models, which fully satisfies the requirement of deployment on embedded devices with limited arithmetic power. Meanwhile, the FPS on the PC side is 141.2 f/s, which achieves a better balance between defect detection accuracy and speed.

4.4	Ablation experiment

To validate the rationality of each enhancement strategy, this paper employs YOLOv8n as the baseline and conducts step-by-step ablation experiments. And predictions are made using the test set to quantitatively analyze the performance effects of different improvement strategies, and the experimental results are shown in Tab. 5.

4.4.1	Effectiveness of PIDC

Comparing Experiments 1 and 3 in Tab. 5, it can be found that the performance of the model is significantly improved after replacing the C2f module with the C2f_PIDC module in the C3, C4, and C5 parts of the Backbone. The APN and APD are improved by 1.9% and 6.5%, respectively, and the mAP is improved by 4.1%. Further data from Experiments 4 and 5 show that the overall detection accuracy is again improved with the introduction of the C2f_PIDC module on top of STEP, with a significant increase in APD of 4.8%.

Through the above experiments, it is effectively verified that the improved C2f_PIDC module is able to efficiently extract the feature information of irregular targets and reduce the computation amount by virtue of its sparse sampling strategy, weight-sharing mechanism, and multi-path information interaction. Thus, the overall performance of the model is improved.

4.4.2	Effectiveness of STEP

Comparing Experiments 1 and 4 in Tab. 5, it can be found that STEP significantly improves the detection accuracy of the model for small target targets compared to the original PAFPN without significantly increasing the model complexity. Among them, APN and APD were improved by 5.4% and 6.9%, respectively, and mAP was improved by 6.1%. The above experiments verify that STEP can effectively improve the detection performance of the model by optimizing the processing of shallow feature information and the fusion strategy of multi-scale information in the small target detection task.

4.4.3	Effectiveness of Wise-MPDIoU

Upon comparing Experiments 1 and 2, as well as Experiments 5 and 6 in Tab. 5, it is evident that the Wise-MPDIoU loss function significantly enhances the model's capability to precisely identify and locate overlapping targets. This improvement is achieved by optimizing the quality of the predicted bounding box without any increase in model complexity, ultimately elevating the mAP to 76.1%. It also improves the inference speed of the model by weighting the region between the predicted and real boxes.

To summarize, the improved algorithm improves APN and APD by 7.5% and 13.4%, and mAP by 10.4%, respectively, compared with baseline, under the premise of a slight increase in Parameters. The inference speed of the model on the PC side is 141.2 f/s. It meets the needs of deployment and real-time detection and achieves a good balance between speed and accuracy, further validating the effectiveness of the improved algorithm.

TABLE 5. Comparative Experimental Results of Different Loss Functions

| Experiment | Wise-MPDIoU | PIDC | STEP | APN | APD | mAP | Params (M) | FPS (f/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | 73.7 | 57.6 | 65.7 | 3.0 | 172.4 |
| 2 | √ |  |  | 74.7 | 62.6 | 68.6 | 3.0 | 169.7 |
| 3 |  | √ |  | 75.6 | 64.1 | 69.8 | 3.1 | 141.2 |
| 4 |  |  | √ | 79.1 | 64.5 | 71.8 | 3.3 | 148.1 |
| 5 |  | √ | √ | 80.5 | 69.3 | 74.9 | 3.4 | 138.8 |
| 6 | √ | √ | √ | 81.2 | 71.0 | 76.1 | 3.4 | 141.2 |

4.5	Improved visualization of effects

To further validate the enhancement in irregular defect feature detection capability offered by our proposed improvement, we employ feature map visualization to illustrate the focal points of the model's neck network's attention across various improvement strategies. The results are presented in Fig. 13. Among them, the darker the color means the model is more interested in the region, and it is the main basis for evaluating the model's defect feature detection ability.

Fig. 13 shows that Exp4 can capture small target information more accurately compared to Exp1 by adopting STEP on the basis of baseline. Exp5 replaces C2f_PIDC on the basis of Exp4, which significantly enhances the anti-interference ability of the algorithm to better focus on the pin-bolt region, and once again verifies that the C2f_PIDC module adaptively extracts irregular small target feature information. Exp6 replaces the CIoU loss function with the Wise-MPDIoU loss function, so that the algorithm can accurately depict the region of interest of the target features.

| Original figure |  |  |  |
| --- | --- | --- | --- |
| Exp1 |  |  |  |
| Exp6 |  |  |  |
| Exp5 |  |  |  |
| Exp6 |  |  |  |
| Fig. 13. Heat maps of neck output during improvement. |  |  |  |

|  | YOLOv8n | AFMFNet |
| --- | --- | --- |
| Smoggy environment |  |  |
| Complex background |  |  |
| Pin defect angle shift |  |  |
| Defect masking |  |  |
| Low light |  |  |
| Fig. 14. Comparison of detection effect in multiple scenes. |  |  |

4.6	Detection results in complex environments

To validate the robustness of the AFMFNET algorithm, challenging test scenarios such as smoggy environments, low-light environments, complex backgrounds, angular shift of pin defects, and pin defect holes being obscured are selected as detection samples. And by comparing the performance of AFMFNet and YOLOv8n algorithms on these samples, utility analysis is carried out. The detection results are shown in Fig. 14.

As can be seen in Fig. 14, the detection performance of YOLOv8n is far inferior to that of AFMFNet in complex environments and backgrounds. Specifically, YOLOv8n fails to detect defective pins under the complex environment of smog, low light, and strong noise, while AFMFNet shows excellent robustness. For the angular shift and mask problems of defective pin holes, YOLOv8n showed wrong and missed detections. In contrast, AFMFNet successfully detected all of them.

Through the above detection experiments, the overall detection performance of AFMFNet is significantly better than that of the benchmark model, and the robustness of the improved algorithm in diversified and complex scenarios is fully verified.

- CONCLUSIONS

To improve the identification and localization of transmission line pinning defects and hazards, overcome the performance bottleneck of existing intelligent detection algorithms in identifying small targets, and satisfy the model's need for generalization to diverse and complex scenarios, the AFMFNet algorithm is proposed, with the following specific innovations:

(1) To improve the feature extraction capability for irregular pin defective bolts, an enhanced version of PIDC with adaptive dynamic adjustment of sensory field is designed.

(2) Due to the increasing surge in the demand for small target detection, a new feature pyramid network SOEP is developed to address the drawbacks of the original PAFPN.

(3) The MPDIoU loss function is improved by borrowing the idea of Wise-IoU to replace the CIoU loss function to improve the convergence speed and regression accuracy of the model.

Experimental results demonstrate that AFMFNet enhances detection accuracy for normal and defective pins by 7.5% and 13.4%, respectively, and boosts mAP by 10.4% compared to the baseline. With 3.4M parameters and an inference speed of 141.2 f/s, it meets deployment and real-time detection requirements. Furthermore, AFMFNet outperforms other algorithms in complex backgrounds, defect masking, and angle shifts due to shooting. Future efforts will focus on expanding defect categories and refining detection accuracy for all-weather real-world applications in transmission line maintenance.

ACKNOWLEDGMENTS

This work was supported by the National Natural Science Foundation of China under Grant, Grant Number: 521770410.

CONFLICT OF INTEREST

The authors declare no potential conflict of interests.

ORCID

Moji Pan: https://orcid.org/0009-0005-7576-427X

Haisen Zhao: https://orcid.org/0000-0003-3178-2490

REFERENCES

Duo, Chunhong, et al. "UAV‐aided distribution line inspection using double‐layer offloading mechanism." IET Generation, Transmission & Distribution 18.13 (2024): 2353-2372.

Yan, Y., et al.: Application status and development trends for intelligent perception of distribution network. High Volt. 6(6), 938–954(2021).

Zhao, H., et al.: Autonomous live working robot navigation with real-time detection and motion planning system on distribution line. High Volt. 7(6), 1204–1216 (2022).

Ren, K. He, R. Girshick and J. Sun, "Faster R-CNN: Towards Real-Time Target Detection with Region Proposal Networks," IEEE Trans. Pattern Anal. Mach. Intell., vol. 39, no. 6, pp. 1137-1149, Jun. 2017.

Ji, Hongxin, et al. "Visual inspection for transformer insulation defects by a patrol robot fish based on deep learning." IET Science, Measurement & Technology 15.7 (2021): 606-618.

K. He, G. Gkioxari, P. Dollár, et al., "Mask R-CNN," 2017, arXiv:1703.06870.

Liu Wei, ANGUELOV D, et al., "SSD：single shot multibox detector," in Computer Vision–ECCV 2016: 14th European Conference, Amsterdam, vol. 9905 , pp. 21-37, Sep. 2016.

REDMON J, DIVVALA S, GIRSHICK R, et al., " You only look once: unified, real-time target detection," 2016, arXiv: 1506.02640.

Liu, Yue, et al. "Insulator defect detection with deep learning: A survey," IET Generation, Transmission & Distribution 17.16 (2023): 3541-3558.

Dong, Chao, et al. "An improved cascade RCNN detection method for key components and defects of transmission lines." IET generation, transmission & distribution 17.19 (2023): 4277-4292.

G. Hua, et al. "An enhanced YOLOv8‐based bolt detection algorithm for transmission line." IET Generation, Transmission & Distribution 18.24 (2024): 4065-4077.

X. Tao, D. Zhang, Z. Wang, X. Liu, H. Zhang and D. Xu, "Detection of Power Line Insulator Defects Using Aerial Images Analyzed With Convolutional Neural Networks," in IEEE Transactions on Systems, Man, and Cybernetics: Systems, vol. 50, no. 4, pp. 1486-1498, April 2020.

J. Gou, S. Du, L. Liu, "Self-explosion detection of transmission line insulators based on improved mask region convolutional neural network," Transactions of China Electrotechnical Society, vol. 38, no. 1, pp. 47-59, Jan. 2019.

X. Li, H. Liu, G. Liu, et al. "Detection of pinning defects in transmission lines based on deep learning," Power System Technology, 2021, pp. 2988-2995.

Z. Zhao, H. Qi, Y. Qi, K. Zhang, Y. Zhai and W. Zhao, "Detection Method Based on Automatic Visual Shape Clustering for Pin-Missing Defect in Transmission Lines," IEEE Trans. Instrum. Meas., vol. 69, no. 9, pp. 6080-6091, Sept. 2020.

Y. Xiao, Z. Li, D. Zhang and L. Teng, "Detection of Pin Defects in Aerial Images Based on Cascaded Convolutional Neural Network,"  IEEE Access, vol. 9, pp. 73071-73082, 2021.

P. Luo, B. Wang, H. Wang, F. Ma, H. Ma and L. Wang, "An Ultrasmall Bolt Defect Detection Method for Transmission Line Inspection," IEEE Trans. Instrum. Meas., vol. 72, pp. 1-12, Feb. 2023, Art no. 5006512.

X. Zhu, H. Hu, S. Lin, et al., "Deformable convnets v2: More deformable, better results," Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit., pp. 9308-9316, 2019.

Q. Hou, D. Zhou, J. Feng, "Coordinate attention for efficient mobile network design," Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit., pp. 13713-13722, 2021.

Raja Sunkara and Tie Luo, "No more strided convolutions or pooling: A new CNN building block for low-resolution images and small targets," 2022, arXiv: 2208.03641.

Wang, Chien-Yao, et al. "CSPNet: A new backbone that can enhance learning capability of CNN," Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit., pp. 390-391, 2020.

X. Wang and J. Song, "ICIoU: Improved Loss Based on Complete Intersection Over Union for Bounding Box Regression," IEEE Access, vol. 9, pp. 105686-105695, Aug. 2021.

S. Ma, Y. Xu, "Mpdiou: a loss for efficient and accurate bounding box regression," 2023, arXiv:2307.07662.

J. Fu, et al., "Analysis of intelligent recognition method for defects in transmission line UAV inspection images," High Voltage Engineering, vol. 49, pp. 103-110, Sep. 2023.

Z. Tong, Y. Chen, Z. Xu, et al., "Wise-IoU: bounding box regression loss with dynamic focusing mechanism," 2023, arXiv:2301.10051.

Y.-F. Zhang, W. Ren, Z. Zhang, Z. Jia, L. Wang and T. Tan, "Focal and efficient IOU loss for accurate bounding box regression," Neurocomputing, vol. 506, pp. 146-157, Sep. 2022.

C. Wang, W. He, Y. Nie, et al., "Gold-YOLO: Efficient target detector via gather-and-distribute mechanism," 2023, arXiv:2309.11331.

Feng, Yifan, et al. "Hyper-yolo: When visual object detection meets hypergraph computation," IEEE Transactions on Pattern Analysis and Machine Intelligence (2024).