Urban vegetation effects on different urban forms to mitigate heat stress, thermal comfort and vulnerability: case of Paraisópolis-Morumbi, São Paulo

Loyde Vieira de Abreu-Harbich1, Adriana Majer Krassin2, and Renato Luiz Sobral Anelli3

1Professor and Researcher at Faculty of Arqchitecture and Urban Planning at Mackenzie Presbyterian University.

2Graduation at Faculty of Arqchitecture and Urban Planning at Mackenzie Presbyterian University.

3Professor and Researcher at Faculty of Arqchitecture and Urban Planning at Mackenzie Presbyterian University.

Corresponding author: Loyde Abreu-Harbich (loyde.harbich@mackenzie.br)

†Rua Itambé, 143, prédio 9, Higienopolis, CEP 01302-907, São Paulo, Brazil

Key Points:

- Street trees and green spaces of planned neighborhoods bring thermal comfort and reduce the impacts of urban heat islands.

- Quantifications of urban heat  at the local scale in the Favela and Planned Neighborhood,  São Paulo, Brazil, by PET and UTCI index.

- There was a difference in air temperature between the favela and the planned neighborhood: the temperature was 7.8º C in 2023 and 11.7º C in 2024.

Abstract

Satellite surface temperatures of São Paulo show heat islands and coolness islands due to urban form. Some planned neighborhoods have street trees, green areas, appropriate urban drainage to prevent flooding in floodplain areas, and adequate thermal comfort and ventilation conditions for urban health and human well-being. Other neighborhoods that originated irregularly within the urban grid or on the city's outskirts suffer from extreme weather problems, such as urban heat islands and localized rainfall that cause flooding and landslides. This work investigates urban heat at the pedestrian scale in different urban forms in Paraisópolis-Morumbi region. Methods: a) Study area selection; c) Field climate data (air temperature and humidity), every 10 min (08/25/2024 to 09/14/2024); d)Quantify heat and thermal comfort by the PET (Physiological Equivalent Temperature) and UTCI (Universal Thermal Climate Index); e)Analysis of current and future scenarios. The maximum air temperature difference between Morumbi (area 1) and Paraisópolis (area 3) was 7.8º C in 2023 and 11,77 ºC in 2024. The difference between Favela is 12.5ºC in PET and 10ºC in UTCI. The maximum relative humidity values showed similar values, around 79%, but the minimum values differed by 14.4%. Although data collection was conducted in the winter, heat conditions can worsen during the rainy season, when temperatures in PET and UTCI can exceed 45 ºC in conditions of extreme thermal stress. The computer simulations presented the current and future scenarios in which temperatures can be cooled with the implementation of a park in the Antonico Valley (area 3).

Plain Language Summary

Satellite thermal images of São Paulo can show the differences between urban heat and cool islands based on surface temperatures. Street trees and green spaces of planned neighborhood bring thermal comfort. Nonetheless, irregular neighborhoods face severe weather challenges, such as extreme heat and localized rainfall resulting in devastating flooding and landslides.

This study quantifies urban heat sensations at local scale in the Paraisópolis (Favela) and Morumbi (Planned Neighborhood), São Paulo, Brazil using thermal comfort index as Physiological Equivalent Temperature (PET) and Universal Thermal Climate Index (UTCI).

Results show temperatures difference between favela and planned neighborhood in terms of air temperature, 7.8º C in 2023, and 11.7º C in 2024. In terms of PET, this difference is 12.5º C, and UTCI, around 10º C. Future scenarios demonstrate that strategic interventions can effectively cool temperatures, such as establishing a park in favela and enhance planting tree.

Not only street trees can minimize the negative effects of heat islands in cities, but also the small green spaces with various urban functions such as conservation of blue and green spaces, leisure and urban cooling, especially in favelas. Municipalities must invest in these nature-based solutions to improve urban resilience and quality of life for residents.

1 Introduction

Extreme weather events have caused significant repercussions on the environmental, economic, and social scenarios, affecting resident populations, especially in vulnerable areas of large cities such as São Paulo. Complex cities such as São Paulo have urban heat and coolness islands, according to the different configurations of the city (Borges, et al., 2022). It is worth noting that the heat island phenomenon is characterized by an increase in temperature in urban centers concerning the surrounding areas, and the materials applied to floors and facades capable of storing heat mainly in denser areas and less afforestation, especially at night (Barros & Lombardo, 2016; Oke, Zeuner, & Jauregui, 1992).

Heat island studies based on thermal images obtained by satellites and georeferenced that consider surface temperatures, mainly roofs and floors, and disregard air temperature and the application of thermal comfort indexes, which is the analysis that most influences the health and well-being of the population. However, to more accurately interpret the real human heat sensation, it is important to have a local monitoring network so that micro-scale interventions can effectively change the urban climate, even in vulnerable regions. Studies indicate that vegetation can reduce thermal sensations by more than 10 °C (de Abreu-Harbich, Labaki, & Matzarakis, 2015) and this microclimate regulation strategy can be used to create islands of urban coolness. It is worth noting that the integration between urban climatology, planning actions, and urban design is a complex modeling process, but it can be a key part of urban planning (Matzarakis, 2022). In addition, assessing urban heat using environmental comfort indexes is an excellent way to investigate the real territory and propose future scenarios for climate improvements adapted to users. It is important to highlight that the application of thermal comfort indexes such as PET (Physiologically Equivalent Temperature) in city mapping has allowed the development of urban guidelines that are more consistent with human thermal perception. In addition to the PET, the UTCI (Universal Thermal Climate Index) was developed to assess the thermal conditions of external environments in all types of climates through human physiological responses independent of the personal characteristics of individuals (Fiala, Havenith, Bröde, Kampmann, & Jendritzky, 2012).

In this context, the main problem is investigating how urban densification and vegetation influence the intensity of heat islands to offer adaptations to urban climate change. The study of the effect of heat islands and urban ventilation will feed a database with relevant information to return urban planning adapted to climate change, mitigating the problems caused by extreme weather. This research aims to determine how urban heat impacts thermal comfort at the pedestrian scale in different urban forms in the Paraisópolis-Morumbi region.

2 Materials and Methods

The methodological procedures must be consistent with the objectives pursued and described in detail so that others can reproduce the work (Fig. 1):

a) Selection of study areas based on urban form and vegetation presence.

b) Thermography data collected.

c) Field meteorological data (air temperature and relative humidity) collected.

d) Analyzing Data.

e) Environmental computer simulation of three-dimensional models of the current and future scenario using the software Envi-Met 5.6 (Bruse, 2024).

Figure 1. Flow chart research outline

2.1 Study area selection

São Paulo has a humid subtropical climate (Cfa/Cwa, according to the Köppen-Geiger classification, (Aparecido, Rolim, ichetti, Souza, & Johann, 2016). The average annual temperature is around 20º C, with dry winters and rainy summers. São Paulo has a data monitoring network, but in some regions with high building density and extreme vulnerability, there is no monitoring network, making it difficult to assess the intensity of heat islands.

The choice of the study area was defined at the 1st CIAMCLIMA Workshop in July 2022 after considering the following criteria: areas in a situation of social and environmental vulnerability, access to social actors (community leaders, population, among others), access to a survey of buildings in the field, and researcher safety to conduct field surveys.

The region has a thermal contrast due to the presence of vegetation and irregular occupations, such as the city of Paraisópolis, which has a high construction density, soil impermeability, and lack of vegetation (Fig. 2).

Figure 2.

Morumbi vs. Paraisópolis features and materials used on façades and roofing and heat adaptations

Morumbi neighborhood was planned to be a residential garden neighborhood, with green areas and dense tree mass, as well as the Paraisópolis subdivision, which did not have the original occupation planned because the uneven topography and poor infrastructure compromised the sale of the lots. In 1960, irregular occupation began in Paraisópolis, one of the most populated favelas in São Paulo, with over 100.000 inhabitants (IBGE, 2022). The region continues to undergo constant improvements, but the conditions of habitability, health, and well-being still leave much to be desired. In addition, there is a need to expand leisure areas and provide vegetation cover to alleviate the heat, as in the neighborhood, Morumbi.

Materials used on façades in Paraisópolis Favela are concrete slabs and ceramic bricks for walls, simple glass and metal Venetian for windows, and concrete slabs of concrete tiles. The principal vernacular architecture adaptation climate adaptation includes greening façades with small plants in vases, narrow streets, cobegos, street shading, and evaporative coolers. In Morumbi, materials used on façades are concrete or ceramic bricks painted with light colors, simple glass for windows are shaded with trees or metal or wood Venetian, ceramic tiles or concrete slabs for roofing (Fig. 2).

2.2 Computer Simulation

The Envi-Met software (Bruse & Fleer, 1998; Bruse M. , 2024) will be used to analyze the influence of urban geometry on human thermal comfort in open areas. This software allows the modeling of urban geometry and vegetation interaction in calculating urban thermal comfort. Depending on the scales of analysis, one software may respond closer to reality than the other.

Computer simulations will occur in two stages: the first to study human thermal comfort obtained in the study area and the second to study future scenarios aimed at improving the thermal conditions of the urban environment. Future scenarios will be designed based on the results of field research, which aims to meet the needs of local users (Fig. 4).

The actual situation and the development of future scenarios for improving urban thermal comfort were analyzed using the Envi-Met software based on the results of research carried out with users.  Data used for future scenarios was obtained by open data  by ClimateOneBuilding. (2024)). This simulation analysis is a fundamental tool for developing projects at the urban design level (Iuorio & Harbich, 2019).

Figure 3.

Actual and future scenarios developed at Envi-Met 5.6

This study selected three sections: a planned neighborhood, region of Colegio Porto Seguro and Lina Bo Bardi House in Morumbi Neighborhood (P1), a Favela’s rebuilt area, Comunidade Fazendinha (P2) , and Favela, the region of Córrego Antonico Valley (P3) (Fig.3).

3 Meteorological Data Monitoring, static and dynamic

Meteorological data from urban stationary stations cannot delineate the city’s microclimate and may have limitations in terms of accuracy, time, and cost ( Writzl, et al., 2024). Therefore, measurements of meteorological variables by transect, whether on foot or in vehicles (bicycles, motorcycles, or cars), present a panorama of the built environment that is closer to the user’s sensation. Much research has been developed using this methodology for thermal and acoustic surveys. It is believed that, with the advancement of artificial intelligence, big data, and the Internet of Things, collected data can be integrated, calculated, and made available to the population in real time as a form of communication (Sulzer, Christen, & Matzarakis, 2022). This research used the methodology of (Kousis, Pigliautile, & Pisello, 2021) where data collection is carried out on a mesoscale with static measurements of climate variables from the installation of dataloggers in specific regions and dynamic measurements from the transect by means of people, bicycles or cars.

Figure 3.

Meteorological Data Monitoring, static and dynamic

For static measurements, HOBO brand MX2301 Temperature and Humidity data loggers with Bluetooth were installed in Area 1 (P1) – Morumbi-, Area 2 (P2) – Comunidade Fazendinha- and area 3 –Floriculture Rael, Corrego Antonico Valey (Fig.4). There were two measurement campaigns: in fall, from 4th to 14th Sep 2023 and 23rd Sep. 2024 to 3rd Oct. 2024. The data interval is every 10 min.

A HOBO brand datalogger was used for dynamic measurements, placed in protection, and fixed at a bike or car. The data interval was every 10 min, and the route took about 1.5 hours. FLIR brand thermal camera, model E5 Pro, area used during static measurement campaigns. The pictures are taken at each measurement point.

To calculate thermal comfort in humans in outdoor environments, the Rayman Pro software (Matzarakis, Rutz, & Mayer, 2010; Matzarakis A. , 2009) was used, and the following indices were selected: PET (Physiological Temperature Equivalent) (VDI, 1998) and UTCI (Universal Thermal Climate Index) (Błażejczyk, et al., 2010). The Rayman Pro software was used to quantify thermal comfort by these two indices (Matzarakis, Rutz, & Mayer, 2010). The input climatic variables were air temperature, relative humidity, wind speed, and Mean Radiant Temperature; and the personal variables were 0.6 Clo, based on the studies of Abreu Harbich, Chaves e Brandstetter (2018) and sitting, 80 W, and cycling, 200 W. To compare the results of UTCI indexes with PET, Table 1 was used, where thermal sensations are divided into sensation classes.

Table 1-

Physiological equivalent temperature (PET) for different grades of thermal sensation and physiological stress on human beings  for Europa, Taiwan e São Paulo

| Thermal Sensation | Physiological stress level | UTCI (oC) ( (Błażejczyk, et al., 2010) | PET para European (oC PET) ( Matzarakis & Mayer, 1997) | PET para Taiwan (oC PET) (Lin & Matzarakis, 2008) | PET para São Paulo (Marques & Monteiro, 2009) |
| --- | --- | --- | --- | --- | --- |
| Very cold | Extreme cold stress | -13 to -27 | <4 | <14 |  |
| Cold | Strong cold stress | -13 to 0 | 4 to 8 | 14 to 18 | < 4 |
| Cool | Moderate cold stress | 0 to 9 | 8 to13 | 18 to 22 | 4 to 12 |
| Slightly Cool | Slight cold stress | 9 to 18 | 13 to 18 | 22 to 26 | 12 to 18 |
| Comfortable | No thermal stress | 18 to 26 | 18 to 23 | 26 to 30 | 18 to 26 |
| Slightly warm | Slight heat stress | 26 to 32 | 23 to 29 | 30 to 34 | 26 to 31 |
| Warm | Moderate heat stress | 32 to 38 | 29 to 35 | 34 to 38 | 31 to 43 |
| Hot | Strong heat stress stress | 38 to 46 | 35 to 41 | 38 to 42 | >43 |
| Very Hot | Extreme heat stress | >46 | >41 | >42 |  |

Source: Adapted by de Abreu-Harbich, Labaki, Matzarakis (2015)

4 Results

4.1 Results of thermography

Figure 5 shows the results of thermography where in point 7a and 7b are the low surfaces temperatures due to tree shades in planned area, Morumbi. If we compare the sidewalks in Planned area, Morumbi (point 8 and 9), Rebuilt Favela, Comunidade Fazendinha (point 5 and 6), and Favela (point 1, 2 and 3),  it can observe that tree shade can improve surface temperatures more than buildings shade. It means that tree shaded can reduce surface temperatures around 5o C. It can be highlighted that sidewalk of rebuilt Favela have a lack of trees, and it can be used to adapted this areas to urban heat.

The hottest image is a roofing of Paraisópolis Favela, point 4. It can be observed that concrete material absorbs a lot of solar radiation, and brings more heat for this area. In this case, cool roofs could be a good strategy for reducing the heat island.

Figure 5.

Results of thermography at study areas

4.2 Results of Data Collected in loco (static)

Figure 6 presents the results of the measurements of the locations’ air temperature and relative humidity. It was observed that the highest temperature found was at the Floriculture do Rael measurement point at 34º C, while at the Casa de Vidro, the maximum found was 28ºC. It was observed that the lowest temperature found was also at Floriculture do Rael, 15ºC. The results suggest that where there is vegetation, there was no great thermal variation due to the thermoregulation work of the vegetation. The thermal amplitude varied around 18º C at Floriculture do Rael, a measurement point close to the canalized Antonico River, inside the favela. In the Casa de Vidro, the trees are capable of thermoregulating temperatures at 13ºC. It was observed that the most humid location was Morumbi, with 90.32% in the early morning and 27.52% in the hottest time of the day. It was observed that the variation found was 62%.

Figure 6.

Results of air temperature and relative humidity in Sep. 2023 and Sep. 2024. In 2023,the higher air temperature was 34 ºC in Favela (Floriculture Rael, Area 3) and the highest difference between air temperature between Planned Neighborhood (Morumbi, area1) and Favela (Floriculture Rael, Area 3) was 7,8 ºC. In 2024,the higher air temperature registered was 41 ºC in Favela (Floriculture Rael, Area 3) and the highest difference between air temperature between Planned Neighborhood (Morumbi, area1) and Favela (Floriculture Rael, Area 3) was 11,7 ºC. In 2023, the maximum relative humidity was 90%. In 2024, the maximum relative humidity was 95% in Favela (Floriculture Rael, Area 3)

.

Planned Neighborhood (Morumbi, area1), 12% in Rebuilt Favela(Fazendinha, area 2) and 15% in Favela (Floriculture Rael, Area 3).

Figure 7 shows the results of thermal temperatures calculated by the PET and UTCI indices. We observe temperatures above 35 in the afternoon, that is, thermal discomfort and generating health risks. Figure 8 shows a comparative analysis of Air Temperature, PET, UTCI and Mean Radiant Temperature frequencies.

Figure 7.

Frequencies of Air Temperatures is between 18 to 23 ºC in all cases. Frequency of PET over 35 ºC was 9% in Planned Neighborhood (Morumbi, area1), 12% in Rebuilt Favela(Fazendinha, area 2) and 15% in Favela (Floriculture Rael, Area 3). Frequency of UTCI between 29 to 35 ºC was 34% in all study cases. Frequency of UTCI over 35 ºC was 36% in all study cases.

4.3 Results of simulation Actual and Future Scenarios

Figure 8 shows results of the simulations of the current and future scenarios for: area 1-Planned Neighborhood (Morumbi); area 2- Rebuilt Favela (Comunidade Fazendinha); area 3-Favela (Floriculture Rael).

The current scenario of Area 1 (Planned Neighborhood) indicates that street trees contribute to lowering temperatures on sidewalks and around buildings, particularly during the afternoon, thermal sensations vary 38 to 46 ºC. The combination of wind and tree shade can reduce temperatures and distribute around the street and buildings. The future scenario of this area shows cooling shade effects during morning, until 4º C. The differences of  thermal sensations were between 36 to 46 º C.

In the current scenario of Area 1 (Planned Neighborhood), street trees contribute to lowering temperatures on sidewalks and around buildings, with air temperatures ranging from 28 to 31 ºC and thermal sensations between 38 to 46 ºC in the afternoon. In future scenario of Area 1 (Planned Neighborhood) – with more trees -, the air temperature above tree became more heat than current scenarios. In terms of PET, it observed a difference between shaded areas and sunny areas. It means that areas with a lot of trees can increase air temperature because the wind cannot circulate around trees and buildings. When comparing relative humidity and results between current and future scenarios, it was observed that humidity increases while wind circulation decreases, leading to a potential accumulation of particulate pollution.

In the current scenario of Area 2 (Rebuilt Favela), the air temperature is between 25 and 28 ºC, with thermal sensations ranging from 24 to 31 ºC. Comparing current and future scenarios of Area 2 (Rebuilt Favela), there is no apparent reduction in air temperature and thermal sensation. However, the distribution of thermal sensation improved around 5 ºC in area near Av. Hebe Camargo. In terms of relative humidity, the future area experienced higher humidity levels throughout the day, approximately 10% in the morning. Additionally, the wind speed decreased, contributing to improved thermal comfort. It is important planting tree with porous canopies structure as Sibipiruna (Caesalpinia pluviosa), Brazilian native species as Abreu, Labaki and Matzarakis (2015) suggested for cooling areas.

In the current scenario of Area 3 (Favela), air temperature range between27 to 30 ºC and thermal sensation, 30 to 50 ºC. Compared with future scenarios, the small park can improve the maxim results of thermal comfort, with capacity to reduce heat stress. The enhancement in thermal sensations is attributed to wind circulation and humidity in the area.

To compare these results with data collected in loco (dynamic), it is observed that Area 1 (Planned Neighborhood) and 3 (Favela) are compatible with results. Area 2 (Rebuilt Favela) simulation indicates that planting trees strategically can reduce thermal sensations. Although pedestrians face heat stress, this is not reflected in the simulation due increasing the albedo due solar radiation that cand increasing their Heat Stress (Krüger & Gonzalez, 2016).

Table 2 – Comparison between observed data and simulated data at 14h

| Study Area | Air Temperature |  |  | Thermal Comfort Sensations (PET) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Observed | Current | Future | Observed | Current | Future |
| Area 1 (Planned Neighborhood) | 24 ºC | 28 - 32ºC | 28-36 ºC | 31 ºC | 34-46 ºC | 30-68 ºC |
| Area 2 (Rebuilt Favela) | 35 ºC | 25-28 ºC | 24-31 ºC | 36 ºC | 25-32 ºC | 23-27 ºC |
| Area 3 (Favela) | 29 ºC | 27 -33 ºC | 28-37 ºC | 34 ºC | 28-50 ºC | 30-36 ºC |

Cluster of trees could not thermoregulate the environment homogeneously in the future scenario of area 1 (Planned Neighborhood, Morumbi) because the wind permeability is compromitted by trees. However, the minimum PET is reduced. The appropriate placement of trees, as illustrated in the future scenario of Area 2 (Rebuilt Favela), can evenly distribute thermal sensations through wind circulation, thereby reducing heat stress risk for pedestrians. Also, small parks change the improvement of thermal features of urban form. The wind can circulate and reduce the potential of air pollution by particles, and it can distribute the benefits of refreshing areas in microclimate. In area 3 (Favela), it is observed that the maxima PET reduction , more than 20 ºC , and reduces heat stress risk.

Figure 8. Results of Actual and Future scenario at study areas

5 Conclusions

Complex cities such as São Paulo have various heat and cooling islands in urban areas. This phenomenon occurs due to differences in materials applied on pavement and buildings, the presence of street trees and green spaces, topography, and land uses. Planned Neighborhoods in Brazilian cities can promote health and well-being due presence of street trees and small parks used to urban drainage, but irregular neighborhoods, such as favelas, have a lack of vegetation, have climate problems due to extreme weather events. This work focuses on quantifying climate change in different urban forms of São Paulo- planned neighborhoods, rebuilt favelas, and favelas. Street trees and small parks can reduce heat stress for pedestrians in different urban forms. However, this vegetation must be uniformly distributed in urban areas to avoid tree clusters that block wind circulation. This effect was observed on field measurements and computer simulations of current and futures scenarios (with more urban trees). Computer simulations can be used as urban planning tools to help the decision-making process in cities to adapt to climate change.

In the context of global warming, plating additional trees plays a crucial role for cooling urban areas and improve air quality, especially in areas with a potential for increasing the albedo due solar radiation and reduce ambient temperatures (Akbari & Rose, 2008; Krüger & Gonzalez, 2016). Comparing Planned Neighborhood (Morumbi) with Favela (Paraisópolis), observed that temperature differences were 7.8º C in 2023 and 11.7º C in 2024. Regarding PET, this difference is 12.5º C, and UTCI is around 10º C. These temperature differences are due to the albedo of materials used in buildings and pavements, combined with solar radiation, and this effect can change the thermal sensations in pedestrian level. This result confirms findings of Krüger and Gonzalez (2016).

Studies shows that each trees species have their different physical or structural characteristics and their location of placement in an urban environment influences in their cooling capacity(de Abreu-Harbich, Labaki, & Matzarakis, 2015; Morakinyo, Ouyang, Lau , Ren, & Ng, 2020). This study observed that the cooling effects of trees depend also on urban forms of residential areas that need to combine distribution of street trees and small parks to reduce temperatures. It confirms results of Morakinyo et al. (2020).

Considering these results, it is important to expand the meteorological data variables such as radiation balance and wind speed in urban areas, to compare the simulations results of current and future scenarios with collected data. Additionally, it is important to investigate cooling effects of green or cooling roofs and green façades in favelas areas.

Given that local climate changes are related to the specific features of the city, such as size, population density, and diurnal and seasonal variations , the phenomenon of a heat island can be characterized based on on-site measurements. It is observed that the magnitude of the heat wave is almost 7.8 °C between the two neighborhoods analyzed. This increase in temperature is due to the excessive increase in temperature in urban centers surrounding areas, i.e., the use of materials on the floor and facades that store heat, dense areas, and less trees tend to present higher temperatures, even at night (Barros & Lombardo, 2016).

The simulations of future scenarios indicated two strategies for microclimate improvement with effects on the entire neighborhood: the insertion of more trees and the opening of the Antonico River to permeate the wind and provide more ventilation in the streets. However, it was observed that to ensure an improvement in comfort, health, and well-being, it is necessary to be aware of how important it is.

Acknowledgments

We thank to Paraisópolis Community, in special Isaac Bezerra for help us on field collected data. We thank the Thermal Comfort Laboratory of Faculty of Architecture and Urban Planning at Mackenzie Presbyterian University for supplying the computational resources essential for this study. This research is supported by National Council for Scientific and Technological Development research project, process no.: 409032/2021-2 and MackPesquisa, process no.:221074.

Conflict of Interest

The authors declare no conflicts of interest relevant to this study.

Open Research

Authors declare that all simulation data utilized in this study are publicly accessible and can be obtained from climate.onebuilding.org. Envi-met 5.6 from M. Bruse (2024), accessible at https://envi-met.com/ was used to distribute temperature in urban areas and quantify comfort in urban areas.

References

Matzarakis, A., & Mayer, H. (1997). Heat stress in Greece. International Journal of Biometeorology, 41, pp. 34–39. doi:https://doi.org/10.1007/s004840050051

Morakinyo, T., Ouyang, W., Lau , K.-L., Ren, C., & Ng, E. (1 de Jun de 2020). Right tree, right place (urban canyon): Tree species selection approach for optimum urban heat mitigation - development and evaluation. Science of The Total Environment, 719, p. 137461. doi:https://doi.org/10.1016/j.scitotenv.2020.137461

Writzl, L., Wollmann, C., Turba, I., Iensse, A., Silva, A. N., Baumhardt, O. d., . . . Matzarakis, A. (2024). Mobile Measurements in the Urban Thermal Environment Using Bicycles: A Systematic Review. RA’E GA - O Espaço Geográfico em Análise, 61. doi:https://doi.org/10.5380/raega.v61i1.96807

Akbari, H., & Rose, L. (2008). Urban Surfaces and Heat Island Mitigation Potentials. Journal of the Human-Environment System, 11(2). doi: https://doi.org/10.1618/jhes.11.85

Aparecido, L. d., Rolim, G. d., ichetti, J., Souza, P., & Johann, J. (2016). Köppen, Thornthwaite and Camargo climate classifications for climatic zoning in the State of Paraná, Brazil. Ciência e Agrotecnologia, 40 . doi:https://doi.org/10.1590/1413-70542016404003916

Barros, H., & Lombardo, M. (2016). A ilha de calor urbana e o uso e cobertura do solo no município de São Paulo-SP. GEOUSP: Espaço e Tempo (Online), 20 . doi:https://doi.org/10.11606/issn.2179-0892.geousp.2016.97783

Błażejczyk, K., Broede, P., Fiala, D., Havenith, G., Holmér, I., Jendritzky, G., . . . Kunert, A. (december de 2010). Principles of the New Universal Thermal Climate Index (UTCI) and its Application to Bioclimatic Research in European Scale. Miscellanea Geographica, 14, pp. 91-102. doi:https://doi.org/10.2478/mgrsd-2010-0009

Borges, V. O., Nacimento, G., Celuppi, M., Lúcio, P., Tejas, G. T., & Gobo, J. A. (2022). Zonas climáticas locais e as ilhas de calor urbanas: uma revisão sistemática. Revista Brasileira de Climatologia, ABClima (Brazilian Association of Climatology), 31. doi:https://doi.org/10.55761/abclima.v31i18.15755

Bruse , M., & Fleer, H. (1998). imulating surface–plant–air interactions inside urban environments with a three dimensional numerical model. nvironmental Modelling & Software, 13(3-4), pp. 373-384. doi:https://doi.org/10.1016/S1364-8152(98)00042-5

Bruse, M. (10 de Jun de 2024). ENVI-MET. Fonte: 3D Modelling Software for Urban Cooling and Climate Adaptive Planning: https://envi-met.com/

Bruse, M. (2024). Leading 3D Modelling Software for Urban Cooling and Climate Adaptive Planning. Fonte: Envi-met: https://envi-met.com/

ClimateOneBuilding. (2024). Climate.OneBuilding.org. Fonte: Repository of Building Simulation Climate Data: https://climate.onebuilding.org/

de Abreu-Harbich, L. V., Chaves, V. L., & Brandstetter, M. C. (1 de May de 2018). Evaluation of strategies that improve the thermal comfort and energy saving of a classroom of an institutional building in a tropical climate. Building and Environment, 135, pp. 257-268. doi:https://doi.org/10.1016/j.buildenv.2018.03.017

de Abreu-Harbich, L. V., Labaki, L. C., & Matzarakis, A. (2015). Effect of tree planting design and tree species on human thermal comfort in the tropics. Landscape and Urban Planning, 138 . doi:https://doi.org/10.1016/j.landurbplan.2015.02.008

Fiala, D., Havenith, G., Bröde, P., Kampmann, B., & Jendritzky, G. (2012). UTCI-Fiala multi-node model of human heat transfer and temperature regulation. International Journal of Biometeorology, 56, pp. 429-441. doi:https://doi.org/10.1007/s00484-011-0424-7

IBGE, I. (2022). Censo Brasileiro de 2020. Censo Brasileiro de 2010: IBGE.

Iuorio, O., & Harbich, L. (2019). Green Networks as a Key of Urban Planning with Thermal Comfort and Well-being. Em F. Lemes de Oliveira, & I. Mell, Planning Cities with Nature. Cities and Nature (Cities and Nature ed., pp. 97–109). Springer, Cham. doi:https://doi.org/10.1007/978-3-030-01866-5_7

Kousis, I., Pigliautile, I., & Pisello, A. (2021). Intra-urban microclimate investigation in urban heat island through a novel mobile monitoring system. Scientific Reports, 11, p. 9732. doi:https://doi.org/10.1038/s41598-021-88344-y

Krüger, E., & Gonzalez, D. (2016). Impacts of changes in surface albedo on microclimate and. Ambiente Construído, 16(3), pp. 89-106. doi:http://dx.doi.org/10.1590/s1678-86212016000300094

Lin, T.-P., & Matzarakis, A. (2008). Tourism climate and thermal comfort in Sun Moon Lake, Taiwan. International Journal of Biometeorology, 52, pp. 281–290. doi:https://doi.org/10.1007/s00484-007-0122-7

Marques, L., & Monteiro, M. (2009). Índices de conforto térmico em espaços abertos. Fórum Patrimônio: ambiente construído e patrimônio sustentável, 3. Fonte: https://repositorio.usp.br/item/001820988

Matzarakis, A. (2009). RayMan Pro. Acesso em 2023, disponível em https://www.urbanclimate.net/rayman/

Matzarakis, A. (2022). Communication Aspects about Heat in an Era of Global Warming—The Lessons Learnt by Germany and Beyond. (M. AG, Ed.) Atmosphere, 13 . doi:https://doi.org/10.3390/atmos13020226

Matzarakis, A., Rutz, F., & Mayer, H. (2010). Modelling radiation fluxes in simple and complex environments: basics of the RayMan model. International Journal of Biometeorology, 54, pp. 131–139. doi:https://doi.org/10.1007/s00484-009-0261-0

Oke, T., Zeuner, G., & Jauregui, E. (1992). The surface energy balance in Mexico City. Atmospheric Environment, 6. doi:https://doi.org/10.1016/0957-1272(92)90050-3

Sulzer, M., Christen, A., & Matzarakis, A. (2022). A Low-Cost Sensor Network for Real-Time Thermal Stress Monitoring and Communication in Occupational Contexts. Sensors, 22(5). doi:https://doi.org/10.3390/s22051828

VDI. (1998). VDI 3787, Part I: environmental meteorology, methods for the human-biometeorological evaluation of climate and air quality for the urban and regional planning at regional level. Part I: climate. Düsseldorf: VDI/DIN-Handbuch Reinhaltung der Luft, Band 1b. Fonte: https://www.dinmedia.de/en/technical-rule/vdi-3787-blatt-2/3353826

.