Iterative Optimization Method for Frequency Stability Constraints in Renewable Energy-Integrated Power Systems

Ruilin Gao, H. F. Wang, Senior Member, IEEE

Abstract-- This paper presents a practical method for allocating inertia and damping in power systems integrated with renewable energy sources (RES). The frequency response model of the power system is first established, with distinct modeling of the dynamics of synchronous generators (SGs) and three types of RES units. A state-space model, which describes the relationship between frequency response and disturbances, is derived through the disturbance allocation process. A fitting method for frequency and power responses is proposed based on modal analysis. Analytical formulas for frequency stability indices (FSIs) and power indices are derived. Furthermore, applying matrix perturbation theory, an iterative optimization method for inertia and damping allocation of RES units is proposed and validated through simulation cases using the modified IEEE 39-bus model.

Index Terms--Frequency stability index (FSI), frequency and power response, state-space model, matrix perturbation theory, inertia and damping allocation

Introduction

W

ITH the large-scale integration of renewable energy sources (RES), the low-inertia characteristics of these units have altered the frequency response behavior of the power system, introducing new challenges to the frequency stability of the grid. To enhance the frequency stability of RES-integrated systems, various frequency control strategies for RES units have been proposed. These include virtual inertia control,[1]-[3] virtual droop control,[4] and virtual synchronous machine control,[5][6] all aimed at improving the system’s ability to respond quickly to frequency deviations. Three main frequency stability indicators (FSIs)—the maximum rate of change of frequency (RoCoF), the frequency nadir, and the quasi-steady-state (QSS) frequency—are commonly used to assess frequency stability. These indicators are essential for analyzing frequency stability or tuning the parameters of frequency controllers. The time-domain simulation approach, based on the Load Frequency Control (LFC) model[7][8] or system differential equations,[9][10] has been initially proposed to obtain accurate frequency responses and FSIs. This method models the complex dynamics of the grid in detail, providing precise results. However, these results are often limited by the specific conditions of the simulation, and the computational load becomes substantial when considering multiple scenarios.

To address these limitations, an analytical method has been developed[11][12] to derive a generic solution that accounts for all possible scenarios. This method constructs an equivalent single-machine frequency response model by linearly combining the inertia and damping characteristics of multiple synchronous generators (SGs). Using inverse Laplace transforms, analytical expressions for the global frequency nadir can be derived. Additionally, revised mathematical formulations are developed to incorporate under-frequency load shedding (UFLS) strategies and wind power frequency control strategies.[13][14] The analytical approach provides high accuracy and is frequently employed as an optimization objective or constraint in the allocation of inertia and damping in energy storage or wind power systems.[15]-[18]

In these analytical methods, network dynamics are typically neglected, especially in small-scale, low-order system scenarios. However, in large-scale RES-integrated systems, efforts have been made to extend the analytical approach, as the frequency dynamics of individual units or regions can differ significantly. Studies[19]-[21] have introduced tie-line power flow equations to model the interconnection of SGs across different regions, resulting in a regional frequency response model used for unit scheduling and inertia placement. A common-mode method has been proposed to account for global frequency responses, incorporating network effects,[22] with analytical solutions for the frequency nadir and average RoCoF derivable accordingly. When considering regional frequencies, mathematical proofs in one study[20] demonstrated that no analytical solutions exist for high-order systems. Moreover, studies[20][22] have established connections between frequency responses and modes, thereby justifying the application of modal analysis in frequency stability assessments.

Several studies have established a connection between frequency stability and modal analysis. The system’s frequency response is typically modeled using state-space equations with disturbance inputs.[23]-[25] A third-order system has been introduced to simulate the global frequency response, which can be decomposed into an exponential decay and an oscillatory decay. These correspond to a real root and a pair of conjugate complex roots derived from the characteristic equation.[22][26] Further mathematical proof demonstrates that the regional frequency response is the superposition of the global frequency response and inter-area oscillations, which are intrinsically linked to the system’s modes.[20]

This paper proposes a frequency stability analysis method based on modal analysis, with the goal of achieving optimal inertia and damping allocation in high-order systems. The primary contributions of this paper are outlined as follows:

(1) A state-space model is derived to represent both global and regional frequency responses in a multi-machine system integrated with RES. Explicit models for four types of generating units are implemented to account for the diversity in renewable generation. The global and regional frequency responses are clearly defined using this model.

(2) The definition of modes and residues within the state-space model is clarified from the perspective of modal analysis, which is commonly used in small-signal stability analysis but is rarely applied to frequency stability analysis. A unified response fitting method based on modes and residues is proposed to highlight the correlation between frequency/power responses and modes. Furthermore, a quasi-analytical method is derived to provide specific solutions for three frequency stability indices and two power indices.

(3) Additionally, by incorporating matrix perturbation theory,[27] an iterative optimization method is proposed to evaluate frequency stability and RES reliability. The increments in FSIs and power indices can be decomposed into increments in modes and residues. This method enables precise inertia and damping allocation in scenarios with different objective functions and various types of RES units.

The remainder of this paper is organized as follows: Section II clarifies the disturbance allocation process between load disturbances and terminal disturbances. Section III provides a theoretical analysis of the increments in FSIs/power indices and modes/residues for the iterative optimization method. Section IV presents case studies to verify the proposed methods. Finally, Section V discusses the research findings and outlines future work.

New energy access interconnection system model and frequency response fitting method

The system response is typically most pronounced only during a short period immediately following a disturbance.[18] Frequency response analysis can, therefore, be conducted under the assumption of linearization. Based on this premise, the following assumptions are made:

(1) Nonlinear phenomena, such as the dead zone of the governor, are neglected in the modeling;

(2) The modeling of excitation control and reactive power control for each generating unit is excluded, assuming the system operates with active power only and no reactive power;

(3) Secondary frequency control is disregarded, and the network model is treated as constant and resistanceless.

Multi-machine interconnection model for RES integration

The power-frequency response of a power system is primarily determined by the generating units with frequency control capabilities.[22][23] All relevant generating units, including synchronous generators (SGs) and RES devices, must be modeled to develop a system frequency response model that accounts for the integration of renewable energy sources (RES). As shown in Fig. 1, four major types of devices are considered in the modeling of RES devices: (a) permanent magnet synchronous generator-based grid-forming devices (PMSG-GFM), (b) PMSG-based grid-following devices (PMSG-GFL), (c) doubly-fed induction generator-based grid-following devices (DFIG-GFL). The key parameters—namely inertiaand damping , along with the frequency control parameters of the devices—play a crucial role in the frequency response for all four types of generating units.

Fig. 1.  Block diagram of power-frequency response of generating units: (a) SG; (b) PMSG-GFM; (c) PMSG-GFL; (d) DFIG-GFL

As illustrated in Fig. 1, generating units are classified into two categories. Grid-forming (GFM) units, such as SGs and PMSG-GFMs, regulate frequency by acting as ideal voltage sources and adjusting their phase angle in response to terminal power imbalances. Grid-following (GFL) units, such as PMSG-GFLs and DFIG-GFLs, behave as controlled current sources, adjusting their output power based on the phase-locked loop (PLL) frequency derived from their terminal phase. The input-output (IO) relationships of these two categories differ significantly and should be addressed separately in subsequent discussions regarding disturbances.

In modeling the SG, the second-order dynamics of the generator swing equation and the first-order dynamics of the prime mover governor are considered, as shown in Fig. 1(a). The nonlinear equation for a single unit can be expressed as follows:

(1)

Here, the first and second equations represent the swing equation, where the frequency deviation of the SG is denoted as The third equation models the dynamics of the prime mover and the governor, which adjusts mechanical power based on the deviation  .  is the sum of the time constants of the prime mover and the governor,  is the SG frequency bias coefficient, and  and  denote the inertia and damping coefficient of SG, respectively.

Based on the nonlinear equation of a single SG in equation (1), the linearized model of a multi-SG system can be expressed as follows:

where  represents the collection of micro-increments of all SG state variables. The matrices in equation (2) are provided in Appendix A1.

Similarly, the PMSG-GFM is modeled as a fourth-order system.[28] This model consists of a second-order virtual synchronous generator (VSG) control, a first-order DC capacitor dynamic, and a first-order DC energy source, with the nonlinear equations expressed as:

where  and  represent the virtual inertia and damping coefficients of the VSG control, which mimic the dynamics of the SG, and  is the active power injection reference. is DC-link capacitor, and represents the DC source. The linearized model of a PMSG-GFM-based farm is similarly expressed in the form of equation (2):

where  denotes the state variables of the N PMSG-GFM units, and the state-space matrices are also presented in Appendix A.

The PMSG-GFL devices are modeled using a three-order PLL and a virtual inertia and damping controller that governs frequency control, as described in Equation (5):

(5)

In (5),  and  correspond to the proportional-integral (PI) parameters of the PLL. The third equation represents the active power output controlled by the virtual inertia and damping control, with   denoting the frequency locked by the PLL.  and  are the virtual inertia and damping control parameters, and  represents the time delay of the frequency controller.

The nonlinear equations for the DFIG-GFLs are presented in Equation (6), which consists of two components: the same PLL and virtual power controller as in Equation (5), along with the rotor speed control and rotor model described in Equation (6). A detailed model of the DFIG-GFL can be found in [29].

whereis the inertia coefficient of DFIG rotor, andare the PI parameter of rotor speed controller. Using (5) and (6), the linearized model of GFL RES units is obtained as shown in (7), is the state variables of N GFL RES units:

In the grid, nodes are categorized into four types based on RES integration: SG node, RES-GFM node, RES-GFL node, and load node. The power grid equation,[8],[9] along with the synchronous torque coefficient between nodes, is described by Equations (8) and (9):

where , ,, and  represent the injected power at the SG node, RES-GFM node, RES-GFL node, and load node, respectively.  is the synchronous torque coefficient between nodes i and j;  denotes the generator rotor angle,  is the angle output of VSG controller,and are the phase angles of the RES-GFL and load nodes. Considering the input-output (IO) relationship between SG and RES units, Equation (8) can be transformed as follows:

Fig. 2 Multi-machine interconnected model with RES units

The matrix elements are provided in Appendix A5. By combining the linearized generation unit models from Equations (2), (4), and (7), along with the network model in Equation (10), the multi-machine interconnected model (MMIM) is obtained, as shown in Equation (11) and Figure 2:

In this model, matrix A corresponds to the state-space matrix of the closed-loop system resulting from the interconnection of the network and generation units. Matrix B represents the correlation matrix related to disturbances, with detailed formulas provided as follows:

The intermediate variables in Equations (12) and (13) are illustrated in Appendix A6. Neglecting load disturbances, the equation remains the same as the closed-loop state-space equation. In this context, the system can be analyzed using classical small-signal stability theories, such as modal analysis. The remaining part of Equation (11) reflects the relationship between the system’s state variables and disturbances. These components form the state-space model, which serves as the foundation for the system frequency response analysis theory presented in this paper.

System Disturbance Allocation Process and Dynamic Response Fitting Method

The power-frequency response model of the system plays a critical role in understanding the settling of disturbances. Ideally, the frequency response is considered as the power adjustment process of the generating units in response to variations in load-side power. Thus, it is essential to examine the correlation between load disturbances and the resulting frequency response. Traditionally, large-scale renewable energy source (RES) integrations have been neglected, with the initial condition for system frequency changes set only as step disturbances in power at the synchronous generator (SG) terminals [30][31]. This approach does not consider the relationship between load disturbances and the generating units. In contrast, the present study adopts a more comprehensive model, incorporating node type classifications in the network model, as outlined in Equation (8), and distinguishing the modeling of generating units based on their input-output (IO) relationships while also accounting for load disturbances.

As shown in Figure 1, Grid-Forming (GFM) units are modeled using state-space equations, where power input leads to phase angle output. In contrast, Grid-Following (GFL) units exhibit a phase angle input to power output relationship. In Equation (10), the network terminals are transformed based on the IO relationships of the generating units. During load disturbances, the initial output of the generating units cannot change instantaneously due to the inherent time delays in their frequency control mechanisms,[30][31] where  describes the relationship between load disturbance and inputs of generating units.

Fig. 3 Disturbance Allocation Process in Power System from load disturbance to terminal disturbance of generating units

Based on Equation (14), power disturbances at the load nodes are allocated to the power changes at the SG and PMSG-GFM inputs  and , and the phase angle shifts at the GFL input  through the network are also allocated. This allocation process is depicted in Figure 3. The state-space model in Equation (11) can also be expressed in the form of Equation (15), considering a step disturbance at the load:[2]

where B corresponds to the input matrix of the linearized models of the generating units from Equations (2), (4), and (7). The matrix  represents the disturbance input of the interconnected system, and the system state variables change in response to these inputs. The relationship between the system frequency response and state variables is given by:

The system frequency response consists of both regional frequency responses associated with each SG and the global frequency response. The matrix C can be expressed as:

,

where the system state variables are arranged in the form of,is the total inertia of GFMs and SGs.

By combining the state-space model in Equation (15) with the frequency response equation in (16), and comparing it with the open-loop state-space equation and output equation,[32] the frequency response (or its derivative) can be expressed in terms of modes and residues:

In reference to the measurement of system controllability and observability,  represents the residue between the k-th mode and the i-th frequency response is defined. Consequently, the problem of calculating Frequency Stability Indices (FSIs) can be reformulated as the calculation of modes and residues. By applying the inverse Laplace transform and assuming a load step disturbance occurs at time , the time-domain frequency derivative response can be derived as follows:

where m represents the total order of the system. The time-domain response of the rate of change of frequency (RoCoF) can be further determined as follows:

Additionally, system frequency response is driven by the dynamic power injections from all generating units. The maximum and quasi-steady-state power, as power indices, must be constrained by current limitations. The frequency-domain power response of renewable energy sources (RES) is given by:

where  in the equations of PMSG-GFMs represents the residue of the k-th mode with respect to the j-th Virtual Synchronous Generator (VSG) frequency response. The power response is derived from the frequency formula of VSG control. Similarly,  represents the residue of the k-th mode with respect to the j-th Grid-Following Load (GFL) active power response. The definitions of the input, output, and feedforward matrix   are obtained from (7).

The time-domain responses in Equations (19) and (20) exhibit a similar form to the state variable solutions derived from linear analysis.[33] These responses can, therefore, be transformed into a unified form:

where represents the contribution factor, which quantifies the contribution of the k-th mode to the i-th state for a given disturbance, and C is a constant irrelevant to time variables. The solution in Equation (22) consists of a sum of oscillatory and exponential waves, each corresponding to a system mode. The magnitudes of the contribution factors  indicate the relative weights of these waves during disturbances, offering valuable insight into identifying the dominant modes in the time-domain response.

Frequency Stability Indexes and Power Indexes

Once the times corresponding to the frequency nadir and the extreme RoCoF, denoted by  and   are known, the values of these two transient FSIs can be obtained using the response fitting formulas in Equations (19) and (20). Here,  represents the nominal system frequency:

As , the third FSI, corresponding to the quasi-steady-state frequency, can be determined:

Similarly, when the time corresponding to peak power  is known, the transient maximum power can be obtained from Equation (21) using the inverse Laplace transform. Here,  represents the nominal power of the j-th RES unit:

As, the second power index, corresponding to the quasi-steady-state power, is derived from Equations (26) and (27):

The quasi-steady-state FSI and power indices correspond to the constant CC in Equation (22). As , the waves in the time-domain responses decay to the constant value C. Using Equations (23) through (29), residues and modes can be easily obtained based on the linearized model of the interconnected system, allowing the calculation of FSIs and power indices.

Iterative Optimization Method of Frequency Parameter Based on Matrix Perturbation Theory

FSI and Power Index Increment Calculation based on Matrix Perturbation Theory

As discussed previously in Section (17), the system frequency responses are influenced by intrinsic characteristics represented by modes and the input-output (IO) relationship reflected by residues. When control parameters of the system are altered, the problem of determining changes in frequency stability indices (FSIs) and power indices can be transformed into a problem of mode and residue variations.

The left and right eigenvectors, as well as the modes of the system, are described in Equation (30) using the modal analysis method:[27][34]

where, , and  are the i-th mode, the corresponding left and right eigenvectors, and the eigenvalue matrix . When considering a small increment in the frequency parameters of renewable energy sources (RES) devices, the changes in the state-space matrix, modes, and eigenvectors are presented in Equation (31):

In this equation, variables marked with a subscript 00 denote the initial values of each variable, while the variables marked with the symbol Δ represent the increments resulting from changes in the frequency parameters. According to matrix perturbation theory,[24] the perturbations in modes and eigenvectors can be expressed using Equations (32) through (34):

In Equations (27) to (29), if the state-space matrix variation is known, the perturbation increments in modes and eigenvectors can be determined based on the initial values obtained from the initial matrix A​. Combining this with Equation (12), the state-space matrix variation can be expressed as:

According to Equations (5) and (7), the state-space variation matrix , remains constant because the SG correlation matrix, the PMSG-GFM output matrix , and the GFL feedforward matrix  are independent of the frequency control parameters of the RES units. Furthermore, the state space variation matrix is a constant matrix. As a result, the computational complexity is relatively low, and the matrix can be solved using Equation (35). Furthermore, based on the calculated perturbation increments in modes and eigenvectors through Equations (32) to (34), the perturbation in residues  can be determined using matrix perturbation theory:

This study focuses on the influence of changes in frequency control parameters on frequency stability indices. The perturbation of FSIs can be expressed as in Equations (37) through (39), derived from the partial derivatives of modes and residues:

where , , , .

Similarly, the perturbation of power indexes is presented in (40)-(43):

(40)

(41)

Modal Analysis-Based Iterative Optimization Method

This section focuses on deriving the distribution of inertia and damping from renewable energy sources (RES). Therefore, a linearized multi-objective cost function is proposed:

(44)

where , , and represent the object function of system frequency stability and the reliability of RES units. Corresponding weights are assigned by . The deployment of inertia and damping improves frequency stability, but it also increases the operating points of RES units. This increase challenges the current inverter limitations and reduces the reliability of RES units. Additionally, the setup costs associated with inertia and damping increase. The optimization model for the allocation problem is formulated to obtain an optimal compromise as follows:

(45)

The detailed formulation of the cost functions is provided in Equations (46) and (47). Equation (46) considers three frequency stability indices (FSIs) for the weighted average calculation: regional and global frequency nadirs ,, extreme RoCoFs, , and QSS global frequency. Two power indices are considered in Equation (47) for the objective function related to RES reliability:

(46)                  (47)

The formulation in Equation (46) addresses system frequency both regionally and globally. In practice, some regional frequencies may be lower than the global frequency, and therefore, improving the minimum value of FSIs should be prioritized.[17] An extreme objective is presented in Equation (48), contrasting with the mean objective in Equation (46):

(48)

Meanwhile, constraints on power indices are incorporated into the optimization. Specifically, the power index should not exceed its constraints during the optimization process:

(49)

The stopping criterion for the optimization is based on FSI constraints. The optimization process terminates when one of the following conditions is satisfied:

(50)

(51)

where Equation (50) represents the expected optimization target, where the three FSIs are constrained within their respective boundaries to ensure frequency stability. Equation (51) is the termination criterion for FSI increments. During the optimization, it is crucial to ensure that FSI increments do not simultaneously become negative, as this would indicate that the allocation of inertia is negatively impacting frequency stability.

Fig. 4 Flow Chart of Iterative Optimization Method

The iterative process for index calculation and multi-objective optimization is devised based on the matrix perturbation theory discussed in the previous section. The flow chart in Fig. 4 details the process, which is divided into the following seven steps:

(1) Initialization: This step involves inputting the parameters for each generation unit and power grid, as well as the parameters for the iterative optimization method, including the step size for frequency control parameter calculation FSI/power index constraints,)

(2) Establishment of the MMIM: This step aims to derive the state-space disturbance equations in Equations (15) and (16), which yield the state-space-related disturbance matrix  disturbance matrix , and the state-space and input variation matrix;

(3) Calculation of Intrinsic Characteristics: Based on the matrix , modes, the left and right eigenvectors and the residue  are calculated using the current model parameters;

(4) Calculation of Perturbations in Intrinsic Characteristics: Based on , perturbations in modes, eigenvectors, and residues are calculated using (20)-(22), (24) with a given step size;

(5) FSI and Power Index Perturbation Calculation: The perturbations in the indexes are computed using Equations (39)-(43);

(6) Objective Function Calculation and Inertia/Damping Allocation: The objective functions in Equations (43), (45), and (46) are calculated, and inertia and damping are allocated at the location corresponding to the largest value in Equation (44).

(7) Iteration and Judgement: Inertia and damping are incremented according to the step size and the parameter allocation at the current step. The state-space matrix for the next step is iterated through the state-space change matrix. If the stopping criterion in Equations (50) and (51) is met, the optimization process concludes; otherwise, the process returns to step (3) for the next iteration.

Case Study

Fig. 5 The modified IEEE 39 bus model

The modified IEEE 39-bus system, depicted in Figure 5, serves as the test system for this study. MATLAB/Simulink is employed for response fitting, iterative optimization, and nonlinear simulation. The disturbance is set to a 200 MW load increase at Node 21, with 10 RES farms integrated at Nodes 8, 31, 14, 24, 27, 25, 18, 38, and 22. Three types of RES are considered: DFIG, PMSG-GFL, and PMSG-GFM.

FSI and power index calculation results

Figs. 6 through 8 illustrate the effects of a stepwise increase in frequency control parameters on a single RES farm, allowing for the observation of variations. Figure 6 compares the results from simulations, fitting methods, and iterative methods. Figure 7 presents results under three different parameter variation scenarios, corresponding to different interconnection locations and choices of inertia and damping. The results with three different types of RES unit interconnections are displayed in Figure 8. Several conclusions can be drawn as follows:

(a) As shown in Figure 6, both the fitting and iterative methods demonstrate high accuracy in predicting index variations at the next step. The maximum error between the iterative method results and the simulation results does not exceed 7%;

(b) Fig. 7 indicates that variations in inertia have minimal impact on the quasi-steady-state (QSS) indexes fss and Pss, while damping variation has a lower impact than inertia on transient indexes such as fmin, RoCoFmax , and Pmax;

(c) Based on conclusion (b), optimizing for improved QSS indexes fss and Pss is primarily related to damping. Therefore, the unit cost for damping is much higher than that for inertia. Consequently, inertia should be prioritized for transient index improvement.

Fig. 6  Comparison of simulation, fitting and iteration results: (a) Frequency nadir; (b) Extreme RoCoF; (c) Transient maximum power.

Fig.7  Comparison of iteration results with different parameter variation scenarios: (a) Frequency nadir; (b) Extreme RoCoF; (c)Quasi-steady-state frequency;  (d) Transient maximum power; (e) Quasi-steady-state power.

Fig. 8 Comparison of iteration results with different RES type interconnection: (a) Frequency nadir; (b) Extreme RoCoF; (c) Transient maximum power.

(d) Damping exhibits the same influence on QSS regional and global frequencies. As such, the global frequency is used as the optimization target in Section III;

(e) Significant regional divergence is observed in index variations with the same quantities of inertia and damping. For example, inertia is more likely to be allocated to Farm 10 than to Farm 7;

(f) Figure 8 indicates that PMSG-GFMs exhibit the best performance in enhancing extreme RoCoFs and certain regional frequency nadirs. However, they negatively affect regional frequencies SG 5 and 8. Additionally, PMSG-GFMs show the highest increase in transient power with the same increment in inertia compared to the other two RES types. The inertia limit for PMSG-GFM may be smaller than that for GFL units. DFIG-GFLs appear to be the least sensitive to parameter variations, as indicated by the lowest increment in FSIs. This behavior is mainly attributed to the cascaded control structure of DFIG-GFLs, where active power response is controlled by rotor-side dynamics, unlike PMSG-GFLs, which are directly managed by virtual controllers.

Case A:  Parameter optimization considering different object functions

The allocation of frequency control parameters is achieved by solving the optimization problem presented in equation (44). PMSG-GFL-based RES farms are initially considered for optimization, with the extreme objective function selected. The FSI constraints are set as 59.8 Hz, 1 Hz/s, and 59.875 Hz for frequency nadir, extreme RoCoF, and QSS frequency, respectively. Additionally, transient and QSS power index constraints are applied. The optimization step size for inertia and damping is set to and p.u.. Damping optimization is performed first to satisfy the quasi-steady-state constraints for FSI and power indexes, based on the conclusions drawn from the single-step iteration. The primary focus is on the inertia allocation process aimed at enhancing transient FSIs while restricting the transient power index, as illustrated in Figure 9.

Fig.9 Iteration Process of PMSG-GFL-based RES farm parameter optimization with step size 1 p.u.: (a) Allocation of inertia J; (b) Transient maximum power; (c) Frequency nadir; (d) extreme RoCoF

A clear preference for inertia allocation emerges in Figure 9(a) and (b). The maximum power  of RES Farm 10 first reaches the power constraint of 1.5 p.u., followed by Farms 5, 6, 8, and 10 as inertia is allocated to the corresponding RES farms. Figures 9(c) and (d) demonstrate that the installation of inertia significantly improves frequency nadirs and reduces extreme RoCoFs, both globally and regionally. The objective of constraining all transient FSIs within their respective boundaries with the minimum amount of inertia is thus achieved. Furthermore, with increasing total inertia, the growth rate of FSI becomes slower, suggesting the presence of boundaries at the turning points for the two transient FSIs.

In Section III, two objective functions for improving FSI performance are introduced: the extreme objective and the average objective. Figures 10 and 11, along with Table I, present the allocation and simulation results for comparison:

Fig. 10 Inertia and Damping allocation results with (a) average object optimization; (b) extreme object optimization

TABLE Ⅰ

Statistical results of optimization with two object functions

|  | base | extreme | Average |
| --- | --- | --- | --- |
|  | 40 | 213.4 | 239.2 |
|  | 0 | 84.6 | 81.4 |
| fmin | 59.690 | 59.808 | 59.810 |
| RoCoFmax | -1.419 | -1.000 | -1.008 |
| fsysmin | 59.756 | 59.841 | 59.847 |
| fss | 59.860 | 59.875 | 59.875 |

A comparison of the allocation results in Figure 10 and Table I indicates that the average objective requires a larger allocation of inertia to promote transient FSIs globally rather than locally (i.e., focusing on the lowest regional frequency in SG7). A higher global frequency nadir is recorded, supporting the comparison of the two objective functions. Additionally, the allocation for the extreme objective reveals a preference for placing inertia and damping close to the disturbance point (Node 21), as Farm 10 has the shortest electrical distance to the disturbance point, followed by Farms 5, 6, 8, and 10. In contrast, Farms 1–3 are less likely to have inertia allocated under the extreme objective. However, Farm 2 plays a leading role in inertia support when the average objective is considered. The allocation results are validated, as all FSIs and power indexes remain within the set boundaries in the time-sequence simulation results shown in Figure 11.

Fig. 11 Simulation results with: (a) average object optimization; (b) extreme object optimization

Case B: Parameter optimization of various types of RESs

In Case B, the optimization process is applied to a system comprising three types of interconnected RES units. The allocation strategy for the PMSG-GFL, detailed in Case A, is adopted. The DFIG-GFL system is analyzed, and the parameter optimization process is presented in Figure 12. The FSI and power index constraints are identical to those in Case A.

Fig.12 Iteration Process of DFIG-GFL-based RES farm parameter optimization with step size 1 p.u.: (a) Allocation of inertia J; (b) Transient maximum power; (c) Frequency nadir; (d) extreme RoCoF

Compared to the inertia optimization process for the PMSG-GFL system, shown in Figure 9, the DFIG-GFL allocation process requires more iterations before the transient FSIs reach the selected boundaries. Both the frequency nadir and extreme RoCoF exhibit turning points, beyond which further increases in total inertia do not result in improvements to the transient FSIs. This phenomenon supports the hypothesis proposed in Case A. Specifically, after iteration 640 (), any additional allocation of inertia to the RES farms in the test system negatively impacts frequency stability. In other words, there are inherent limits to the FSIs when FSI constraints are chosen prior to the optimization process. The optimization becomes more challenging with a larger frequency nadir constraint or a smaller extreme RoCoF constraint.

Fig.13 Feasible region and FSI trajectory of DFIG-GFL optimization

In Figure 13(a), a trajectory representing the two transient FSIs is plotted in red to facilitate iteration process analysis and FSI constraint definition. The trajectory can be divided into three stages: Stage I: The process begins at the initial point, with both FSIs improving simultaneously through inertia JJ allocation. The PMSG-GFL iteration process in Case A remains in this stage throughout. Stage II: Following the blue trajectory to the expected endpoint, only the extreme RoCoF improves as inertia JJ increases. The maximum frequency nadir of 59.763 Hz is recorded at the transition point between Stage I and Stage II. Stage III: As inertia JJ increases, both the frequency nadir and extreme RoCoF decrease. The boundary value of extreme RoCoF, 1.051 Hz/s, is recorded at the turning point between Stage II and Stage III. This point marks the end of the optimization process, as additional allocation of inertia no longer improves the two transient FSIs.

Using the boundary values and the trajectory of the two FSIs, a feasible region is defined to verify the FSI constraints, as shown in Figure 13(a). The selection of FSI constraints must remain within the boundary defined by the frequency nadir and extreme RoCoF, as well as the curve from Stage II. The feasible region’s boundary is further validated through a 27×56 optimization scenario with flexible FSI constraints within the specified range ,, as shown in Figure 13(b). The optimization results confirm the effectiveness of this feasible region definition.

TABLE Ⅱ

Statistical results of optimization with two object functions

|  | PMSG-GFL |  | DFIG-GFL |  | PMSG-GFM |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Base | opt | Base | opt | base | opt |
|  | 40 | 122.3 | 40 | 538.7 | 40 | 58.2 |
|  | 0 | 84.6 | 0 | 192.2 | 20 | 231.1 |
| fmin | 59.690 | 59.780 | 59.640 | 59.755 | 59.629 | 59.763 |
| RoCoFmax | -1.419 | -1.099 | -1.594 | -1.100 | 1.680 | -1.100 |
| fss | 59.860 | 59.875 | 59.843 | 59.875 | 59.840 | 59.875 |

Following the discussion of FSI constraints during the DFIG-GFL parameter optimization, it is clear that the previously selected FSI constraints , can no longer be applied, as no further improvement in the FSIs is observed within these limits. Conservative values of FSI constraints , are chosen for comparison, allowing the inertia and damping allocation results for the three types of RES units to be compared, as shown in Table II and Figure 14. As concluded in Section A (f), the total inertia demand for the three types follows the relationship: DFIG-GFL > PMSG-GFL > PMSG-GFM. The PMSG-GFM integrated scenario requires only 18.2 p.u. of additional inertia to meet the FSI constraints, while the DFIG-GFL system requires 538.7 p.u. of inertia, nearly ten times that needed for the PMSG-GFM system. This disparity is primarily attributed to the control structure of the DFIG-GFL system, as well as the significantly lower initial transient FSIs among the three types.

Fig.14 Inertia and Damping allocation results with: (a) PMSG-GFLs; (b) DFIG-GFLs;(c) PMSG-GFMs

Conclusion and Future Works

This paper investigates the frequency stability of renewable energy sources (RES)-integrated systems. The key conclusions are as follows:

(1) A state-space disturbance equation (SSDE) for frequency stability analysis is proposed based on the modeling of four types of generating units. This equation clarifies the load disturbance allocation mechanism, which equivalently converts power step disturbances in the load into step disturbances at the terminals of synchronous generators (SGs) and RES units.

(2) The definition of modes and residues within the disturbance input-frequency output structure is introduced based on the SSDE. A response-fitting method, inspired by modal analysis, is proposed. Analytical solutions for frequency stability and power indices are derived to reduce computational burdens;

(3) An iterative optimization method for frequency parameter tuning is presented, based on the SSDE and matrix perturbation theory, and verified through various optimization scenarios. A feasible region is defined to guide the selection of frequency stability index (FSI) constraints. The boundary of these constraints indicates that there are limits to frequency parameter tuning, and extensive inertia allocation is not recommended for frequency stability concerns.

Future work will focus on the theoretical analysis of the feasible region’s boundary, which may be closely related to variations in the peaks and troughs of oscillatory waves in frequency responses.

Appendix

The matrices of the SG model in Equation (2) are presented in (A1):

(A1)

The matrixes of the PMSG-GFM model in (4) are:

,    (A2)

The matrixes of the PMSG-GFL model in (7) can be expressed as:

(A3)

The fifth-order matrixes of DFIG-GFL in (7):

(A4)

The matrix elements in (10) can be expressed as follows:

(A5)

(A6)

The matrix elements in (12) (13) can be expressed as follows:

(A7)

(A8)

References

- R. Fu, X. Wang, Y. Zhang and L. Li, “Inertial and primary frequency response of pll synchronized vsc interfaced energy resources,” IEEE Trans. on Power Syst., vol. 37, no. 4, pp. 2998-3013, July 2022.

- R. Eriksson, N. Modig, and K. Elkington, “Synthetic inertia versus fast frequency response: A definition,” IET Renew. Power Gener., vol. 12, no. 5, pp. 507–514, Apr. 2018.

- Y. Wu, W. Yang, Y. Hu, and P. Q. Dzung, ‘‘Frequency regulation at a wind farm using time-varying inertia and droop controls,’’ IEEE Trans. Ind. Appl., vol. 55, no. 1, pp. 213–224, Feb. 2019.

- M. Garcia and R. Baldick, “Real-time co-optimization: Interdependent reserve types for primary frequency response,” in Proc. 10th ACM Int. Conf. Future Energy Syst., 2019, pp. 550–555.

- J.Driesen and K.Visscher, “Virtual synchronous generators,” 2008 IEEE Power and Energy Society General Meeting - Conversion and Delivery of Electrical Energy in the 21st Century, Pittsburgh,PA,USA, 2008, pp. 1-3.

- S. Wang, J. Hu and X. Yuan, “Virtual synchronous control for grid-connected dfig-based wind turbines,” IEEE J. Emerg. Select. Topics Power Electron., vol. 3, no. 4, pp. 932-944, Dec. 2015.

- P. Kundur, Power System Stability and Control, 1st ed. New York, NY,

USA: McGraw-Hill Educ., 1994.

- H. Bevrani, Robust Power System Frequency Control. New York, NY,

USA: Springer, 2009.

- J. Fradley, R. Preece and M. Barnes, “The influence of network factors on frequency stability,” IEEE Trans. on Power Syst., vol. 35, no. 4, pp. 2826-2834, July 2020.

- D. -J. Lee and L. Wang, “Small-signal stability analysis of an autonomous hybrid renewable energy power generation/energy storage system Part I: time-domain simulations,” in IEEE Transactions on Energy Conversion, vol. 23, no. 1, pp. 311-320, March 2008.

- P. M. Anderson and M. Mirheydar, “A low-order system frequency response model,” IEEE Trans. on Power Syst., vol. 5, no. 3, pp. 720-729, Aug. 1990.

- I. Egido, F. Fernandez-Bernal, P. Centeno and L. Rouco, “Maximum frequency deviation calculation in small isolated power systems,” IEEE Trans. on Power Syst., vol. 24, no. 4, pp. 1731-1738, Nov. 2009.

- Denis Lee Hau Aik, “A general-order system frequency response model incorporating load shedding: analytic modeling and applications,” IEEE Trans. on Power Syst., vol. 21, no. 2, pp. 709-717, May 2006.

- H. Ye, W. Pei and Z. Qi, “Analytical modeling of inertial and droop responses from a wind farm for short-term frequency regulation in power systems,” IEEE Trans. on Power Syst., vol. 31, no. 5, pp. 3414-3423, Sept. 2016.

- A. Bera, B. R. Chalamala, R. H. Byrne and J. Mitra, “Sizing of energy storage for grid inertial support in presence of renewable energy,” IEEE Trans. on Power Syst.,  vol. 37, no. 5, pp. 3769-3778, Sept. 2022.

- N. Nguyen and J. Mitra, “Reliability of power system with high wind penetration under frequency stability constraint,” IEEE Trans. on Power Syst., vol. 33, no. 1, pp. 985-994, Jan. 2018.

- C. Liang, P. Wang, X. Han, W. Qin, R. Billinton and W. Li, “Operational reliability and economics of power systems with considering frequency control processes,” IEEE Trans. on Power Syst., vol. 32, no. 4, pp. 2570-2580, July 2017.

- H. Ahmadi and H. Ghasemi, “Security-constrained unit commitment with linearized system frequency limit constraints,” IEEE Trans. on Power Syst., vol. 29, no. 4, pp. 1536-1545, July 2014.

- S. S. Oskouee, S. Kamali and T. Amraee, “Primary frequency support in unit commitment using a multi-area frequency model with flywheel energy storage,” IEEE Trans. on Power Syst., vol. 36, no. 6, pp. 5105-5119, Nov. 2021.

- L. Badesa, F. Teng and G. Strbac, “Conditions for regional frequency stability in power system scheduling—part I: theory,” IEEE Trans. on Power Syst., vol. 36, no. 6, pp. 5558-5566, Nov. 2021.

- L. Badesa, F. Teng and G. Strbac, “Conditions for regional frequency stability in power system scheduling—part II: application to unit commitment,” IEEE Trans. on Power Syst., vol. 36, no. 6, pp. 5567-5577, Nov. 2021.

- H. Gao et al., “Common-mode frequency in converter-integrated power systems: definition, analysis, and quantitative evaluation,” IEEE Trans. on Power Syst.,  vol. 37, no. 6, pp. 4846-4860, Nov. 2022.

- B. K. Poolla, D. Groß and F. Dörfler, “Placement and implementation of grid-forming and grid-following virtual inertia and fast frequency response,” IEEE Trans. on Power Syst.,  vol. 34, no. 4, pp. 3035-3046, July 2019.

- S. S. Guggilam, C. Zhao, E. Dall’Anese, Y. C. Chen and S. V. Dhople, “Optimizing DER participation in inertial and primary-frequency response,” IEEE Trans. on Power Syst., vol. 33, no. 5, pp. 5194-5205, Sept. 2018.

- M. Toulabi, S. Bahrami and A. M. Ranjbar, “An Input-to-State Stability Approach to Inertial Frequency Response Analysis of Doubly-Fed Induction Generator-Based Wind Turbines,” IEEE Transactions on Energy Conversion, vol. 32, no. 4, pp. 1418-1431, Dec. 2017.

- P. Ju, Y. Zheng, Y. Jin, C. Qin, Y. Jiang, and L. Cao. “Analytic assessment of the power system frequency security,” IET Gener. Transm. Distrib., vol.15, no.15, pp. 2215-2225, March 2021.

- S. H. Chen, Matrix Perturbation Theory in Structural Dynamic Design. Beijing, China: Science, 2007.

- A. Tayyebi, D. Groß, A. Anta, F. Kupzog and F. Dörfler, “Frequency stability of synchronous machines and grid-forming power converters,” IEEE J. Emerg. Select. Topics Power Electron., vol. 8, no. 2, pp. 1004-1018, June 2020

- J. Ying, X. Yuan, J. Hu and W. He, “Impact of inertia control of DFIG-based WT on electromechanical oscillation damping of SG,” IEEE Trans. on Power Syst., vol. 33, no. 3, pp. 3450-3459, May 2018

- J. Fradley, R. Preece and M. Barnes, “The influence of network factors on frequency stability,” IEEE Trans. on Power Syst., vol. 35, no. 4, pp. 2826-2834, July 2020.

- P. M. Anderson and A. A. Fouad, Power System Control and Stability. Hoboken, NJ, USA: Wiley, 2002

- G. C. Verghese, I. J. Perez-Arriaga, and F. C. Schweppe, “Selective
modal analysis with applications to electric power systems, Part
II: The dynamic stability problem,” IEEE Trans. Power App. Syst.,
vol. PAS-101, no. 9, pp. 3126–3134, Sep. 1982.

- J. J. Sanchez-Gasca et al., “Inclusion of higher order terms for small-signal (modal) analysis: committee report-task force on assessing the need to include higher order terms for small-signal (modal) analysis,” IEEE Trans. on Power Syst., vol. 20, no. 4, pp. 1886-1904, Nov. 2005

- T. S. Borsche and F. Dorfler, “On placement of synthetic inertia with ¨explicit time-domain constraints,” 2017, arXiv:1705.03244.

Ruilin Gao, H F Wang are with the School of Electrical Engineering, Sichuan University, Chengdu 610065, China. Corresponding author is Ruilin Gao, Email: ruilin_gao_scu@163.com.