Received: Added at production	Revised: Added at production	Accepted: Added at production

DOI: xxx/xxxx

A R T I C L E T Y P E

Compact All-frequency Reflectionless Filtering Power Divider with High-Isolation and Extended Out-of-Band Suppression

Gangxiong Wu1,2,3       Yang Jin2       Hao Wu1       Wei Zhang1,2,3       Ruirui Jiang1,3       Jin Shi1,3

1 School of Information Science and Technology, Nantong University, Nantong, China

2 School of Microelectronics and School of Integrated Circuits, Nantong University, Nantong, China

3 Research Center for Intelligent Information Technology, Nantong University, Nantong, China

Correspondence

Jin Shi School of Information Science and Technology, Nantong University, Nantong226019 China.

Email: jinshi0601@hotmail.com

Funding information

This work was supported in part by the National Natural Science Foundation of China under Grant 62201292, 62101266, and 62301285, by the Natural Science Research Project of Jiangsu Higher Education Institutions under Grand 22KJB140004 and Grand 23KJB510024, and by the China Scholarship Council under Grand 202308320210.

Abstract

This paper presents a filtering power divider (FPD) that integrates all-frequency absorptive and isolative functionalities while achieving extended out-of-band suppression. The proposed design employs an absorption-isolation network (AIN), composed of microstrip lines and resistors, to ensure simultaneous reflectionless input operation and high isolation without degrading filtering performance or requiring additional absorption circuits. A cascaded coupled-line (CL) structure facilitates broadband filtering and power division, while parallel half-wavelength stubs enhance out-of-band suppression. Additionally, a π-type defected ground structure (DGS) is incorporated to mitigate high-frequency spurious signals, further extending out-of-band suppression without increasing circuit footprint. Theoretical analysis and parametric optimization are performed to establish impedance conditions for achieving reflectionless operation and optimizing circuit performance. A fabricated prototype, centered at 2 GHz, exhibits a 72% 3-dB bandwidth, input reflection below -10 dB across the entire frequency range, all-band isolation exceeding 21.1 dB, and stopband attenuation greater than 25.2 dB up to 6.8f₀. These results validate the proposed FPD as a compact and high-performance solution for modern RF front-end applications requiring spectral purity and robust isolation.

K E Y W O R D S

Filtering power divider (FPD), reflectionless, isolation, out-of-band suppression, defected ground structure (DGS)

- INTRODUCTION

Filters and power dividers (PDs) are fundamental components in radio frequency (RF) front-end systems, enabling frequency selection and signal distribution. Over the past decades, filters have seen significant advancements in broadband architecture1, high-selectivity designs2, reconfigurable solutions3, and wide-stopband implementations for RF interference suppression4-9. Similarly, PDs have evolved to support arbitrary power division ratios10, multi-band operation11, and high isolation12. Traditionally, filters and PDs were designed as separate components with distinct functionalities. However, the increasing demand for compact, multifunctional RF systems has led to the development of filtering power dividers (FPDs), which integrate filtering and power distribution within a single, compact structure13-15. This integration reduces circuit size, minimizes insertion loss (IL), and enhances overall system efficiency, making FPDs highly attractive for modern RF applications.

Reflectionless filtering power dividers (RFPDs) have gained attention for their ability to suppress out-of-band reflections, mitigate electromagnetic interference (EMI), and enhance system stability16. While effective in reducing stopband reflections, their role in electromagnetic compatibility (EMC) remains underexplored. As RF systems become increasingly congested, there is a need for power dividers that minimize reflections, provide wideband absorption, and ensure high isolation to prevent signal degradation and interference. Several techniques have been proposed to achieve absorptive circuits in RFPDs. The design in 17 employs bandstop filters (BSFs) with resistors but suffers from large circuit sizes and limited broadband performance. Absorptive stubs18, while compact, often degrade passband flatness and require additional isolation resistors. Similarly, two-stub configurations19 exhibit suboptimal passband response and insufficient out-of-band suppression. Most RFPD designs treat absorption and isolation circuits separately, increasing circuit complexity while reducing efficiency. To address these challenges, recent research has focused on integrated design methodologies that simultaneously optimize reflectionless performance and filtering characteristics20-23. For instance, the composite structure in 20 merges absorption and isolation using a single resistor, yet it increases circuit complexity and degrades filtering selectivity. The isolation network proposed in 22 achieves output reflectionless behavior, but it lacks compactness and does not fully optimize filtering performance. Furthermore, designs utilizing three-line coupled structures24-26 offer a compact footprint, but they often suffer from degraded isolation, and filtering performance or require additional absorptive stubs to enhance reflectionless performance. Additionally, a major challenge in FPD design is the suppression of high levels of out-of-band signals, including harmonics and spurious emissions, which can interfere with adjacent frequency channels and violate EMC regulations27. Moreover, insufficient isolation between output ports can lead to crosstalk and signal degradation, further compromising EMC performance. Consequently, the development of a compact RFPD that simultaneously achieves broadband reflectionless operation and strong EMI suppression is crucial for modern RF applications.

This paper presents a novel compact RFPD that integrates an absorption-isolation network (AIN) for full-frequency reflectionless operation and a π-type defected ground structure (DGS) for enhanced stopband suppression. The AIN minimizes unwanted reflections, reducing EMI emissions and ensuring spectral purity, while the π-type DGS extends out-of-band suppression without increasing circuit complexity. The proposed RFPD achieves: 1). All-frequency reflectionless operation; 2). Enhanced stopband suppression; 3) Compact and integrated architecture; 4). Improved and all-band isolation performance. 5). Low IL and excellent frequency selectivity.

- CIRCUIT STRUCTURE AND DESIGN THEORY

The topology of the proposed compact all-frequency RBPF, illustrated in Figure 1, consists of a passband filtering circuit and an AIN. The bandpass response is achieved using cascaded coupled lines (CLs) loaded with half-wavelength open-circuit stubs (Zs2), where Ze1 and Zo1 represent the even-mode and odd-mode characteristic impedances of the input CL, while Ze2 and Zo2 correspond to the output CLs. The AIN is composed of a pair of quarter-wavelength transmission lines (TLs) with an impedance of Z2, a half-wavelength TL (Z3), ground resistors (R1), and a parallel resistor (R2), ensuring both input reflectionless operation and output isolation. The electrical length of the TLs (θ) is set to 90° at the center frequency (f0), optimizing performance across the desired frequency range.

- (b)                                                                                 (c)

F I G U R E 2  Equivalent circuits. (a) left-hand even-mode excitation. (b) right-hand even-mode excitation. (c) right-hand odd-mode excitation.

FIGURE 1.  Schematic of the proposed RFPD.

Due to the inherent symmetry of the proposed configuration (Figure 1), it can be analytically decomposed into its odd- and even-mode equivalent circuits, as illustrated in Figure 2. Specifically, Figure 2(a) corresponds to the even-mode excitation at the left input, Figure 2(b) represents the even-mode excitation at the right input, and Figure 2(c) illustrates the odd-mode excitation at the left input respectively. These equivalent circuits provide a foundation for theoretical analysis. The current and voltage conditions in CL1, which is terminated by a single impedance termination (Zi), and the two-port CL2 in Figure 2. are defined as follows: I2 = 0, I4 = -(V4/Zi) and I2 = I4 =0, where i=e (even-mode) or o (odd-mode). The even- and odd-mode impedances Ze and Zo of the integrated AIN under respective excitations are derived and can be expressed by:

(1)

(2)

Based on circuit theory ([V]=[I][Z]) and TL theory28, the impedance matrix [Z] of CL1 () and CL2 () can be calculated as follows:

(3)

(4)

where Za1 = (Ze1 +Zo1)/2, Zb1 = (Ze1 -Zo1/2), Za2 = (Ze2 +Zo2)/2, and Zb2 = (Ze2 -Zo2)/2.

(1) Left-Side Even-Mode Excitation: For the left-side even-mode excitation illustrated in Figure 2(a), the input impedance ZEline is calculated as:

(5)

(2) Right-Side Even-Mode Excitation: For the right-side even-mode excitation depicted in Figure 2(b), the input impedance ZERin is derived as:

(6)

(3) Right-Side Odd-Mode Excitation: For the right-side odd-mode excitation depicted in Figure 2(c), the input impedance ZORin is expressed as:

(7)

Therefore, the return loss (RL) of the RFPD can be derived from the reflection coefficient () at port 1 under even-mode excitation (as shown in Figure 2(a)):

(8)

Similarly, the IL (S21) and isolation (S32) can be extracted from the  and odd-even reflection coefficient (& ) at port 2:

(9)

(10)

To achieve perfect reflectionless characteristics across the entire frequency band, it is essential to ensure |S11|==0 at f = 0 (DC) and f = 2nf0 (n=0, 1, 2, …). Based on Eqs. (5) and (8), this condition is satisfied when:

R1=2Z0                                                                                               (11)

Additionally, since the electrical length θ = 90◦ at f0, the AIN achieves excellent isolation, as indicated by |S32|=0. From Eqs. (2)-(10), R2 is determined as:

(12)

From the above analysis, it is evident that the parameters of the filter’s performance depend on only five key parameters: Ze1, Zo1, Ze2, Zo2, and Z1. These parameters are determined using Eqs. (1)-(12), ensure superior bandwidth and frequency selectivity characteristics. As a result, the numerically optimized parameters are as follows: Ze1=157 Ω, Zo1=43 Ω, Ze2=128 Ω, Zo2=38 Ω, and Z1=39 Ω. The resistor values, calculated from Eqs. (11) and (12) are: R1=100 Ω and R2=360 Ω. Finally, Z2 and Z3 can be fine-tuned to achieve optimal all-frequency reflectionless and isolation.

Figure 3(a) illustrates the numerically calculated performance of the proposed RFPD. The results show that the input RL and output-port isolation consistently exceed 12.6 dB and 20.5 dB, respectively, across the entire frequency range. Additionally, the 3-dB fractional bandwidth (FBW) reaches 74%, with a stopband rejection of over 32.5 dB. To further refine the design, a detailed parameter analysis is conducted to identify key factors influencing the RFPD’s performance. Figure 3(b) demonstrates the impact of Z1. As Z1 increases, both the passband FBW and the absorption performance improve, whereas the stopband rejection and isolation deteriorate. Figures 3(c)-(f) examine the effect of AIN. Notably, the parameters Z2, Z3, R1 and R2 do not affect the IL (S21). However, their specific impact on other performance metrics is as follows:

Z2: As Z₂ increases, in-band isolation initially improves but later deteriorates. Meanwhile, matching performance is enhanced, whereas absorption at the band edges declines.

Z3: Increasing Z3 enhances the absorption levels but degrades in-band matching performance. Notably, Z3 does not affect isolation.

R1: This parameter significantly influences both absorption and isolation. As R1 increases, absorption levels and isolation improve initially but degrade beyond a certain threshold.

R2: The impact of R2 is limited to isolation, with no effect on absorption. Specifically, as R2 increases, the isolation at the center frequency initially improves and then deteriorates, while the isolation at the passband edges shows consistent improvement.

These results provide a comprehensive understanding of the interplay between design parameters and the RFPD’s performance metrics, guiding further optimization of the proposed structure to achieve balanced performance across reflectionless operation, isolation, absorption, and bandwidth.

| (a) | (b) | (c) |
| --- | --- | --- |
| (d) | (e) | (f) |

FIGURE 3.  (a) Theory results of the proposed RFPD. (Ze1=157 Ω, Zo1=43 Ω, Ze2=128 Ω, Zo2=38 Ω, Z1=39 Ω, Z1=39Ω, Z2=150Ω, Z3=145Ω, R1=100 Ω, and R2=360 Ω). Effects of (b) Z1. (c) Z2. (d) Z3. (e) R1. (f) R2.

| (a) | (b) |
| --- | --- |
| (c) |  |
| FIGURE 4. Proposed LPF scheme. (a) Full-wave simulation structure. (b) It’s the LC equivalent circuit. (c) S-parameters. |  |

STOPBAND EXTENSION OF THE RFPD

Harmonics and spurious signals often emerge beyond the passband due to non-ideal transmission characteristics, parasitic mode excitations, and material imperfections. These unwanted signals degrade system performance by introducing interference and reducing signal purity. Effective stopband suppression is therefore crucial for enhancing selectivity, ensuring signal integrity and meeting the stringent requirements of modern high-frequency communication systems. To address these challenges, the proposed RFPD integrates a π-shaped DGS, which acts as a resonant lowpass filter (LPF) to suppress harmonics in the stopband. Implemented on the substrate’s backside, the DGS enhances stopband attenuation without increasing the circuit footprint, thereby optimizing both performance and layout efficiency.

Figure 4(a) provides an overview and dimensional details of the proposed π-shaped DGS, which consists of a 50-Ω microstrip line (ML) positioned above the substrate and three π-shaped slots etched into the ground plane. The DGS slots can be equivalently modelled as a parallel LC circuit, facilitating theoretical analysis. In this configuration, the conducting metal regions act as inductors, while the slot gaps function as capacitors. The corresponding equivalent LC circuit, along with the extracted lumped values, is illustrated in Figure 4(b).

Figure 4(c) compares the results of the equivalent circuit model and full-wave electromagnetic simulations, showing strong agreement in both magnitude and overall trend, thereby validating the proposed equivalent circuit representation. As a result, integrating the π-shaped DGS significantly enhances upper-band suppression without compromising the circuit’s original performance or compact form factor. To implement this enhancement, the π-shaped DGS-based LPF is seamlessly incorporated into the output ports of the RFPD. This integration preserves the circuit’s compactness while improving selectivity.

FIGURE 5.  Comparison of responses with and without the DGS.

- (b)

FIGURE 7. The fabricated prototype. (a) Bottom view, (b) Top view.

FIGURE 6. The layout and dimensions of the proposed RFPD.

EXAMPLES FOR ENUNCIATIONS

To validate the practical feasibility of the proposed RFPD with enhanced out-of-band suppression, a prototype was simulated, fabricated and measured on a Rogers RO4003C substrate (εr = 3.55, tanδ = 0.0027) with a thickness of 1.524 mm. The proposed RFPD was simulated using the commercial 3D electromagnetic simulation software CST Microwave Studio (CST MWS), and its optimized layout is shown in Figure 6. As depicted in Figure 6, the final optimized circuital dimensions of the RFPD are as follows: L1=25.7 mm, w1=3 mm, Lc1=23.6 mm, wc1=0.6 mm, s1=0.16 mm, Lc2=23.4 mm, wc2=0.85 mm, s2=0.16 mm, L4=46.4 mm, w4=3.3 mm, L2=21.9 mm, w2=0.23 mm, L3=45.1 mm, w3=0.31 mm, L5=51.5 mm, w5=3 mm, R1=91 Ω, and R2=360 Ω. A fabricated prototype, produced using standard PCB technology, is shown in Figure 7. The core circuit occupies a compact area of 0.16λg2, where λg is the effective wavelength at the center frequency (f0=2.0 GHz).

Figure 8 presents a comparison of the simulated and measured results of the fabricated RFPD, demonstrating excellent agreement in magnitude and trend. As shown in Figure 8(a), the measured 3-dB FBW is 72% (1.28~2.72 GHz), with a minimum in-band IL of 0.68 dB. Two out-of-band transmission zeros (TZs), generated by the half-wavelength stubs, are positioned at 1.0 GHz and 3.0 GHz, significantly enhancing selectivity. The selectivity factor, defined as the ratio of 20-dB to 3-dB bandwidths, is 1.47. Furthermore, as shown in Figure 8(b), the measured RL (|S11|) at the input port exceeds 10 dB across the entire frequency range from DC to 4.5 GHz, demonstrating an absorptive bandwidth of 225%. Additionally, as illustrated in Figure 8(c), the measured isolation (|S32| & |S23|) between the output ports is greater than 21.2 dB within the passband and exceeds 28.3 dB outside the passband, demonstrating excellent isolation across the entire frequency range. Meanwhile, output-port matching (|S22| & |S33|) is well-maintained, with phase differences between ports 2 and 3 remaining at 0.36o. Figure 8(d) illustrates the measured stopband suppression, demonstrating that the RFPD, integrated with the π-type DGS, achieves a broad stopband with suppression levels exceeding 25.2 dB up to 6.8f0. Table 1 presents a comparative analysis of the proposed RBPF against existing designs in literature. The proposed design demonstrates superior performance, featuring wide bandwidth, low IL, high isolation, full frequency reflectionless operation, excellent frequency selectivity, compact circuit size, and broad, highly effective stopband suppression.

| (a) | (b) |
| --- | --- |
| (c) | (d) |
| FIGURE 8. Simulated and measured results. (a) S11 and S21. (b) S22 and S32. (c) The phase difference between ports 2 and 3. (d) S21 from DC to 16 GHz. |  |

| T A B L E 1 Performance comparison between the proposed and reported rbpfs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref. | f0 (GHz) | ILmin (dB) | 3-dB FBW (%) | In-band IO & Stopband (dB) | 10-dB ABW (GHz) | RC | Upper-Band Suppression | A.A.S | Size (λg2) |
| 17 | 2.04 | 0.9 | 13.6 | 13.5 | 1.11-2.65 | 3.34* | No | Yes | - |
| 23 | 3.0 | 1.48 | 12 | 20 | 1.97-4.03 | 1.39* | No | Yes | 2.44* |
| 20 | 2.0 | 2.1 | 5 | 18 | 1.5-2.5 | 2.44* | No | Yes | 3.4 |
| 21 | 1.0 | 1.3 | 15 | 18 | 0.5-1.5 | 2.43* | No | No | - |
| 22 | 2.0 | 0.62* | 40.1 | 8.5* | 0.94-2.77 | 2.2* | No | No | 1.67* |
| 19 | 2.0 | 0.2 | 82.5/99.5 | 18 | All-band | 1.38/1.22 | No | Yes | 0.19 |
| 25 | 2.0 | 0.49 | 29.2 | 17.3 | All-band | 1.49 | No | No | 0.38 |
| 24 | 2.36 | 0.32 | 50.8* | 16 | All-band | 1.4* | No | Yes | 0.21 |
| 26 | 1.8 | 0.4 | 70 | 8* | All-band | 1.8* | No | Yes | 0.04 |
| 18 | 1.75 | 0.9 | 51.4 | 14.3 | All-band | 1.56 | 2.6f0 >20dB | Yes | 0.12 |
| This work | 2.0 | 0.68 | 72 | 21.1 | All-band | 1.47 | 6.8f0 > 25.2dB | No | 0.16 |
| RC=¦FBW20dB/FBW3dB¦, ABW: Absorptive Bandwidth, A.A.S: Additional Absorptive Stubs, IO: Isolation between the two output ports, *: Estimated from the results. |  |  |  |  |  |  |  |  |  |

- CONCLUSIONS

In this paper, a wide-stopband FPD with an absorption-isolation composite network, offering high integration and excellent performance. The design utilizes cascaded CLs for wideband filtering and smooth passband response, while a half-wavelength open stub introduces controllable transmission zeros for enhanced selectivity. The absorption-isolation network ensures reflectionless and isolation properties without compromising filtering performance. Furthermore, the incorporation of the π-type DGS structure at the output port achieves effective out-of-band rejection while maintaining the compact size of the circuit. The proposed RBPF exhibits wide bandwidth, all-band reflectionless and isolation characteristics, as well as exceptional high-stopband suppression. These attributes make it highly suitable for applications in modern RF front-end systems, including wireless communication, radar, and satellite systems where EMC compliance is essential.

ACKNOWLEDGMENTS

This work was supported in part by the National Natural Science Foundation of China under Grant 62201292, 62101266, and 62301285, by the Natural Science Research Project of Jiangsu Higher Education Institutions under Grand 22KJB140004 and Grand 23KJB510024, and by the China Scholarship Council under Grand 202308320210.

DATA AVAILABILITY STATEMENT

The data that support the findings of this study are available on request from the corresponding author. The data are not publicly available dueto privacy or ethical restriction.

CONFLICT OF INTEREST

The authors declare no potential conflict of interest.

REFERENCES

- Huang Z, Wang X, Zhu L, Lu G. A novel ring type wideband bandpass filter: Proposal, design, and verification. IEE Trans. Circuits Syst. II, Exp. Briefs.2022; 9(3): 49-953.

- Wu Y-W, Hao Z-C, Lu R, Hong J-S. A high-selectivity D-band mixed-mode filter based on the coupled overmode cavities. IEE Trans. Microw. Theory Techn.2020; 68(6): 2331-2342.

- Zhang R, Gómez-García R, Peroulis D. Multifunctional bandpass filters with reconfigurable and switchable band control. IEEE Trans. Microw. Theory Techn.2019; 67(6): 2355-2369.

- Malherbe J. Wideband bandpass filter with extremely wide upper stopband. IEEE Trans. Microw. Theory Techn.2018; 66(6): 2822-2827.

- Narayane V-B, Kumar G. A selective wideband bandpass filter with wide stopband using mixed lumped-distributed circuits. IEEE Trans. Circuits Syst. II, Exp. Briefs.2022; 69(9): 3764-3768.

- Li J, Zhang Y, Zhou X. A compact ultra-wideband multimode bandpass filter with sharp rejection using stepped impedance open stub and series transformers. IEEE Trans. Microw. Theory Techn. 2020; 68(10): 4521-4530.

- Wang C, Liu S, Gómez-García R. A hybrid 3-D frequency selective structure featuring enhanced stopband suppression and reduced insertion loss. IEEE Trans. Antennas Propag 2023; 71(1): 234-242.

- Zhang B, Wu H, Yang, L. Design of multifunctional balanced-to-single-ended HTS ultrawideband filtering balun for suppressing radio frequency interference. IEEE Trans. Appl. Supercond 2023; 33(4): 1-7.

- Sun Z, Gao M, Zhang R. Wideband reconfigurable multifunctional absorber/reflector with bandpass/bandstop filtering and band-notch absorption for RCS and EMI shielding. IEEE Trans. Electromagn. Compat2023; 65(2): 654-664.

- Zhang N, Wang X, Zhu L, Lu G. A novel unequal power divider with general isolation topology: Design and verification. IEEE Microw. Wireless Technol. Lett. 2023; 33(1):19-22.

- Hayati M, Malakooti, S-A, Abdipour A. A novel design of triple-band Gysel power divider. IEEE Trans. Microw. Theory Techn. 2013; 61(10): 3558-3567.

- Song K, Xia F, Zhou Y, Guo S, Fan Y. Microstrip/slotline-coupling substrate integrated waveguide power divider with high output isolation. IEEE Microw. Wireless Compon. Lett. 2019; 29(2): 95-97.

- Liu S, Zhao S, Wang W, Wu Y. Packaged wideband highly selective filtering power divider with arbitrary phase difference and power dividing ratio on multilayer stripline. IEEE Trans. Microw. Theory Techn. 2023; 71(3): 1550-1562.

- Zhang X, Zhu L, Feng Q. Design of the filtering power divider with highly-selective bandpass filters. IEEE Trans. Microw. Theory Techn. 2022; 70(9): 3550-3562.

- Zhang Y, Liu H, Wang S, Chen Z. A synthesis theory of stripline dual-band filtering power divider with controllable full-frequency isolation. IEEE Trans. Circuits Syst. II, Exp. Briefs. 2023; 70(1): 155-159.

- Chen S, Liu H, Ma T, Zhang Z. Wideband unequal four-way filtering power divider with absorptive feature. Prog. Electromagn. Res. C. 2024; 147:1-10.

- Gómez-García R, Muñoz-Ferreras J-M, Psychogiou D. RF reflectionless filtering power dividers. IEEE Trans. Circuits Syst. II, Exp. Briefs. 2019; 66(6): 933-937.

- Zhang Y, Wu Y, Yan J, Wang W. Wideband high-selectivity filtering all-frequency absorptive power divider with deep out-of-band suppression. IEEE Trans. Plasma Sci. 2021; 49(7): 2099-2106.

- Zhang S, Liu H, Chen S, Wang Z, Fang S. Wideband filtering power divider with unequal power division ratio and all-frequency input absorptive feature. IEEE Trans. Circuits Syst. II, Exp. Briefs. 2024;71(3): 1136-1140.

- Lee B, Lee J, Lee G, Lee J. All-port-reflectionless narrowband filtering power divider topology with generic equations. IEEE Trans. Circuits Syst. I, Reg. Papers. 2022; 69(4): 1417-1426.

- Lee B, Nam S, Lee J. Filtering power divider with reflectionless response and wide isolation at output ports. IEEE Trans. Microw. Theory Techn. 2019; 67(7): 2684-2692.

- Chen Q, Zhang H, Wang Z, Liu H, Fang S.-J. A novel balanced-to-unbalanced all-port reflectionless filtering power divider without loading additional absorptive branches at input and output ports. Prog. Electromagn. Res. M. 2023; 121:157-167.

- Zhao X-B, Wei F, Yang L, Gómez-García R. RF balanced-to-single-ended out-of-phase/3-dB filtering power divider with differential-mode input-quasi-reflectionless behavior. IEEE Microw. Wireless Technol. Lett. 2024; 34(6): 733-736.

- Zhu Y-H, Cai J, Cao Y, Chen J-X. Compact wideband absorptive filtering power divider with a reused composite T-shape network. IEEE Trans. Circuits Syst. II, Exp. Briefs. 2023; 70(3): 899-903.

- Wu G-X, Jin Y, Zhang W, Wu H, Jiang R.-R, Shi J. Multi-functional balanced-to-single-ended filtering power divider with all-band output reflectionless and common-mode suppression. AEU-Int. J. Electron. Commun. 2024; 178: 155266.

- Zhang Y, Liu H-M, Chen S-Y, Wang, Z, Fang S. All-port-reflectionless wideband filtering power divider using five-line coupled structure. IEEE Microw. Wireless Technol. Lett. 2025; 35(1):31-34.

- Wang J, Zhang H, Li S. Harmonic suppressed Wilkinson power divider using parallel resonant shunt stub. IEEE Trans. Microw. Theory Techn 2021; 69(8): 3750-3758.

- Pozar D-M. Microwave engineering.4th ed. New York, NY: Wiley, 2001.