# Influence of pedestrian grouping dynamics on evacuation efficiency in underground flooded stairs: a 3D numerical assessment

Carlos H. Aparicio Uribe a , Beniamino Russo a , Jackson Téllez-Álvarez a, b , Eduardo Martínez-Gomariz a, c

a Flumen Research Institute, Universitat Politècnica de Catalunya – Centre Internacional de Mètodes Numèrics en Enginyeria, Campus Nord, Carrer de Jordi Girona, 1-3, B0 S1, 08034, Barcelona, Spain

b Consorci Besòs Tordera—CCB Serveis Mediambientals, SAU; 08403, Granollers, Spain

c Innovation Department, Aigües de Barcelona, 08009, Barcelona, Spain

- Beniamino Russo beniamino.russo@upc.edu

- Carlos Humberto Aparicio Uribe carlos.humberto.aparicio@upc.edu

Abstract: The increasingly recurrent urban floods pose significant risks to underground spaces and their users. Often essential in these scenarios, stairs present unique challenges during floods as they serve as pathways for evacuation and for the incoming water. With most of the prior studies focusing on studying a single individual, the impact of group dynamics on flooded stairs remains underexplored. This study addresses this gap investigating how these dynamics influence evacuation efficiency and safety through a 3D numerical analysis. This research introduces realistic human body dummies along the stairs to simulate various pedestrian arrangements under a discharge of 0.423 m³/s. Hydrodynamic forces and Momentum of Flow (MF) and Specific Force Per Unit Width (SFPUW) safety criteria are calculated to evaluate hazardous conditions. Pedestrian arrangements include a single individual at different positions, as well as tandem and side-by-side arrangements. Results revealed significant variations in the water-dragging forces based on positions and grouping dynamics. Notably, compared to isolated cases, side-by-side arrangements exhibited increased water-exerted force on both side-by-side and downstream individuals, while tandem configurations showed reductions in dragging forces on trailing pedestrians. The affectation of forces diminished when the separation between them increased. This study highlights potential bottlenecks, evaluates evacuation strategies, and provides insights to enhance safety protocols in underground infrastructure during flood events, contributing to urban resilience and flood risk mitigation strategies.

Keywords: Urban resilience, underground space, human instability, flooded stairs, flood risk mitigation.

## INTRODUCTION

In the last decades, cities have experienced a demographic explosion. It is reflected in the densification or development of habitable areas for human activities. According to the United Nations (2018), this trend will continue and by 2050, approximately two-thirds of the global population will reside in urban areas. This process is directly linked to an increased concentration of people in specific places, consequently triggering some environmental disruptions (Aparicio Uribe et al., 2022). For instance, a common disturbance is the reduction of permeable soils which exacerbates flood events (Ribeiro et al., 2021). Under current climate change conditions, floods have become the most frequent natural hazard in recent years (CRED-UNDRR, 2020). Therefore, cities are expected to experience more recurrent and severe floods (Kvočka et al., 2016; Martínez-Gomariz et al., 2019; Monjo et al., 2022). Recent examples between 2021 and 2024 could illustrate this (BBC News, 2021a, 2021b; Iagua Magazine, 2021; El Mundo News, 2022a, 2022b; Torres & Olaizola, 2023; & Franca et al., 2024).

In this context, underground infrastructures and their associated spaces, by their nature, are particularly susceptible to flooding (Yu et al., 2019; Forero-Ortiz et al., 2020a, 2020b; Forero-Ortiz & Martínez-Gomariz, 2020). These infrastructures are vital to the stability of cities by contributing to economic security and facilitating human and cargo transport (Avci & Ozbulut, 2018). Thus, it is paramount to ensure an effective response to extreme weather events that could impact them, maintaining both; functionality and safety during critical episodes (Russo et al., 2020a, 2020b). Thus, practical knowledge to understand their vulnerabilities is necessary to strengthen urban resilience (Lyu et al., 2018).

In underground spaces, stairs could be seen as a critical element for evacuation. Likely, evacuation routes include transit over stairs. Besides, during floods, they became at the same time the preferred pathway for the incoming flux. The study of underground evacuation through flooded stairs has been triggered in recent years. Several authors have studied it theoretically, numerically and experimentally (Aparicio Uribe et al., 2024). Important inputs like mechanics-based analysis of human instability or safety criteria among others have been provided by several authors (Takedomi et al., 2001; CCIDUS, 2002, Ishigaki et al., 2005, 2008a, 2008b, 2009, 2010; Onishi et al., 2007; Shao et al., 2015; Joo & Kim, 2015; Lee et al., 2016; Kim et al., 2018; Hou et al., 2022; Tellez-Alvarez et al., 2022; Li et al., 2022 & Liang et al., 2023). But, as a common factor, these studies were developed under the idealised assumption of a single person using the stairs. However, these spaces can become crowded during emergencies, with multiple people attempting to evacuate simultaneously (Jiang et al., 2014).

Given the inevitability of human activity in flood-prone spaces (Cox et al., 2010; Russo et al., 2013, Martínez-Gomariz et al., 2016a), improving our knowledge of evacuation dynamics and flood-related hazards in underground areas is crucial for effective mitigation measurements (Musolino et al., 2020; Dias et al., 2021; Okamoto et al., 2022). These spaces tend to have high concentrations of people (e.g. peak hours), increasing the likelihood of accidents and fatalities. Thus, further research on the specific constraints and characteristics in the urban cluster, such as stairs-human-flow interactions is needed. Thoroughly understanding how this interaction might affect the safety conditions outlined in previous studies is pertinent. This could be addressed through experimental and/or numerical campaigns involving simultaneously two or more subjects on the stairs.

The main objective of this paper is to study, through a 3D numerical campaign, how pedestrian grouping dynamics impact evacuation efficiency in flooded stairs. This also aims to assess evacuation strategies, identify possible bottlenecks and offer insights into optimising evacuation procedures in such conditions. Based on a real-scale model constructed at a laboratory of the Technical University of Catalonia (UPC), the commercial software FLOW-3D (Flow Science 2023) was employed for numerical analysis. A force analysis on human dummies is performed and to complement the results the flood-related hazard assessment is carried out based on the safety criteria presented in the literature.

## METHODOLOGICAL APPROACH

### Mechanics-based analysis of human instability in flooded stairs

The three possible instability mechanisms during human interactions with floods are: floating, toppling and sliding. The first is common in high water depths and low velocities, contrasting with sliding common in low water depths and high velocities, meanwhile toppling could be situated in the middle of both. Urban pluvial flooding scenarios are characterised by low water depths and high velocities (Martínez-Gomariz et al., 2016b). Consequently, the likelihood of floating instability is low compared to the other two mechanisms because water depth requires surpassing a person's height (Xia et al., 2014). Thus, these cases' most relevant instability mechanisms are toppling and sliding. Their acting forces and equations were thoroughly studied and described by Li et al. (2022). According to Lind et al. (2004), toppling could be better resisted by humans by shifting stance. This agreed with Xia et al. (2014) who concluded that the critical instability mechanism in this situation is sliding and relies on the drag force induced by the flow.

### Studies about subjects' interactions in flooded stairs

Jiang et al. (2014) is the only study found in the literature focused on multiple users interacting in flooded stairs. The authors conducted an experimental campaign measuring the hydrodynamic forces exerted on a circular cylinder. This cylinder represented an idealized leg and was tested along with four different cylinder arrangements: side-by-side, in tandem, and two staggered patterns to simulate the impact of surrounding individuals' legs (Figure 1). For the side-by-side arrangement, the force in the central cylinder also increased when the number of cylinders increased. For the tandem arrangement, a reduction in the force exerted by the flux in the cylinders behind the first one was observed. This effect gradually weakened as the distance between the cylinders increased. Finally, for the staggered setups, forces on the cylinder downward varied significantly. They can be reduced for small separations between the side-by-side cylinders, or an extra force could be applied when separations between them increase. The effect of the side-by-side cylinders also decreased when the downstream cylinder separation increased.

Figure 1. Cylinder arrangements studied by Jiang et al. (2014): a) side-by-side, b) tandem, c) and d) staggered arrangements.

### Safety criteria for evacuation of flooded stairs

Several studies suggested safe evacuation limits mostly based on water depth (Aparicio Uribe et al., 2024). For comparative purposes in this study, two safety criteria will be used to evaluate hazardous conditions in the numerical models. The first criterion: Momentum of Flow (MF) was proposed by Takedomi et al. (2001). Based on water depth (D) and velocity (V), it suggested a critical value for the safe evacuation of 1.5 m³/s². This criterion was re-evaluated by Ishigaki et al. (2005) after testing 49 volunteers (16 females and 33 males) in a real-scale experimental facility. The authors found that a critical value of (MF) = 1.5 m³/s² is above the obtained experimental safety values for evacuees. Hence, a new and more restrictive critical value of 1.2 m³/s² was put forward (Eq. 1).

|  | MF=V2·D=1.2 m3/s² | ( 1) |
| --- | --- | --- |

The second criterion to be used in this study was proposed by Ishigaki et al. (2008): Specific Force Per Unit Width “SFPUW” (M0). This was based on a new testing condition in the experimental facility in which the lower pathway beneath the stairs had an initial water depth (referred to as the water stagnation level). According to the authors, the water velocity decreases considerably in the lower pathway under this condition. Besides water depth (D) and velocity (V), the gravitational force (g) was considered (Eq. 2). A safe evacuation limit (M0) = 0.125 m³/m was proposed, whereas (M0) = 0.25 m³/m was defined as a critical evacuation limit. The latter value represents the maximum condition under which a healthy adult can evacuate along the stairs without external assistance. Matzuo et al (2011) compared the criteria (MF) = 1.2 m³/s² and (M0) = 0.125 m³/m concluding that the latter is the most accurate criterion.

|  | M0=V2Dg+D22=0.125 m3/m | ( 2) |
| --- | --- | --- |

However, Ishigaki et al. (2010) later suggested a differentiation by sex (male, elderly male, female and elderly female) to the SFPUW criteria. For simplicity, in this paper, only the safe evacuation limit proposed by Ishigaki et al. (2008) is considered (M0) = 0.125 m³/m to represent the SFPUW. In the case of the Momentum of Flow, the safe evacuation limit of (MF) = 1.2 m³/s² is used.

### Model description

Numerical modelling activities aim to obtain the water-exerted force (dragging forces) that human subjects face in different parts of the domain when they are climbing straight-run flooded stairs, the most commonly used stairs in underground space entrances (BUEDRI, 2005; ICC, 2011; and MOHURD, 2013). These forces will be measured on different configurations, either with a single human body or several ones (e.g. tandem or side-by-side). Also, the numerical model will be sought to derive the hydraulic variables water depth (D) and velocity (V) to define the safety criteria (MF) and (M0). The hydraulic behaviour of the real-scale model is entirely based on the previously calibrated and validated numerical model carried out by (Aparicio Uribe et al., 2023a; 2023b). In these studies, authors characterized the flow on the stairs without considering human interactions. Thus, the present paper can be seen as a research line continuation of the previously mentioned studies. All possible arrangements will be tested in the numerical model using a discharge of 0.423 m³/s, the maximum discharge available in the experimental facility at UPC. In this way, the numerical results will give a glance at the occurrence of possible instability conditions during future experimental campaigns including testing volunteers walking the flooded stairs.

The three-dimensional solid representation in the model consists of the human body and the stairs prototype. The first is a realistic human body (3DPEOPLE dummy), which is set as walking stairs, therefore every leg will be positioned in a different step. This human body representation was presented in a high-definition text-based 3D file (.obj) format. The second one is identical to a real-scale model (1.0 m width) constructed at the UPC labs that, in turn, resembles one of the access stairs to the Paral·lel metro station belonging to the “Transports Metropolitans de Barcelona” (TMB) in Spain. It was generated using AutoCAD Civil 3D (Autodesk, 2023). They were imported to the commercial Computational Fluid Dynamics (CFD) software FLOW-3D in “Standard Triangle Language” (.stl) format to serve as the surfaces interacting with the flux (Figure 2).

Figure 2 Three-dimensional solid representations of a human body (a) and the stairs model constructed with AutoCAD (b).

FLOW-3D employs the Volume of Fluid (VOF) method to model the free-surface dynamics and solves the physical fluid problems through the Reynolds-Averaged Navier-Stokes (RANS) equations: continuity and momentum (Tellez-Alvarez et al., 2020; 2021). It includes various turbulence models, among which the Renormalization-Group (RNG) stands out (Flow Science, 2023). Aparicio Uribe et al. (2023a; & 2023b) compared RNG with K-ɛ and K-ω turbulence models evidencing the best performance with RNG. Besides, to reproduce the impact of the aeration in the fluid due to the particular changing flow dynamics throughout the stairs domain, the numerical model included the following modules (widgets in FLOW-3D): “Gravity and Non-inertial Reference” which adds a body force representing gravity, “Viscosity and Turbulence” which accounts for viscous stresses in the fluid and turbulence models, “Air Entrainment” which models the motion and effects of air entrained at turbulent free surfaces, “Density Evaluation” which accounts for variations in fluid density due to air bubbles and “Bubble and Phase Change” which is used to control the pressure-volume relationship for void regions and the model liquid phase change.

FLOW-3D uses structured rectangular grids for the numerical approach. Complex geometry regions are constructed by assembling solid objects that define the flow regions for a simulation. The flow geometry is embedded in the computational grid using a discrete method called: Fractional Area/Volume Obstacle Representation (FAVORTM) that incorporates geometry effects into the governing equations based on the computational grid. The processor generates area fractions for each cell face in the grid by determining which corners of the face are inside of a defined geometry. Then, area fractions are computed assuming straight-line connections from these intersection points within the face. This assumption introduces a small error in the fractional area when the geometry is curved inside the cell (Flow Science, 2023). Therefore, although this approach offers a simple and accurate way to represent complex surfaces within the domain without requiring a body-fitted grid, it is affected by the resolution of the computation mesh. This is an important conditioner for the realistic representation of the human body (Figure 3).

Figure 3 Affectation of the computational mesh: object definition (above) and FAVORTM created object (below).

The numerical domain was divided into three mesh blocks: one for the upper platform and the tank (Mesh block A), the second one for the stairs’ steps (Mesh block B) in which the human dummies will be allocated, and one more for the lower platform (Mesh block C). These aimed to replicate real conditions as boundary conditions (BC) and reduce the number of total elements in the system. BC in Mesh block A were the specific pressure “P” (atmospheric), the barrier walls “W” and the symmetry “S” to transmit the volume discharge to the next mesh (Mesh block B) considering the surrounding geometry. The initial conditions (IC) defined only in Mesh block A were the volume flow rate (inflow) that surges vertically to replicate the facility pumping conditions, and an initial water level in the tank (purposed to reduce computational time). BC in Mesh blocks B and C were specific pressure “P”, symmetry “S”, and barrier walls “W”. Additionally, in Mesh block C two outflows were located (Figure 4).

Figure 4. Incorporated geometry within mesh blocks, and their respective boundary and initial conditions.

#### Sensitivity analysis

The model relies upon a previous study that calibrated and validated the hydraulic behaviour. The sensitivity analysis is centred on the adequacy of Mesh Block B to replicate the human dummy and the dragging forces exerted on it by the flow, while simultaneously optimising the relationship between accuracy and computational cost. Three different cell sizes were used in Mesh Block B: 0.04, 0.02, and 0.01 m. A human dummy was allocated as shown in Figure 5. This specific location was selected considering that the critical hazardous zone for evacuation, according to previous studies, is located in the lower part of the stairs (Aparicio Uribe et al. 2024). Under stable discharge conditions, the water-exerted force on the dummy is measured for at least 5 sec and then compared. For the 0.01 m cell size the configuration exceeded 35,000,000 elements, and the file size surpassed 16 GB. The 0.02 setup exceeded 4,500,000 elements, and the file size was approximately 4 GB. Finally, the 0.04 m setup exceeded 500,000 elements, and the file was approximately 2 GB in size.

Figure 5. Schematics of a human dummy located between steps 13 and 14 in the stairs’ centreline for sensitivity analysis. U.P. (Upper Platform) and L.P. (Lower Platform).

The mean force in the 0.04 m cell size setup was 17 % higher (146 N) than the other two configurations (120 N). Comparing the maximum and minimum forces registered they were: 167 - 122 N, 135 - 105 N and 127 - 108 N for 0.04, 0.02 and 0.01 cell size setups respectively. Also, 50 % of the total data oscillates between 151-139 N, 124 - 116 N and 124 - 117 N for 0.04, 0.02 and 0.01 cell size setups respectively. The data lies within a relatively narrow range, reflecting moderate variability around the mean. This consistency indicates stable flow conditions over the dummy during force measurements. The difference in the dragging forces between the 0.04 m cell size setup and the others could be due to the poor resolution in the dummy geometry representation (see Figure 3), therefore, this setup is discarded. The other two setups (0.02 and 0.01 m) offer similar results although extreme oscillations in the 0.01 m setup are lower compared to the 0.02 m setup (8 % average against 12 %) and the computational cost is considerably higher. After the previous analysis, the best option for optimising results quality and computational cost is the 0.02 m cell size configuration, thus, it will be used to test the following pedestrian arrangements.

Figure 6. Water dragging forces exerted on the dummy in steps 13-14 for the three different cell size setups.

### Definition of the pedestrians' arrangements to be tested

Three pedestrian arrangements will be proposed to evaluate the flow dragging force on a dummy and how this is altered due to different dummies’ grouping dynamics around the evaluated dummy. All the models will be tested under a stable discharge of 0.423 m³/s. Additionally, to compare these results with previous studies, the arrangements will be similar to those proposed by Jiang et al. (2014).

#### A single pedestrian positioned in different parts along the stairs

For the first case, a single dummy is positioned in six locations (A, B, C, D, E and F) along the stairs’ centre line (Figure 7). This is intended to determine the dragging force that affects a human dummy according to its position on the stairs. This case replicates the idealised assumption that only one person will be on the flooded stairs during an evacuation emergency.

Figure 7. Schematics of a single pedestrian positioned in six locations along the stairs’ centreline: A, B, C, D, E and F. (Upper Platform) and L.P. (Lower Platform).

#### Pedestrians in tandem arrangements

The second case considers four tandem arrangements (Figure 8). In arrangements I, II, and III, two identical dummies are placed along the centreline of the stairs, with one at location C and the other positioned behind it at locations D, E and F, respectively. In arrangement IV, four identical dummies are placed from location C through F. The objective of studying these tandem arrangements is to foresee whether the leading dummy reduces the water-exerted force on those behind it (comparing it to a single dummy (2.5.1)) and how this reduction is related to the distance between dummies.

Figure 8. Schematics of the pedestrians positioned in different tandem arrangements along the stairs centre-line.

#### Pedestrians in a side-by-side arrangement with another pedestrian downstream

Finally, in two side-by-side arrangements three identical dummies were positioned, two in location D (side-by-side) and the other behind them in the stairs' centreline, first in location E and then in F (Figure 9). Dummies positioned at D are close to the stairs’ walls, leaving space along the stairs’ centreline for water to flow. The first goal is to quantify the water-exerted forces when two people try to evacuate side-by-side. The second goal is to foresee whether the leading dummy (Location D) modified the forces exerted by water on the dummy behind. Results have been compared with those obtained in section (2.5.1).

Figure 9. Schematics of two pedestrians in side-by-side arrangements considering another pedestrian behind.

## RESULTS

Numerical simulations (Figure 10) offered the hydraulic parameters water depth (D) and velocity (V) in the entire domain to compute safety criteria (MF) and (M0). Additionally, it provided all the water-exerted forces on the dummies for the defined arrangements.

Figure 10. Render of water velocity in the stairs numerical model under a discharge of 0.423 m³/s for (a) four dummies in tandem arrangement and (b) two dummies in a side-by-side arrangement with another dummy behind them.

### A single pedestrian positioned in different parts along the stairs

Using a box plot, in Figure 11 the water-dragging force that the dummy faces in the locations along the stairs is presented. Average values range from 60.0 to 124.9 N, extreme values are approximately 53.7 N for minimum and 140.9 N for maximum. Below, the respective average values of (MF) and (M0) safety criteria are presented for every step, indicating, according to each criterion, where the hazardous zones are presented in the stairs under the discharge of 0.423 m³/s.

Figure 11. Dragging forces on a single human dummy positioned along the stairs’ centre line with their respective MF and M0 safety criteria by steps (below).

### Pedestrians in tandem arrangement

In Figure 12, results from Figure 11 (grey) are overlapped with those of the tandem arrangements (red). For cases I, II, and III, it could be seen that the water-exerted force in the dummy downstream (D, E and F) decreases when it is close to the dummy upstream (C). However, the reduction lessens when the distance between the dummies enlarges. For case IV a stable decrease in forces for all the dummies behind is evidenced. This will be addressed in detail in the discussion section.

Figure 12.  Dragging forces on human dummies in tandem along the stairs’ centre line.

### Pedestrians in a side-by-side arrangement with another pedestrian downstream

Finally, in Figure 13 results from the side-by-side arrangements (blue) were overlapped with Figure 11 (grey). This evidenced an increase in the water-exerted force on dummies compared with a single dummy positioned in the same location, passing from 124.9 N to approximately 138.2 N. Water-exerted force for the dummy downstream the side-by-side arrangement (E and F) showed that the force on this one also increased for closer distances, in location E the force was 141.0. In contrast, in the single arrangement, it was 120.2 N. However, for location F, forces were equal to 121.9 N.

Figure 13. Dragging forces on human dummies in side-by-side arrangements with another dummy behind.

## DISCUSSION

During flood events in underground spaces, it is feasible that several people attempt to simultaneously evacuate using stairs. In literature, most of the previous numerical and experimental studies solely focused on single-person evacuation. Through a 3D numerical approach, this paper studied how some pedestrian grouping dynamics impact evacuation in flooded stairs. This was possible by computing the (MF) and (M0) safety criteria, evaluating the forces faced by a single dummy located in different parts of the stairs and then comparing it with multiple tandem and side-by-side arrangements. Results evidenced that the water-dragging force experienced by a subject can be affected by flow-subject interactions upstream or even in the same steps.

Results for a single pedestrian positioned along the stairs under a discharge of 0.423 m³/s evidenced an ascending tendency in the water-exerted force in the stairs’ upper half that reaches greater and almost stable values in the lower half. Force average values were 60.0, 95.0, 99.5, 124.9, 120.2, and 121.9 N for locations A, B, C, D, E and F respectively. This is congruent with previous findings that identified the stairs' lower part as the most hazardous zone during evacuations. Extreme values for each location varied between 10 and 13 % reflecting moderated variability along the measurement time (no less than 5 sec).

The average values of (MF) and (M0) safety criteria offered similar results. However, regarding (MF), hazardous conditions appeared in step 13 when the criterion value exceeded 1.2 m³/s² and the dummy faced a force of 120.2 N. Moreover, (M0) showed hazardous conditions from step 10 when the criterion values exceeded 0.12 m³/m. The dummy faced a force of 124.9 N. Contrasting criteria results and the water-exerted forces on the dummy, it can be stated that the accuracy of the (M0) is higher. Forces on the dummy are similar after step 10 for positions D, E and F: approximately 120.0 N. Therefore, whether instability conditions are present in step 13 (MF), they will also be present in step 10 (M0).

According to the results, a dragging force around 120.0 N (60 N by leg) in a dummy surpasses safety conditions. This is similar to the critical dragging force obtained by Martínez-Gomariz et al. (2016c) for mild-slopes and plane areas 114 N (57 N by leg). Using similar discharges on a single-leg model, Ishigaki et al. (2005) defined the critical water-dragging force as around 60 N (varying between 45 and 75 N depending on the footwear). Using cylinders in scaled (1:2) stairs, Jiang et al. (2014) defined 45 N as the critical limit.

The comparison between a single dummy and tandem arrangements was as follows: case (I), force in the dummy in location D passed from 124.9 to 75.4 Nw, 39 % less. In case (II), the force in the dummy in location E passed from 120.2 to 101.5 N, 15 % less. In case (III), force in the dummy in location F remained at 121.9 N. Thus, tandem arrangements lessen the drag force faced by dummies downstream compared with only one. However, this force reduction decreased as the distance between the dummies increased, in this case being negligible after 8 steps. This agrees with the Jiang et al. (2014) results which concluded this was because of the concave region generated upstream. This is a region where the fluid surface curves inward by interacting with a solid surface (in this case the dummy in location C) reducing water depth and consequently the water-exerted dragging force. A way to maintain the water-exerted force reduction is shown in case (IV) where three dummies were consecutively positioned downstream of the dummy in C. The forces in location E passed from 124.9 to 86.7 N, 30 % less, in location F from 120.2 to 74.1 N, 38 % less and in location G from 121.9 to 79.9 N, 34 % less evidencing a relatively stable force reduction for all dummies downstream.

The side-by-side arrangements showed an increment in the water-exerted force on subjects compared to a single dummy stepping in the same place, increasing the force by approximately 10 %. Two dummies (D(right) and D(left)) positioned in steps 10-11 faced an average water dragging force of 136.7 N each; whereas a single dummy located in D faced a 124.9 N force. Results also highlighted this arrangement's influence downstream. A dummy in E faced 141.0 N when two dummies were side-by-side upstream in location D, but a single dummy in the same place faced 120.2 N representing an increment of about 17 % in the water-dragging force. However, a single dummy located in F faced 121.9 N, similarly, when two dummies were side-by-side upstream in location D the force barely changed to 123.1 N. Again, as in tandem arrangements, this shows that the reduction in the force (upstream influence) decreased as the distance between the subjects increased.

## CONCLUSIONS

This study has provided a detailed assessment of the influence of pedestrian grouping dynamics on evacuation efficiency in flooded underground stairs. Through a three-dimensional numerical approach, it has been demonstrated that the arrangement of evacuees significantly affects the hydrodynamic forces they experience and, consequently, their safety. This study identified possible bottlenecks and offered tools to develop safe evacuation strategies. This could, in turn, be considered by stakeholders and policymakers to improve evacuation procedures.

Evaluation of the water-dragging forces on a single dummy proved its ascending tendency from the stairs’ upper part, reaching high and stable values after the middle of the stairs and the lower part. This ratifies previous findings, validating that the stairs’ lower part is the most hazardous zone during flood evacuations. Additionally, the safety criteria results evidenced that the maximum achievable discharge at the UPC experimental facility (0.423 m³/s) could generate instabilities during campaigns testing volunteers; this can be verified by future research. The safety criteria accuracy was tested by comparing results in the presence of hazardous forces (120 N or more). Although both criteria predicted hazardous conditions, (M0) coincided with the values of the water-dragging force while (MF) foresaw these conditions some steps downwards. Thus, it could be concluded that both safety criteria could detect hazardous conditions, but (M0) is more accurate in forecasting the specific zones where they will be present.

The results indicate that tandem arrangements can reduce the drag force exerted on downstream individuals, potentially enhancing their stability and safety. With just one step, the force can be reduced by 38 %, and this force reduction vanishes when the separation between subjects is 8 steps. If several subjects align with one step between each other this reduction can be maintained for all the subjects downstream. Conversely, side-by-side configurations increase resistance to flow, creating a damming effect that amplifies possible instabilities and hinders evacuation. Subjects in the same steps could face forces 10 % higher than a single dummy in the same steps. A subject downstream the side-by-side group faced forces 17 % higher just one step behind. This influence rapidly decreased, in this case, after 5 steps below the influence is negligible.

This 3D numerical approach has offered valuable societal benefits and implications in evacuation procedures during flood emergencies. These findings are crucial for optimising evacuation protocols in flood-prone underground infrastructure. For instance, identifying optimal pedestrian evacuation strategies for safety protocols in metro stations, tunnels, and other underground facilities or enhancing stairs designs and access routes in flood-prone areas, ensuring smoother evacuation. Besides, the findings can be incorporated into emergency response training and simulation models, aiding decision-making during critical flood events and contributing to more resilient cities amidst climate change scenarios.

Despite the advancements, further experimental validation involving real subjects is necessary to confirm the trends observed in the numerical simulations. Additionally, future research should explore the interaction of evacuees with other stairs’ typologies or structural and non-structural elements in underground staircases. In conclusion, this research provides valuable tools to enhance safety in flooded stairway evacuations, while also contributing to the development of more resilient urban environments capable of adapting to the increasing challenges posed by climate change.

DECLARATION OF COMPETING INTEREST

The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

ACKNOWLEDGEMENTS

This work was sponsored by the Spanish Ministry of Science and Innovation - project PID2019-110965RB-I00 “Analysis of flood hazard considering vehicles and other floating elements”.

Special recognition to the support offered by the FLUMEN laboratory team at the UPC in the construction and operation of the facility, the FLOW-3D technical team for the assistance during the development of the numerical models and Transports Metropolitans de Barcelona (TMB) for providing the information needed to replicate the stairs in the laboratory.

DATA AVAILABILITY

The data that support the findings of this study are available from the corresponding author upon reasonable request.

## References

Autodesk. (2023). AutoCAD Civil 3D, 2023 (https://www.autodesk.es/products/civil-3d/overview?us_oa=dotcom-us&us_si=9c1e1af1-e29c-4f16-9cd3-a754bd545d14&us_pt=CIV3D&us_at=Civil%203D&term=1-YEAR&tab=subscription&plc=CIV3D). Last visit: 15.04.2024

Aparicio Uribe, C. H., Bonilla Brenes, R., & Hack, J. (2022). Potential of retrofitted urban green infrastructure to reduce runoff - A model implementation with site-specific constraints at neighborhood scale. Urban Forestry and Urban Greening, 69. https://doi.org/10.1016/j.ufug.2022.127499

Aparicio, C.; Russo, B.; Tellez, J.; Blade, E. (2023a). Numerical modeling of water intrusion in underground spaces throughout stairs: study case of Paral·lel metro station in the city of Barcelona. Advances in Hydroinformatics – SimHydro. (1). 239 - 250. ISBN: 978-981-97-4072-7. https://doi.org/10.1007/978-981-97-4072-7

Aparicio, C.; Russo, B.; Tellez, J.; Blade, E. (2023b) Análisis experimental y numérico del comportamiento hidráulico de escaleras de acceso a estaciones de metro. El caso de estudio de la estación de metro de Paral·lel en Barcelona. JIA 2023: VII Jornadas de Ingeniería del Agua. 323 - 326. ISBN/ISSN: 978-84-17853-72-3. https://doi.org/10.31428/10317/12821

Aparicio Uribe, C. H., Russo, B., Téllez-Álvarez, J., & Martínez-Gomariz, E. (2024). Flood-related hazard criteria during the human evacuation of underground spaces through stairs: a state-of-the-art review. Natural Hazards. https://doi.org/10.1007/s11069-024-07002-4

Avci, O., & Ozbulut, O. (2018). Threat and vulnerability risk assessment for existing subway stations: A simplified approach. Case Studies on Transport Policy, 6(4), 663-673. https://doi.org/10.1016/j.cstp.2018.08.005

BBC News. (2021a, July 21). China floods: 12 dead in Zhengzhou train and thousands evacuated in Henan. https://www.bbc.com/news/world-asia-china-57861067

BBC News. (2021b, September 2). Storm Ida: Floodwater pours into New York City subway station. https://www.bbc.com/news/av/world-us-canada-58418627

BUEDRI (Beijing Urban Engineering Design & Research Institute Co., Ltd.). (2005). Code for design of metro (In Chinese).

CCIDUS (Committee of Countermeasures against Inundation Disasters in Underground Spaces). (2002). Guideline for Measures against Inundation of Underground Spaces. Ministry of Land, Infrastructure, Transport and Tourism (MLIT), Japan (in Japanese).

Cox, R. J., Shand, T. D., & Blacka, M. J. (2010). Australian Rainfall and Runoff Revision Project 10: Appropriate Safety Criteria for People, Stage 1 report P10/S1/006, prepared by the Water Research Laboratory, University of New South Wales, Sydney.

CRED-UNDRR. (2020). The human cost of disasters: An overview of the last 20 years (2000-2019). Centre for Research on the Epidemiology of Disasters - UN Office for Disaster Risk Reduction. https://www.undrr.org/media/48008/download?startDownload=true

Dias, C., Rahman, N. A., & Zaiter, A. (2021). Evacuation under flooded conditions: Experimental investigation of the influence of water depth on walking behaviors. International Journal of Disaster Risk Reduction, 58. https://doi.org/10.1016/j.ijdrr.2021.102192

El Mundo. (2022a, August 9). Las peores lluvias en 80 años en Corea del Sur dejan al menos 8 muertos. https://www.elmundo.es/internacional/2022/08/09/62f22ca2fdddffb92d8b45ba.html

El Mundo. (2022b, August 17). París, colapsada por las fuertes lluvias y el viento. https://www.elmundo.es/internacional/2022/08/17/62fc908fe4d4d848498b45a3.html

Flow Science, Inc., Santa Fe, NM, USA. FLOW-3D® Version 2023R1 (2023).

Forero-Ortiz, E., & Martínez-Gomariz, E. (2020). Hazards threatening underground transport systems. Natural Hazards, 100(3). https://doi.org/10.1007/s11069-020-03860-w

Forero-Ortiz, E., Martínez-Gomariz, E., Porcuna, M. C., Locatelli, L., & Russo, B. (2020). Flood risk assessment in an underground railway system under the impact of climate change-A case study of the Barcelona Metro. Sustainability (Switzerland), 12(13). https://doi.org/10.3390/su12135291

Franca M., Valero D., Bayon A., Martínez-Gomariz E. & Russo B. (2024) EGU Blogs. October 2024 floods in the Valencia region, Eastern Spain: One example too many of hazardous debris. Accessed on February 3, 2025 https://blogs.egu.eu/divisions/hs/2024/11/06/floods-valencia-hazardous-debris/

ICC (International Code Council). (2011). International Building Code (IBC). (distributed by Cengage Learning).

Ishigaki, T., Baba, Y., Toda, K., & Inoue, K. (2005). Experimental study on evacuation from underground space in urban flood. 31st IAHR Congress, 1116-1123.

Ishigaki, T., Onishi, Y., Baba, Y., & Toda, K. (2008). Criterion and its application for safety evacuation during underground flooding. Proceedings of Hydraulic Engineering, 52, 841-846. https://doi.org/10.2208/prohe.52.841

Ishigaki, T., Asai, Y., Nakahata, Y., Shimada, H., Baba, Y., & Toda, K. (2010). Evacuation of aged persons from inundated underground space. Water Science and Technology, 62(8). https://doi.org/10.2166/wst.2010.455

Jiang, L. Jie, Shao, W. Yun, Zhu, D. Z., & Sun, Z. Lin. (2014). Forces on surface-piercing vertical circular cylinder groups on flooding staircase. Journal of Fluids and Structures, 46, 17-28. https://doi.org/10.1016/j.jfluidstructs.2013.12.011

Joo, J., & Kim, T.-H. (2015). An experimental study on evacuation ability during underground space inundation. Journal of Korean Society of Hazard Mitigation, 15(2), 189-196. https://doi.org/10.9798/kosham.2015.15.2.189

Kim, M., Lee, D. H., Kim, J. S., & Eom, J. H. (2018). A study on evacuation safety at inundated stairs by using real-scale hydraulic model experiment. E3S Web of Conferences, 40. https://doi.org/10.1051/e3sconf/20184006016

Kvočka, D., Falconer, R. A., & Bray, M. (2016). Flood hazard assessment for extreme flood events. Natural Hazards, 84(3), 1569-1599. https://doi.org/10.1007/s11069-016-2501-z

Lee, D. H., Kim, M., & Rhee, D. S. (2016). Evacuation safety evaluation of inundated stairs using 3D numerical simulation. International Journal of Smart Home, 10(3), 149-158. https://doi.org/10.14257/ijsh.2016.10.3.15

Liang, C., Guo, K., & Guan, M. (2023). Influence of staircase design on flood characteristics in underground spaces. Tunnelling and Underground Space Technology, 144. https://doi.org/10.1016/j.tust.2023.105511

Li, Q., Xia, J., Xie, Z., Zhou, M., & Deng, S. (2022). Hazard and vulnerability in urban inundated underground space: Hydrodynamic analysis of human instability for stairway evacuation. International Journal of Disaster Risk Reduction, 70. https://doi.org/10.1016/j.ijdrr.2021.102754

Lyu, H. M., Sun, W. J., Shen, S. L., & Arulrajah, A. (2018). Flood risk assessment in metro systems of mega-cities using a GIS-based modeling approach. Science of the Total Environment, 626, 1012-1025. https://doi.org/10.1016/j.scitotenv.2018.01.138

Martínez-Gomariz, E., Gómez, M., & Russo, B. (2016a). Estabilidad de personas en flujos de agua. Ingeniería Del Agua, 20(1), 43–58. https://doi.org/10.4995/ia.2016.4231

Martínez-Gomariz, E., Gómez, M., & Russo, B. (2016b). Experimental study of the stability of pedestrians exposed to urban pluvial flooding. Natural Hazards, 82(2), 1259-1278. https://doi.org/10.1007/s11069-016-2242-z

Martínez-Gomariz, E., Gómez, M., Russo, B. (2016c). Simulaciones 3D aplicadas al análisis de la estabilidad de peatones durante inundaciones en zonas urbanas, XXVII Congreso Latinoamericano de Hidráulica. Realidades S. A. C. Lima, Perú.

Martínez-Gomariz, E., Locatelli, L., Guerrero, M., Russo, B., & Martínez, M. (2019). Socio-economic potential impacts due to urban pluvial floods in Badalona (Spain) in a context of climate change. Water (Switzerland), 11(12), 2658. https://doi.org/10.3390/W11122658

Martínez-Gomariz, E., Russo, B., Gómez, M., Plumed, A. (2020). An approach to the modelling of stability of waste containers during urban flooding. Journal of Flood Risk Management, 1(13), http://doi.org/10.1111/jfr3.12558

MOHURD (Ministry of Housing and Urban-rural Development of the People’s Republic of China). (2013). Code for design of civil buildings. Chinese Architecture and Building Press (in Chinese).

Musolino, G., Ahmadian, R., & Falconer, R. A. (2020). Comparison of flood hazard assessment criteria for pedestrians with a refined mechanics-based method. Journal of Hydrology X, 9. https://doi.org/10.1016/j.hydroa.2020.100067

Onishi, Y., Ishigaki, T., Baba, Y., & Toda, K. (2007). Difficulty of evacuation during underground inundation and opinions of users on urban flood. Proceedings of Hydraulic Engineering. 51, 559-564. https://doi.org/10.2208/prohe.51.559

Ribeiro, H. D. B., Simões, A. L. A., Kelly, L. D. D., Mangieri, L. S. G., Schulz, H. E., & Schulz, H. E. (2021). Stability of solids in stepped flume nappe flows: subsidies for human stability in flows. Journal of Applied Fluid Mechanics, 14(3), 681-690. https://doi.org/10.47176/jafm.14.03.31815

Russo, B., Gómez, M., & Macchione, F. (2013). Pedestrian hazard criteria for flooded urban areas. Natural Hazards, 69(1), 251-265. https://doi.org/10.1007/s11069-013-0702-2

Russo, B., Velasco, M., Locatelli, L., Sunyer, D., Yubero, D., Monjo, R., Martínez-Gomariz, E., Forero-Ortiz, E., Sánchez-Muñoz, D., Evans, B., & Gómez, A. G. (2020a). Assessment of urban flood resilience in Barcelona for current and future scenarios. The RESCCUE project. Sustainability (Switzerland), 12(14), 5638. https://doi.org/10.3390/su12145638

Russo, B., Velasco, M., Monjo, R., Martínez-Gomariz, E., Sánchez, D., Domínguez, J. L., Gabàs, A., & Gonzalez, A. (2020b). Evaluación de la resiliencia de los servicios urbanos frente a episodios de inundación en Barcelona. El Proyecto RESCCUE. Ingeniería Del Agua, 24(2), 101. https://doi.org/10.4995/ia.2020.12179

Shao, W. Yun., Jiang, L. Jie., Fang, Lei., Zhu, D. Z., & Sun, Z. Lin. (2015). Assessment of the safe evacuation of people walking through flooding staircases based on numerical simulation. Journal of Zhejiang University: Science A, 16(2), 117-130. https://doi.org/10.1631/jzus.A1400154

Takedomi, K., Tachi, K., Mizukusa, K., & Yoshitani, J. (2001). An experimental study on the danger to walkers caused by inundation flow over stairs into underground space. Proceedings 56th Annual Conference on JSCE (in Japanese), Part 2, 244-245.

Tellez-Alvarez, J., Gómez, M., & Russo, B. (2020). Modelling of surcharge flow through grated inlet. Advances in Hydroinformatics, Singapore. https://doi.org/10.1007/978-981-15-5436-0_65

Tellez-Alvarez, J., Gómez, M., Russo, B., & Amezaga-Kutija, M. (2021). Numerical and experimental approaches to estimate discharge coefficients and energy loss coefficients in pressurized grated inlets. Hydrology, 8(4), 162. https://doi.org/10.3390/hydrology8040162

Tellez-Alvarez, J., Gomez, M., & Russo, B. (2022). Numerical simulation of the hydraulic behavior of stepped stairs in a metro station. Advance in Hydroinformatics. Springer Water. Singapore, 1001-1009. https://doi.org/10.1007/978-981-19-1600-7_62

Torres, V., & Olaizola, B. (2023, August 15). La lluvia provoca cortes en el Metro de Madrid, la M-30 y la M-40 y 337 salidas de los bomberos: “No podemos vivir ahí.” El Mundo. https://elpais.com/espana/madrid/2023-09-15/cortes-en-dos-lineas-de-metro-y-en-la-m-30-a-causa-de-la-lluvia.html#

United Nations, D. of E. and S. A. P. D. (2018). The World’s Cities in 2018. Data booklet (ST/ESA/ SER.A/417). https://www.un.org/en/development/desa/population/publications/pdf/urbanization/the_worlds_cities_in_2018_data_booklet.pdf ISBN / ISSN 9789211483062

Xia, J., Falconer, R. A., Xiao, X., & Wang, Y. (2014). New criterion for the stability of a human body in floodwaters. Journal of Hydraulic Research, 52(1), 93-104. https://doi.org/10.1080/00221686.2013.875073

Yu, H., Liang, C., Li, P., Niu, K., Du, F., Shao, J., & Liu, Y. (2019). Evaluation of waterlogging risk in an urban subway station. Advances in Civil Engineering. https://doi.org/10.1155/2019/5393171

3D PEOPLE. (2024, September 10). UG, Köln, Germany. https://www.3dpeople.com/en/