Novel high-speed level up-shifter design for improved delay, duty cycle and skew.

Praveen Pendyala1

1 AMD Inc., 2485 Augustine Drive, Santa Clara, CA 95054 USA

Email: praveen.pendyala@amd.com.

Fig 1. (a) Conventional Contention Mitigated Level shifter (CMLS).  (b) Proposed Novel Single Ended to Differential LS with centered mean duty cycle.  (c) Graphs showing the interpolation action on the internal nodes to average the delay from the input to the output.

High-speed level-up shifters (HSLS) are key components in low-power, high-speed CMOS I/O links. Conventional HSLS designs rely on single-ended to differential converters, which add delay, power, and degrade output duty cycle accuracy. This work introduces a novel HSLS that eliminates the need for explicit conversion by using delay interpolation, improving differential duty cycle centering while reducing power and latency. Additionally, conventional HSLS architectures suffer from single ended duty-cycle distortion and P/N skew. To address this, an auxiliary source follower is proposed to correct these distortions systematically. The combined techniques result in a superior HSLS architecture, demonstrated through PVT and Monte Carlo simulations (1000 samples), with performance compared to a traditional design.

Introduction: High-speed level-up shifters (HSLS) are widely used in serial link circuits, either in the read or write path. With the move toward CMOS logic and low-power design, HSLS with zero static power are increasingly important. To reduce power, single-ended signaling is used until the HSLS, where a single-ended to differential converter generates complementary inputs for level shifting. The conventional Contention Mitigated Level Shifter (CMLS) [1], shown in Fig. 1(a), is widely adopted. Prior designs [2]–[10] often use constant current sources for power-delay optimization but overlook duty cycle and skew correction. This paper presents a novel HSLS architecture improving differential duty cycle, single-ended duty cycle, and P/N skew. Discussion section details the techniques; Results compares simulation results against CMLS.

Discussion: A conventional low-power level-up shifter (Fig. 1(a)) introduces mean differential duty cycle distortion due to asymmetry when driven by a single-ended input. This can be corrected using a single-ended to differential (SE2Diff) converter to ensure complementary, zero-skew inputs. However, SE2Diff blocks typically add three or more gates, increasing power and latency. Figure 1(b) presents a delay-interpolation-based approach that eliminates the need for a SE2Diff. MP1/MN1 and MP2/MN2 are split into half-strength parallel devices, and delay-matched inverters (Inv1, Inv2) ensure that nets D and E switch simultaneously. As illustrated in Fig. 1(c), the effective delay from input to each side is equalized, resulting in zero mean differential duty cycle error. Additional power is drawn only from the lower supply.

Even with ideal complementary inputs, the conventional CMLS introduces inherent duty cycle error on each output. As shown in Fig. 2(a), when A rises and Ab falls, MN1p turns on while MN1n turns off. However, Bb remains at 0V briefly since PMOS MPccn is not yet on. B begins pulling low first, gradually activating MPccn, which then pulls Bb high with delay—causing systematic duty cycle errors and skew between B and Bb (Fig. 2(c)).

Fig 2. (a) Conventional Contention Mitigated Level shifter (CMLS) – Level up stage.  (b) Proposed Novel Level up stage with PMOS source follower stage.  (c) Graphs showing CMLS outputs with systematic error and proposed solution to center each output duty cycle by transition coupling.

A common fix is a cross-coupled inverter at the output, but it adds power and risks metastability. Instead, Fig. 2(b) introduces pulldown PMOS devices (MP3p, MP3n) that delay C and Cb transitions until Bb and B switch, respectively. MP3p slows C’s rise until Bb rises, and MP3n aids MN2n to accelerate Cb’s fall.

Fig 3. Proposed HSLS with combined ideas presented in Fig 1 and 2.

This method re-centers the duty cycles to 50%, reduces P/N skew, and consumes less power than cross-coupled solutions, with no area or delay penalty. Device strengths are simply redistributed, not increased. The outputs remain robust without contention since auxiliary control doesn’t interfere with core switching. Additionally, Monte Carlo variation and PVT robustness improve due to systematic mismatch cancellation.

| Table 1. Simulation Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Units | Values Simulated |  |
| Process (TSMC N4P) |  | - | SS, SF, FS, FF |  |
| Voltage | Vhi | V | 0.945/1.21 |  |
|  | Vlo | V | 0.585/0.935 |  |
| Temperature |  | °C | -40, 0, 100, 125 |  |

Power isolation is also essential in CMOS level shifters to prevent crowbar current when the input supply (VLO) is unstable, but the output supply (VHI) is active. The proposed HSLS in Fig2b can be modified by introducing a powerOk signal from the VHI domain. Large NMOS switches on sources of MN1p/MN2p and small PMOS pull-ups switched on nets B and Bb need to be added that are driven directly by the powerOk signal to provide the correct isolation.

Results and Comparisons: We combine both techniques discussed above to design an improved HSLS, shown in Fig 3, and compare it with the conventional CMLS. PVT and Monte Carlo simulations evaluate delay, power, duty cycle, and differential skew. Table 1 lists PVT conditions. Fig 4 illustrates transient waveforms at the SS corner (0°C, VHI = 0.945V, VLO = 0.585V). Table 2 summarizes worst-case PVT and Monte Carlo (1000 samples) results. Fig 5 shows performance improvement spread across corners. Duty cycle is measured in ps and converted to percentage using Equation (1).

| Dps=D%100×2×Tbit | (1) |
| --- | --- |

Table 2. Simulation Results Summary showing PVT and Monte Carlo simulation result comparisons. Improvements highlighted in green.

| - PVT (Worst Case) Results Comparison |  |  |  |  |  |  |  | (b) Monte Carlo (Worst PVT) Results Comparison |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measurement (Best Improvement across PVT) | Design | VLO/VHI |  |  |  |  |  | MC Parameter | Design | Measurement (ps) |  |  |  |  |  |  |
|  |  | 0.585/ 0.945 | 0.585/ 1.2 | 0.935/ 0.945 | 0.935/ 1.2 |  |  |  |  | Out P DCE |  | Out N DCE |  |  | Differential DCE | P/N Skew |
| Insertion Delay (ps) (~19%) | CMLS | 89.5 | 90.0 | 46.2 | 44.6 |  |  | µ | CMLS | 17.2 | 25.6 |  |  | 9.9 |  | 15.0 |  |
|  | Proposed | 81.2 | 92.2 | 37.4 | 37.7 |  |  |  | Proposed | 5.1 | 9.9 |  |  | 4.9 |  | 7.7 |  |
| Output P/N Skew (ps) (~ 50%) | CMLS | 14.7 | 5.2 | 17.5 | 13.4 |  |  |  | Improvement | 70.7% | 61.3% |  |  | 50.1% |  | 49.1% |  |
|  | Proposed | 10.3 | 3.7 | 10.4 | 6.6 |  |  | σ | CMLS | 4.3 | 3.9 |  |  | 3.9 |  | 0.9 |  |
| Power (uW) (~ 22%) | CMLS | 670.6 | 1429 | 586.5 | 1137 |  |  |  | Proposed | 3.5 | 3.3 |  |  | 3.3 |  | 1.0 |  |
|  | Proposed | 623.2 | 1114 | 696.1 | 1160 |  |  |  | Improvement | 17.9% | 15.8% |  |  | 16.0% |  | -4.1% |  |
| Differential DCE (ps) (~ 77%) | CMLS | 16.2 | 28.5 | 2.7 | 4.4 |  |  | Worst µ+/-3σ | CMLS | 21.7 | 37.3 |  |  | 21.7 |  | 17.7 |  |
|  | Proposed | 3.6 | 11.6 | 3.9 | 3.7 |  |  |  | Proposed | 14.8 | 16.5 |  |  | 14.8 |  | 10.2 |  |
| Single Ended DCE (ps) (~ 76%) | CMLS | 40.5 | 33.8 | 28.1 | 23.8 |  |  |  | Improvement | 31.8% | 55.7% |  |  | 31.6% |  | 42.4% |  |
|  | Proposed | 9.6 | 12.9 | 14.1 | 9.4 |  |  |  |  |  |  |  |  |  |  |  |

Conclusion: A novel single-ended to differential high-speed CMOS level-up shifter was presented for use in high-speed I/O links. Compared to the conventional CMLS, the proposed HSLS shows up to 75% improvement in duty cycle metrics and 50% reduction in P/N skew, with negligible area, power, and delay overhead. Slight regressions in unconventional corners (highlighted in Table 2) occur due to temporary contention, mainly when input/output voltages are similar. However, overall benefits significantly outweigh these limitations.

Fig 4.   Waveforms inside the HSLS block in Fig 3. Dashed lines show the waveforms in a conventional CMLS, and the solid lines show the equivalent waveforms in the proposed HSLS.  (SS corner, 0°C, VHI=0.945V, VLO = 0.585V as an example, Data rate = 8533 Mbps).

Acknowledgments: The author would like to acknowledge the Circuit Technology DDR group at AMD for providing the resources to work on this idea.

Conﬂict of interest: The authors have declared no conﬂict of interest.

Funding information: There are no funders to report for this submission.

Data availability statement: The data that support the ﬁndings of this study are available from the corresponding author upon reasonable re-quest.

2025 The Authors. Electronics Letters published by John Wiley & Sons Ltd on behalf of The Institution of Engineering and Technology

This is an open access article under the terms of the Creative Commons Attribution License, which permits use, distribution and reproduction in any medium, provided the original work is properly cited.

Received: xx January 2025   Accepted: xx March 2025

doi: 10.1049/ell2.10001

Fig 5.   Simulation Results across PVT. Clockwise from top left: Single Ended (P and N) Output Duty cycle Error, Differential Duty cycle error at output, Total Power consumption, Insertion Delay, Output P/N Skew. Each plot contains overlays for 4 VLO VHI combinations as shown in Table 1.

References

1.	C. Q. Tran et al., "Low-power high-speed level shifter design for block-level dynamic voltage scaling environment," 2005 International Conference on Integrated Circuit Design and Technology, 2005. ICICDT 2005., Austin, TX, USA, 2005.

2.	Y. Osaki et al., “A low-power level shifter with logic error correction for extremely low-voltage digital CMOS LSIs,” IEEE J. Solid-State Circuits, vol. 47, no. 7, Jul. 2012.

3.	S.-C. Luo et al., “A wide-range level shifter using a modified Wilson current mirror hybrid buffer,” IEEE Trans. Circuits Syst. I, Reg. Papers, vol. 61, no. 6, Jun. 2014.

4.	M. Lanuzza et al., “Fast and wide range voltage conversion in multisupply voltage designs,” IEEE Trans. Very Large Scale Integr. (VLSI) Syst., vol. 23, no. 2, Feb. 2015.

5.	L. Wen et al., “Subthreshold level shifter with self-controlled current limiter by detecting output error,” IEEE Trans. Circuits Syst. II, Exp. Briefs, vol. 63, no. 4, Apr. 2016.

6.	E. Maghsoudloo et al., “A high-speed and ultra-low-power subthreshold signal level shifter,” IEEE Trans. Circuits Syst. I, Reg. Papers, vol. 64, no. 5, May 2017.

7.	S. R. Hosseini et al., “A high-speed and power-efficient voltage level shifter for dual-supply applications,” IEEE Trans. Very Large Scale Integr. (VLSI) Syst., vol. 25, no. 3, Mar. 2017.

8.	M. Lanuzza et al., “An ultralow-voltage energy-efficient level shifter,” IEEE Trans. Circuits Syst. II, Exp. Briefs, vol. 64, no. 1, Jan. 2017.

9.	R. Lotfi et al., “Energy-efficient wide-range voltage level shifters reaching 4.2 fJ/transition,” IEEE Solid-State Circuits Lett., vol. 1, no. 2, Feb. 2018.