Distributionally Robust Dispatch of Thermal Units with Multiple Energy Storage Considering Oxygen-enriched Deep Peaking Technology

Xiong Wu, Member, IEEE, Yonglong Fan, Junji Zhou, Dong Liu

Abstract—The growing penetration of renewable energy needs a large amount of flexible resource to regulate in power systems. Current research mainly focuses on individual flexibility of resources, the synergistic flexible potential of thermal units with multi-energy storage systems remains underexplored. Therefore, this paper proposes a distributionally robust optimal dispatch method for multiple flexible resources including thermal units and multiple energy storage. Firstly, a refined operation model of units and energy storage is constructed. Specifically, oxygen-enriched deep peaking of units and coordinated operation of battery and hydrogen energy storage are considered. Secondly, a data-driven distributionally robust optimization (DRO) model is established to cope with the uncertainties of renewable energy. An inexact columns and constraint generation (i-C&CG) method is then developed to solve the model efficiently. Finally, a case study of a provincial power system is conducted, and the results verify the validity of the proposed method.

Index Terms—Flexibility enhancement, multiple energy storage, deep peaking of thermal units, distributionally robust optimal dispatch, inexact columns and constraint generation methods.

- NOMENCLATURE

Parameters

|  | Startup/shutdown costs of unit g |
| --- | --- |
|  | Cost coefficient of the unit's conventional operating curve |
|  | Additional coal consumption cost function for deep peaking |
|  | Coefficient of water consumption cost/ additional loss cost for the electrolyzer |
|  | Startup/shutdown costs of the electrolyzer |
|  | Cost loss coefficients of the battery storage/ hydrogen fuel cell |
|  | Penalty coefficient of wind/polar abandonment |
|  | Penalty coefficient of load shedding |
|  | Penalty coefficient for insufficient flexibility shortage |
|  | Slope corresponding to section α of the coal regular/ additional consumption curve |
|  | Efficiency of conversion to oxygen/hydrogen |
|  | Minimum duration of electrolyzer operation in overloaded/underloaded condition |
|  | Minimum shutdown time of the electrolyzer |
|  | Gas storage n maximum permissible gas flow rate |
|  | Minimum/Maximum output limits for thermal generating units g |
|  | Minimum continuous startup/shutdown time for thermal unit g |
|  | Thermal unit g climbing limit in start-stop/regular operation/deep peaking state |
|  | Minimum magnitude of output of unit g during deep peaking at time t |
|  | Upper and lower limits of capacity of battery storage k |
|  | Charge/discharge efficiency of battery storage k |
|  | Installed capacity of wind farm station w and solar power plant pv |
|  | Annual normalized output curves for wind farm station w and solar power plant pv |

Variables

|  | Startup operation variable of unit g at time t |
| --- | --- |
|  | Operating / Deep peaking state variable of unit g at time t |
|  | Downtime status binary variables of the electrolyzer n at time t |
|  | Startup/shutdown action variables of the electrolyzer n at time t |
|  | Power absorbed by the electrolyzer n at time t |
|  | State variables of the nth electrolyzer is in low/overloaded/normal operation at time t |
|  | Charging/discharging power of battery storage k at time t |
|  | Power generated by the hydrogen fuel cell n at time t |
| / | Abandoned power of wind/polar at time t |
|  | Load shedding power of node b at time t |
|  | Upward/downward flexibility deficit power of the system at time t |
|  | Amount of oxygen/hydrogen produced by the electrolyzer n at time t |
|  | Capacity of gas storage unit n at time t |
|  | Charge/discharge state indicator variable for battery storage k at time t |
|  | Value of power transmission of line l at time t |
|  | Total upward/downward demand of the system at time t |
|  | Upward/downward flexibility supply of thermal unit g at time t |
|  | Ratio of the minimum output decrease during deep peaking of the unit g at time t |
|  | Upward/downward flexibility supply of hydrogen fuel cell n at time t |
|  | Upward/downward flexibility supply of electrolyzer n at time t |
|  | Upward/downward flexibility supply of battery storage k at time t |
|  | Total upward/downward flexibility supply of the system at time t |

- INTRODUCTION

The increasing penetration of renewable energy introduces significant volatility and stochasticity into power systems, and it results in a pronounced imbalance between supply and demand for system flexibility. Accordingly, the flexibility requirements of the system also increase dramatically. Existing research on flexibility analysis mainly focused on traditional flexibility resources, and the emerging resources has not yet been studied adequately. In fact, some flexibility technologies such as oxygen-enriched deep peaking technology in thermal units and hydrogen storage technology are gradually applied in power system, and they can improve the system flexibility to an extent. Thus the generation dispatch of these technologies remains to be explored.

Flexibility retrofitting of thermal units is one of the effective ways to enhance the system flexibility supply. To characterize its operating behaviors more accurately, reference [1] characterizes the feasible regions of unit output in adjacent time periods under multiple time scales. It accurately responds to the operating characteristics of the unit deep peaking period in actual operation characteristics. Reference [2] established the improved ramp-up constraint and the minimum deep peaking operation time constraint. It also considered the conventional cost and the additional cost in the deep peaking state respectively. Reference [3] investigated the technical path of oxygen-enriched combustion flexibility modification to enable the thermal unit to realize lower stable output.

Energy storage has flexible bidirectional regulation capability. Reasonable allocation of energy storage is another way to enhance system flexibility[4]. Reference [5] studied the coordinated operation of energy storage with thermal units, which significantly improved the flexibility. Hydrogen energy storage has a broad prospect in new energy high penetration scenarios due to the advantages of clean and low self-loss rate [6]. References [7] established refined hydrogen energy flow constraints for hydrogen production, manufacturing and storage processes. Considering power fluctuation and frequent shutting down and up can greatly affect the service life of the electrolyzer, reference [8] established a multi-state operation model and proposed a framework of the multi-electrolyzer array combination. Reference [9] investigated the power-efficiency dynamic characteristics of alkaline and PEM electrolyzers separately. It proposed a coordinated dispatch operation strategy for multiple arrays of hybrid alkaline-PEM electrolyzers. Reference [10] considered the dynamic heat transfer and hydrogen-oxygen impurity crossover process to improve the flexibility of the hydrogen production.

The references above have conducted detailed studies on the deep peaking flexibility retrofitting of thermal units and the energy storage. However, they lacked the in-depth excavation of the synergistic operation mechanism of unit and multiple energy storage. How to reasonably evaluate the flexibility of system operation has triggered extensive research by scholars. Reference [11] summarized the operational characteristics of various types of typical flexibility resources. Reference [12] quantifies the flexibility of the system from both the supply and demand perspectives. It effectively identifies the potential flexibility shortfall, and improves the economic efficiency. Reference [13] investigated the flexibility supply capacity of electric vehicles and demand-side response. It balanced the operation economy and flexibility by adding a flexibility deficit penalty to the objective. Reference [14] transforms the flexibility of multiple time scales into a feature region and describes the flexibility equilibrium constraints based on the theory of flexibility.

The strong uncertainty brought by the renew energy is the main factor affecting system flexibility. To better cope with the uncertainty, DRO has been widely used since it can balance the economy and robustness. The C&CG method achieves efficient solution and has been widely used in solving DRO model. However, when solving large-scale problems containing integer variables, the complexity of the master problem increases as the number of iterations increases, resulting in a great computational burden. Thus, a series of improved algorithms need to be investigated to enhance the solution efficiency. Reference [15] proposed a feasible pump-based column and constraint generation algorithm. It relaxes the subproblem into LP problem and generates feasible integer solutions in combination with feasibility pumps to reduce the iteration time. Reference [16] allows inexact solution of the master problem. It alleviates the computational burden associated with large-scale MILP. A backtracking procedure is proposed to make a trade-off between generating constraints and improving the accuracy. Reference [17] improved the C&CG method by introducing auxiliary variables to avoid the inner C&CG loop and reduce the model complexity. Although the above references have investigated a series of measures to enhance the efficiency, they have not alleviated the difficulty due to the large number of binary variables. Therefore, how to realize the acceleration of solving a problem with a large number of integer variables while guarantying certain accuracy deserves further study.

In summary, the existing research on flexible resources mainly focuses on one single flexible resource. It lacks the in-depth excavation of the synergistic operation mechanism of thermal unit and multiple energy storage. To solve the above problems, a distributionally robust dispatch of thermal units with multiple energy storage considering oxygen-enriched deep peaking technology is proposed. Firstly, a refined cooperative operation model of thermal units and multiple energy storage is constructed. Secondly, a data-driven DRO dispatch strategy is established by integrating economy and flexibility. Then, an i-C&CG method is employed to realize the efficient solution of the model. Finally, the effectiveness of the proposed method is verified by case studies. The main contributions are summarized as follows.

A synergistic operation model of thermal units considering the oxygen-rich deep peaking and multiple energy storage systems considering the multi-state operation of electrolyzer is investigated. The flexibility regulation capacity of various resources is quantified.

A data-driven DRO dispatch strategy employing a comprehensive paradigm to limit the uncertainty of the probability distribution is established. An i-C&CG method is developed to solve the model, which greatly improves the solving efficiency.

The rest of this paper is organized as follows. A dispatch model considering operational flexibility is proposed in section II. A data-driven based DRO model is proposed in section III. Section IV demonstrates the solution method to solve the model. A case study is illustrated in section V. Finally, conclusions are given in section VI.

Dispatch Model Considering Operational Flexibility

The dispatch model includes the coordinated operation of thermal units and multiple energy storage. Multiple energy storage includes battery energy storage and hydrogen energy storage. Among them, the oxygen-enriched deep peaking of thermal units can be performed by fully utilizing oxygen—the byproduct of hydrogen production from electrolyzer. Thus, the synergistic operation of thermal units and hydrogen storage is realized, as shown in Fig. 1. Retrofitting the conventional units to oxygen-enriched deep peaking units can effectively reduce the minimum technical output and improve the flexibility.

Fig. 1 Schematic diagram of hydrogen energy storage and thermal units synergistic and flexible operation.

Objective Function

From equation  , the objective function of the dispatch model consists of unit operation cost, electrolyzer operation cost, energy storage operation cost, power abandonment and load cutting penalty cost and system flexibility shortage penalty cost. Equation  indicates that the thermal unit operating cost consists of two parts: the unit startup and shutdown cost and the coal consumption cost. The coal consumption cost includes the normal operation and the additional operation cost during deep peaking. Equation  indicates that the operating cost of the electrolyzer includes the water consumption cost, startup/shutdown cost and the loss cost caused by the operation in non-normal operating condition. Although the electrolyzer can be operated in overloaded and low loaded condition, it will damage the electrode material, affecting the service life and the safety of the system. Equation  quantifies the losses to the system under low and overload conditions by introducing auxiliary variables to calculate the absorbed power under unconventional conditions. Equation  represents the operating cost of battery energy storage and the operating cost of hydrogen fuel cells. Equation  represents the system penalty cost considering the abandonment of wind and solar energy and loss of load during operation. Equation  represents the inflexibility penalty, including upward and downward adjustments of the inflexibility penalty. Constraints - quantify the upward and downward flexibility shortfalls at each moment by  calculating flexibility margins.

Among them, the coal consumption cost in the equation  is a nonlinear function. In order to facilitate the solution, the original equation is linearized in segments, and the binary auxiliary variable  is introduced to divide the curve of the coal consumption cost and the additional coal consumption cost into m segments. The simplified results are shown in equations -. At the same time, equations  still contains the nonlinear term of continuous variables multiplying 0-1 variables with continuous variables, and the auxiliary variable  is introduced to further linearize the process, as shown in equations -.

Constraints

Hydrogen energy storage operation constraints

The hydrogen energy storage system contains three parts: electrolyzer, gas storage tank and hydrogen fuel cell. Among them, the electrolyzer adopts the proton exchange membrane electrolyzer. It has better flexibility and can be operated in both low and overloaded states, providing higher flexibility and operational efficiency. Its multi-state operation transition relationship is shown in Fig.2.

Fig. 2  Schematic diagram of multi-state operation transition of electrolyzer

Constraint  restricts the start/stop state of the electrolyzer. Constraint  restricts the logical coupling relationship between the electrolyzer shutdown state and various operation states between adjacent time periods. Constraint  restricts the electrolyzer to one state at a time. It is in line with the actual operation requirements. In order to avoid frequent transitions between the regular state and the overload and low load states, the constraints - limit the minimum duration of the overload and low load to ensure security. Considering the operating safety and hydrogen production efficiency, the power varies in the range of [0.5,1]. The electrolyzer can be operated in overloaded and underloaded states for a short time, with power variations in the ranges of [1,1.6] and [0.15,0.5], respectively. Constraints  limits the range of power variations in multiple operating modes through binary variables. Constraints - limit the relationship between the oxygen, hydrogen and the power output. Constraint  limits the relationship between hydrogen consumption and power generation. Considering the actual operation limitations, the hydrogen fuel cell and the electrolyzer cannot operate at the same time. Thus, constraint  limits the range of variation of fuel cell power generation. Constraint  limits the change of the hydrogen storage capacity in the adjacent time period. Constraint  limits the range of variation of gas production and consumption. Constraint  limits the upper and lower limits of the storage capacity. Constraint  limits the storage capacity to remain constant at the start and end of the dispatch period.

Among them, constraints - contain nonlinear terms of multiplication of two 0-1 variables, which need to be linearized. Taking constraint  as an example, it is linearized by introducing auxiliary variables and transformed into constraints -.

Oxygen-Rich Deep Conditioning Modified Unit Operation Constraints

Constraint  describes the start-up and shutdown limit of units. Constraint  represents the minimum start-up time limit of a thermal power unit. Constraint  represents the minimum shutdown time limit of a thermal power unit. Constraint  limits the logical relationship between the unit's deep peaking state and the operating state. Constraint  limits the output range of unit when it is in deep peaking. Constraint  limits the amount of output of the thermal unit. Its minimum power output decreases significantly when it is in deep regulation. Constraint  limits the climbing power of the unit. Considering the operational requirements, it is reduced when in deep regulation. Constraint  limits the unit's deep peaking minimum output in relation to the oxygen supply rate, oxygen storage capacity, and theoretical minimum output.

Among them, constraint  is a nonlinear constraint. Linearization using Big-M method is shown as constraint  .

Battery storage operational constraints

In addition to hydrogen storage, battery storage also offers operational flexibility in both directions. Constraints  and  limit the charging and discharging power of the energy storage. Constraints  limits the simultaneous charging and discharging of the energy storage. Constraint  limits the amount of power stored in the energy storage. Constraint  limits the capacity change of energy storage in adjacent time periods. Based on the actual grid operation requirements, constraint  restricts the storage capacity to be equal at the beginning and end of each day.

Renewable Energy Unit Operating Constraints

Constraint  indicates that the output power of the wind farm station is the difference between the predicted output power and the abandoned power. Constraint  indicates that the abandoned power cannot exceed the active power provided by the wind turbines. Constraint  indicates that the output power of the PV field station is the difference between the predicted output and the abandoned power. Constraint  indicates that the abandoned PV power cannot exceed the active power provided by the PV plants.

Power Balance Constraints and Power Flow Constraints

Constraint  -  consider the power balance constraints for each region. If the power conservation cannot be met, a demand side response is taken to remove part of the load. Constraint - is based on the DC power flow, which limits the transmission capacity of critical sections.

Constraints of Quantifying System Flexibility Demand

To improve the flexibility of the system, it is necessary to quantify the supply and demand for flexibility. Considering the stochastic fluctuation of wind power and load, the flexibility demand of the system is quantified according to the maximum forecast deviation. Taking load as an example, the method of flexibility quantification is shown in Fig. 3. Constraints - limit the upper and lower limits of load, solar and wind power fluctuations at each moment. Constraints - quantify the flexibility demand of system load fluctuations by comparing the magnitude of the upper and lower limits of load fluctuations in adjacent time periods. Solar and wind power, as power sources, have just the opposite flexibility requirements, as shown in constraints -. Constraints - limit the total system flexibility demand at all moments.

Fig. 3  Schematic for quantifying flexibility demand

Constraints of Quantifying System Flexibility Supply

The system contains multiple flexibility resources, including thermal units, electrolyzers, hydrogen fuel cells, and battery. Constraint  limits the supply of up-regulation flexibility to thermal units. It is determined by the lesser of the upper creeping power and the up-regulatable margin. Constraint  limits the supply of down-regulation flexibility to thermal units. It is determined by the down-climbing power and the down-tunable margin. It can be seen that when the thermal unit is in deep peaking condition, its down-regulation flexibility supply will increase significantly. Constraint  limits the relationship between the minimum power output factor and the unit's deep peaking operating state. Constraints - limit the relationship among electrolyzer flexibility supply and electrolyzer operating state. Constraints - limit the relationship between the flexibility supply of the hydrogen fuel cell and the operating state of the electrolyzer. Constraints - limit the relationship between the flexibility supply of the battery and its capacity and maximum charging and discharging power. Constraints - limit the total flexibility supply of the system to the sum of the flexibility provided by all flexibility resources.

Data-Driven Based DRO Dispatch Model

The dispatch model constructed in the previous section is a two-stage model. The first stage variables are decision variables that are not affected by the scenario prediction error. The second stage variables are the decision variables that are adjusted according to the real-time output of renewable energy. The optimization model is written with a compact form, as shown in equation .

where and  are the coefficient matrices of the objective functions corresponding to the first and second stage variables, respectively.  ,  and  are the coefficient matrices of the corresponding variables in the model constraints, respectively.  is the coefficient matrix of the constant term, and  is the scenario predicted power.

However, due to the high penetration of renewable energy, its power fluctuation greatly affects the economy and robustness of system. Therefore, a data-driven model based on multiple discrete scenarios is developed. After obtaining the dispatch results of the units and multiple energy storage in the first stage, the second stage of the model searches for the worst probability distribution for a given set of scenarios. The objective function can be expressed as equation .

where  is the feasible domain of the first-stage variable  qualified by the relevant constraints.denotes the probability distribution of each scenario.  denotes the set of scenario probability distributions. denotes the expected value when the scenario probability distribution is .

Since the exact distribution of the uncertain scenario set is difficult to obtain, we use multiple discrete typical scenarios and the occurrence probability to characterize the uncertainty, as shown in equation -. In order to control the probability distribution of each scenario to vary within a reasonable range, the integrated paradigm constraint  is established using the predicted probability distribution as a benchmark.

where is the 1-paradigm,  is the -paradigm,  is the allowed deviation from the probability distribution, respectively. is the initial given scenario probability distribution, which satisfies the confidence constraints of equation . The right-hand side of equation  is the uncertainty probability confidence levels  and  , then  and  can be solved according to equation .

Among them, constraint  contains a nonlinear constraint. An auxiliary variable  is introduced to transform it, as shown in equations -.

Solution Methods

In this paper, we propose the i-C&CG method to realize the accelerated solution. Its core advantage is the dynamic allocation of computational resources to boundary improvement and problem size control. Specifically, it adds a backtracking process to the traditional C&CG method. By dynamically adjusting the inexact convergence gap, the main problem is allowed to terminate with a relatively optimal gap when solving. It effectively reduces the computational burden and balances the efficiency and accuracy. And it can ensure the convergence within a finite number of steps.

The DRO model established in the previous section is divided into a master problem and sub problem, which are solved by iterating over each other, as shown in equations -. The objective of the master problem is to find the optimal solution under all the recognized scenario distributions and to give the model a reference lower bound.

where is the number of iterations. is the slack variable of the subproblem.  is the worst-case probability distribution of the scenarios set by the subproblem at the kth iteration. is the auxiliary variable of the second stage generated in the kth iteration.  is the lower bound value of the current solution.

The objective of the subproblem is to find the worst distribution of the scenario. According to equation , it can be seen that there is no coupling relationship between the inner layer problems for each typical scenario. Therefore, the subproblem can be transformed into two single-layer problems solved sequentially as shown in equation -.

The specific flow of i-C&CG algorithm-based solution is shown in Fig. 4 and the main steps are:

Step 1: Initialize the system parameters. Set the upper and lower bounds of the initial solution , the iteration convergence gap  , , the main problem convergence gap  , , and the number of iterations .

Step 2: Solve the master problem. Solve the main problem  with the relative optimal gap , and record the found optimal feasible solution . Record the lower bound and the upper bound of the main problem solving result, respectively. If  , set  and update the lower bound .

Step 3: Solve the subproblem. Based on the optimization result  of the main problem, solve the subproblem , record the optimization result  and the optimal value , and update the upper bound .

Step 4: Optimality check and backtracking process. Judge whether the result of this iteration satisfies , if so, the iterative convergence requirements are met, the solution is finished, and the optimal solution  is returned. Otherwise, judge whether the result satisfies  , if so, enter the development step: update , narrow the master problem solution gap  for all , return to Step 2 to solve again; if not, enter the exploration step: update the worst probability distribution , and generate new variables with corresponding constraints to add into the master problem.

Step 5: Update the number of iterations  , and return to Step 2 to solve again.

Fig. 4 Algorithm Solution Flowchart of i-C&CG

Case Study

To verify the effectiveness of the proposed method, a case study is carried out using a  provincial power system in China consisting of 6 sub-regions. Clustering cuts are performed based on the identified typical scenarios as shown in Fig. 5.The transmission capacity of each regional link has been indicated in Fig. 6. According to the planning in 2030, the maximum load will be 180 GW, the total installed capacity of thermal units, gas-fired units, nuclear units, wind power and solar power will be 107 GW, 27 GW, 12 GW, 45 GW and 105 GW. The maximum deviation coefficient of wind, solar and load predicted output  is 0.1/0.08/0.05, and  is 0.5 and 0.95, respectively. The specific parameters of the system are shown in TABLE I.

Four cases are set up to verify the effectiveness of the proposed method. As shown in TABLE II, Case1 is set up as the base case. Case2 only considers the deep peaking of units. Case3 only considers the multistate operation of electrolyzer. Case4 is the operation strategy proposed in this paper.

TABLE I

System Operation Parameters

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
|  |  |  | 0.01% |
| / |  |  | 2h |
|  |  |  | 2h |
|  | 0.2 |  | 0.8 |
|  | 0.004% |  | 0.95 |

TABLE II

Comparison of Case Settings

| Case | Oxygen-enriched deep peaking of thermal units | Multi-state operation of electrolyzer |
| --- | --- | --- |
| Case 1 | × | × |
| Case 2 | √ | × |
| Case 3 | × | √ |
| Case 4 | √ | √ |

| (a) Typical scenarios |  |
| --- | --- |
| (b)Scenarios with high wind power | (c)Scenarios with high solar power |
| (d) hot and windless scenario | (e)cold wave and low-light scenario |

Fig. 5 Typical scenario wind, light and load output curves

Fig. 6 Schematic diagram of the regional division of provincial power grids

Dispatch Results Analysis

The dispatch results are shown in TABLE III. Compared with Case1, the power abandonment rate in Case2 decreased by 15.5%, and flexibility shortage penalty cost decreased by about 60%. It shows that considering the deep peaking of thermal unit can effectively improve the system flexibility supply. The system's new energy consumption capacity and flexibility adjustment capacity are significantly improved. The system flexibility is further improved after considering the deep conditioning of the units and the multi-state operation of the electrolyzer. Compared with Case1, the power abandonment rate of Case4 decreased by 19.47%, the flexibility shortage penalty cost decreased by 73%, the total cost decreased by 20%. This verifies the effectiveness of the synergistic operation strategy.

TABLE III

Comparison of Cases Dispatch Results

| Case | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| Unit Costs (millions) | 385.54 | 421.36 | 376.98 | 417.64 |
| Electrolyzer Costs (millions) | 0.8 | 0.6 | 1.66 | 1.16 |
| Flexibility Deficiency Penalty Cost (millions) | 76.49 | 31.01 | 68.52 | 20.63 |
| Total cost (millions) | 667.46 | 557.66 | 632.16 | 531.21 |
| Power Abandonment Rate | 29.97% | 13.4% | 25.9% | 10.5% |

The power balance of the system before and after deep peaking of units is shown in Fig. 7. Comparing Fig. 7 (a) and (b), it can be seen that after considering the deep peaking, the unit output can be reduced to a lower level. It has better flexible regulation capability. The operating power curves of the electrolyzer is shown in Fig. 8. It can be seen that it is shut down at 8:00-9:00 a.m. morning peak load and 18:00-21:00 p.m. evening peak load. In the midday from 10:00 to 14:00 when the solar output is large, it runs in overload state to promote new energy consumption. When the load is low and the wind power output fluctuates in the evening, it is operated at a low load state to provide flexibility for downward adjustment. It can be concluded that considering multi-state operation effectively improve the flexibility.

Supply and Demand Analysis of System Flexibility

Take Case4 as an example to analyze the system flexibility supply-demand balance. As shown in Fig. 9, during the period from 7:00 to 9:00 and from 18:00 to 20:00, the load level is high while the new energy output is low. When the load climbs, the system upward demand is larger than the upward supply, resulting in a shortage of upward flexibility. During the period from 11:00 to 14:00, the net load level is low due to the high share of solar output. When the renewable energy output increases, the system downward demand is larger than the downward supply, resulting in a downward flexibility deficit.

(a) Deep peaking is not considered

(b) Deep peaking is considered

Fig. 7 Schematic diagram of system power balance before and after considering oxygen-enriched deep peaking of thermal units

Fig. 8 Operating power curve of the electrolyzer under the comparison case

Fig. 9 Analysis of system supply and demand for flexibility at each moment

Based on the above analysis, calculate the flexibility index of the system. As shown in equations -, quantitatively analyze the proportion of flexibility resources that bear the system flexibility demand at each moment.

where are the upward/downward flexibility indices of the i-th flexibility resource at moment t, including thermal units, electrolyzers, battery storage and hydrogen fuel cells.

When the total flexibility supply is larger than the flexibility demand, the system flexibility index is bigger than 1. It indicates that the system flexibility regulation ability is stronger and the system flexibility is abundant. On the contrary, when the index is less than 1, it indicates that the system flexibility adjustment ability is weak and there is a flexibility deficit. As shown in Fig. 10, during the hours of 7:00 to 9:00 and 18:00 to 20:00, the load climbs and the units climb rapidly resulting in a decrease in upward flexibility supply. In the 11:00 to 15:00 hours, the solar output is large. In order to consume more renewable energy, the units reduce output, resulting in a significant reduction in the downward flexibility supply. Battery storage can provide effective flexibility supply during most dispatch hours. However, due to its power and capacity constraints, it has limited regulation capability when there is a large unidirectional flexibility demand. At the same time, the electrolyzer provides effective flexibility support in the morning and evening peaks. It enhances the system's ability to consume renewable energy and maintain power supply.

(a) Upward revision of the flexibility index

(b) Downward revision of the flexibility index

Fig. 10 Schematic diagram of the flexibility index at various moments

DRO confidence level analysis

The allowable deviation of the probability distribution of each discrete scenario varies when the confidence level varies. The results under the integrated paradigm constraints are compared and analyzed by taking  , respectively. They are shown in TABLE IV.

TABLE IV

Comparison of System Cost Results at Different Confidence Levels

|  | =0.1 | =0.5 | =0.95 |
| --- | --- | --- | --- |
| =0.1 | 5.2045 | 5.2644 | 5.3087 |
| =0.5 | 5.2051 | 5.2651 | 5.3121 |
| =0.95 | 5.2052 | 5.2684 | 5.3315 |

It can be seen that when  increases, the total operating cost of the system all increases. This is due to the fact that when the confidence level increases, the probability distribution of the system will contain more extreme scenarios, forcing the system to adopt a more conservative strategy and set aside more operational reserve. It leads to an increase in the total cost. The decision makers can balance the robustness and economy by choosing a reasonable confidence level.

Comparison of solution methods

In this paper, the i-C&CG algorithm is used to solve the DRO model, and the is set to 0.001, , and , . The iterative convergence process is shown in Fig. 11. As the number of iterations increases, the gap between the master problem and the subproblem is gradually reduced. After four iterations the convergence threshold is reached at the end of the solution. The effectiveness of the algorithm is verified.

Fig. 11 Algorithm solving iterative convergence process

Using the same case and solution accuracy, i-C&CG and C&CG methods are compared. The results are shown in TABLE V. As can be seen, due to the fact that i-C&CG solves the master problem inexactly, the solution accuracy is not as well as the C&CG method. But the relative error is still within the allowable range. As for the solving time, the i-C&CG solution takes 26% less time compared with the C&CG method. It indicates that the i-C&CG method accelerates significantly and has higher solving efficiency.

TABLE V

Comparison of Method Results

| Method | Total cost(billions) | Number of iterations | Solving time |
| --- | --- | --- | --- |
| C&CG | 5.3109 | 4 | 143min |
| i-C&CG | 5.3121 | 4 | 105min |
| Gap | 0.02% | 0 | 26.6% |

Comparative analysis with other dispatch methods

In order to verify the effectiveness of the DRO based dispatch strategy, it is compared with RO and SO based methods. As an example, the algorithm in this chapter solves the optimal solution of the decision variables in the first stage of each method separately. Then 1,000 scenarios are generated for intraday economic dispatch using Monte Carlo method.  The cost expectations of comparing the methods are shown in TABLE VI.

TABLE VI

Comparison of Results of Multiple Dispatching Methods

|  |  | DRO | RO | SO |
| --- | --- | --- | --- | --- |
| Stage 1 cost (billions) |  | 0.63 | 0.86 | 0.61 |
| Probability Distribution Test Cost Expectation (billions) | Mean value | 5.66 | 6.13 | 5.79 |
|  | Maximum value | 9.63 | 11.14 | 10.26 |

It can be seen that the total system cost of the RO is the highest among the three methods due to the optimal decision for the worst-case scenario. In order to make the results robust, the dispatch strategy will tend to start more units and storage devices to cope with the uncertainty.

Comparing the DRO based results with the SO based results, the DRO based results are higher than the SO based ones at the first stage. It is because the DRO considers the worst distribution of the scenario set and chooses a more conservative strategy.

Conclusion

In this paper, a distributionally robust dispatch model of thermal units with multiple energy storage considering oxygen-enriched deep peaking technology is proposed. A refined operation model considering oxygen-enriched deep peaking of units and coordinated operation of battery and hydrogen energy storage is constructed. Additionally, a data-driven DRO model is established to cope with the uncertainty. An i-C&CG method is then developed to solve the model efficiently. The case study reveals that multiple energy storage can effectively improve system flexibility. After considering the synergistic operation of energy storage and units, the cost of inflexibility penalty is reduced by 73%, and the power abandonment rate is reduced by 19.47%, which effectively alleviates the problem of inflexibility. In addition, the time required for i-C&CG solving is reduced by 26% compared with the C&CG method. This indicates that the proposed method accelerates significantly when facing large-scale MILP with higher efficiency.

Future work will continue to quantify the flexibility of emerging resources, such as electric vehicles and virtual power plants, and incorporate them into a collaborative dispatch framework. A more comprehensive quantitative model of flexibility will be constructed to further enhance the economy and flexibility of system.

References

[1]	     L. Yang, N. Zhou and G. Zhou, "An accurate ladder-type ramp rate constraint derived from field test data for thermal power unit with deep peak regulation," IEEE Transactions on Power Systems, vol. 39, pp. 1408-1420, 2024.

[2]	M. Teng, C. Chen and Y. Zhao, “A distributionally robust optimized day-ahead dispatch method for power systems with uncertain wind power access taking into account deep regulation of thermal units and energy storage,” Grid Technology, vol. 48, pp. 3122-3132, 2024.

[3]	Y. Lu, P. Yang and Z. Dong, “Coordinated optimal dispatch considering the operational flexibility of hydrogen-oxygen dual-cycle system,” Power System Automation, vol. 47, pp. 91-99, 2023.

[4]	K. Bruninx, Y. Dvorkin and E. Delarue, "Coupling pumped hydro energy storage with unit commitment," IEEE Transactions on Sustainable Energy, vol. 7, pp. 786-796, 2016.

[5]	H. Liu, L. Chen and S. Cui, "Distributionally robust dispatch of power system with advanced adiabatic compressed air energy storage for frequency security," Journal of Energy Storage, vol. 98, p. 112782, 2024.

[6]	D. Zhao, C. Xu and R. Tao, “A review of the research on flexible regulation of multifarious distributed energy storage on the distribution side of novel power systems,” Chinese Journal of Electrical Engineering, vol. 43, pp. 1776-1799, 2023.

[7]	J. Zhang, K. Sun and C. Li, "Economic scheduling of gaseous-liquid hydrogen generation and storage plants considering complementarity of multiple products," Journal of Modern Power Systems and Clean Energy, vol. 11, pp. 223-233, 2023.

[8]	C. Chen, Y. Zhang, and S. Zhu, “Optimal dispatch of an integrated electric-hydrogen-thermal energy system considering multiple electrolysers with multiple operating conditions in combined operation,” Power Grid Technology, vol. 49, pp. 542-551, 2025.

[9]	W. Zhang, J. Zhou and M. Li, “Multi-row optimization strategy for wind-scenic hydrogen production in alkaline-proton exchange membrane hybrid electrolysers taking into account energy consumption characteristics,” Power Grid Technology, vol. 49, pp. 63-72, 2025.

[10]	Y. Qiu, B. Zhou and T. Zang, "Extended load flexibility of utility-scale P2H plants: Optimal production scheduling considering dynamic thermal and HTO impurity effects," Renewable Energy, vol. 217, p. 119198, 2023.

[11]	Z. Lu, Y. Lin, and Y. Qiao, “Flexibility supply-demand balance of very high percentage renewable power systems,” Power System Automation, vol. 46, pp. 3-16, 2022.

[12]	K. Sun, D. Zhang and J. Wang, "Optimal dispatch scheme considering system operational flexibility," Electric Power Systems Research, vol. 239, p. 111217, 2025.

[13]	Y. Tong, J. Hu and X. Liu, “Quantification of flexible supply and demand in new energy power systems and distributionally robust optimal dispatch,” Power System Automation, vol. 47, pp. 80-90, 2023.

[14]	H. Li, N. Zhang and W. Bao, "Modeling and planning of multi-timescale flexible resources in power systems," CSEE Journal of Power and Energy Systems, pp. 1-10, 2022.

[15]	C. Zhang, Z. Dong and L. Yang, "A feasibility pump based solution algorithm for two-stage robust optimization with integer recourses of energy storage systems," IEEE Transactions on Sustainable Energy, vol. 12, pp. 1834-1837, 2021.

[16]	M. Y. Tsang, K. S. Shehadeh and F. E. Curtis, "An inexact column-and-constraint generation method to solve two-stage robust optimization problems," Operations Research Letters, vol. 51, pp. 92-98, 2023.

[17]	Z. Zhang, M. Zhou and Z. Guo, “A cooperative robust planning method for transmission grid and energy storage considering dynamic thermal calibration of overhead lines,” Chinese Journal of Electrical Engineering, vol. 45, pp. 1-14, 2025.

This research was supported by the National Key R&D Program of China (No.2022YFB2403100).

X. Wu, Y. Fan, J. Zhou are with the School of Electrical Engineering, Xi’an Jiaotong University, Xi’an 710049, China. D. Liu is with the State Grid Economic and Technological Research Institute Co., Ltd, Beijing, China.

(e-mail: wuxiong@mail.xjtu.edu.cn; fyl690619@stu.xjtu.edu.cn; zjj010627@stu.xjtu.edu.cn; liudong@chinasperi.sgcc.com.cn).

Digital Object Identifier