Multi-MG Electricity-Carbon Joint Optimization Method Considering Robust Market Clearing

Yang Xina, Wang Xulia, Zhou Fana, Wang Leib⁎

a State Grid Anhui Electric Power Co., Ltd., Economic and Technical Research Institute, Hefei, 230022 China.

b Anhui Provincial Laboratory of Renewable Energy Utilization and Energy Saving (Hefei University of Technology), Hefei, 230009 China.

A R T I C L E  I N F O

Keywords:

Micro-Grid

marginal cost pricing theory

electricity-carbon trading

accelerated-adaptive alternating direction multiplier method

asymmetric Nash bargaining

A B S T R A C T

The large-scale access of renewable power generation, the prosperity of the carbon market and distributed power generation trading market, and the influx of capital into the power market have created a context in which Micro-Grid (MGs) can be seen as a key technology for promoting renewable energy consumption. In this context, it is likely that multiple MGs will participate in the market. In light of the inherent unpredictability of wind power and other clean energy sources within each MG, a multi MG s electricity-carbon joint optimization operation method that considers robust market clearing is proposed. This method ensures the economic viability of the multi- MG s alliance while adhering to the principles of low-carbon operations and security. The constructed model comprises two layers. The upper layer establishes a robust clearing model of the upper-level electricity-carbon joint market with optimal accruals and benefits based on the marginal pricing theory. The lower layer considers the coupling of electricity-carbon joint trading, establishes a multi- MG s low-carbon joint optimization model based on the Nash negotiation theory, and proposes an electricity-carbon revenue allocation method based on the contribution degree of the interaction products to realize the reasonable allocation of the revenue of each MG. In conclusion, the accelerated-adaptive alternating direction multiplier method (AA-ADMM) is put forth as a means of solving the model. The results of the illustrative example serve to corroborate the efficacy of the methodology proposed in this paper.

1. Introduction

As the proportion of renewable energy generation increases and flexible resources, such as distributed energy storage and electric vehicles, become more widely available, the discrepancy between the spatial and temporal characteristics of power system demand and supply is becoming increasingly pronounced. For this reason, Micro-Grid (MGs), which utilize advanced communication technologies to effectively aggregate distributed power sources, controllable loads, and energy storage in order to respond flexibly to scheduling commands and participate in power market transactions, have emerged as a solution [2-3].

Corresponding author. Wang Lei

E-mail addresses: wanglei818818@sina.cn

There are not only spatial and temporal complementary characteristics of electricity and power among multi-MGs in regional power grids, but also there are interest associations and conflicts among multi-MGs decision makers [4]. The current game-theoretic approach to energy trading between multiple players can be divided into two categories: non-cooperative game and cooperative game. In a non-cooperative game, participants compete with one another by attempting to attain the optimal cost for each individual, with the objective of achieving the lowest energy cost possible, in accordance with the principles of Nash equilibrium. In their study, Literature [5] proposed a distributed trading method based on a non-cooperative game to achieve energy sharing among multiple microgrids. Literature [6-7] investigated the pricing strategies between different MGs and distributed resources, establishing a non-cooperative dynamic. A game pricing model for multiple MGs was developed by literature [8], who also employed stochastic programming methods to create and resolve an energy bidding method for a three-tier Cournot-Nash game. This was done in order to address the impacts of market competition and uncertainty resulting from renewable energy access. In a non-cooperative game, each subject is regarded as self-interested and adversarial, thereby failing to consider the broader interest of the group. In contrast, a cooperative game emphasizes that participants can collaborate and form alliances to advance their shared interests. The literature [9] is based on the coalitional game optimization approach, whereby joint gains are distributed among the members of the coalition in accordance with the Shapley value. The literature [10] employs the Nash bargaining model to address the shortcomings of the coalitional game, namely its high computational complexity and difficulty in ensuring the maximization of global benefits. While the aforementioned studies have demonstrated the potential for collaborative optimization among multiple MGs decision-makers, several challenges remain. Firstly, the impact of the MG on market clearing outcomes has not been fully considered. Currently, the MG's role is limited to that of a price acceptor within the market, which represents an internal optimization within the MG. Secondly, the internal interests of the multi-MGs are not distributed equitably.

In order to address the aforementioned issues, this paper proposes the implementation of a clearing tariff based on the locational marginal price (LMP) as a means of encouraging the participation of all relevant parties in the transaction [11-12]. The objective is to establish the interaction between the dynamic pricing of the upper power market and the operation strategy of MG, and to realize the energy management of multi-MGs. LMP contains information on energy demand, network loss, nodal grid, branch circuit blockage, and so forth. It also guarantees the characteristics of real-time power balance and economic dispatch in a market-oriented way, which is very suitable for short-term decision-making in the spot market. Furthermore, in accordance with the literature [13], the MG comprehensive contribution factor is introduced to assess the influence of electricity-carbon sharing trading volume among MGs on profit distribution, thereby facilitating a more equitable distribution of profits. Furthermore, in practical operation, MG is significantly influenced by the stochastic nature of renewable energy resources and load power uncertainty. However, the aforementioned literature has not yet conducted a comprehensive analysis of this uncertainty. In order to account for the additional cost of stabilizing fluctuations in renewable energy resources and loads, this paper employs the uncertainty locational marginal price (ULMP) to charge the uncertain resources at the time of market clearing. This allows for the measurement of source-load uncertainty and system flexibility cost [14–15].

At present, an increasing number of countries have implemented policies pertaining to energy conservation and carbon emission reduction, thereby establishing multi-MGs low-carbon cooperative operation as a novel area of research. [16] A multi-MGs joint optimization operation model was constructed based on a stepped carbon price. [17] put forth a peer-to-peer trading model of electricity-carbon standby as a means of incentivizing the sharing of multiple products among MGs. [18] puts forth a revenue-sharing methodology for MG alliances that considers the role of carbon emission reduction. Nevertheless, all of the aforementioned studies employed regional average carbon emission factors, thereby neglecting the influence of spatial and temporal variability on the operational strategy of multi-MGs low-carbon collaboration. In recent years, the in-depth excavation of demand-side emission reduction potential has led to a shift in the apportionment of carbon emission responsibility from direct to indirect. Some scholars have put forward the theory of carbon emission flow, which obtains the node dynamic carbon emission factor that can reflect the characteristics of spatial and temporal variability of carbon. The tracing of carbon emissions [19-20] provides a clear illustration of the transfer path of indirect carbon emission responsibility. However, the apportionment result is influenced by the node location [21], and it is unable to reflect the impact of users' electricity consumption behavior changes on system carbon emissions. In order to address the shortcomings of the carbon emission flow theory, some scholars have proposed the marginal carbon intensity theory based on LMP [22]. This theory adopts the marginal carbon intensity (MCI) to quantify the user's electricity consumption carbon emission responsibility, excludes the impact of the location factor on the apportionment of carbon emissions, and reflects the relationship between the node power change and the system carbon emission micro-increase at the same time. This can be simultaneously derived by the market-clearing model.

In the context of multi-MGs co-optimization algorithms, [23] employed Kriging meta-model fitting and substitution of the virtual power plant energy internal management model to derive the trading tariffs and output schedules of MGs. [24] employed the second-order cone relaxation technique to supplant the nonlinear component of the multi-MGs optimization model. [25] utilized the Lagrange dyadic relaxation method to decompose and coordinate the multi-MGs optimization scheduling problem. However, the aforementioned literature is unable to achieve a balance between model solution speed and solution efficiency while ensuring MG privacy. The adaptive alternating direction method of multipliers (A-ADMM) is employed to address the issue of disparate interest subject transactions. It is capable of effectively mitigating the dependency of the alternating direction method of multipliers (ADMM) on the initial value through the dynamic adjustment of the penalty parameter during the iteration process while safeguarding the privacy of members [26]. Nevertheless, the primary benefit of the adaptive mechanism in the A-ADMM algorithm lies in its capacity to obtain an acceptable convergence speed when the optimal penalty factor is uncertain. This circumvents the issue of slow convergence that can arise from selecting an inappropriate value. However, there is still scope for enhancing the algorithm's solution speed. This paper improves the A-ADMM algorithm by the accelerated ADMM in reference [27], dynamically adjusting the value of the penalty factor, and proposes a solution method with faster speed and better privacy in the joint optimization of multi-MGs electricity and carbon.

In conclusion, this paper puts forth a multi-MGs electricity-carbon joint double layer optimization and solution method based on market robust clearing. The principal innovations and contributions of this paper are as follows:

(1) An electricity-carbon joint market clearing model based on the principle of marginal pricing is established with the objective of efficiency optimization. The electricity price, reserve price, and carbon intensity at each node are cleared using LMP, ULMP, and MCI, which interact with the lower-layer MG operation strategy.

(2) In consideration of the interdependence of electricity and carbon trading, a multi-MGs low-carbon joint optimization lower-layer model is established, based on Nash negotiation theory. A comprehensive MG contribution factor is introduced to ensure the fairness of benefit distribution.

(3) The A-ADMM is enhanced, and the improved method markedly accelerates the solution process while safeguarding the confidentiality of members.

The rest of the paper is organized as follows. Section 2 briefly describes the multi-MG low-carbon cooperative architecture based on market-robust clearing proposed in this paper. Section 3 describes the upper-layer electricity-carbon joint market robust clearing model and price calculation method proposed in this paper. Section 4 introduces the multi-MG electricity-carbon joint optimization operation model constructed in the lower layer. Section 5 introduces the solution method of the model. Section 6 verifies the effectiveness of the multi-MG cooperative optimization method proposed in this paper through an arithmetic example. Finally, Section 7 gives the conclusion.

2. A Multi-MGs Low-Carbon Cooperative Framework that Considers Robust Market Clearing

Figure 1 illustrates the schematic diagram of the multi-MGs electricity-carbon joint optimization operation, as proposed in this paper, which comprises two distinct parts: the electricity-carbon joint market and the multi-MGs electricity-carbon cooperative game.

Fig. 1. Schematic diagram of multi-MG electricity-carbon combined optimization operation

The electricity-carbon joint market comprises the electricity market, the carbon market, and the electricity-carbon cloud service platform (ECCSP), which is equipped with functions such as data accounting, data storage, smart contracts, and information transmission. The ECCSP plays a pivotal role in the electricity-carbon joint market, facilitating the safe operation of the electricity market and the carbon market. It plays a pivotal role in the joint electricity-carbon market, facilitating the safe operation of both the electricity market and the carbon market. The power market and carbon market take into account the inherent uncertainty and operate in accordance with their optimal strategies, subsequently uploading market transaction data to the ECCSP. The ECCSP then disseminates information, including LMP, ULMP, MCI, and carbon quota price, to the lower system based on the transaction data and the operational status of the upper power grid. This enables the MGs to participate in the optimal operation of the electricity-carbon cooperative alliance [28].

The cooperative gaming electricity-carbon multi-MGs is comprised of two distinct entities: the MG and the cooperative gaming electricity-carbon trading center (CGE-CTC). The MGs, which are primarily comprised of wind turbines, photovoltaic cells, energy storage systems, interruptible loads, gas turbines, and other components, are highly flexible. In contrast, the primary function of the CGE-CTC is to collect and aggregate the volume and price of electricity traded. The operators of the MGs determine the final results of the energy trading and formulate the corresponding revenue distribution scheme through data interaction and iterative calculations. The final trading results are then sent to the MGs through the communication network. The final transaction results are transmitted to the MGs via the communication network. Subsequently, each MG receives the information issued by the electricity-carbon joint market and utilizes it to rationally arrange dispatching, formulate electricity-carbon trading strategies through CGE-CTC, and feed back the demand for trading of electricity, reserves, and carbon quotas to the electricity. A consensus must be reached on the trading of carbon within the joint market in order to fully utilize the advantages inherent in the complementary nature of cross-regional resources. This will enable the interactive sharing of electricity and carbon quotas while simultaneously reducing the impact of uncertainty in sources and loads.

The electricity-carbon joint market and the multi-MGs cooperative game are two distinct yet interdependent entities. The market decision information exerts influence on the scheduling arrangement of each MG, while the MG trading demand serves to constrain the operation plan of the market players [29].

3. Robust Clearing Model for the Electricity-Carbon Joint Market

The electricity-carbon joint market accurately conveys the information of electricity price, reserve price, nodal carbon intensity, and carbon quota price to each MG, which is a prerequisite for the cooperative operation of each MG. In this paper, we consider the impact of source-load uncertainty, assume that the electricity-carbon joint market is based on the node marginal price for clearing, and use the DC current model to construct a robust clearing model for the electricity-carbon joint market by ignoring the reactive power and line losses of the grid [30].

3.1. Modeling Uncertainty

Given that polyhedral uncertainty sets are simple in structure, easy to regulate, and provide protection for decision making when uncertain parameters are unknown, they ensure worst-case performance. Therefore, in this paper, polyhedral uncertainty sets are used to model the uncertainty of renewable energy resources and loads.

Define the polyhedral uncertainty sets Ui,wt, Ui,pv, and Ui,load for wind power, PV, and load, respectively:

(1)

where Pi,wt,t, Pi,pv,t, and Pi,load,t are the predicted values of wind power, PV, and load output at node i at time t, respectively. Pi,wt,a,t, Pi,pv,a,t, and Pi,load,a,t are the actual values of wind power, PV, and load output at node i at time t. ∆Pi,wt,t,max, ∆Pi,pv,t,max, and ∆Pi,load,t,max are the maximum values of the deviation of the wind power, PV, and load output at node i at time t. Γi,wt, Γi,pv, and Γi,load are the uncertainty regulation parameters of wind power, PV, and load output at node i at time t, and T is the total number of time periods.

3.2. Market Robust Clearing Modeling

The marginal pricing theory is used to obtain the price of electricity at each node in the power system, and a two-stage robust clearing model of the market taking into account the source-load uncertainty and carbon trading is established with the minimization of the production cost of the power producer and the carbon trading cost as the objective function in consideration of the low-carbon economic operation of the market:

(2)

where Ωge is the set of power plants, πi,ge,t, Pi,ge,t are the electricity price and power generation of power plant i in time period t. πc0 and Ei,p,t, are the carbon trading price and carbon quota trading volume of the day, respectively.

The constraints include nodal power balance constraints, branch circuit tidal current constraints, generator output constraints, carbon quota constraints, etc., as shown below:

(3)

(4)

(5)

(6)

(7)

(8)

(9)

(10)

(11)

(12)

(13)

where JGi, JLi, JMG, U denote the set of units connected to node i, the set of loads, the set of MGs, and the set of uncertain elements, respectively. N denotes the number of nodes. Pi,MNt, and Pi,load,t are the MG interacting power and loads on node i at moment t. ei,ge is the carbon intensity of the power plant. and χE is the carbon quota assigned to a unit of electricity supply. Ei,ex,t is the amount of carbon quota traded between MGi and the superior grid at moment t. Tl-i denotes the power transfer factor of node i to line l. Llmax denotes the capacity limit of line l. Pi,ge,t,min, Pi,ge,t,max, Ri,ge,min, Ri,ge,max are the minimum and maximum values of the output of power plant i and the small and maximum values of the climb, respectively. δ denotes the uncertain elements, including the renewable energy output and load fluctuation values. λ, μ are the Lagrange multipliers of Eq. (3)-(8) and Eq. (9)-(13), respectively. Eq. (3)-(8) denote the nodal balance constraints, carbon quota constraints, line tidal current constraints, unit output constraints, and ramp-up constraints in the day-ahead market, respectively. Eq. (9)-(13) denote the corresponding constraints after taking into account the impacts of uncertainty.

3.3 Worst-Case Scenarios for MG Operation

In this paper, we consider the impact of MG's worst-case scenario under uncertainty on the upper-tier electricity market. Based on the traditional three-layer two-stage robust optimization without considering intra-coalition transactions, and construct a two-stage robust optimization model in the form of min-max-min, and the model is written in compact mode:

(14)

where U denotes the uncertainty set, Ω(x,z,θ) denotes the feasible domain of y under a certain determination (x,z,θ). the first row of constraints denotes the constraints related to variables x and z in the first stage. The second row of constraints denotes the constraints related to variables x, y, z, and θ in the second stage. a, b are the column vectors of the constant coefficients in the objective function. A, B, C, D, E, and F are the matrices of constant coefficients in the constraints, and c, d are the column vectors of constant coefficients in the constraints. In this paper, according to the principle of column and constraint generation (C&CG) algorithm [31], (24) is decomposed into an outer min problem and an inner max-min problem, due to the inner max-min problem is a two-layer optimization problem, which is converted into a single-layer max optimization problem by using the strong dyadic theory or the Karush-Kuhn-Tucker (KKT) condition, and the nonlinear terms appearing in it are linearized by using the big M method, and the worst-case scenarios of each MG are obtained finally. operation scenario, and the specific solution procedure is shown in Appendix A1.

3.4. Calculation of Electricity Prices and Carbon Intensity in the Electricity-Carbon Joint Market

(1) Calculation of electricity prices

The LMP considering carbon trading can be obtained from the derivation of the Lagrange function of the robust clearing model of the electricity-carbon joint market, which can be expressed as follows:

(15)

where L is the Lagrange function of the robust clearing model for the electricity-carbon joint market. LMPi,t is represented by three monomials, with the first term denoting the energy component of the marginal tariff at that node, the second term denoting the blocking component pricing, and the third term the carbon trading component. It is worth noting that the coupling of carbon trading costs in the tariff clearing is to change the marginal unit mix of the clearing, and the tariff information issued does not contain the carbon trading component.

Referring to [14], the ULMP can be viewed as the marginal cost of an incremental unit of uncertainty in the case of market clearing. It is worth noting that in calculating the ULMP, this paper considers renewable energy output as a negative load. Specifically, it can be expressed as:

(16)

where δi,t is the output deviation of the uncertain component at node i at time t. πi,u,t denotes the ULMP of node i at moment t. Equating the volatility cost due to renewable energy output increments with the volatility cost due to load demand shortfalls is denoted by πi,up,t and vice versa by πi,down,t:

(17)

(2) Calculation of carbon price

With LMP and ULMP, the charges and credits of market participants become transparent and fair.

In environments where carbon trading is considered, unit offers typically contain a non-carbon component and a carbon emission component, and the nodal tariff at node n is a linear combination of the system marginal unit offers:

(18)

where aT n is the corresponding linear combination of coefficient column vectors satisfying ETan=1 and c denotes the tariff component of each unit in the LMP, the node marginal carbon intensity can be expressed as:

(19)

4. Multi-MGs Electricity-Carbon Joint Optimization Operation Model

The MG studied in this paper consists of wind turbine, PV, energy storage, interruptible load, and gas turbine, which can be traded with the joint electricity-carbon market for both electricity and carbon trading, as well as with other MGs for cooperative trading. To meet the demand of each aggregation unit of the MG itself, it is necessary to consider the operating constraints of each unit of the MG and make a trade-off between economy and low carbon. By optimizing the scheduling equipment to determine the appropriate trading strategy, the efficiency of the MG cooperative alliance and the optimization of resource utilization can be achieved [32].

4.1 Electricity-Carbon Joint Market Tariff Calculation

(1) Model objective function

Each MG has as its optimization objective the minimization of the operating cost Ci,t consisting of three components: electricity transaction cost, carbon cost and equipment operation and maintenance cost:

(20)

where Ci,power,t, Ci,carbon,t, and Ci,om,t are the MGi electricity transaction cost, carbon cost and equipment O&M cost at moment t, respectively.

1）Electric energy transaction costs

Electricity transaction costs include MGi's transaction costs with other MGs and transaction costs with the upper power grid.

(21)

where Cij,e,t is the electricity transaction cost between MGi and MGj at time t. Ci,MN,e,t is the electricity transaction cost between MGi and the superior grid at time t. πij,e,t, and Pij,t  are the transaction price and interactive power between MGi and MGj at time t. πMN,e,t is the transaction price between MGi and the superior grid at time t.

2) Cost of carbon emissions

The cost of carbon emissions in this paper consists of the cost of carbon allowance trading between MGs and the cost of carbon allowance trading between MGs and their parent grid [33].

(22)

where Ci,carbon,t is the cost of carbon emission, Ci,ex,c,t and Cij,c,t are the cost of carbon allowance trading between MG and the superior grid and the cost of carbon allowance trading between MGs at moment t, respectively. Ei,ex,t is the amount of carbon allowances traded with the market in MGi at time t. πij,c,t and Eij,t are the unit price and trading volume of carbon allowances traded between MGi and MGj at time t, respectively. ρi,t is the marginal carbon intensity of the node where MGi is located at moment t. Pi,MT,t and ei,MT are the gas turbine output power and carbon intensity in MGi at moment t.

3) Equipment operation and maintenance costs

Equipment O&M costs include interruptible load, energy storage and gas turbine O&M costs.

(23)

(24)

where Ci,IL,t , Ci,ESS,t and Ci,MT,t are the cost of calling interruptible load, energy storage and gas turbine in MGi at time t, respectively. M, R, and Q are the total number of interruptible loads, energy storage, and gas turbines in MGi, respectively. K1, K2, and K3 are positive parameters. P2 i,IL,m,t is the amount of interruptions calling interruptible loads m in MGi at moment t. θm is the parameter of the type of interruptible loads. νESS,t is the unit calling cost of the energy storage. Pi,ESS,r,t is the MGi's energy storage unit r at moment t charging and discharging power. Pi,dis,r,t, Pi,ch,r,t, and ηi,r are the discharging power, charging power, and efficiency of the energy storage unit r in the MGi at moment t, respectively. And C(Pi,MT,q,t) is the calling cost of the gas turbine q in the MGi at moment t.

(2) Model constraints

The multi-MGs cooperative operation optimization model constraints are shown in Appendix A2, equations (A6)-(A14).

4.2 Multi-MGs Cooperation Operation of an Asymmetric Nash Negotiation Model

The revenue sharing model for the multi-MGs power-carbon cooperative operation based on asymmetric Nash bargaining is as follows:

(25)

where C* i,0 is the optimal robust cost of MGi when it operates independently. Ci is the operating cost of MGi when it participates in a cooperative alliance.

The multi-MGs cooperative operation Nash negotiation model is a non-convex nonlinear optimization problem in nature, and it is more difficult to solve the model directly. In order to overcome this problem, this paper draws on the model-equivalent transformation method in the literature [34], and transforms the multi-MGs cooperative operation Nash negotiation model into two sub-problems: the MG coalition operation cost minimization sub-problem and the transaction benefit distribution sub-problem.

Subproblem 1: MG coalition operating cost minimization sub-problem.

(26)

Subproblem 2: Subproblem of trading revenue allocation based on combined contribution factors.

(27)

where ω* MG,i is the optimal value of the objective function of MGi in subproblem 1.

4.3 MG Combined Contribution Factor

For the problem of profit distribution arising from multi-MGs cooperation, existing studies mainly use the symmetric Nash negotiation model to analyze, in which each participant has the same bargaining power in profit distribution. However, each MG has different behaviors and contributions in electricity-carbon trading, and in order to measure the impact of electricity-carbon shared trading volume among MGs on profit distribution and to achieve a more equitable profit distribution, this paper adopts the asymmetric Nash negotiation model [35].

The combined contribution factor r* i to the MG is defined as follows:

(28)

where NMG is the total number of MGs. T is 24h, γe and γc are the weights of MG electricity trading and carbon trading, respectively.

Equation (28) has the following characteristics: 1) Any MG that has traded electricity and carbon in the cooperative alliance will receive a contribution. 2) The larger the trading volume of electricity-carbon trading in the cooperative alliance, the larger the contribution of the MG, and the larger the profit it receives. 3) The contribution of MGs trading the same products in the cooperative alliance is different at different times, and the larger the price difference between products traded inside and outside of the cooperative alliance, the larger the contribution The larger the contribution, the more profit will be gained. If the trading price between MGs is equal to the trading price between MGs and the market, the contribution of MGs to the alliance is zero, and the corresponding profit distribution is zero.

In order to ensure that the above game can be carried out normally, the following two conditions need to be satisfied: (1) the price of electricity traded between MGs is between the market purchase and sale price of electricity, as shown in Equation (29). (2) the price of carbon traded between MGs is between the market purchase and sale price of carbon, as shown in Equation (30).

(29)

(30)

where πMN,e,sell,t, πMN,e,buy,t are the prices at which the MG sells and buys electricity from the grid, respectively. πc0,sell,t, πc0,buy,t are the prices at which the MG sells and buys carbon allowances from the market, respectively.

5. Model Solving

5.1 Upper Layer Model Solving

The upper power grid is cleared under the market mechanism based on the node marginal price, and the upper power grid robust clearing model can also be solved by the C&CG algorithm through MATLAB+YALMIP+CPLEX, with Eq. (1) as the objective function, and Eq. (2) to (13) as the constraints, and solved to obtain the optimization results, and at the same time to obtain the results of the node marginal carbon intensity, and the specific solving process is shown in Appendix A2.

5.2 Equivalent Transformation and Solution of the Lower-Level Model

This paper presents an improvement to the A-ADMM algorithm, as described in reference [27], which is known as AA-ADMM. This new method dynamically adjusts the value of the penalty factor, as shown in Equation (31), in order to optimise the solution to subproblems 1 and 2.

(31)

where ρk+1 is the penalization factor, β is the scaling factor of the original residuals and the pairwise residuals. τiner and τdeer are the scaling factors of the step update.

When the original residuals are large, the relationship between the variables is adjusted by increasing the step size to accelerate the convergence of the original residuals, when the pairwise residuals are large, the convergence of the variables is promoted by decreasing the step size to reduce the oscillations of the objective function, and when there is not a big difference between the pairwise residuals and the original residuals, the penalty factor is dynamically adjusted to increase the convergence speed of the algorithm.

(1) MG coalition operation cost minimization subproblem solving

When Eq. (32) is satisfied, it means that the subjects of the MG cooperative alliance reach a trading consensus.

(32)

Introduce the Lagrange multipliers λij,e,t, λij,c,t and the penalization factors ψij,e, ψij,c to construct the augmented generalized Lagrange functional form of Subproblem 1:

(33)

Decomposition of Eq. (33) yields a distributed optimization model for each MG subproblem 1:

(34)

(2) Transaction revenue allocation subproblem solving.

When Eq. (35) is satisfied, it indicates that the MGs have reached a consensus on the revenue allocation subproblem.

(35)

The optimal values of P* ij,t, E* ij,t for the amount of electricity and carbon quota traded between each MG are obtained by solving Subproblem 1, and substituting into Eq. (27):

(36)

Introducing Lagrange multipliers μij,e,t, μij,c,t and penalty factors σij,e, σij,c, the augmented generalized Lagrange function of subproblem 2 is constructed and decomposed according to the principle of A-ADMM to obtain the distributed optimization model of each MG subproblem 2:

(37)

For subproblems 1 and 2, the specific iterative solution steps are shown in Appendix A3.

5.3. Two-Layer Model Solving

The two-layer model proposed in this paper presents a system of two-way information interactions, comprising electricity, carbon intensity, uncertainty components, demand, and price of carbon quotas. These interactions can be solved by iterative algorithms, with the iterative solving process of the model illustrated in Fig. 2. The specific steps involved are as follows:

Fig. 2.  Schematic diagram of the iterative solution of the two-layer model

(a) The initial value of power demand for each MG is set based on the historical data of each MG, and k is set to 1.

(b) The upper market clearing model is solved based on the deterministic market of the previous day, the upper grid is scheduled, and the LMP, ULMP, and MCI are updated.

(c) The worst-case scenario for each MG operation is solved using the C&CG algorithm based on the updated clearing results, and return to the upper layer grid for robust power price clearing.

(d) The lower-layer multi-MGs electricity-carbon cooperation game model is solved, the number of iterations of the lower-layer model is set to a=0, and the maximum number of iterations and convergence accuracy of the lower-layer model are determined.

(e) Each MG in the lower-level model is optimized separately, and the dy is updated. The subsequent variables are to be calculated.

(f) The original residuals and dyadic residuals of the lower layer model are to be calculated, and the penalty factor is to be updated.

(g) The number of iterations, a, is to be updated and the convergence of the lower-layer model is to be determined. If the model converges, the iteration is to be terminated, and the demand for power purchase from each MG to the upper power is to be obtained. In the event that the grid and the demand for carbon quota trading are not satisfied, the process should be repeated from step (d) until the convergence condition is met.

(g) k=k+1, return to step (b), and repeat solving the two-layer model until either the convergence condition is satisfied or the maximum number of iterations is reached.

6. Example Analysis

The algorithm sets the MG coalition to contain three MGs, whose optimized operation period is 24 h, and the scheduling interval is 1 h. The generation types of the superior grid include wind and thermal power generation, and the network node topology and generator parameters of the superior grid are shown in Fig. 3 and Table B1, and the MGs are connected to different nodes of the superior grid, and the related parameters are shown in Table B2 and Table B3. The predicted output of each MG wind, PV, and load day ahead is shown in Fig. 4. The uncertainty of PV and wind power is set to 0.15 of the day-ahead forecast, the uncertainty of load is set to 0.1 of the day-ahead forecast, and the uncertainty of source-load fluctuation is set to 9. The prices of MGs for purchasing and selling carbon allowances to the upper market are$0.3 and $0.15/kg, respectively, and the weights of the revenue allocation are γe=0.5 and γc=0.5, respectively. The calculations presented in this paper utilize the Gurobi optimization tool in MATLAB software via the YALMIP toolkit.

Fig.3  Nodal diagram of the higher grid

Fig. 4.  MGs Predicted Output

6.1. Analysis of Market Robust Clearing Results

The LMP, ULMP, and MCI obtained from the constructed electricity-carbon joint market clearing are shown in Fig. 5. The system clearing results are consistent for the majority of the time period, indicating that there is no blockage on the line. However, there are two notable exceptions: the first is between 12-13, and the second is between 19-21. This results in nodes 4 and 6 having a higher purchase price. The value of the ULMP is dependent upon the system's marginal unit mix and the line blockage. When the source load within the system is certain, the value of the ULMP is 0. However, when there is uncertainty regarding the injected power at any node within the system, the ULMP It is possible that the value of the MCI may not be equal to zero. The MCI is analogous to the LMP, which is determined by the system marginal unit mix, the unit's carbon emission intensity, and the line blockage. It is also noteworthy that in the electricity-carbon joint market, the transaction cost of the carbon quota is also taken into account in the decision-making process regarding marginal unit mix.

(a) LMP of the 6-bus system at hour 24

(b) ULMP of the 6-bus system at hour 24

(c) MCI of the 6-bus system at hour 24

Fig. 5.  The results of the robust market

6.2. Analysis of Robust Clearing in the Electricity-Carbon Joint Market

Three models are set up to comparatively analyze the impact of spatio-temporal variability of MG node marginal carbon potentials on the operation of multi-MGs electricity-carbon cooperation:

Model 1: Multi-MGs electricity-carbon joint optimization without considering market robust clearing and node marginal carbon intensity.

Model 2: Multi-MGs electricity-carbon joint optimization without considering market robust clearing but accounting for node marginal carbon intensity.

Model 3: Multi-MGs electricity-carbon joint optimization considering market robust clearing and taking into account the nodal marginal carbon intensity, which is the model proposed in this paper.

MG

Fig. 6.  Comparison of power purchases from the upper grid for different models

Fig. 7. Comparison of power trading across MGs under different models

Figures 6 and 7 present a comparative analysis of the various scenarios associated with each MG, with a particular focus on the volume of power purchased from the upper market and the trading of electricity among MGs in Models 1, 2, and 3. Figure 4 employs a positive value to indicate instances where a MG purchases products from the upper grid, while a negative value is used to denote instances where a MG sells products to the upper grid. Model 2 considers the node marginal carbon potentials. In this model, MGs with smaller marginal carbon potentials purchase more low-carbon electricity when the line is blocked and provide excess low-carbon energy to MGs with larger carbon intensities through cooperative trading. This cooperative trading enables MGs to reduce the necessity to purchase high-carbon emission electricity from the upper market, which in turn reduces carbon emissions and serves to alleviate line blockage. While Model 3 considers robustness, when the system source load fluctuates significantly and the load is high, the internal trading within the MG will be more frequent in order to reduce the additional cost due to uncertainty.

6.3. Analysis of the Operation of Multi-MGs Electricity-Carbon Cooperation

Table 1 shows the analysis of the benefits of each MG before and after the cooperation, through the cooperative operation, the operational benefits of each MG are increased by $753 , $437 and $891 , respectively, and the total benefits of the cooperative alliance are increased by $2,081. And in the robust case, the cost of each MG increased by $1,247, $913 and $1,489 , respectively. And the total cost increased by $3,649, compared to the cooperative run without considering robustness.

Table 1  Benefit analysis before and after MG cooperation

|  | MG1 | MG2 | MG3 | MG Collection |
| --- | --- | --- | --- | --- |
| Individual running benefit/$ | 7356 | 7510 | -7578 | 7288 |
| Cooperative running benefit/$ | 8109 | 7947 | -6631 | 425 |
| Benefits of cooperative operation in the robust case/$ | 6862 | 7034 | -8120 | 5776 |
| Benefit added value/$ | 753 | 437 | 891 | 2081 |
| Robustness cost increase/$ | 1247 | 913 | 1489 | 3649 |

Table 2  Uncertainty cost analysis before and after cooperation

|  | MG1 | MG2 | MG3 | MG Collection |
| --- | --- | --- | --- | --- |
| Individual running benefit/$ | 289 | 102 | 301 | 692 |
| Cooperative running benefit/$ | 112 | 57 | 172 | 341 |
| Cost reduction value/$ | 177 | 45 | 129 | 351 |

Table 3  Analysis of carbon emissions before and after MG cooperation

|  | MG1 | MG2 | MG3 | MG Collection |
| --- | --- | --- | --- | --- |
| Carbon emissions before cooperation/t | 3.2769 | 2.5674 | 5.6958 | 11.5401 |
| Carbon emission/t after cooperation | 3.4589 | 2.6732 | 1.8944 | 8.0265 |
| Pre-cooperation carbon cost/$ | -796 | -241 | 567 | -470 |
| Post-cooperation carbon cost/$ | -570 | -222 | -507 | -1299 |

Table 2 presents the results of the cost analysis pertaining to the source load uncertainty, both before and after the commencement of cooperation between the various MGs. In comparison to the pre-cooperation period, the additional cost of uncertainty for each MG following cooperation has been reduced, with a decrease of $177, $45, and $129, respectively. Consequently, the total cost of the cooperative alliance has been reduced by $351 . It can be concluded that the impact of uncertainty on the system can be effectively reduced when multiple MGs operate in a cooperative manner.

Table 3 illustrates the carbon emissions and cost analysis before and after the cooperation of each MG. In comparison to the pre-cooperation period, the carbon trading benefit of each MG increases by $829 following the cooperation. Following cooperation, the carbon emissions of MG1 and MG2 exhibited a slight increase, while those of MG3 experienced a notable decline. This resulted in a substantial reduction in low-carbon emissions, largely attributable to the following factors: firstly, the cooperative operation curtailed the purchased electricity of MG1 and MG2. PP3, and the indirect carbon emissions of MG3 are reduced, thereby achieving a reduction in overall carbon emissions. Secondly, should MG3 have a requirement for the acquisition of carbon allowances, it can engage in direct trading within the MG alliance, obviating the need to purchase from higher markets. The necessity for purchasing from the upper market is thus negated, thereby facilitating a reduction in carbon emissions. Consequently, the operation of multi-MGs cooperation is conducive to the reduction of alliance carbon emissions and plays a positive role in improving the economies of all parties.

(a) Price of electricity traded between MGs

(b) Volume of electricity traded between MGs

Fig. 8. Results of electricity trading between MGs

(a) Price of carbon trading between MGs

(b) Volume of carbon traded between MGs

Fig. 9. Results of carbon trading between MGs

Figure 8 and 9 illustrates the outcomes of electricity-carbon trading among MGs. The variability of load demand for renewable energy resources within different MGs gives rise to differing electricity-carbon trading tendencies in different time periods. During the nocturnal period between 0:00 and 8:00, MG1 and MG2 exhibit augmented renewable energy output and diminished marginal carbon intensity within the upper power grid. Consequently, they are able to procure low-carbon electricity from the upper power grid and sell the surplus renewable energy output and purchased low-carbon electricity to MG3, which has a demand for electricity. This enables a reduction in the energy purchase cost and carbon emissions of MG3. However, during the 10:00-15:00 time period, MG1 has a reduction in renewable energy output and an increase in load, MG2 maintains a consistent level of renewable energy output but experiences an increase in load due to PV output, and MG1 and MG2 have higher prices and carbon intensity for purchasing electricity from the higher grid. Conversely, MG3 experiences a surplus of power during this time period. Consequently, MG1 and MG2 elect to procure electricity from MG3 to satisfy their internal load requirements. For MG3, PV units are employed as renewable energy sources, with output contingent upon solar irradiation. In instances where solar irradiation is insufficient to meet load demand within MG3, the latter primarily procures electricity from other MGs via the cooperative energy trading center to bridge the gap.

6.4. Interactive Product Contribution Impact Analysis

The background to the joint optimization of electricity and carbon in Multi-MGs operations is set out. In accordance with the different bargaining methods, two allocation strategies for benefit distribution have been established. The benefits accruing to individual MGs under the different allocation strategies are set out in Table 4 and Table 5.

Strategy 1: The application of the standard Nash bargaining benefit allocation.

Strategy 2: asymmetric Nash bargaining benefit allocation, which considers the comprehensive contribution factor of the MG.

A comparison of Tables 1 and 3 reveals that the benefit increase for MG1 and MG3 under Strategy 2 is greater than that of Strategy 1, whereas the benefits for MG2 are both reduced. This is due to the high proportion of renewable power generation in MG1, which has a particularly high relative contribution to carbon trading, while MG3 participates in the trading of cooperative products and has a high relative contribution to participation in cooperation. The benefit allocation through the combined contribution factor increases the cooperative benefits of MG1 and MG2. In contrast, the standard Nash bargaining benefit allocation does not take into account the relative contribution degree of the members and thus directly distributes the cooperation benefits equally. Therefore, the asymmetric Nash bargaining benefit allocation, which takes into account the comprehensive contribution factor of MG, is a fair and reasonable approach..

Table 4  Distribution of proceeds in the standard Nash bargaining scenario

|  | MG1 | MG2 | MG3 | MG Collection |
| --- | --- | --- | --- | --- |
| Individual running benefit/$ | 7356 | 7510 | -7578 | 7288 |
| Cooperative running benefit/$ | 8050 | 8204 | - 884 | 9425 |
| Benefit added value/$ | 693.67 | 693.67 | 693.67 | 2081 |

In the asymmetric Nash bargaining revenue distribution, the revenue distribution of the alliance can be flexibly adjusted by changing the weighting coefficients of electric energy trading and carbon trading γe and γc according to the actual situation, if γe =γc means that electric energy trading and carbon trading have the same weighting. If γe>γc means that the alliance encourages members to trade electric energy and the cooperation revenue is allocated to members with a large proportion of electric energy trading. If γe<γc means that the alliance encourages members to trade carbon and the cooperation revenue is allocated to members with a large proportion of carbon trading. If  is used, it means that the coalition encourages members to engage in carbon trading, and the benefits of cooperation are allocated to members with a high proportion of carbon trading. Table 5 shows the analysis of the impact of different weighting coefficients on the distribution of the coalition's revenue, with the increase of the carbon trading weight, the increase of the revenue gained by MG1 becomes more obvious.

Table 5  Analysis of the weighting coefficients on earnings distribution

| Carbon trading weights | MG1 benefits Value added/$ | MG2 Benefits Value added/$ | MG3 Benefits Value added/$ |
| --- | --- | --- | --- |
| 0.2 | 659.98 | 758.43 | 728.31 |
| 0.4 | 681.49 | 426.64 | 872.73 |
| 0.6 | 804.68 | 363.82 | 935.83 |
| 0.8 | 1 002.68 | 216.59 | 948.75 |

6.5. Convergence Analysis of the Iterative Solution

In order to ascertain the efficacy of the AA-ADMM method, a comparative analysis was conducted with the conventional ADMM and A-ADMM. The results of this analysis are presented in Table 6. An excessive or insufficient step size setting will result in an increased number of iterations and time. The traditional ADMM method is more dependent on the step length, and the number of iterations and time show obvious changes under different step lengths. In contrast, the A-ADMM method reduces the dependence on the initial step length by modifying the step length, which reflects that the iteration time of the method is basically the same under The AA-ADMM method has the capacity to enhance the stability of convergence, irrespective of the selected step length. Furthermore, the AA-ADMM method can improve the convergence speed of the algorithm while ensuring the stability by dynamically adjusting the penalty factor. A comparison of the results obtained from the three algorithms reveals that the AA-ADMM method requires fewer iterations and a shorter iteration time when the initial step length is maintained constant. This observation lends support to the efficacy of the algorithm..

Table 6  Comparative analysis of convergence of each iterative method

| Method | Step length | Number of iterations/times | Iteration time/s |
| --- | --- | --- | --- |
| Traditional ADMM | 0.0001 | 201 | 648 |
|  | 0.0010 | 189 | 568 |
|  | 0.0100 | 398 | 1 265 |
| A-ADMM | 0.0001 | 135 | 389 |
|  | 0.0010 | 129 | 487 |
|  | 0.0100 | 146 | 496 |
| AA-ADMM | 0.0001 | 105 | 314 |
|  | 0.0010 | 116 | 298 |
|  | 0.0100 | 103 | 347 |

7. Conclusion

This paper proposes a multi-MGs electricity-carbon joint optimization operation method that considers robust market clearing. This method is designed to account for the potential worst-case scenario of MG operation. It is based on the node margin theory and employs a two-layer model of robust market clearing and multi-MGs cooperative operation. The model is solved using C&CG and AA-ADMM. On this basis, a revenue allocation method based on the contribution degree of interaction products is proposed, taking into account the differing relative contributions of each MG to the cooperative alliance, in order to ensure a fair and reasonable revenue allocation. The following conclusions may be drawn from the analysis of the arithmetic example:

(1) The robust market-based multi-MGs electricity-carbon cooperative alliance can effectively guide MGs to spontaneously adjust their electricity consumption behaviors. Furthermore, through the trading behaviors of the MG cooperative alliance, it can reduce the impacts brought by system uncertainty. This approach can also help to alleviate system line blockage and improve the overall reliability of the system operation. Furthermore, it allows for a deeper exploration of the potential for electricity-carbon cooperation among MGs, with a view to considering both the economic and environmental benefits of the alliance.

(2) By considering the asymmetric Nash bargaining revenue allocation method for carbon quota trading, it can effectively reflect the multi-dimensional inputs and contributions of MGs in the alliance, improve the fairness and reasonableness of revenue allocation, and contribute to the stability of the long-term cooperative relationship between MGs.

(3) The proposed AA-ADMM can markedly enhance the algorithm's convergence velocity and stability by modifying and rectifying the step length throughout each iteration through optimization of the step length updating mechanism. Furthermore, it can augment the computational efficiency of the model while effectively safeguarding the privacy of MG information.

In conclusion, this paper examines and analyzes the method of multi-MGs cooperative operation under uncertainty. Further research could consider the impact of uncertainty pertaining to each MG on the contribution of asymmetric Nash bargaining and the trading of reserves among MGs when distributing the proceeds of multi-MGs.

8. Acknowledgments

This work is supported by the Science and technology project of State Grid Anhui Electric Power Co., Ltd (No. B3120925000G)

References

[1] 	LI Jiamei, Ai Qian, Yin Shuangrui. Market mechanism of virtual power plant participation in peaking and frequency regulation services with reference to foreign experience[J]. Chinese Journal of Electrical Engineering,2022,42(01):37-56.

[2] 	KANG Chongqing, CHEN Qixin, SU Jian, et al. Scientific problems and research framework of virtual power plant with enormous flexible distributed energy resources in new power system[J]. Automation of Electric Power Systems, 2022, 46(18): 3-14.

[3]	LIU Siyuan, AI Qian, ZHENG Jianping, et al. Bi-level coordination mechanism and operation strategy of multi-time scale multiple Micro-Grid[J]. Proceedings of the CSEE, 2018, 38(3): 753-761.

[4]	E. G. Kardakos, C. K. Simoglou and A. G. Bakirtzis, "Optimal Offering Strategy of a Virtual Power Plant: A Stochastic Bi-Level Approach," in IEEE Transactions on Smart Grid, vol. 7, no. 2, pp. 794-806, March 2016.

[5]	ZHAO T, CHOO F H, ZHANG L, et al. Game theory based distributed energy trading for microgrids parks[C/OL]//2017 Asian Conference on Energy, Power and Transportation Electrification (ACEPT), Singapore. 2017: 1-7.

[6]	ZHOU Bo, LÜ Lin, GAO Hongjun, et al. Robust day-ahead trading strategy for multiple Micro-Grid[J]. Power System Technology, 2018, 42(8): 2694-2703.

[7]	XU Z, GUO Y, SUN H B. Competitive pricing game of Micro-Grid: models, strategies, and equilibria[J]. IEEE Transactions on Smart Grid, 2022, 13(6): 4583-4595.

[8]	LI Z, WU L, XU Y, et al. Distributed tri-layer risk-averse stochastic game approach for energy trading among multi-energy microgrids[J/OL]. Applied Energy, 2023: 120282.

[9]	[1]CHIS A, KOIVUNEN V. Coalitional Game-Based Cost Optimization of Energy Portfolio in Smart Grid Communities[J/OL]. IEEE Transactions on Smart Grid, 2019: 1960-1970.

[10]	XIA Yuanxing, XU Qingshan, LI Shujuan, et al. Dynamic and static compound game in transactive energy market and its pricing mechanism[J]. Automation of Electric Power Systems, 2023, 47(6):178-184.

[11]	LI Peijie,LU Yong,BAI Xiaoqing,et al. Decentralized optimization for dynamic economic dispatch based on alternating direction methodof multipliers[J]. Proceedings of the CSEE, 2015, 35(10): 2428-2435(in Chinese).

[12]	LI Gaojunjie, YANG Jun, ZHU Xu, et al. Real-time energy sharing mechanism for charging stations considering user response characteristics of electric vehicles[J]. Automation of Electric Power Systems, 2022, 46(12):56-66.

[13]	CUI S, WANG Y W, LIU X K, et al. Economic Storage Sharing Framework: Asymmetric Bargaining-Based Energy Cooperation[J/OL]. IEEE Transactions on Industrial Informatics, 2021: 7489-7500.

[14]	YE H, GE Y, SHAHIDEHPOUR M, et al. Uncertainty Marginal Price, Transmission Reserve, and Day-Ahead Market Clearing With Robust Unit Commitment[J/OL]. IEEE Transactions on Power Systems, 2017: 1782-1795.

[15]	WANG L, ZHU Z, JIANG C, et al. Bi-Level Robust Optimization for Distribution System With Multiple Microgrids Considering Uncertainty Distribution Locational Marginal Price[J/OL]. IEEE Transactions on Smart Grid, 2021: 1104-1117.

[16]	HOU Haoyu, GE Xiaolin, CAO Xudan. Coalition game optimization method for multiple Micro-Grid considering carbon trading[J]. Proceedings of the CSU-EPSA, 2023, 35(3): 77-85.

[17]	SHEN Sichen, HAN Haiteng, ZHOU Yizhou, et al. Electricity-carbon-reserve peer-to-peer trading model for multiple Micro-Grid based on conditional value-at-risk[J]. Automation of Electric Power Systems, 2022, 46(18): 147-157.

[18]	GE Xiaolin, CAO Xudan, LI Yiling, et al. Optimal operation method for multiple Micro-Grid considering risk and carbon flow[J]. Proceedings of the CSU-EPSA, 2023, 35(8): 126-135.

[19]	ZHOU Tianrui, KANG Chongqing, XU Qianyao, et al. Preliminary Theoretical Investigation on Power System Carbon Emission Flow[J]. Automation of Electric Power Systems, 2012, 36(7):38-43.

[20]	CHENG Y, ZHANG N, WANG Y, et al. Modeling Carbon Emission Flow in Multiple Energy Systems[J/OL]. IEEE Transactions on Smart Grid, 2019: 3562-3574.

[21]	ZHOU Quan, FENG Donghan, XU Changbao, et al. Methods for Allocating Carbon Obligation in Demand Side: a Comparative Study[J]. Automation of Electric Power Systems, 2015, 39(17):153-159.

[22]	RUIZ P A, RUDKEVICH A. Analysis of Marginal Carbon Intensities in Constrained Power Networks[C/OL]//2010 43rd Hawaii International Conference on System Sciences, Honolulu, Hawaii, USA. 2010.

[23]	YAN X Y, GAO C W, MING H, et al. Optimal scheduling strategy and benefit allocation of multiple Micro-Grid based on general Nash bargaining theory[J]. International Journal of Electrical Power & Energy Systems, 2023, 152: 109218.

[24]	PAN J, LIU X O, HUANG J J. Multi-level games optimal scheduling strategy of multiple Micro-Grid considering carbon emission flow and carbon trade[J]. Electric Power Systems Research, 2023, 223: 109669.

[25]	LI Xiangyu, ZHAO Dongmei. Distributed coordinated optimal scheduling of multiple Micro-Grid based on decentralized control structure[J]. Transactions of China Electrotechnical Society, 2023, 38(7): 1852-1863.

[26]	WANG Q, WU W C, WANG B, et al. Asynchronous decomposition method for the coordinated operation of Micro-Grid[J]. IEEE Transactions on Power Systems, 2023, 38(1): 767-782.

[27]	LUO Ping, ZHOU Haobing, XU Lin, et al. Day-ahead optimal scheduling of multi-microgrids with combined cooling, heating and power based on interval optimization[J]. Automation of Electric Power Systems, 2022, 46(9): 137-146.

[28]	FENG Changsen, XIE Fangrui, WEN Fushuan, et al. Design and implementation of joint trading market for green power certificate and carbon based on smart contract[J]. Automation of Electric Power Systems, 2021, 45(23): 1-11.

[29]	CUI S C, WANG Y W, XIAO J W. Peer-to-peer energy sharing among smart energy buildings by distributed transaction[J]. IEEE Transactions on Smart Grid, 2019, 10(6): 6491-6501.

[30]	YAN M Y, SHAHIDEHPOUR M, ALABDULWAHAB A, et al. Blockchain for transacting energy and carbon allowance in networked microgrids[J]. IEEE Transactions on Smart Grid, 2021, 12(6): 4702-4714.

[31]	ZHENG Xiaodong,CHEN Haoyong,DUAN Shengzhi,et al. Distributionally robust planning of transmission network and energy storage based on scenario probability-driven[J] . Electric Power Automation Equipment,2022,42(6):170-178.

[32]	CHENG Y H, ZHANG N, ZHANG B S, et al. Low-carbon operation of multiple energy systems based on energy-carbon integrated prices[J]. IEEE Transactions on Smart Grid, 2020, 11(2): 1307-1318.

[33]	YUAN Guili, LIU Huaqi, YU Jianfang, et al. Combined heat and power optimal dispatching in virtual power plant with carbon capture cogeneration unit[J]. Proceedings of the CSEE, 2022, 42(12): 4440-4449.

[34]	LI Jiyong, ZHAO Xinzhe, ZHENG Yifei, et al. Optimal configuration of a regional integrated energy system considering energy sharing based on Nash negotiation[J]. Power System Protection and Control, 2023, 51(5): 22-32.

[35]	WU Jinling, LOU Ping, GUAN Minyuan, et al. Operation optimization strategy of multi-microgrids energy sharing based on asymmetric Nash bargaining[J]. Power System Technology, 2022, 46(7): 2711-2723.

Appendix A

A1 CC&G solves the upper modeling approach

The three-layer model of Eq. (24) is converted into a master problem and subproblem iterative solution according to the C&CG algorithm, which is represented as follows.

Main problem:

(A1)

where η is an introduced auxiliary variable, denoting the predicted value of the objective function of the subproblem. θ* denotes the optimization result of the worst-case scenario with uncertain variables obtained by the subproblem, which is a known quantity in the main problem.

Subquestion:

(A2)

where ϕ is the dyadic variable of y in the constraints. z* and x* are the optimization results obtained from the master problem and are known quantities in the subproblem. Since the subproblem is a two-layer optimization problem, KKT conditions are used to convert it to a single-layer max optimization problem with the introduced dyadic constraints and complementary relaxation constraints:

(A3)

The complementary relaxation constraint in the second line of the above equation, where ϕTθ is a bilinear term, is linearly transformed by the big M method as follows:

(A4)

where M is the introduced extremely large positive number. υ is the introduced 0-1 variable. Thus, the subproblem is converted from a hard-to-solve two-layer optimization problem to a mixed-integer linear programming problem, which can be solved by cplex, expressed as follows:

(A5)

The model convergence accuracy was set to 10-2.

A2 MG coalition optimization model constraints

1) Power balance constraint

(A6)

(A7)

where Pi,IL,t and Pi,DER,t are the amount of interruptible load calls and DER generation output in MGi at time t, respectively. Pi,ESS,t is the amount of energy storage resource calls in MGi at time t, Pi,PV,t and Pi,WT,t are the amount of photovoltaic and wind generation output in MGi at time t, respectively. θi,t indicates the worst-case scenario for MG operation.

2) Transaction price constraint

(A8)

(A9)

(A10)

where πij,e,max, πij,e,min, are the upper and lower limits of electricity price between virtual power plant i and other MG. πMN,e,max , πMN,e,min are the upper and lower limits of electricity price between virtual power plant i and the upper grid. πij,c,max , πij,c,min are the upper and lower limits of carbon quota price between virtual power plant i and other MG, respectively.

3) Interruptible load constraint

(A11)

where PIL,q,max is the maximum call of the interruptible load q. UIL,m,t is the state of the interruptible load m at time t, which is a 0/1 variable. TIL,m,max is the maximum interruption time. LIL,m, LIL,m,max are the number of interruptions of the interruptible load m in the day and the maximum number of interruptions per day, respectively. VIL,m,t is a 0/1 variable, and the value of 1 indicates that the interruptible load m starts to be interrupted at time t, and nm is the minimum time interval between two interruptions of the interruptible load m.

4) Gas turbine constraint

(A12)

where uq is the 0/1 variable to determine the operating state of gas turbine q, 1 means start and 0 means shutdown, PMT,q,t is the output of gas turbine q in time t, ΔMT is the climbing constraint of gas turbine, PMT,q,max and PMT,q,min indicate the maximum and minimum output of gas turbine, respectively.

(A13)

(A14)

where SOC,e,t is the charge state of energy storage unit e at time t, σch,e,t and σdis,e,t are the 0/1 variables for determining the charge and discharge of energy storage unit, respectively, Ee,N is the rated capacity of energy storage unit e, Pch,e,max , Pch,e min , Pdis,e,max , Pdis,e,min , SOC,e,max , SOC,e,min are the upper and lower limits of charge power, upper and lower limits of discharge power, and upper and lower limits of charge state of energy storage unit e, respectively.

A3 Accelerated Adaptive ADMM Iterative Solution Subproblem 1&2 Steps

1) Subproblem 1 is solved using MATLAB+YALMIP+GUROBI;

2) Set the maximum number of iterations to 200 and the convergence accuracy to 10-3, the initial number of iterations is 0, each MG electric energy and carbon quota trading volume is 0, the initial Lagrange multiplier is 0, and the penalty factor is 10-4, the adaptive parameters μe=μc=1;

3) For the MGs in the coalition, solve the model in sequential order: for the first MG, it receives the kth iteration result Pj1,k,t, Ej1,k,t from the 2nd, 3rd, … , and NMGth MGs, for the non-first MGi, it receives the k+1th iteration result Pji,k+1,t, Eji,k+1,t from the 1st, 2nd, … , and i-1th MGs, and kth iteration results Pji,k,t, Eji,k,t from the i+1th, i+2th, …, and NMGth MGs. iteration result 10, 11, 14. the MGs in the coalition solve the model based on the iteration results from the other participants to obtain Pji,k+1,t, Eji,k+1,t.

4) After completing one round of iterations, the Lagrange multipliers are updated according to equation (A15):

(A15)

5) Calculate the raw and pairwise residuals:

(A16)

(A17)

(A18)

(A19)

6) Update the penalty factor according to equation (A20) (A21):

(A20)

(A21)

7) Update the number of iterations: k=k+1;

8) Judge the convergence according to the algorithm, if any MGi in the coalition satisfies the iteration termination condition of equation (A22), then stop the iteration and output the final result, otherwise return to step 3 and repeat the calculation until the convergence condition ξ1 is satisfied or the maximum number of iterations kmax1 is reached.

(A22)

Subproblem 2 is solved in the same steps as Subproblem 1, with the convergence condition changed to:

(A23)

Appendix B

Table B1  Parameters of upper grid generator sets

| Serial number | Type of power generation | Maximum output/kW | Quoted cost factor/($/kWh) | carbon intensity/(kgCO2/kWh) |
| --- | --- | --- | --- | --- |
| 1 | wind power | 4500 | 0.35 | 0.042 |
| 2 | coal fuel | 3000 | 0.45 | 0.850 |
| 3 | coal fuel | 2500 | 0.37 | 0.800 |
| 4 | petroleum | 3400 | 0.61 | 0.575 |
| 5 | coal fuel | 1900 | 0.26 | 0.910 |

Table B2  MG internal distributed resources

| MG serial number | Wind Power Business | Photovoltaic Sending ecommerce | Micro Gas Turbine Unit | Energy Storage Systems | Interruptible load |
| --- | --- | --- | --- | --- | --- |
| MG1 | 1 group |  |  | 1 group | 1 group |
| MG2 | 1 group | 1 group |  | 1 group | 1 group |
| MG3 |  | 1 group | 1 group | 1 group | 2 groups |

Table B3  MG internal device parameters

| Equipment | Numerical value |  |
| --- | --- | --- |
| Fans | Capacity | 3000kW |
| Photovoltaic | Capacity | 1000kW |
| Energy Storage | Maximum power storage | 1000kW |
|  | Minimum storage capacity | 100kW |
|  | Maximum charge/discharge power | 200kW |
|  | Charging and discharging efficiency | 0.94 |
| Gas turbine | Maximum output power | 800kW |
|  | Minimum output power | 0kW |
|  | Climbing rate | 100kw/h |